3GPP TSG RAN WG2 Meeting #133                	                	                          R2-260xxxx

Gothenburg, Sweden, Feb. 9th-13th 2026

Agenda Item:	x.x.x

Source:	Xiaomi

Title:	Report of [POST132][007][UE caps] BC capability CRs (Xiaomi)

Document for:	Discussion and Decision

Introduction

This report provides a summary for the following post-meeting email discussion:

- [POST132][007][UE caps] BC capability CRs (Xiaomi)

Intended outcome: Review CRs and agree on whether we start from R16

Deadline:  Long

Companies providing input to this email discussion are requested to leave contact information below.

| Company | Name | Email Address |
| --- | --- | --- |
| Xiaomi | Ziyi Li | liziyi5@xiaomi.com |
| Ericsson | Lian Araujo | lian.araujo@ericsson.com |
| Huawei, HiSilicon | Tong Sha | shatong3@hisilicon.com |

Discussion

During RAN2 #132 meeting, the following agreement on ‘per band and per BC’ were agreed:

Agreements from RAN2 perspective

- No matter CA is configured or not, if the capability/component is not counted across CCs, the minimum capability between per BC capability and per band capability should be applied for a band in case of band combination (CA).

- For the capability/component is counted across CCs, wait for RAN1 for the conclusion.

- When per BC capability is indicated but some subset of the bands in the BC doesn’t indicate per band capability, the capability/component is not supported in the band without per band capability, the capability/component is supported only in the band with per band capability signalled. This applies to both capability/component that is counted/not counted across CCs.

- When the UE reports the per band capability but does not include the per BC capability for a certain BC, the UE supports the feature as indicated in the per band capability without further per BC limitations. This applies to both capability/component that is counted/not counted across CCs.

- If Feature B is prerequisite of Feature A and both features are ‘per band and per band combination’:

a. A UE supporting feature A per band shall also indicate support of feature B at the given band;

b. A UE supporting feature A per BC shall also indicate support of feature B at the given BC.

6	For per-band per-BC capabilities for MIMO codebook capabilities:

-	If a UE supports such capabilities in a set of bands separately (per-band) then the UE must always support those capabilities in any CA combination composed of the respective bands (per-BC). No further enhancement is needed for this case.

8	Inform RAN1 about our agreements

7	Update at least Rel-19 CRs.  FFS whether we need Rel-16, 17, 18 CRs

Additionally, RAN1 further agreed the following agreements in RAN1 #123 meeting in R1-2509496 [1]:

Agreement: - When UE indicates both per band and per BC capability, if the capability/component is counted across CCs and CA is not configured, then - gNB only considers per Band capability regardless of reported per BC capability (i.e., per BC capability can be ignored in this case) - When UE indicates both per band and per BC capability, if the capability/component is counted across CCs and CA is configured, then - if all CCs over the CA are within the same band (intra-band CA), the CA is restricted by the per band capability/component corresponding to the CA - if the CCs of the CA are associated with a band combination (inter-band CA), the CA is restricted by the per BC capability/component corresponding to the CA

Based on the above, rapporteur summarizes the following general principle of ‘per band and per BC’ and its prerequisite:

When both per band and per BC capability are indicated: - if capability/component is not counted across CCs: - No matter CA is configured or not, the minimum capability between per BC capability and per band capability should be applied for a band in case of band combination (CA); - if capability/component is counted across CCs: - if CA is not configured by network, only per band capability regardless of reported per BC capability; - if CA is configured by network, - if all CCs over the CA are within the same band (intra-band CA), the CA is restricted by the per band capability/component corresponding to the CA - if the CCs of the CA are associated with a band combination (inter-band CA), the CA is restricted by the per BC capability/component corresponding to the CA When per BC capability is indicated but some subset of the bands in the BC doesn’t indicate per band capability: The capability/component is not supported in the band without per band capability, the capability/component is supported only in the band with per band capability signalled. This applies to both capability/component that is counted/not counted across CCs. When the UE reports the per band capability but does not include the per BC capability for a certain BC: The UE supports the feature as indicated in the per band capability without further per BC limitations. This applies to both capability/component that is counted/not counted across CCs. If a UE supports the per band capability in a set of bands separately, the UE should also support the corresponding per BC capabilities in any CA combination composed of the respective bands. If Feature B is prerequisite of Feature A and both features are ‘per band and per band combination’: a. A UE supporting feature A per band shall also indicate support of feature B at the given band; b. A UE supporting feature A per BC shall also indicate support of feature B at the given BC.

During previous discussion, the discussion on ‘per band and per band combination’ was initiated from MIMO codebook capabilities, who are defined in both per band and per BC with the same feature components. However, there were some concerns on whether applying the above interpretation for ‘per band and per band combination’ capabilities to capabilities such as power class, etc. However, rapporteur observes that power class comprises different components with some components defined as per band (highlighted in green) while others defined as per BC (highlighted in yellow), which is not using ‘per band and per BC’ granularity (as for MIMO codebook capabilities) in RAN1/4 feature list [2].

| 2-9 | UE power class | 1) Support of FR1 UE power class 2) Support of FR2 UE power class 3) Support of FR1 UE power class for EN-DC 4) Support of FR1 UE power class for NR-CA |  | Yes | gNB cannot know the UE power class | Type 1 and Type 3 (see remarks column) | No Need | No Need |  | Capability signalling - FR1 UE power class ( Type 1 (per band)) - FR2 UE power class ( Type 1 (per band)) - FR1 UE power class for EN-DC (Type 3 (per band combination)) - FR1 UE power class for NR CA (Type 3 (per band combination)) Note that default power class for each component is indicated in TS38.101-1/2/3. If the default power class is not indicated, UE shall report supported power class. Note that the component 2) is also used as power class for intra-band contiguous NR-CA in FR2 | RAN4 | Mandatory to support at least one power class with capability. The capability signalling is absent if UE supports only default power class. | Mandatory to report non-default power class if UE supports |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Therefore, rapporteur suggests to consider define ‘per band and per band combination’ as a new granularity with the following definition:

Per band and per BC capability means that a UE capability parameter defined in both per band and per BC with the same feature components. The capability is signalled in per band and also signalled in per band combination.

Q1. Does company agree to define ‘per band and per band combination’ as a new granularity and its definition above?

| Company | Yes/No | Comment to definition |
| --- | --- | --- |
| Ericsson | No strong view | As long as we clarify in 38.306 what this per band and per band combination signaling is there may not be much impact on whether we define it as a new granularity or not. |
| Huawei, HiSilicon | Yes from Rel-19 | We are fine to define a new capability granularity to make the capability usage clear to both the UE and the network from Rel-19. |

Following RAN2 #132 meeting agreement

7	Update at least Rel-19 CRs.  FFS whether we need Rel-16, 17, 18 CRs

Rapporteur provided Rel-19 draft CR, including the above ‘per band and per band combination’ definition and general principle,

Q2. Please companies provide comment to the draft CR in below table (to avoid changes over changes and so many bubble comment in draft CR):

| Company | Yes/No | Whether to capture ‘per band and per band combination’ definition in Section 3.1? (Yes/No) | Suggestion on changes in Section 4.2.1 |
| --- | --- | --- | --- |
| Ericsson |  | Yes, but some changes are needed. We should usually spell out “band combination” rather than saying “BC”, and the second sentence seems misleading since in some cases the UE may not report the per BC component. Hence we suggest the following: Per band and per BCband combination capability means that a UE capability parameter defined in both per band and per BCband combination with the same feature components. The capability is signalled in per band and also signalled in per band combination. | The sentence below talks about “set of bands separately” which is difficult to understand: If a UE supports the "per band" capability in a set of bands separately, the UE should also support the corresponding "per BC" capabilities in any CA combination composed of the respective bands. we suggest to change it to: If a UE supports the "per band" capability in a set of bands separately, the UE should also support the signalled “per band” corresponding "per BC" capabilities in any CA combination composed of the respective bands. The paragraph below is difficult to follow. The main idea is that for CA within a band we are limited by the per-band capability, and for CA involving more than one band then we are limited by per-BC capability, and counting is performed across all CCs. But at the moment the sentence is not so clear, we could discuss more during the meeting to try to find a better wording. if the comprised parameter(s) is counted across CCs of intra-band CA (all CCs over the CA are within the same band), the "per band" capability is applied across CCs within the corresponding intra-band CA; if the comprised parameter(s) is counted across CCs of inter-band CA (all CCs over the CA are associated with a band combination), the "per BC" capability to the corresponding inter-band CA. |
| Huawei, HiSilicon | See comments | No. There is no granularity definition in section 3.1, we think it is enough to capture something in section 4.2.1. | If the change is added in section 4.2.1, we suggest something like “A feature component of a UE capability parameter defined in both per band and per BC with same feature components. The capability of the feature component is signalled in per band and also signalled in per band combination.” The removed parts require that all the components should be same for perband and perBC capabilities. But we think it is possible that that only parts of the components are overlapped between per band and perBC capabilities. Besides, whether such principle can be applied for NR-DC case is not clear, considering the perBC capabilities can be included in the ca-ParametersNR-DC branch. |

In the end, considering capabilities with ‘per band and per band combination’ are widely captured in TS 38.306 since Rel-16 (almost all MIMO codebook capabilities), to keep consistent and avoid ambiguity, rapporteur proposes the above changes starting from Rel-16.

Q3. Does company agree to start to make the changes on ‘per band and per band combination’ (as in Rel-19, including changes in Section 3.1 and Section 4.2.1) from Rel-16 TS 38.306 (if agreed, rapporteur will provide the corresponding CR with same changes as in Rel-19 from the agreed release):

| Company | Yes/No | If not, which release to start the change? (Rel-17, Rel-18) |
| --- | --- | --- |
| Ericsson | Yes |  |
| Huawei, HiSilicon | No | Generally speaking, we should be very careful to add some “general principles” on earlier releases. It has the potential risk of NBC. We would like to invite proponents to clarify which exact legacy capabilities have ambiguity issues. After corresponding capabilities identified clearly, we can check whether the CR is NBC to any. |

Conclusion

<to be updated>

Reference

[1] R1-2509496	LS on updated Rel-19 RAN1 UE features lists for NR after RAN1#123 Wednesday

[2] R4-1907593 "RAN4 NR UE features", contribution to TSG-RAN WG4 meeting #XX.