3GPP TSG-RAN WG2 #133	Tdoc R2-26xxxxx

Gothenburg, Sweden, Feb. 9th-13th 2026

Agenda Item:	x.x.x.x

Source:	Ericsson

Title:	[POST132][018][6G] ASN.1 structure

Document for:	Discussion, Decision

# 1	Introduction

This document captures the progress and outcome of the email discussion [POST132][018] on the 6G ASN.1 structure. According to the agreements at RAN2-132 it is supposed to “identify possible ways to address the issues agreed in the meeting and capture any additional observation”.

In the first phase of this email discussion companies should review the listed problems and observations. Comments should preferably be provided in the tables at the end of the respective sub-section. Companies may also add new sub-sections for problem areas that fit into the scope of this email discussion and that haven’t been mentioned yet.

Based on the contributions to RAN2-132 most companies seemed to be well aligned regarding the problems observed in 5G NR. Hence, the rapporteur proposes that the first phase ends by December 12th.

In the second phase this email discussion should aim to discuss and “identify possible ways to address the issues”. The second phase shall end by January 23rd the latest. However, companies are encouraged to provide and review inputs earlier and continuously to facilitate a fruitful discussion.

# 2	Contact Information

Please fill in the following table for contact information:

| Company | Email |
| --- | --- |
| Ericsson | Henning.Wiemann [at] ericsson (dot) com |
| Toyota ITC | Erik.Sunell [at] toyota (dot) com |
| Nokia | Tero.Henttonen [at] Nokia (dot) com |
| Samsung | seungri.jin@samsung.com |
| OPPO | qianxi.lu@oppo.com |
| Jio Platforms Limited | umur.karabulut [at] jio (dot) eu |
| Huawei, HiSilicon | caozhenzhen@huawei.com |
| Xiaomi | xiaoxiao26@xiaomi.com |
| MediaTek | pasi.laitinen [at] mediatek (dot) com |
| ZTE | liu.jing30@zte.com.cn |
| Apple | fangli_xu@apple.com |
| InterDigital | martino.freda@interdigital.com |
| Qualcomm | uphuyal@qti.qualcomm.com |
| LGE | han.cha@lge.com |

# 3	Problem areas

The following sub-sections describe problems that companies observed with the 5G ASN.1 structure and that they described in their contributions to RAN2-132.

## 3.1	Delta signalling

Several contributions (e.g. R2-2508618 (Huawei), R2-2508450 (Apple), R2-2508614 (Ericsson), R2-2508080 (Xiaomi), R2-2508115 (OPPO), R2-2508098 (CATT), R2-2508386 (InterDigital)…) observed that “the need code introducing additional restraints (e.g., Need S) and conditions (e.g. conditional presence) are the main causes of implementation complexity and compatibility issues in delta configuration”. Especially the textual rules whether and when a UE shall maintain or release fields makes it impossible to automate delta signalling. The required manual parsing makes the solution error prone. ZTE and others explained during this discussion that NR’s AddMod/Release lists are one main building block of delta signalling and suffer from the same ambiguity problems as the single-element cases. MediaTek and Samsung clarified that also the hierarchical and parallel (common/dedicated) structure made delta-signalling ambiguous.

MediaTek highlighted during this discussion that the ambiguity in delta-signalling is also/primarily caused by the fact that many fields were made optional for multiple reasons (“overloaded”). This makes it difficult to describe and verify when the NW may (not) include a field and what the UE shall do if the network did (not) include it.

It has also been mentioned (R2-2508614 (Ericsson), R2-2508618 (Huawei), R2-2508406 (ZTE)) that NR’s delta signalling is particularly problematic for inter-node reconfigurations from network implementation perspective. The challenge is that the target node must be able to release branches of the source configuration which belongs to features that it does not support itself. Ericsson clarified that the gNB must fall back to a full configuration if it cannot ensure that.

R2-2508406 (ZTE) highlighted the problem that NR’s AddMod/Release-Lists are difficult to implement and even more difficult to extend in subsequent releases. Extending the length or the individual elements tends to result in complex structures with parallel- or extension lists tends to result in implementation problems. Many companies agreed with that during the discussion.

During this discussion Nokia highlighted some key design goals for 6G’s ASN.1 structure and for delta-signalling in particular:

- The signalling structure should allow the network to update only parts/branches of the configuration while keeping other parts unchanged.

- The signalling structure must also be able to represent the UE’s entire current configuration (which the UE might have received in several “deltas”) for e.g. inter-node handover.

- When creating the ASN.1 signalling structure RAN2 should have a plan how to extend it in “any” possible direction (Where to put extension markers? When to use parallel lists? …).

Investigate how to make the rules for delta signalling more explicit inside the signalling structure and thereby less ambiguous and less error prone to implement including the case of inter-node mobility.

In the context of delta signalling, investigate especially how to improve the definition and extensibility of (AddMod/Release) lists.

Ensure that delta signalling allows the network to modify/replace one part of the configuration without having to re-send also unchanged parts of the configuration. Discuss how to dimension and define those “parts” to avoid unnecessary “re-transmissions”.

Ensure that the signalling structure is able to represent the UE’s entire current configuration (which the UE might have received in several “deltas”), e.g. for inter-node signalling in case of inter-node handover.

| Company Name | Comment on problem |
| --- | --- |
| Ericsson | We would like to echo what ZTE raised about the AddMod/Release lists. Those lists are a particularly important tool in the context of delta signalling since they were meant to allow the network to (re-)configure potentially long lists of potentially large elements in a signalling-efficient manner. But while AddMod/Release lists follow similar patterns and rules, their realizations are too different to automate their use in practice. And this is especially the case when those lists are extended (in size or element-type) in subsequent releases. We discovered e.g. related problems when trying to add or modify dedicated BWPs. In summary, we think that NR’s AddMod/Release lists suffer from the same ambiguity problem that several companies confirmed for the single-element cases (“Need M”, “Need S” and “Cond”). |
| Toyota ITC | We believe that the main limitation of Need codes and delta signalling is that Need codes are included as comment lines within the ASN.1 syntax rather than being integrated into the formal compiled ASN.1 schema. While the network side generally complies with these codes and restrictions, the UE implementation must still account for all possible encoding outcomes, including those that may never occur. Since signalling relies on the designer’s interpretation of these codes, errors and ambiguities still remain a risk. The current approach is based on failure handling, but recovery is never immediate. Such delays and errors can have serious consequences for critical applications, such as vehicle safety systems. Therefore, exploring alternative and more robust methods integrated in the formal ASN.1 schema would be beneficial and help these problems. The possibility of releasing unsupported source configuration branches suggests that the configuration release mechanisms should be improved but as of now we do not have any proposal. |
| Nokia | We have never fully defined delta signalling except “in spirit”, and it only really appears via the need codes, each of which was also introduced for a specific reason: - Need M accounts for cases with delta signalling - Need R was the “typical” case without delta signalling - Need S was for the parameters with default values that may use delta signalling depending on conditions - Need N was for one-shot IEs that are only used once and therefore not subject to delta signalling - Conditions account for complicated cases that may use delta signalling depending on conditions What we want to achieve with all of these is that one RRC reconfiguration can change only one part of the configuration without affecting the other parts of the configuration. For example: If network wants to change the measurement configuration, it should not be required to change bearer configuration, UP and bearer configuration or PHY configuration. We think it would be best to focus on what we aim to achieve with the need codes: Keeping the signalling size to as small as possible. In summary: We see the following requirements for RRC signalling (delta or non-delta): - Network is able to provide UE with only necessary configuration updates, i.e. it shall be possible to explicitly signal only changing parts of UE configuration via RRC reconfiguration. - It shall be possible to uniquely represent the UE RRC configuration, i.e. network shall be able to easily create the currently used UE configuration e.g. for handover requests. - It shall be possible to extend any configuration, e.g. when an IE is created it is already understood how any extensions could be done (i.e. typically each IE could have ellipsis-based extensions available, and it shall be possible to address any mistakes done when defining signalling). |
| Samsung | We also share the above problems identified by many companies. The delta configuration is currently ambiguous due to the existing RRC ASN.1 structure. The deeply nested and hierarchical nature of this structure complicates the procedure, making it challenging to implement and manage effectively. For the last problem raised by ZTE, it is true the current AddMod/Release-Lists are difficult to implement and even more difficult to extend in subsequent releases but we think this AddMod/Release-Lists structure is the simplest way to achieve the delta configuration i.e. update the additional list/parameters using this structure. So, as suggested by ZTE that new solutions for extending list structures (e.g. always introduce a new list that completely replaces the old one) could be considered in 6G. |
| OPPO | For the two sub-questions of this clause: - “the need code introducing additional restraints (e.g., Need S) and conditions (e.g., conditional presence) are the main causes of implementation complexity and compatibility issues in delta configuration”, “Especially the textual rules whether and when a UE shall maintain or release fields makes it impossible to automate delta signalling. The required manual parsing makes the solution error prone.” In our view, the use of Need-S and conditions for optional fields does introduce a certain level of complexity. However, it should be emphasized that this complexity stems not from the descriptive tools themselves, such as Need-S or conditional statements, but rather from the inherent intricacy of the underlying behaviors and logic they are intended to represent. This complexity would persist regardless of the method used to specify such behavior in the specification (other than the issue discussed in clause 3.2). Therefore, while we acknowledge the existence of complexity in these constructs, we remain uncertain so somehow negative to this, as to what specific improvements could yield a significantly clearer or more manageable solution. - “NR’s AddMod/Release-Lists are difficult to implement and even more difficult to extend in subsequent releases”, In our view, most of the difficulties highlighted in R2-2508406 could be resolved if an extension marker had already been included in the elements of the original list. In such a case, the sole purpose of list extension would be to support an increased number of entries. This restriction would effectively address the two main issues identified in 8406: - “However, sometimes, we define that old list can also release the entry that established by the new list, but sometimes not.”, if we can limit the extension to increased number of entries, it is always the case that old (or new) Release list release the old (or new) AddMod list; - “Due to lack of ID/Index in the new list, when the network wants to release the new parameter, the network must release the entire entry.” If we can limit the extension to increased number of entries, there is no such problem any more. We are open to proposals regarding how to ensure “extension marker has already been included in the element of the old list”. |
| Jio Platforms | While it is widely recognized that the complexity of delta signalling originates from Need codes, conditional-presence logic, and the hierarchical RRC structure, we would like to highlight an additional issue that persists even under ideal implementations. Need codes are expressed only as textual annotations in ASN.1 comments, not as part of the formal schema, and must therefore be manually reconstructed in implementation. Developers must interpret behaviours such as “maintain”, “release”, or “specified elsewhere”, often in combination with conditional presence (CondX), which creates cross-layer logic and opportunities for ambiguity. Even if these challenges are perfectly addressed, deeper technical problems remain. A fundamental limitation appears in inter-node delta signalling, especially between vendors or across releases. In HO preparation, the source node supplies an RRC configuration based on its own release, while the target node generates the configuration for the target cell. If the target node supports a lower release, it cannot interpret fields originating from newer features and silently omits them when constructing its own configuration. When this reduced configuration is delivered to the UE, many fields are absent not due to network intent but due to unsupported features. The UE, however, cannot distinguish this. Every absent optional field must be interpreted strictly via its Need code. As a result, the UE treats the target-node configuration as a genuine delta relative to the source and maintains, releases, or defaults parameters accordingly. This may produce a UE configuration that the target node neither expects nor reconstructs, yet neither side has visibility of the mismatch. Similar inconsistencies occur in other domains where vendors use different allocation strategies or structuring rules. For example, while Logical Channel IDs (LCIDs) themselves do not carry Need-code semantics, the configuration IEs associated with each LCID, such as LogicalChannelConfig, RLC-BearerConfig, and other channel-specific substructures, contain numerous OPTIONAL fields annotated with Need codes. When vendors allocate LCIDs differently, or when a target node omits or restructures LCID-related configuration that it does not support, these configuration branches may be absent in the target node’s RRC message. The UE must then interpret such absences strictly through Need-code rules, potentially maintaining or releasing logical-channel state in ways the target node did not intend. Measurement configuration identifiers (measObjectId, reportConfigId, measId) present a similar issue: differing vendor allocation strategies mean that unsupported identifiers are simply discarded by the target node, causing the UE again to treat their absence as intentional. Likewise, optional substructures in CellGroupConfig, such as RLC, PDCP, and SDAP entities, may be omitted or defaulted differently across implementations; the UE interprets these omissions as deltas, which can lead to misalignment of bearer, PDCP, or SDAP state, and in some cases even result in data-path disruption immediately after handover. These examples show that the fragility of delta signalling is not limited to implementation difficulty. The underlying issue is that the UE must interpret any absent field according to rigid Need-code rules, even when the omission is caused by inter-vendor or inter-release differences rather than network intent. This structural limitation affects multiple configuration areas and leads to inconsistent UE behaviour across heterogeneous deployments. [Ericsson] To avoid the above-mentioned mismatch, an NR gNB is supposed to apply a full-config if it notices extensions that it does not comprehend, i.e., fields corresponding to extensions of a newer release or branches of the configuration for which it has not implemented to actively release of Need-M/S-fields or AddMod/Release lists. That implies also that the target node, if it decides to create a delta configuration, must ensure consistency among all the IDs that reference between the mentioned information elements. Hence, while we fully agree with the root problem for inter-node mobility, the outlined consequences of a wrongly interpreted delta-configuration won’t occur if the NR gNB applies a full-configuration when necessary. |
| Huawei, HiSilicon | Regarding the issue of extending element types in the AddMod/Release lists for NR, we believe this is a common problem: when extending list-type IEs that do not have an extension marker reserved at the element level (due to concerns about the overhead of extension markers or due to forgetting to reserve them in the original version), it is required to create parallel lists to introduce extensions for the elements. Since there is no explicit association between the original list and the extended list, the extended list needs to contain the same number of elements in the same order as the original list. This approach incurs some cost when only a few elements need to be extended. However, generally speaking, this is not a major issue because in most cases, the length of the lists is not particularly long. This problem becomes more significant only when the lists are very long and have undergone multiple extensions (e.g., band combination lists). Regarding the issue of extending the size of the AddMod/Release list for NR, we believe this may not be a real problem but rather a compromise consideration. We could introduce an ID that is longer than actually needed in the first version to avoid this issue, but doing so would incur more overhead before there is a real need for longer IDs. |
| Xiaomi | Delta signaling is a useful tool for signaling size optimization, and should be confirmed as still supported in 6GR. Based on the listed contributions and the comments provided by companies above, the main problems that prevent the current delta signaling being correctly/efficiently deployed in 5G NR are two folded from our perspective: - Problem w.r.t. the Need code for single-element field: With Need code type not a part of ASN.1 coding compilation, how to cope with the absence of an "Optional" field in 5G NR has to be (manually) parsed/implemented by the implementors in a field-by-field manner, making it impossible to automate the delta signaling implementation and thus resulting in delta signaling error-prone. - Problem w.r.t. the list specific delta signaling (e.g. ToAddMod/ToRelese: How to efficiently/unambiguously implement the list specific delta signaling, especially for the extension of the list, e.g., the extension of the list size, the extension of each entry element, etc. Another problem that may be worth considering is how to deal with the "mandatory" parameters which impact the benefits really achievable by delta signaling. Delta signaling only applies to OPTIONAL fields, but in NR/LTE there are quite a number of mandatory parameters, each of which has to be present in every RRCReconfig msg, no matter it is changed or not. Even if delta signaling is applied, the benefit (e.g. the ratio of total signaling volume saved) remains rather limited due to the presence of these mandatory parameters. If there are some cases that NW configuration is relatively static (e.g. due to same UE capability, in same deployment, for same services as proposed by some companies) and only a small portion of the NW configuration change frequently, it looks more desirable to avoid the benefit from delta signaling being drained by the mandatory parameters. [Rapporteur] Typically, NR uses mandatory fields only on the lowest level, i.e., for primitive IE types within a SEQUENCE. Delta-signalling (by Need M/S or by Add/Mod-Lists) is typically done above that level, i.e., for the larger IE types (SEQUENCE). If the gNB does not want to change any field inside such SEQUENCE, it may omit the entire SEQUENCE and the UE will maintain all fields therein, irrespective whether they are mandatory or optional and irrespective of their Need code. But if the gNB would like to change one parameter within such a SEQUENCE, it must re-provide all other mandatory fields and all OPTIONAL fields that are marked Need R. We believe that the optional (Need R) fields contribute typically more to the amount of “unnecessary” data than the fewer and smaller mandatory fields. We suggest to study instead how to decide for which fields to enable delta signalling and for which fields to accept an unnecessary retransmission if any of their “siblings” changes. |
| MediaTek | We agree that the need codes and Cond's are the main contributors to implementation complexity and interoperabity issues in the scope of delta signalling. The reason why they are such is what Toyota mentioned - they cannot be automated, but implementations rely on designers' interpretations. However, we think the need codes are Cond's are not the root cause of the delta signalling related issues. The need codes and Cond's exist, because optionality of the fields is overloaded and this is the root cause. In ideal signalling, an optional field would be used only for optional configuration parameters. However, due to delta signalling, many mandatory configuration parameters (for a certain feature/functionality, or within certain IE) are carried by optional fields; this is what we refer by overloading. Yes, this arrangement makes it possible for the network to omit these fields in the subsequent signalling, thus implement the actual delta signalling, but as a side-effect it causes the issues presently discussed **. Overall, we think the solution to address the delta signalling problems should focus on the mentioned root cause. Regarding the list issue mentioned by ZTE, we agree that extending the content of a list element using parallel extension list is a source for implementation complexity and interoperability issues, especially as it is often combined with an approach where a field in the extension list element fully or partially replaces existing field in corresponding legacy list element. If only the size of the list is extended using a size extension list (which has the same element IE as the legacy list), we do not see that as a major issue. (** Refer to our contribution R2-2508112 where we observed that ambiguity “functionally mandatory UE configuration parameters can be absent in over-the-air RRC messages for initial configuration of a feature/functionality" is typically a side-effect of the delta signalling mechanism.) |
| ZTE | We agree the problems that caused by “Need S” and conditions in field description are mainly because they are not machine-readable. Sometimes, the conditions were even added/modified in later release, which requires the network and the UE to be upgraded at the same time. In addition, there are many configuration implications that are not explicitly specified, which also cause problem in delta configuration. So far, we are not sure we can find a perfect way to “replace” the Need S or conditions, maybe we can find some typical example when discussing the solutions (in phase II). On the issue about AddMod/Release list, it is observed in 5G implementation. And according to previous RAN2 discussion, there are several papers/CRs to clarify or fix the issue caused by the extended list. In 5G, we have been using a patching approach to expand the list, thinking this would reduce signaling. However, the complexity this introduces actually outweighs the gains. In 6G, such problem needs to be avoided. We suggest to introduce a new list because it is simple and avoids the signaling burden caused by extension marks. But we are also open to discuss other solutions. |
| Apple | The purpose of the delta configuration design is to provide only the configurations that have changed. In 5G Need code is designed to achieve this goal. However, as mentioned by companies, the current number of optional parameters is extremely large, the nested RRC configuration structure is very complex, and there are multiple ways to describe the conditions related to optional parameters. This has caused difficulties in understanding the design during the spec drafting, reading, and development phases. To improve the design of 6G, two approaches can be considered: 1) Full automation without manual parsing: As Toyota indicated, the condition/restriction can be integrated in the formal ASN.1 schema. 2) With manual parsing: the situation is similar as today, to make it easier to design and understand the RRC configuration, we can use a unified description method or keywords to describe the same type of configuration parameters, or group configurations with the same attributes together. Regarding the “AddMod/Release-Lists” issue, we also acknowledge that the current RRC design is a little complex. We understand this is due to a lack of the consideration for forward scalability and compatibility in the initial 5G RRC structure design, leading to increased complexity in later RRC release due to the number and parameters of extensions. Therefore, in the 6G RRC structure design, we need to ensure forward compatibility design of “AddMod/Release-Lists”. |
| InterDigital | We agree with the basic principles mentioned by Nokia that: - The network should be able to change only part of the configuration at the UE while keeping the current configuration of other parts unchanged. - There should be a way for the network to represent the UE’s current configuration (after the application of multiple delta configuration). For the second principle, we think the benefit of this is to avoid the use of full configuration. |

## 3.2	Conditional presence/absence rules

Several contributions (R2-2508112 (MediaTek), R2-2508649 (Toyota), R2-2508450 (Apple)…) raised that the specifications describe configuration constraints inconsistently, incompletely and ambiguously. And some companies mentioned that this may result in errors where “functionally mandatory UE configuration parameters can be absent in over-the-air RRC messages for initial configuration of a feature/functionality" or that "UE configuration parameters which shouldn't be modified after initial configuration of a feature/functionality can be sent in subsequent over-the-air RRC messages with new values”. Many companies agreed to this observation and supported investigating means to overcome or reduce this problem in 6G. Nokia, OPPO and other stated that it is important to ensure that such solutions don’t result in excessive additional specification-, implementation- or signalling overhead.

Huawei, OPPO, MediaTek and others pointed out that this issues is directly related to the first issue since conditions and need codes are both used to describe UE behaviour upon absence of an optional field – one one hand to realize delta signalling and secondly to express configuration constraints.

R2-2508386 (InterDigital) and R2-2508614 (Ericsson) observed that continued extensions of large IEs and the use of large IEs for different purposes tend to make it ambiguous which combination of parameters and values therein is valid and thereby contributes to the above-mentioned ambiguities.

Investigate the configuration constraints to specify and how to specify them unambiguously and clearly distinguishable from delta signalling.

| Company Name | Comment on problem |  |
| --- | --- | --- |
| Ericsson | We observe that RAN2 used conditions primarily for two reasons: 1) A constraint of the functionality being configured. For example, changing the RLC-SN could screw up the ARQ window and is hence forbidden unless the NW reestablishes the RLC entity. However, there are also cases (e.g. “SearchSpace-> controlResourceSetId -- Cond SetupOnly”) where it is not as obvious why the NW could not change the mapping in a later reconfiguration. It should be noted that there are other constraints which were not reflected in NR’s “Cond” tags: UE capabilities restrict also which fields the NW may include and which values it may set them to. What is the relation between “conditions” and “UE capabilities”? Any constraint that could differ among different UEs should be described (entirely?) in UE capabilities. RAN2 should avoid describing related constraints partially in conditions and partially in UE capabilities. 2) Re-use of (larger) IEs in several places where some fields within the IE may only be present in one context but not in others (e.g. PCell vs. SCell). RAN2 should consider to introduce separate IE types that are tailor-made for the specific purpose. It seems advisable to re-use only low-level (primitive) ASN.1 types. Of course, this comes also at an additional implementation cost for UE and NW and was therefore often avoided in NR. |  |
| Toyota ITC | We are willing to introduce additional, distinct information element types to address specific purposes, even if this results in a larger ASN.1 codebase. This approach would reduce reliance on textual descriptions and help maintain a concise and readable specification. One possible approach is leveraging ASN.1 sub-types with explicitly declared presence and absence. Extensions to information elements and messages should not be limited to non-critical extensions. Critical extensions should also be considered to avoid overly long, complex, and fragmented structures. The best practice is to evaluate extension types on a case-by-case basis for each Release, rather than applying a universal rule where only one extension type is used. |  |
| Nokia | We support the intent of machine-readability of specifications, including the RRC rules on conditional presence or absence of fields. However, it is difficult to give exact “rules” for all eventualities, so unfortunately some case-by-case logic may have to be applied. In most cases, all of the parameters can be present or absent at addition or modification, so making Add- and Mod-parts separate would lead to ASN.1 duplication, which creates maintenance issues. Some general ways to avoid complications are: - Avoid complicated conditions: The typical condition should be present when X occurs, absent when Y occurs, released otherwise. - Use procedural text for complicated conditions: If/when such are needed - this allows more “pseudo-code” type of rules that are machine-readable and easier to maintain. Example is shown below: Conditional presence in 5G RRC ServingCellConfig: firstActiveDownlinkBWP-Id BWP-Id OPTIONAL, -- Cond SyncAndCellAdd SyncAndCellAdd; This field is mandatory present for a SpCell upon reconfiguration with reconfigurationWithSync and upon RRCSetup/RRCResume. The field is optionally present for an SpCell, Need N, upon reconfiguration without reconfigurationWithSync. The field is mandatory present for an SCell upon addition, and absent for SCell in other cases, Need M.. Corresponding procedural text to handle the same: ### X.Y.Z RRC conditions #### X.Y.Z.1 SyncAndCellAdd This field shall be included on configuration according to following: - if the configuration is included for - SpCell upon reconfiguration with reconfigurationWithSync; OR - RRCSetup/RRCResume, OR - Scell upon SCell addition: 2> the field is mandatory present - else if the configuration is included for SCell - reconfiguration without reconfigurationWithSync: 2> the field is optionally present and is not retained by the UE after using - else 2> the field is not present, Need M - Make IEs small enough to be unambiguous: The smaller the configuration parts, the easier it is to discern what needs to be included/changed in case of reconfiguration. However, this cannot be always done as some IEs will be large by nature. - Only case-by-case separation to “Setup” and “Modify” – parts: This should be considered as a possibility but not applied automatically. It could still be used when it is clear how the extensions would be done and duplication is kept to minimum. It would also be good to avoid creating too many versions of the signalling (to avoid the problems we had in UMTS RRC). In summary: Only use simple conditions and write complicated conditions via procedural text. Separation to “Setup” and “Modify” branches can be considered case-by-case but not as a rule for everything. |
| Samsung |  | We also share the above problems identified by companies and RAN2 should consider to solve the issues. The multiple rules in ASN associated with conditions along with need codes make the ASN structure difficult to understand. Additionally, the lack of a modular ASN structure and the ad hoc addition of IEs further complicate the system. We think this issue could be discussed with RRC modular structure. |
| OPPO |  | This issue is fundamentally related to the conditional presence of a field. While the distinction compared to the issue discussed in Clause 3.1 lies in the proponent emphasizing the condition based on whether it pertains to "initial configuration" or "subsequent configuration," we generally concur with the concern raised. Although there may be a potential downside in addressing it—namely, an excessive proliferation of distinct information element (IE) types tailored for specific purposes—the primary concern is not the growth of the ASN.1 codebase per purpose, but rather the risk of errors introduced during the maintenance and extension of IE sets in future releases. Given that human error cannot be entirely eliminated during such processes (“during the maintenance and extension of IE sets in future releases”), it is essential to explore solutions that do not lead to a significant increase in the number of IEs. From this perspective, we support the use of leveraging ASN.1 sub-types with explicitly declared presence and absence conditions as a more robust and maintainable approach. |
| Huawei, HiSilicon |  | We also think this issue somehow overlaps with the first issue (i.e., “the need code introducing additional restraints (e.g., Need S) and conditions (e.g. conditional presence) are the main causes of implementation complexity and compatibility/ambiguity issues in delta configuration”.) and they can be discussed together. |
| Xiaomi |  | In general, we are open to discuss how to improve the handling of the conditional presence/absence, to eliminate the implementation ambiguity and avoid error cases happening. As per above contributions listed and companies' input, it seems that this can be done either by IE type definition (e.g. Add-/Mod-type IEs as proposed by MediaTek) or by the procedural texts that specify when the parameters shall be present/absent (as proposed by Nokia). - For the way relying on IE type definition: it may need to be further considered how to reduce the signalling complication and avoid the ASN.1 duplication in different IEs, especially for the fields that can be present or absent at addition or modification. - For the way relying on procedural texts: it may need to be further considered how to make this way future-proof, as it seems undesirable to add the text procedures for a number of parameters introduced every release. Regarding the ambiguity due to continued extensions for those complicated IEs, we are also open to discuss whether it is possible to rely on more critical extension than the non-critical extension as in 4G/5G, since one of the main causes for such ambiguity is the non-compatibility/non-coexistence between new extended fields/parameters introduced in a later release and the old ones already existing from an earlier release (e.g. overriding earlier release mandatory field by the extension, no simultaneous presence, etc.). In addition, we disagree with considering UE capability as some forms of "Constraints" for NW configuration and thus disagree with involving in UE capability in this discussion, as mentioned by Ericsson above. In 5G NR, it is specified that the NW needs to respect to UE capability and this should be ensured based on appropriate NW implementation. Therefore, in principle, UE capability actually functions as sort of requirement for the NW to follow, instead of placing any restriction for the NW configuration. [Ericsson] No worries. We did not intend to re-focus this discussion on UE capabilities and we don’t need to discuss it in this email discussion. But we do see that both (capabilities and conditions) restrict what the NW is allowed to configure. Hence, RAN2 should pay attention to keep them logically separate, i.e., use conditions (and field descriptions) only for describing logical/functional constraints. |
| MediaTek |  | We agree that the problems identified by the companies are valid and should be addressed. We agree with Ericsson about the two usages of Cond's. The third usage is delta signalling discussed in Chapter 3.1 (e.g., Cond Setup or Cond SetupOnly), so the topics in Chapter 3.1 and 3.2 are partially related to each other. We (similar to Ericsson and Toyta) think the Cond's would not be needed so much, if the ASN.1 contained more ASN.1 IEs for different purposes. For example, instead of having single IE which is used for both PCell and SCell and where differences of possible PCell and SCell configurations are captured using Cond's (such as Cond SCell), separate IEs for PCell and SCell could be defined. As an another example, instead of having single IE which is used for initial configuration and subsequent reconfiguration of a feature/functionality and where differences of initial configuration and subsequent reconfiguration are captured using Cond's (such as Cond Setup or SetupOnly), separate IEs for initial configuration and subsequent reconfiguration could be defined. We agree with other companies that creating more ASN.1 IEs for different purposes causes maintenable overhead, but we think such overhead would be acceptable for more machine-readable ASN.1. We agree with Nokia that it is difficult (in practice nearly impossible) to find a solution that fits for all purposes and which could be applied everywhere by default. (For example, it might be practical using separate ASN.1 IEs for PCell and SCell, but not necessarily for PUCCH-SCell and PUCCH-less SCell; it might be practical using separate ASN.1 IEs for initial configuration and subsequent reconfiguration for certain low level IE originating from RAN1, but not necessarily for certain higher lever IE 'owned' by RAN2, etc.) However, even if a perfect solution couldn't be found, there is a possibility to improve the conditional presence/absence handling a lot compared to 5G. Overall, we think the solution direction should be towards identifying a toolbox and guidelines on occasions in which each tool could be usable. The actual usage of the tools should then be decided case-by-case when the ASN.1 definitions are designed. |
| ZTE |  | Usually, the first issue should be identified and fixed during IODT. But we admit that sometimes, the IODT test cannot cover all the cases. However, we are not sure it worth define separate Add- and Mod- IE structures for this purpose (same concern as Nokia). Also, sometimes, a field could be initially defined for initial configuration, but in later release, it can be enhanced to support modification. How to deal with that? e.g. dummy the field in Add- and add it to Mod-? This may bring more complexities. For Nokia’s proposal on using the “text procedure” to describe the presence condition, we may need more time to check the details, e.g. whether it is machine-readable. Based on 5G experiences, many complex presence conditions are introduced in later releases. Therefore, instead of defining a simple condition in the first version, more importantly, we need to consider how to pursue a future-proof solution so that we can avoid introducing complex conditions in future versions. [MediaTek] If a field is enhanced in later release to support modification, it is just added to 'mod' IE variant. There is no change to 'add' UE variant. Please refer to 4.2.1 for details. |
| Apple |  | We agree that distinguishing between "add without modify" and "add and modify " is beneficial. From the solution perspective, we think it's unnecessary to introduce two separate IEs for different uses (i.e. add and mod) of the same parameter which would increase the size of asn.1 file. Perhaps we could consider using different types of need codes to differentiate the attribute is “add only” or “add and mod”. |
| InterDigital |  | We agree with this issue and agree that conditional presence/absence rules should be less ambiguous. The solutions from the companies can be studied. In addition, we agree with companies that there is a tradeoff between introducing different IEs for different purposes and the overhead of RRC signalling and specification size this may create and so it may be best to avoid extensive use of this approach. |

## 3.3	Dependencies between common- and dedicated signalling

Several companies (R2-2508112 (MediaTek), R2-2508614 (Ericsson), …) consider dependencies between the common configuration (which the UE acquires via MIB/SIB1) and the dedicated configuration (which the gNB provides in the RRCReconfiguration) as a problem. R2-2508112 (MediaTek) explains that they “make the 5G UE configuration structure complex for both the UE and the network. From UE's perspective, the split of the UE configuration to common and dedicated parts has no functional significance”. During this discussion, Nokia, Samsung, Apple and others observed the same problem. Huawei and MediaTek pointed that the problems are especially related to the common/dedicated branches in the context of BWPs. Huawei described issues related to provisioning of “common” branches via dedicated signalling (UE capabilities).

Xiaomi pointed out that it should be investigate which common (broadcast) parameters are at all applicable/relevant for connected UEs. For those parameters the signalling structure must indicated whether the UE shall apply the broadcast parameter or a UE specific value. ZTE agreed with that.

Avoid splitting the connected mode configuration into common- and dedicated branches.

Discuss whether it is necessary that UEs (re-)acquire parameters from system information. If so, seek for means to specify/configure unambiguously which parameter the UE shall (re-)acquire from system information and which ones it shall take from the dedicated configuration.

| Company Name | Comment on problem |
| --- | --- |
| Toyota ITC | We have no strong opinion on this topic, but the separation between common and dedicated configurations likely originates from UMTS legacy that continues to influence the current thinking. Given that the logical channel structure already clearly defines common and dedicated signalling, the types of information elements and configurations carried within these channels do not necessarily require such a distinction. |
| Nokia | We think the split between “common” and “dedicated” is not very useful and actively obfuscates the usage of RRC configuration. The reason it exists is the separation of UE configuration to SIB-acquired parts (for PCell) and RRC-configured parts (for all serving cells), which is just a design choice. For UE, all RRC configurations can be considered to be UE-specific: UE need not care what other UEs do so there is no obvious need to make distinction between “cell-specific” and “UE-specific” configurations in RRC specification. Even if some configurations are actually same for all UEs in the cell, this would not matter for one UE who receives his RRC configuration from network. Hence, we see the following guidelines on splitting UE configurations in RRC: - MIB/SIB configurations: Used in initial RRC access (i.e. RRC setup/resume) only. These configurations would never require network to reserve specific resources for a single UE. - RRC configuration: Provided by network when UE is in or moves to CONNECTED mode. Can reuse (part of) MIB/SIB1 configuration during CONNECTED mode. - Network is in control of overriding configurations: Network shall be able to indicate which configuration UE follows. For example, network may indicate UE to override MIB/SIB1 configuration completely or to reuse it. -- SIB1 configuration: SIB1 ::= SEQUENCE { field1 INTEGER (0..7), field2 INTEGER (0..255), field3 BOOLEAN, ... } -- RRC configuration in CONNECTED mode: Allows reusing SIB1 or overriding it completely: RRC-Config ::= SEQUENCE { sib1-Config CHOICE { useSIB1 NULL, dedicatedSIB1 SIB1 } field4 BOOLEAN OPTIONAL, ... } This would give network explicit and full control over what UE does, while also avoiding defining the same parameters twice in ASN.1 and having complicated definitions of what to do when SIB1 configuration is received via “common (PBCH)” or “dedicated (RRC)” signalling. In summary: Do not define separation to “common” and “dedicated” configuration in 6G RRC but define one structure for UE configuration. The parts of configuration that relate to SIBs must be possible to override by RRC signalling in CONNECTED. |
| Samsung | We also share that NR RRC configurations are impacted for any feature/service addition, primarily due to the high interdependencies between RRC modules. These interdependencies, combined with the common and dedicated configuration signalling approach, introduce significant challenges. So we think these interdependencies increases specification efforts, complexity and ambiguity in configuration management. |
| OPPO | The core concern in this discussion is not entirely clear to us. First, it is inherently necessary to support configuration delivery both via SIBx and via RRCReconfiguration. Consequently, a well-defined rule is required for the UE to determine whether to retain the configuration provided in SIBx or to apply the one conveyed in RRCReconfiguration—which by design overrides the SIBx configuration (However, we do not currently see a compelling need to introduce an explicit ASN.1 indication as suggested, especially considering that legacy systems have consistently followed the principle that any configuration included in RRCReconfiguration takes precedence over the corresponding configuration in SIBx.). Secondly, or if the underlying concern arises from having two separate configurations targeting the same parameter (e.g., pdcch-ConfigCommon and pdcch-Config), and the objective is to unify these into a single configuration applicable in both SIBx and RRCReconfiguration contexts, such an approach would appear to conflict with the intent outlined in Clause 3.2. Specifically, Clause 3.2 advocates for separating information elements based on their intended purpose, thereby enabling clearer presence/absence semantics that are more amenable to machine processing. So in either case, the nature of the issue being raised remains unclear to us. |
| Huawei, HiSilicon | The current design logic is that dedicated signaling is built on top of the common part (cell-specific). The dedicated part consists of additional UE-specific configurations in addition to the cell common configuration. For most of Common part and dedicated part configurations, the UE typically obtains the common part from SIBs (during initial access) or through dedicated signaling (during handover or SCell addition), and the common part is used by both IDLE and CONNECTED states, and the common part for CONNECTED UE is subsequently overwritten by the configuration in SIB1. However, in NR the BWP-Common and BWP-dedicated in BWP configurations except for initialBWP is another issue. The separation is not useful from the UE point of view, as the BWP-Common is not used by IDLE UEs and the UE will never update BWP-Common from SIB1. The common-dedicated parts (expect for BWP-Common/dedicated) separation is generally acceptable, and using SIB1 to update the common part helps to reduce the overhead of dedicated signaling when the cell specific configurations need to be updated. However, we have indeed observed some compatibility issues for this mechanism. In particular, there is debate about whether the gNB should tailor the common part according to the UE's capabilities when the common part is transmitted via dedicated signalling. For example, whether to remove SUL configurations for UEs that do not support SUL, or whether the PUCCH configuration carried in the common part of an SCell actually indicates that the SCell's PUCCH has been configured for the UE. These issues have led to extensive compatibility discussions. Therefore, for 6G, a consistent principle should be established first to avoid potential problems. |
| Xiaomi | From our perspective, two issues may need to be addressed before entering the detailed solution on how/whether to eliminate the dependency of the common- and UE specific configuration: - In which case/for which configuration a CONNECTED UE has to acquire/maintain both the common- and UE specific configurations? - For such configuration in bullet 1 (if any), how does the UE decide whether/when it needs to use common-configuration, UE specific configuration or (the combination of) both. Especially for the first bullet above, it is also related to the general principle on how the NW is assumed to provide the configuration, e.g.: if it can be assumed that NW can always provide the UE-specific configuration that can already cover the up-to-date common-configurations, there seems no need for the UE to obtain the common-configuration from SI, and it seems this problem on dependency does not exist anymore. |
| MediaTek | We (obviously) think that separation of configuration parameters to common and dedicated parts (IEs) serves no functional purpose and it only makes the signalling structure complex. However, we are mainly concerned about separation within dedicated RRC signalling, such as BWP-DownlinkCommon/BWP-UplinkCommon and BWP-DownlinkDedicated/BWP-UplinkDedicated within RRCReconfiguration, so we do not see SIB1 having a large role in this discussion (except of course that SIB1 and the common part carried in dedicated RRC signalling carry logically same information for the initial BWP). However, regarding SIB1, we have different view than Nokia for solution direction. We would prefer solution direction where IE(s) for MIB/SIB1 would not be re-used in dedicated RRC signalling, as that would re-create the problem which exist in 5G where the UE needs to combine several IEs within the dedicated RRC signalling to form a view of a configuration; instead, IEs for the dedicated RRC signalling should be defined to be as self-contained as possible. |
| ZTE | We have no strong opinion on combining the common and dedicated configurations since the benefit is unclear. But we echo the comment from companies that the network should be allowed to provide dedicated configuration that overrides the common configuration, in this case, the UE should not override it again when it obtains the SIB1. In addition, we may need to consider how to modify the common configuration? In 5G, the common configuration is assumed to be the same for all the UEs, so, update of common configuration for connected UEs requires reconfigurationWithSync, it’s better to support a more flexible way (without HO) in 6G. |
| Apple | We agree with MTK’s view that common and dedicated parts (IEs) serves no functional purpose. Regarding the configuration used in connected mode, UE does not care what is common and what is UE specific, so it’s no problem for NW to provide all the required config directly to UE. |
| InterDigital | We agree with others that UE configuration is always dedicated, and that the UE does not need to be concerned with which parts of the configuration are shared with other UEs. However, the split between common and dedicated configurations in 5G was used in order to re-use configuration parameters that were present in both SIB as well as dedicated configuration as a means to reduce ASN.1 redundancy (e.g., for the BWP configuration). Therefore, we think both common and dedicated UE configuration as we have in 5G may be useful also for 6G. What may need to be addressed a UE’s dedicated configuration is somehow impacted by the signalling in SIB. We agree with ZTE that this is worth studying, for example, to allow the network to change configuration that is common to all UEs in a more efficient way. |

## 3.4	Nested ASN.1 structure

Several companies (e.g. R2-2508874 (Samsung), R2-2508080 (Xiaomi), R2-2508386 (InterDigital), R2-2508139 (LG), R2-2508614 (Ericsson), R2-2508406 (ZTE), R2-2508450 (Apple)) indicated that NR’s deeply nested (tree-like) configuration structure makes future extensions and delta signalling difficult. Xiaomi clarified during the discussion that deep nesting may make delta-signalling less efficient if/since higher level IEs in the tree structure must be re-provisioned when trying to update only a parameter in a low-level IE.

MediaTek explained that the primary problem of the nested structure is not related to delta signalling: “the deep (but originally logical) structure of cell groups, serving cells, BWP, etc. makes it difficult to add new features in clean way. In general, the more informative a structure is, the more tied it is to the known set of features and concepts at the time of the initial creation of the structure. Then, once a new feature which does not fit into the structure well is specified, it needs to be added in an unoptimal manner and then the main benefit of the original, once logical, structure mostly disappears”.

Several companies agreed with MediaTek that the nesting (and possible alternatives) should first be discussed in the context of the other email discussion on RRC modularization.

The problems of a hierarchical/nested ASN.1 structure (and possible alternatives) should first be discussed in the context of the other email discussion on RRC modularization.

| Company Name | Comment on problem |
| --- | --- |
| Toyota ITC | We agree that the NR structures are deeply nested and complex. However, this issue assumes that 6GR ASN.1 will follow the same design methodology. Discussions around 6GR extension types and signalling restrictions, for example, may suggest different structuring approaches. |
| Nokia | We think this is better discussed over the RRC structure email discussion, and it is easier to make progress if we e.g. look at how the existing Rel-15 RRC configuration would be different if done avoiding as many nested levels. While we agree that deep nesting is problematic, it is not obvious how the nesting truly impacts delta signalling. The basic principles should be to avoid more than 2-3 levels of nesting for the most common elements (e.g. protocol layer, serving cell, bearer and UP, measurement, security and PHY configurations). In summary: Avoid deep nesting but do not specify a “limit” to configuration nesting levels in RRC. |

| Samsung | We also share that the multiple parameters across different messages in ad-hoc manner due to complex RRC ASN.1 structure make the future extensions and delta signalling difficult. By this structure, the development and testing & maintenance efforts are increasing and difficult to understand new features. |
| --- | --- |
| OPPO | Nest itself seems not an issue, but the consequence it caused may be an issue. Yet if we look at the harmful consequence highlighted by proponents (if we take 8406 as an example) - “However, due to the tree-like hierarchical structure, even if the network intends to modify only one or two physical channel parameters, it might still need to carry the entire tree structure. The mandatory fields and their associated fields at each level contribute to the final increase in RRC signalling size.”. However, even in the absence of a hierarchical structure, mandatory fields would still contribute to the overall signaling size. Therefore, the total signaling overhead is not determined by nesting but also by the number and size of mandatory elements at the same level of the target parameter. [ZTE] The intention is to avoid sending mandatory fields duplicately if they are already part of the UE’s current configuration. The goal is like what Nokia indicated in 3.1, “Network is able to provide UE with only necessary configuration updates, i.e. it shall be possible to explicitly signal only changing parts of UE configuration via RRC reconfiguration.” - “Consequently, when the BWP configuration is updated or the PUCCH configuration under a BWP is modified, the corresponding CSI-MeasConfig also requires modification.” Yes, cross-level field dependencies may exist; however, it is unclear what actual adverse impact such dependencies would have. Are the concerns primarily related to specification readability, or do they affect implementation, maintainability, or interoperability? [ZTE] This does not relate to specification readability, it relates to NW implementation, e.g. on evaluating which parameter worth delta configuration and its complexity. - During HO, “Sometimes, the target cell even cannot decode some IEs, so it is impossible for the target cell to switch-off those functions”. The core issue appears to lie not in the nested structure itself, but rather in the mixing of features that are supported and unsupported by the target cell. [ZTE] Agree this is not because of the nested structure. It is mainly because of the definition of “SetupRelease{} ---OPTIONAL, Need M”. The target cell needs to explicitly indicate “release” to disable a function, but in HO, the target cell (with lower release) is even incapable to decode the ASN.1. - “for different UE types (e.g. RedCap, NCR-MT), their newly introduced specific parameters are often scattered across various branches”. The fundamental issue appears to stem not from the nested structure itself, but rather from the intermixing of features that are supported and unsupported across different UE types. [ZTE] This was raised by other companies, but in our (ZTE) view, this is not a high priority issue. So that issue-1) and 2) are not clear to us, while issue-3) and 4) are more related to the modular design, so out of scope of this email. [ZTE] Issue 1) , 3) and 4) are more related to modular RRC design, but issue 3）is more related to delta configuration, and it is already mentioned in 3.1. |
| Huawei, HiSilicon | We need to identify specific/concrete issues in deeply nested (tree-like) configurations (for example, how they hinder future extensions and make delta signaling difficult), and determine whether these issues are valid, whether these issues overlap with other issues, or can be resolved through modular design. Moreover, more specific/concrete problems will also help us later in discussing how to address these issues. |
| Xiaomi | From our perspective, deep nesting may also reduce the benefit of delta signaling. Specifically, IE/field at each nesting level itself needs to introduce extra overhead due to the signalling structure it applies, so the more nesting level, the more such overhead introduced. If the delta signalling is applied at the very low nesting level, all the higher nesting level parent IEs need be present and cost their own signalling overhead, as long as any fields/IE included in the lower-nesting level needs to be present. This makes the total signalling overhead not benefited much from the use of delta signalling. But we are fine to discuss this issue either here or in the RRC structure email discussion. |
| MediaTek | We agree that the deep (but originally logical) structure of cell groups, serving cells, BWP, etc. makes it difficult to add new features in clean way. In general, the more informative a structure is, the more tied it is to the known set of features and concepts at the time of the initial creation of the structure. Then, once a new feature which does not fit into the structure well is specified, it needs to be added in an unoptimal manner and then the main benefit of the original, once logical, structure mostly disappears. In other words, due to backward compatibility requirement, we do not obviously have the luxury of refactoring a structure whenever a new feature is being defined. To summarize, we'd prefer more flat structure that would retain its shape better upon addition of new features. However, similar to Nokia, we think this topic suits better in "RRC structure discussion" where also modularity is discussed. We think this because if the deep structure (of CellGroupConfig in practice) was changed to more flat structure, maybe then also several problems which are candidates to be resolved by modularity would disappear? (After all, we have modularity in 5G RRC already, as several configurations have their own top-level IEs, such as measConfig or radioBearerConfig, or even separate messages such as MBS or logged measurements.) |
| ZTE | We tend to agree with others that deep nested structure relates to the discussion on modular RRC. As we provide some responses to OPPO’s comments. |
| Apple | We may first need to know what the tree-like RRC structure of 6G looks like. In the 5G nested RRC structure, we understand that this architecture causes some complexity in delta configuration and some duplication of parameter configurations. This is something we hope to avoid in the 6G RRC structure design. |
| Interdigital | We agree that this discussion is related to modular RRC discussion. |

## 3.5	Overhead due to IDs

NR uses IDs for to manage elements in AddMod/Release lists which are one of the main building blocks of NR’s delta signalling (see 3.1). Those IDs are also used to reference from one IE to another IE. While this is useful and efficient in principle, there are occasions where this led to substantial overhead.

During the discussion several companies pointed out that linking by references (IDs) is generally better than referencing by hierarchy. Benefits in terms of flexibility and possible overhead reduction were mentioned. Ericsson pointed out that it may also increase overhead if used in too many levels and for too small IEs. Huawei agreed with Ericsson that NR’s TCI-states and CSI-RS configuration are the main contributors to signalling overhead but that in general ID-based linking is a good tool. OPPO warned that too small ID ranges in the initial release tend to cause cumbersome extensions in subsequent releases.

Nokia, Xiaomi and MediaTek mentioned “referencing by ID” could result in an “ID salad” if used excessively, i.e., it may make it difficult to comprehend the configuration.

Investigate how to use ID-based linking of configuration components while avoiding unfavourable signalling overhead and lack of readability.

| Company Name | Comment on problem |
| --- | --- |
| Ericsson | In our view, linking by references (IDs) is a much better practice than linking by hierarchy. Links by IDs can be multi-dimensional which isn’t possible in a hierarchical tree structure. However, if the IEs are small, the size of their ID (integer) can be significant compared to the size of their actual information. In addition, to the list where the elements and their IDs are defined, the IDs recur also in the IEs that refer to the elements. This may still be a good trade-off, if the (small) elements are defined once but re-used (= referenced) many times. But if that is not the case, such structure can be both complex and signalling-heavy. A prominent example is the CSI-RS resource configuration. CSI-RS resources, resource sets and resource configurations are small in actual size but refer to each other by means of IDs. We see examples in FR2 where this structure is by far the most substantial contributor (>50%) to the overall DL RRC message size. Smaller but still significant in size is the configuration of “TCI states”. They are associated with IDs and refer to SSBs or CSI-RSs by their respective IDs. And, like most of the above-mentioned CSI-RS configuration, they recur for each serving cell. Naturally, configuration flexibility comes with a cost in terms of complexity and overhead. But when designing 6G, RAN2 should look more carefully at real-world configuration examples, identify the actual overhead contributors and aim to eliminate those in 6G. The good news is that the configuration size of most of NR’s protocol layers and physical channels was already fairly small in practice! |
| Toyota ITC | We agree that linking with references is a better practice than linking by hierarchy because it decouples the hierarchy from the identities which is generally desirable. We should solve the problems first and then think about encoding size optimizations, not the other way around. |
| Nokia | Using IDs in an element to refer to other elements can be a useful way to lower overhead for RRC signalling (which we see as one of the real issue that we should aim to solve). We would also note that making a “pool” of all configuration components might also make the configuration much harder to understand (since the effective configuration could become an “ID salad” where it’s difficult to understand what is actually configured and how the separate elements interact with each other), but we would still agree that this is a good direction to investigate more. As with all things, this may not work in all cases but could be a good solution in complicated cases where the same configuration has to be referred to by multiple instances. As long as it’s clear what problems we are solving, we can use the best solution for each case. Finally, this discussion is also close to the topics in the email discussion [POST132][017][6G] RRC structure. In summary: Referring to configuration components (i.e. “configuration modules”) by ID should be investigated further based on practical examples. |
| Samsung | We are fine to study to refer the configuration componets by ID, we think this is the key signaling aspect if the new modular concept will be introduced. |
| OPPO | ID-based linking is a useful mechanism. However, if the concern stems from the overhead introduced by the ID itself, leading to consideration of reverting to hierarchy-based linking or deliberately limiting the ID value range in the initial release, this approach carries significant risks. The current assumption of having only a small number of configurations may prove short-sighted, as we've already witnessed with the expansion of the AddMod list - future requirements could demand substantially more configurations. From our perspective, while reducing signaling overhead through compression techniques is worth exploring, it shouldn't come at the expense of system flexibility. Such compromises may not represent the optimal direction for long-term solution development. |
| Huawei, HiSilicon | We believe that the overhead caused by IDs does exist, but it may not be a very widespread issue. We also observed that CSI-RS configurations and TCI state configurations are the main contributors to signaling size, which can exceed 7 KB in certain carrier aggregation scenarios. In particular, for TCI states, the TCI state ID and its associated CSI-RS ID and BWP ID are the decisive factors determining the size of the TCI state configuration. However, this is primarily a specific problem within TCI state configurations. For most other ID-based associations, this issue is far less pronounced. We could potentially design more compact signaling structures in Stage 3, such as using some implicit IDs for association, but these would require structural design optimizations tailored to specific signaling formats. |
| Xiaomi | In general, using ID/Index and a pool of complete configuration sets can save the signalling overhead, by avoiding the same set of configuration parameters being duplicated many times (e.g. duplicated in each entry of a list). At the same time, the difficulty for understanding the related configurations full of IDs/Indices may need be well balanced as well. The applicability of this referencing/ID based signalling design should be an issue bearing in mind when studying the solution in this direction. |
| MediaTek | We think this topic is somehow related to 'Nested ASN.1 structure' in chapter 3.4. In general, we think referencing by ID is better than linking by hierarchy, due to the reasons we mentioned in chapter 3.4 (i.e., addition of new features tend to mess up hierarchies). However, we agree with Nokia that care must be applied to avoid a "pool" of all configurations; the ambiguity and machine-readability aspects must be kept in mind. |
| ZTE | It seems companies are talking about two different things: - Signalling overhead caused by IDs (e.g. CSI-RS resources) - Whether to apply “pool” concept for all configuration in 6G, so that IDs will be widely used in 6G? We think 2 relates to modular RRC design that can be discussed in [POST132][017]. For 1, we are wondering what we can do in RAN2? Considering the value ranges are mostly come from RAN1/4. (The RAN2 defined lists are usually not very long) |
| Apple | The purpose of the ID-based apporach is to reduce signaling overhead, but the introduction of ID will also introduce new signaling overhead. Therefore, the benefits of this approach depend on the use case and the scenario, requiring the case-by-case analysis. |
| InterDigital | We think ID-based referencing generally reduces signalling overhead and should be use for 6G. The way this is used will depend on the use case and should be further studied. |

## 3.6	…

# 4	Solution Directions

In this section we aim to discuss potential solution directions that could help to address the problems and pain points agreed upon in section 3.

The level-2 sub-sections are meant to address one problem area each.

The level-3 sub-sections therein provide descriptions of possible solution directions. As rapporteur we started to pre-fill those solution directions based on company contributions or based on input provided already during phase 1. However, the proponents are encouraged to review, clarify and compete the solution direction or design principle that they propose. Companies are also requested to summarize their proposal as “Proposed design principle” which RAN2 could possibly try to agree upon in the next meeting (depending on the progress of the email discussion).

## 4.1	How to realize Delta-Signalling?

This section discusses solutions addressing primarily the problems identified in section 3.1, i.e., the following proposals:

Proposal 1: Investigate how to make the rules for delta signalling more explicit inside the signalling structure and thereby less ambiguous and less error prone to implement including the case of inter-node mobility.

Proposal 2: In the context of delta signalling, investigate especially how to improve the definition and extensibility of (AddMod/Release) lists.

Proposal 3: Ensure that delta signalling allows the network to modify/replace one part of the configuration without having to re-send also unchanged parts of the configuration. Discuss how to dimension and define those “parts” to avoid unnecessary “re-transmissions”.

Proposal 4: Ensure that the signalling structure is able to represent the UE’s entire current configuration (which the UE might have received in several “deltas”), e.g. for inter-node signalling in case of inter-node handover.

### 4.1.1	Release all optional fields + Keep/Set by parameterized types

R2-2508614 (Ericsson) proposed …

- Specify that the UE releases all OPTIONAL fields if they were configured before and are absent in a subsequent reconfiguration. This would make delta signalling feasible during inter-node mobility since the target gNB could rely on that the UE releases fields that the source node configured but that the target node might not support (and hence wouldn’t know how to release explicitly).

- Introduce parameterized types (e.g. one for individual elements, one for (large) lists) which contain a CHOICE structure by which the network can instruct the UE to keep the previous configuration or set/change it. Since the CHOICE’s options “survive” the ASN.1 compiler, the NW’s and UE’s application logic could detect which branches of the configuration are subject to delta signalling and automate the handling thereof.

Proposed design principle: Specify that the UE releases all OPTIONAL fields if they were configured before and are absent in a subsequent reconfiguration. Introduce parameterized types (e.g. one for individual elements, one for (large) lists) which contain a CHOICE structure by which the network can instruct the UE to keep the previous configuration or set/change it.

| Company Name | Comment on problem |
| --- | --- |
| MediaTek | While the solution where the UE releases all optional fields if they are absent would resolve the inter-node reconfiguration (mobility) problem from network perspective, such solution would reduce the efficiency of delta signalling for normal (intra-node) reconfigurations, as all optional fields would need to be signalled again in each reconfiguration no matter they are actually changed or not (unless of course all optional fields were defined using the proposed parameterized types using which the NW can signal "keep the previous value"). The fact that the parameterized types survive the ASN.1 compilation and therefore open up possibilities for automation is very important, and we think we should explore such solutions more (also for conditional presence/absence problems in 4.2). (For automation, we should not rely only on validation during ASN.1 compilation itself, but implementations can be expected to have pre/post-processing for ASN.1 encoding/decoding where certain fixed and known aspects in signalling, such as certain kind of CHOICE or certain field name defined by a parameterized type, is utilized. Enablers for this kind of automation would be very beneficial.) |
| OPPO | The proposal to "specify that the UE releases all OPTIONAL fields" leads to some ambiguity regarding its scope. Specifically, it is unclear whether this applies to all fields, given that at least need-N and need-S cannot be avoided. If the intention is to enforce this strictly, the proposal seems more focused on eliminating need-M (?) Even in that case, the value of formally removing all need-M fields—while relying solely on a CHOICE bit to retain previous configurations—is debatable. Key considerations include: 1/ Each affected field would require an additional bit. 2/ If the lowest-level parent IE uses need-R, it already addresses inter-node mobility (as noted by the proponent). Meanwhile, retaining need-M for child IEs enables delta signaling benefits without the need for extra CHOICE bits in parameterized types. Comparing the Two Solutions, if we focus on the lowest-level parent IE, the core alternatives are: 1/ Legacy Approach (need-M + SetupRelease): Allows releasing the IE. 2/ Alternative Approach (need-R + New Parameterized Type): Allows maintaining the IE. The critical question is whether the legacy approach (Alt-1) poses implementation challenges for networks. To resolve this, it would be valuable to gather input from network vendors. [ZTE-Liujing] To answer OPPO’s question. The main problem of Alt-1 is that releasing the IE still requires the network to explicitly indicate “release”. In inter-node handover, it is possible the target cell has lower version, which does not support the upgrade ASN.1, thus, it is not able to decode the configuration and not able to set the “release” indication. Therefore, fullConfig should be used and impacts the performance. [Ericsson] We agree with ZTE’s explanation. In addition, the problem in NR is not only about the target node having a lower release. It is also about lack of support of features. I.e., even if the target supports the same ASN.1 version as the source, it has no means to guarantee that it knows whether/how to release all configuration IEs that the NW might have used but that the target does not support. |
| Apple | This solution can be used to address the delta inter-node reconfiguration, when the target node is unable to release certain configurations due to capacity issues. However, for the normal RRC reconfiguration, this solution may have the following issues: 1) This proposal can not handle the “Need N”, “Need S”, conditional configuration. 2) This adds signaling overhead for Need M parameters. First time to set the choice to configureElement and next time to set the choice to keepElementUnchanged. This has to be done for each optional Need M parameter. Currently we have 780 optional IE with Need M code in the NR RRC spec. 3) This adds processing overhead at both NW and UE for Need M parameters in case no change in previous config. Currently if no change, then NW and UE does nothing, but in this proposal NW has to send the union with keep and UE has to check if union is set to keep or not. 4) We should not make the default neither Need M nor Need R because both used a lot in current NR spec (Need M used 780 times and Need R used 1211 times). |
| ZTE | We understand this solution can meet the requirements different scenarios: HO vs non-HO, but we have some concern on the signalling overhead and implementation burden. As explained in R2-2508614, with this new structure: - NR: 1 bit for keeping the element, 2 bit for releasing it - 6G: 2 bit for keeping the element, 1 bit for releasing it Typically, inter-node handover happens less than intra-node handover and reconfiguration. For intra-node case, most configurations need to be kept and few configurations need to be updated. So, there are more chances and more cases that 2 bits will be needed for keeping the unchanged elements. On the other hand, from network implementation point of view, if the network wants to reconfigure one element that is under the deep level of RRC signalling, since the network cannot indicate “keep” for its parent IE and parent’s parent IE, the network needs to set “keep” indications for all other paralleled IEs which are under the same parent IE, this requires very careful implementation. We think a preferred way to have special treatment for concerned parameters, by default releasing all OPTIONAL fields may be a bit overkill. |
| InterDigital | Similar to comments from other companies, we think releasing all optional parameters results in the need to reconfigure these parameters for the cases where Need M is intended (e.g., intra-node reconfiguration). Use of parametrized types may provide a solution to this, but the results in overhead, as mentioned by Apple. |
| CATT | In our view, full configuration is used to address this issue above in case of inter-node mobility. We are open to discuss other candidate solutions, but low complexity design is preferred. For now it’s pre-mature to agree to “Release all optional fields + Keep/Set by parameterized types” due to the concern raised by other companies. |
| Samsung | In our opinion, if we first define the ASN structure in a more modular manner, including the necessary code types, etc. we can then evaluate the details of the proposed solution to see whether it brings any benefits. The delta configuration is currently ambiguous due to the existing RRC ASN.1 structure and design. One approach to address this ambiguity is the introduction of a modular RRC structure in 6G, which could provide a more structured and efficient framework. Our initial thoughts on the proposed solution are that releasing all optional fields may not be very optimal from the UE perspective. Additionally, we need to evaluate further if it introduces any inconsistencies. We also need to assess whether a parameterized approach would result in signalling overhead, as it may be required for every other configuration. |
| Ericsson | In response to the answers above: 1) Overhead: We think that delta signaling (i.e., the equivalent to NR’s “Need M” and “AddMod/Release” lists) should only be applied to fields of “significant” size (not to small individual fields on the lowest level). And for those larger fields the relative overhead of the optionality bit and an additional “Keep” or “Release” bit is insignificant. Note that also NR’s “Need M” should always be used in combination with SetupRelease. Otherwise, the NR gNB cannot release the field itself. It would need to traverse the object tree upwards until it finds a (grand-)parent that it can “release and add” in a single step. In NR this is extremely difficult since the handling of delta signaling cannot be automated reliably (as observed by many companies). If the parent IEs don’t offer the option to “release and add” the gNB has no other means than a fullConfig. All of those options add in practice a lot more signaling overhead than what could have been saved by omitting some SetupRelease wrappers. 2) Need S; Apple’s bullet 1): NR uses “Need S” to define a default behavior upon absence. That occurs e.g. when the parent IE configures e.g. a physical channel and when the Need S child field determines a parameter thereof. The cleaner solution is to define the default value using the DEFAULT key word. If deemed beneficial, the default can also be specified in the field description (similar to today’s Need S). But one should really avoid defining delta signaling rules in a “Need S”-statement. That is what broke NR’s delta signaling in practice. 3) Processing: Apple’s bullet 3): This is similar to the signaling overhead: NR’s “Need M” in a SetupRelease requires setting/checking one bit to keep the element but two bit to set, change or release it. Our proposal would turn this around: 1 bit for releasing, 2 bit for keeping, setting or changing. Overall, the processing cost for checking those individual bits seem negligible compared to the other processing of the ASN.1 messages and the UE/NW-internal control processing. 4) Implementation complexity: ZTE mentioned that the network would need to set all “ChangeKeep” bits in the tree carefully to avoid that the UE unintentionally releases a branch. We agree with this and your figure depicts this nicely. However, the task of including the ChangeKeep elements and setting them to “Keep” is logically simple and can be easily automated when we mark them by a recognizable field name (ensured by the parameterized type). |
| LGE | Agree that the solution addresses the delta configuration in inter-node reconfiguration. However, the solution can lead to signaling overhead for keeping some parameters as commented by other companies. If intra-node reconfiguration is more frequent than inter-node reconfiguration, the signaling overhead can be significant. |
| Huawei, HiSilicon | In our proposal for modularization, we provide the way to release a module that is not comprehended either by the UE or by the target node (separate encoding in a OCTET STRING, added/modified/released using ToAddModList). We agree with Samsung that releasing all fields by default might not be optimal, and that further evaluation would be useful. |
| Xiaomi | If our understanding is correct, release all optional fields if not present is used together with ChangeKeep structure. We need to evaluate the new ChangKeep structure and compare it with existing SetupRelease structure to see which one is more efficient, especially for non-HO case. For HO case, looks like it may avoid the full configuration if different network supports different features/releases. We also need to understand whether it happens frequently or not in the commercial deployment. |
| Nokia | Release of fields: This would essentially mean all OPTIONAL fields to implicitly use Need R without having to write this down. First, we think this is possible but we have often seen implicit UE behaviour as having more problems than explicit UE behaviour. Second, there is also a difference in a RRC message between when a field is absent (i.e. not present at all in the UE configuration or in the sent configuration message) and “not present” in the signalling (i.e. field is configured but not present in signalling when OPTIONAL): In the former case nothing should be released (and since UE configuration doesn’t change, no actions should be triggered), while in the latter case some actions on release could be triggered (since UE configuration changes). Making the release implicit might make it harder to distinguish such cases. Third, as many fields will likely be optional (including any later-release fields), this would mean potential signalling size just to retain the same configuration in case specific parameters need to be used. The alternative (or complement) to this would be to explicitly make some IEs as “modules” that are defined using parameterized types that can explicitly indicate whether the field is kept or retained. Fourth, this would also mean that releasing any parent field would automatically release all child fields, and releasing only a particular child field would always require the presence of the parent field (i.e. same as with Need M parent fields now). Finally, RRCReconfiguration typically retains fields more than it releases them (with fullConfig being the sole exception, but that wouldn’t affect this case at all if we adopt it at parent message level), so if keeping an element cost 2 bits and releasing costs 1 bit, we are increasing the overhead for the more typical case. Parameterized types: We agree with the use of parameterized types as such, although if course we should discuss the exact format but the principle seems sound. Summary: We are somewhat hesitant to adopt implicit handling of field release, but would like to look at practical examples before determining the best way forward: Coupled with the parameter types, this might also have some beneficial ways e.g. for machine-readability (but the main problem with that is the conditions, not Need R vs. Need M). We can evaluate this further and consider better whether the costs are acceptable. |

### 4.1.2	New type of need code

R2-2508406 (ZTE) proposed to study a new type of Need Code which fits the different requirements in different scenarios. …

| Company Name | Comment on problem |
| --- | --- |
| MediaTek | We think that specifying new type of Need Code which triggers different UE behaviour in non-handover case (absent = maintain) and in inter-node case (absent = release) would effectively implement the same result for inter-node case which "release all optional fields" suggested by Ericsson in 4.1.1 does. However, the difference is that this solution would not reduce the efficiency of delta signalling for normal (intra-node) reconfiguration, whereas the "release all optional fields" solution will reduce the efficiency, see our comment in 4.1.1. A problem with new Need code (or any Need code based solution) is that obviously Need codes remain non-machine readable. In 4.1.1, Ericsson raises a view that using parameterized types to describe certain aspects of signalling has the benefit to survive the ASN.1 compilation, which is essential for machine readability. In that sense, we tend to think that detailed solutions based on parameterized types for various purposes could be explored more (also for conditional presence/absence problems in 4.2). Another alternative solution to the inter-node reconfiguration problem could be more fine-grained full configuration discussed in [017]. |
| Qualcomm | We see some benefits of potentially having new Need Code(s). However, our approach to new need code(s) is somewhat different from having different meaning of the SAME need code for different cases e.g. HO vs non-HO as suggested by ZTE. As explained in our paper R2-2508758, one of the issues of delta signalling is related to whether the UE is able to fully comply with the signalling. To enable the UE to partially apply reconfiguration without declaring RLF, there would likely be a need to differentiate parameters that are ‘critical’ that must be applied (or else reestablishment failure shall be triggered) and not-so-critical (‘best-effort’) that are best if applied successfully but in the worst case possible to be ‘downgraded’ without interrupting the connection. One possible way to achieve this is to have a new Need Code for such parameters. Another potential use case for new Need Code would to be indicate whether UE should keep a configuration upon RLF or re-establishment, which can help in reducing the size of re-establishment message. So, RAN2 could further study scenario-specific Need codes (instead of scenario-specific meaning of the same need code) as not all IEs need to be released/kept homogeneously for all scenarios e.g., for HO, RLF, reconfiguration failure. This can help in minimizing reconfigurations as well as balancing UE memory requirements depending on the scenario. |
| OPPO | This proposal shares similarities with the approach outlined in Section 4.1.1, aiming to leverage a need-R-like operation for inter-node mobility scenarios while employing a need-M-like operation for non-mobility cases. While this represents a potential alternative to the 4.1.1 proposal, it introduces additional complexity in distinguishing between mobility and non-mobility scenarios. This differentiation could become further complicated if there is additional need to distinguish inter-TRP/DU/CU mobility cases, or even dual connectivity (DC) scenarios should 6G continue to support such features. Moreover, there exists a potential chicken-and-egg dilemma: the interpretation of need-codes may depend on specific parameters within RRC messages (used to distinguish between mobility and non-mobility signaling), while these parameters themselves might already be associated with need-codes. From this perspective, we maintain a generally negative stance toward this approach. Instead, we recommend adopting a self-contained ASN.1 decoding methodology (including need-code interpretation) that eliminates additional dependencies. |
| Apple | The proposal is to address the inter-node delta configuration issue. According to the background indicated in ZTE’s contribution, the proposal could be interpreted in the way that one parameter will be handled as Need M in case of reconfiguration from serving cell and Need R in case of HO. Currently there is one flow per parameter (e.g., Need M or Need R). if our understanding on this proposal is correct, it doubles the effort per parameter to maintain 2 flows for each parameter. Due to the large number of parameters, it may introduce some complexity. |
| ZTE | (Proponent) The idea of this solution is to define a new Need Code so that the UE behavior can vary depending on the scenarios. Regarding the comments from OPPO, we think the UE does not need to check the specific parameter to recognize the scenario. For simplicity, the network can explicitly indicate scenario at the beginning of RRC signalling. (for example, 1bit =0 means absent = maintain; 1bit =1 means absent = release). We understand the main difference between 4.1.1 and 4.1.2 is whether the approach applies to all OPTIONAL fields, or can be applied to some specific fields. |
| InterDigital | We think associating need code interpretation with the type of reconfiguration (HO vs non-HO) should be avoided as it creates an additional dependency between the ASN.1 and the UE’s behavior. |
| CATT | In this phase of study, we may just reach consensus that need code can be enhanced to fit the different requirements in different scenarios. Then subsequent discussion can be organized case by case and driven by paper. Our preference is that new enhancements will not bring extra complexity compared to existing designs. |
| Samsung | We should further study the existing code for possible enhancements and evaluate the need for new types of code to determine if they bring improvements in readability and maintainability. |
| Ericsson | We agree with ZTE’s intention that the UE should preferably release optional fields upon HO to avoid keeping parts of the source configuration which the target gNB may not use/comprehend. But doesn’t this approach imply that there is basically no delta signaling during mobility? That is, if the network indicates in the Reconfiguration message “1bit =1 means absent = release” the UE releases all optional fields marked with this “Need M”-like need code. So, during this (handover) scenario the NW has no means to tell the UE to keep part of the configuration. Did I misunderstand the proposal? |
| LGE | This solution can lead to signaling overhead similar with 4.1.1. Based on ZTE comment, we can think several approaches to introduce explicit scenario indication as follows, e.g.: 1) indication per reconfiguration message (i.e., single bit; 2) indication per a parameter with new need code. With approach 1, the UE releases all parameters with absent and new need code when the indication is set to 1. This implies that, in order for UE to maintain some parameters with new need code, the target should include the same parameters as the source into the reconfiguration message. This can result in signaling overhead. We think approach 2 is similar with the solution described in 4.1.1. As mentioned in 4.1.1, this solution can lead to signaling overhead for keeping some parameters (i.e., single indicator for every parameter with new need code should be provided). |
| Huawei, HiSilicon | We are opened to discuss a new need code, but a new need code means more possibly more discussion and errors for each new field introduced. To reduce ambiguities and errors, we should also consider removing Need codes, or removing options. For example, we could entirely rule out the use of plain Need M, and use either SetupRelease or Need R. |
| Xiaomi | Maintains the parameters in two different structures (in non-handover case, absent = maintain; in handover case, absent = release) will increase the complexity which should be avoided. |
| Nokia | We think adding more need codes will likely not solve the delta signalling problems so we should see practical examples and understand how frequently they would be used. This would need a concrete example based on existing specifications to show where this could be beneficial. |

### 4.1.x	…

## 4.2	How to capture conditional presence/absence of fields?

This section discusses solutions addressing primarily the problems identified in section 3.2, i.e., the following proposals:

Proposal 5: Investigate the configuration constraints to specify and how to specify them unambiguously and clearly distinguishable from delta signalling.

### 4.2.1	Different IE types for initial configuration and reconfiguration

R2-2508112 (MediaTek) proposed to specify two variants of an IE types where one is intended for the initial configuration and the other for subsequent reconfigurations. A field that NR would have marked with “Setup” would be mandatory in the first IE variant (to enforce its presence when the network configures the parent field for the first time) and optional in the second variant (to allow the NW to change it in a subsequent re-configuration. A field that would have been market SetupOnly in NR would now be absent in the second variant of the parent IE to ensure that the NW cannot change it after having configured the parent IE.

[MediaTek] For this solution, configuration parameters can be categorized into three groups, based on each parameter's functional mandatoriness and modifiability. Based on the functional mandatoriness, field for the parameter is defined either as optional or mandatory field in the first ('add') IE variant of the parent field. Based on the modifiability, field for the parameter is either omitted or defined as an optional field in the second ('modify') IE variant of the parent field. Table below summarizes this and ASN.1 example below clarifies how the solution is applied in practice.

NOTE 1: For simplicity, we expect that all functionally non-mandatory configuration parameters (group 3) can also be modified upon reconfiguration of the parent field, so modifiability plays a role only for functionally mandatory configuration parameters.

NOTE 2: It should be noted that if the type of the field is ASN.1 sequence (and not a basic type, such as INTEGER), then more complex definition (a CHOICE of 'add' and 'mod' IE variants) is required for functionally non-mandatory configuration parameters (group 3) in the 'modify' IE variant of the parent field, since the parameter can either be configured for the first time or reconfigured upon reconfiguration of the parent field. The CHOICE can be defined using a new parameterized type SetupModify, as shown the ASN.1 example below.

| Group | Is functionally mandatory for the parent field? | Is modifiable upon reconfiguration of the parent field? | ASN.1 definition | Comments |
| --- | --- | --- | --- | --- |
| #1 | Yes | Yes | In 'add' IE variant of parent: mandatory field In 'modify' IE variant of parent: optional field | In NR, this kind of parameter could have '-- Cond Setup' or similar, but many times (especially RAN1 defined parameters) it has nothing to describe the mandatoriness |
| #2 | Yes | No | In 'add' IE variant of parent: mandatory field In 'modify' IE variant of parent: no field | In NR, this kind of parameter could have '-- Cond SetupOnly' or similar, but many times (especially RAN1 defined parameters) it has nothing to describe the mandatoriness and it being non-modifiable |
| #3 | No | n/a | In 'add' IE variant of parent: optional field In 'modify' IE variant of parent: optional field (as a CHOICE of 'add' and 'mod' IE variants, if this field is ASN.1 sequence itself) | In NR, this kind of parameter is very common, i.e., 'normal' optional configuration parameter |

SetupModifyRelease { ElementAddTypeParam, ElementModTypeParam } ::= CHOICE {

release    NULL,

setup      ElementTypeAddParam,

modify     ElementTypeModParam

}

SetupModify { ElementAddTypeParam, ElementModTypeParam } ::= CHOICE {

setup      ElementTypeAddParam,

modify     ElementTypeModParam

}

UpperLevelConfig :: = SEQUENCE {

-- For x-Config, separate 'add' and 'modify' IE variants are not defined,

-- so it is signalled as in NR

x-Config SetupRelease { X-Config } OPTIONAL, -- Need M

-- For lowerLevelConfig, separate 'add' and 'modify' IE variants are defined

lowerLevelConfig SetupModifyRelease { LowerLevelConfigAdd, LowerLevelConfigMod } OPTIONAL, -- Need M

...

}

-- Within LowerLevelConfig, lowerLevelSubConfig1 is group 1 parameter, lowerLevelSubConfig2 is group 2 parameter,

-- and lowerLevelSubConfig3 is group 3 parameter

LowerLevelConfigAdd ::= SEQUENCE {

lowerLevelSubConfig1 LowerLevelSubConfig1Add,

lowerLevelSubConfig2 LowerLevelSubConfig2Add,

lowerLevelSubConfig3 LowerLevelSubConfig3Add OPTIONAL,

...

}

LowerLevelConfigMod ::= SEQUENCE {

lowerLevelSubConfig1 LowerLevelSubConfig1Mod OPTIONAL, -- Need M

lowerLevelSubConfig3 SetupModify { LowerLevelSubConfig3Add, LowerLevelSubConfig3Mod } OPTIONAL, -- Need M

...

}

-- Within LowerLevelSubConfig1, param1 is group1 parameter, param2 is group 2 parameter, and param3 is group3

-- parameter

LowerLevelSubConfig1Add :: = SEQUENCE {

param1 INTEGER,

param2 BOOLEAN,

param3 ENUMERATED { yes, no, maybe } OPTIONAL,

...

}

LowerLevelSubConfig1Mod :: = SEQUENCE {

param1 INTEGER OPTIONAL, -- Need M

param3 ENUMERATED { yes, no, maybe } OPTIONAL, -- Need M

...

}

-- LowerLevelSubConfig2Add, LowerLevelSubConfig3Add, and LowerLevelSubConfig3Mod are omitted

-- for simplicity of the example

Proposed design principle: For lower level configuration IE, define separate 'add' and 'modify' IE variants, unless the IE is very simple and has no extension possibility. Introduce new parameterized types to improve readability and maintainability of ASN.1 and to enable automation in implementations.

| Company Name | Comment on problem |
| --- | --- |
| MediaTek | We think this solution could be applicable to lower level (RAN1) IEs, since those IEs usully configure a mixture of functionally mandatory and non-mandatory configuration parameters. Also, most of the observed interoperability issues are related to ambiguities in RAN1 configurations. For upper-level configuration IEs ('owned' by RAN2), we do not think this solution gives enough benefit vs. the maintenance effort. The fact that the parameterized types survive the ASN.1 compilation and therefore open up possibilities for automation is very important. (For automation, we should not rely only on validation during ASN.1 compilation itself, but implementations can be expected to have pre/post-processing for ASN.1 encoding/decoding where certain fixed and known aspects in signalling, such as certain kind of CHOICE or certain field name defined by a parameterized type, is utilized. Enablers for this kind of automation would be very beneficial.) |
| Qualcomm | This can provide spec clarity on when the NW should or should not include a field, however, it cannot make it impossible by signalling design to include SetupOnly fields in signalling after having configured it already (because a signalling CHOICE including setup is still valid from ASN.1 syntax point of view during modify), and vice versa. So, it is unclear how this can effectively solve the issue of UE configuration parameters which shouldn't be modified after initial configuration of a feature/functionality can be sent in subsequent over-the-air RRC messages with new values. [MediaTek] The key improvement is that checking the correct usage of each CHOICE against the current UE configuration (i.e., that the network does not send 'add' IE if the corresponding configuration has already been setup before, and vice versa for 'mod' IE) can be automatized in the UE implementation (post-processing of ASN.1 decoding) as checking it does not require detailed understanding of presence requirements of each and every RAN1 parameter. In other words, once the UE has checked that the network signals correct version of an IE, it can be sure that the actual parameters within the IE do not cause the "UE configuration parameters which shouldn't be modified after initial configuration of a feature/functionality can be sent in subsequent over-the-air RRC messages with new values" or the "functionally mandatory UE configuration parameters can be absent in over-the-air RRC messages for initial configuration of a feature/functionality" issues anymore and the UE needs no further check for correct presence of each contained (RAN1) parameter. |
| OPPO | We support the intention to establish clearer differentiation between initial and subsequent configurations. However, we have reservations about implementing this through different Information Element (IE) variants, as this approach would: 1/ Increase the ASN.1 specification effort: Each field would require two versions (initial setup and subsequent configuration), leading to numerous possible combinations. For example, Field 1 might use initial setup while Field 2 uses subsequent configuration, resulting in overly complex ASN.1 configurations. 2/ Potentially create maintenance and extension challenges for future releases: A field's presence/absence restriction defined in Release X might need modification in Release X+1 to accommodate new features, creating versioning challenges. Therefore, we recommend exploring alternative approaches that offer greater flexibility and cleaner implementation, such as utilizing sub-type methods, rather than mandating IE variant specifications. |
| Apple | The motivation behind explicitly reflecting the usage of each parameter in ASN.1 part is good. However, according to the proposal, for many data types, there will be 2 separate parameters for the same configuration (one for add and the other for modify). Expectation is that this will be happening not only on the very top level but deeply in the nested structures as well. So at the end there will be too many additional IE types. This will impact the ASN.1 generated code because there will be 2 different look-up tables for each IE (one table for the add showing the included IEs and one for the Modify for the included IEs) One alternative option to reflect the usage (i.e. setup only or setup&modify), conditional presence can be added as in the current 5G we have (Cond SCellAdd, Cond ServCellAdd, Cond condReconfigAdd …). |
| ZTE | We share the concern from OPPO regarding the spec maintenance effort. We understanding the issue raised by MediaTek is valid, but the question is that besides IoT test, do we really need a big, standardized solution for this, considering the number of parameters that are prone to be problems is actually limited in the field. Another phenomenon we observed is that the scenarios encountered in actual deployment may not be the typical scenarios that discussed in RAN2 (note: usually, this not relates to a specific parameter, but relates to a combination of several parameters). So, in order to make sure the initial design of IE structures is flawless, it requires companies to explore all possible configuration scenarios at the beginning, this will also increase the burden of standardization. |
| InterDigital | We also share the concerns in terms of ASN.1 code size and spec maintenance for this approach. |
| CATT | We are open to discuss this as a potential RRC signaling enhancement, but it’s pre-mature to conclude on “define separate 'add' and 'modify' IE variants” due to the concerns above raised by other companies. |
| Samsung | This kind of approach may not be scalable considering every device/feature/service may have different requirements. This may bring some ASN complexity. We can first define the minimal set of parameters which are common for any service/feature/device and then can see if such approaches can fit well into new defined modular structure |
| Ericsson | We agree with MediaTek’s intention to convey conditions (== restrictions for valid configurations) in a more consistent and systematic way. We would also be willing to accept a bit more effort in specifying additional IE types if that would reliably avoid ambiguities in implementation. But we are also not certain that the solution offers the best trade-off among “specification complexity” and “implementation robustness”. Using constrained sub-types as suggested by Toyota in 4.2.2 seems a little lighter for the cases where it is applicable. The example provided by MediaTek seems to grow complex especially since there are different “Conditions” on different nesting levels, right? I am not sure whether we have many such cases in NR. But if so, we did likely not notice them while writing the specifications. But product development will run into the complexity problem during implementation and testing – as MediaTek described. Hence, maybe the primary observation and conclusion should be that RAN2 should avoid specifying nested conditions. I am not sure this is always possible but if we manage to reduce the nesting levels (see email discussion 017) that would be a good first step for reducing also the risk of nested conditions. If RAN2 runs into nested conditions anyway, we would be open to explore MediaTek’s proposal to different IE types for initial configuration and reconfiguration. And if RAN2 concludes that it is too complex to specify the conditions in this way, this is likely a sign that the nested conditions themselves are too complicated. |
| LGE | We also share the concern from OPPO. While we understand that defining different IE types for initial configuration and reconfiguration can improve specification clarity, this approach appears to require excessive ANS.1 specification effort to address a single problematic case. It seems that modifying the ASN.1 signalling structure for such case-by-case issues, provides limited benefit. |
| Huawei, HiSilicon | The solution has significant complexity, so we would like to look more carefully at examples to see if there could not be a simpler solution, even with some more signalling overhead e.g., - not using non-modifiable fields - making short fields, of which only a small number can be configured, mandatory In NR, the main source of overhead is TCI-States and CSI-RS configuration, but we are not aware of any such issue for them. |
| Xiaomi | We agree with MediaTek’s intention. However as mentioned by other companies, we need to consider whether it will increase specification maintenance efforts significantly, for instance too many IEs if we use it for every low level fields. It may be considered case by case. |
| Nokia | We think this could be useful in some cases but not as a rule to be used everywhere – case-by-case judgment would be needed. The example also looks rather complex with nested parameterized types, which are quite complex. The downside of this proposal is ASN.1 duplication, which might create need to have different procedural text as well. We are not sure there are many cases where both the add- and modify-branches end up being very different. |

### 4.2.2	ASN.1 feature for constraint sub-types

R2-2508649 (Toyota) suggested to capture the configuration constraints by an in-built ASN.1 feature referred to as “constraint subtype”. Unlike the proposal in 4.1.2, the IE types would be defined only once and hence there would be only one source-code representation (e.g. in C++) that product developers need to deal with in their implementation. However, RAN2 could specify two (or more) sub-types of an IE which constrain which fields the sender (here NW) may/shall include in each of the variants.

Proposed design principle: …

| Company Name | Comment on problem |
| --- | --- |
| MediaTek | We think this solution could be usable for cases where the same IE is used for several, slightly different purposes, such as for PCell and SCell. Effectively it would mean to derive PCell version of the IE and SCell version of the IE from the same "baseline" IE. Based on above, this solution initially sounds also a good replacement for separate add/mod IEs, but we do not think this solution is suitable for that purpose, as it does not scale well to parent-child structure which we discuss in detail in 4.2.1. As a summary, we think this solution should be evaluated more and it seems a valuable tool in the toolbox towards more machine readable ASN.1. |
| Qualcomm | Constraint subtypes can be a good way to enforce the encoding rules within ASN.1 itself. So, we are sympathetic to such approach. However, we would like to point out potential drawbacks - Constraining all the conditional cases may not be straightforward or possible. - Updating the constraints would possibly be “ASN.1 NBC” (as opposed to today’s possibility of updating procedural text/field description/ Need Code/ Conditions without having to create NBC in the ASN.1 blocks). |
| OPPO | As answered for clause 4.2.1, we tend to agree to explore this method, as a complementary part for the proposal in 4.2.1, in order to avoid mandating two IE variants for every field. |
| Toyota | Following ITU recommendations, the core principle is to define a “base” IE that includes, e.g., optional content. Constraints on this content are then applied through sub-types referencing the “base” IE. This approach enforces the presence or absence of specific content, as well as particular values or value ranges. However, implementing this requires some form of branching, i.e., a choice construct, to select the appropriate constrained version of the IE. This branching can occur either at the message level or within the referencing IEs themselves. Since the sub-types are designed to be extensible with extension markers in the same way as other ASN.1 elements, this method should maintain backward compatibility without introducing differences compared to current methods. The advantage is that there is no ambiguity about whether optional content should be present or absent, making it possible to specifying UE behaviour upon presence or absence without Need codes. Unlike Need codes, constraint sub-types are machine readable. Conceptually the difference compared to 4.2.1. seems quite small or at least there are many similarities. |
| Apple | It is a good tool that is added on top of the existing 5G mechanism for defining the constraints. But the proposal cannot solve all the constraints. It can be used only in some cases where the scenario is very clear then the message can exactly specify which IEs to be included or not. [Toyota] The usage and applicability of constraints can be different in uplink vs downlink, and system information may also require some considerations. |
| ZTE | We are interested in this proposal and are willing to study it further. In addition to the issues raised by Qualcomm, we also want to know whether this can cover all the cases, for example, within a structure IE, some constraints relate to more than one fields, if IE-a is set to X, the IE-b should be present, but if the IE-a is set to Y, the IE-b can be optional or should be absent. |
| InterDigital | We are open to studying this further and see if it is possible to address all scenarios and cases. |
| Ericsson | We agree with Toyota and others that this is a nice tool that was somewhat forgotten in RAN2. We support exploring this tool as means for capturing conditional presence. We agree with MediaTek that the constraint sub-types don’t seem to support nested conditions. Whenever possible we should avoid those anyway. But if we run into nested conditions, we could fall back to MediaTek’s suggestion in 4.2.1. We also agree with ZTE that constrained sub-types may not be able to express more complicated rules including constraints on value ranges. In such cases it may be beneficial to define two versions of the IE in which those conditions occur (see 4.2.3) |
| Huawei, HiSilicon | The proposal seems to have reasonably low complexity, so it looks interesting. |
| Xiaomi | We agree with Toyota’s intention. RAN2 further study is needed on the details, e.g., whether it will increase the maintenance efforts by adding components in each structures ,etc. |
| Nokia | We think this could be a tool to consider for some cases, although the syntax becomes a bit heavy if there are multiple constraints. We should at least consider this if we have constraints that are unlikely to change between releases. We have also one question to clarify: Can the constraints for a field defined in Rel-X using WITH COMPONENTS be changed in a later release? Is that possible? To illustrate the example: If Rel-X+1 changes the constraints for allowed values, what happens in a Rel-X network where the old constraints are used but Rel-X+1 UE no longer follows them? |

[Toyota] Below is a short example including extensions.

Note: Field ‘param1’ is present in all configurations. It does not need to be captured in the constraints because it is mandatory present.

Configuration :: = CHOICE {

configuration1       IE (WITH COMPONENTS { ..., param2 PRESENT, param3 ABSENT }),

configuration2       IE (WITH COMPONENTS { ..., param2 PRESENT, param3 PRESENT }),

...,

configuration3       IE (WITH COMPONENTS { ..., param2 ABSENT, param3 ABSENT, param2-ext PRESENT }),

}

IE :: = SEQUENCE {

param1                 BOOLEAN,

param2                 INTEGER(0..127)                 OPTIONAL,

param3                 ENUMERATED {on, off}            OPTIONAL,

...,

[[

param2-ext              INTEGER(128..255)               OPTIONAL

]]

}

Here is another variant where the param2 value range is extended from 0..127 to 128..255 by only changing the constraints whereas the information element defines an unbounded integer type without any value range:

Configuration :: = CHOICE {

configuration1       IE (WITH COMPONENTS { ..., param2 (0..127) PRESENT, param3 ABSENT }),

configuration2       IE (WITH COMPONENTS { ..., param2 PRESENT, param3 PRESENT }),

...,

configuration3       IE (WITH COMPONENTS { ..., param2 (128..255) PRESENT, param3 ABSENT }),

}

IE :: = SEQUENCE {

param1                 BOOLEAN,

param2                 INTEGER                         OPTIONAL,

param3                 ENUMERATED {on, off}            OPTIONAL,

...

}

### 4.2.3	Accommodate for critical extensions of IE types

R2-2508614 (Ericsson) suggested to address the problem that allowed combinations of fields become ambiguous when RAN2 adds too many new fields to an existing IEs type (after “…”).

The contribution suggested to accommodate for critical extensions of lower-level configuration IEs using the same CHOICE structure, which RAN2 used commonly for NR’s (near-)top-level IEs. Using them instead on lower level IEs (e.g. PDSCH-Config) would allow RAN2 to introduce a new variant (e.g. PDSCH-Config-r23) where needed while avoiding an entirely new configuration structure. The latter would have happened if RAN2 had ever used the critical extension of near-top-level IEs such as RRCReconfiguration.

The contribution highlighted that such it would be important that instances of the new and old type can be referred to by the same legacy ID type (e.g. PDSCH-ConfigId). Otherwise, RAN2 would also have to change all other IEs that referred to the old type.

Proposed design principle: Accommodate for critical extensions of lower-level configuration IEs (e.g. PDSCH-Config, SearchSpace-Config, …) using CHOICE structure and for non-critical extensions by an extension marker (“…”). Decide on a case-by-case basis which extension mechanisms to apply.

Proposed design principle: When IEs are associated with an ID (e.g. in an AddMod list) their critical extensions should be addressed by the same ID type. This minimizes changes in other IEs which use ID type for referencing.

| Company Name | Comment on problem |
| --- | --- |
| MediaTek | We see this as a reasonable solution without obvious drawbacks and think the initial ASN.1 structure should be defined in a way (i.e., add necessary placeholders) that such critical extensions of low-level (RAN1) IEs would be possible, if later seen necessary. |
| Qualcomm | Similar view as MediaTek. This should be ok in principle to allow. However, making a decision case-by-case could be hard in each RAN2 meeting for each IE/parameter without a proper general guideline. |
| OPPO | We do not see there is a critical issue here that requires wider use of critical extension yet. It seems the main source of this proposal is the complication of AddMod list. If so, as long as the extension mark can be secured for IE in the first release of the list, there seems no major issue left to solve. |
| Toyota | We have no strong preference regarding the specific implementation of the critical extension mechanism, as many options exist. However, critical extensions should be employed to prevent the creation of overly complex structures like those currently in use, i.e. if we have a critical extension mechanism, it should be used when the signalling is at risk of getting very convoluted with non-critical extensions. |
| Apple | It's good to cluster the parameters introduced in different versions of the same configuration in one place. However, regarding the proposed CHOICE structure, we are not sure how to provide the full set of the configuration (parameters in multiple versions) with this choice structure. Let’s take the NR SearchSpace configuration to explain our understanding. If we extend the search space config by defining SearchSpace, SearchSpaceExt-r16, SearchSpaceExt-v1700, SearchSpaceExt-v1800 and all those 4 versions are added in one choice, network can only provide the parameters in one version at one time. In NR, the full set of the searchspace configuration may include the parameters in different versions. In NR RRC, network can provide the parameters in multiple versions in one configuration; but with the choice structure, it cannot support it. [Ericsson] Is your question how the CHOICE structure (proposed for handling “critical extensions”) would support the case where RAN2 introduces a “non-critical extension” in parallel lists instead? We agree that the proposed structure itself does not support the “parallel list” approach that RAN2 used to extend SearchSpace. It would rather favour an extension marker in the original SearchSpace IE. If we want to simplify the use of parallel lists (for non-critical extensions) we should look into those enhancements, too (see e.g. 4.4.1 proposed by QC). |
| ZTE | We understand this solution works, and we tend to agree that this should be discussed case by case. For smaller structure, non-critical extension should be sufficient. But for the below example in R2-2508614, we have few questions for clarification: 1. For a UE with Rel-23 version, whether the network can only use pdsch-r23 to configure the UE, or the UE is supposed to support both pdsch-r21 and pdsch-r23, and it is up to the network to decide which structure is used (depending on the enable functions); [Ericsson] As today, the network must comply with the UE capabilities. The UE capabilities would need to indicate (implicitly or explicitly) whether the UE supports the CHOICE alternative “pdsch-r23”. If the UE does not the network will not configure this flavour. This is logically the same as today where the network must not configure a field in a later extension (after “...”) or in a parallel list if the UE does not support those. Note however, that also a UE that supports only “pdsch-r21” will anyway be able to decode the CHOICE structure, i.e., it knows that it comprises of 3 regular alternatives (of which it may not know the name nor the type) and an extension marker. However, it may assume that the NW will not send any of those. In other words, this example UE may assume that the 2 CHOICE bits will always be set to “00” (pdsch-r21). For any other option the UE can conclude immediately that this configuration was faulty. 2. (may not directly relate to this solution) Why PDSCH-Id is defined? Does it relate to the flatten RRC structure design? The PDSCH-Id is not defined under PDSCH structure, so the ID value is based on its position in the list? [Ericsson] Right. The PDSCH-Id should be associated with the PDSCH, i.e., not just with one of the “sub-types”. Therefore, we propose not to include the “ID” into the sub-type (here PDSCH-r21, PDSCH-r23...) but rather do that association inside the AddMod list structure... which we we suggest to realize by a ParameterizedType, too. But this is not depicted here and a bit orthogonal to this proposal. |
| Samsung | We are fine to use the critical extensions more cases rather than the NR and this can be a good way to avoid the hierarchy ASN.1 structure. But as other companies said this could be used case-by-case. |
| LGE | We see that the solution can improve machine readability. However, we are not sure how frequently the issue — the critical extension of IEs associated with an ID (e.g., in an AddMod list) — actually occurs, or whether this approach is the optimal way to address it. |
| Huawei, HiSilicon | Critical extensions imply: - extra UE implementation work (need to support multiple branches) - extra work for non-critical extensions to multiple critical extension branches UMTS was using critical extensions, which was suffering heavily from the above issues, which is why it was excluded in LTE, and the CHOICE for critical extensions of messages was not used in 12 releases of LTE and in 5 releases of NR. |
| Xiaomi | It is ok to consider critical extension as well. But it should be done case by case. |
| Nokia | This kind of branching could make sense for some fundamental lower level elements that are often extended (e.g. PHY channel elements), but likely not for the highest level parent IEs. To us this is another kind of “modularity” where an IE can be versioned, which also means that any such cases with the CHOICE for critical extensions should be wrapped in a parameterized type that allows resetting, updating or releasing the entire module in one step. |

### 4.2.4	Modular design for delta signalling

R2-2508349 (Nokia) proposes modularity in the sense of splitting configuration to “pieces” (e.g. using parameterized type) that allows replace of existing config could help to reduce need of fullConfig over the entire configuration. This would allow “full config” for the “modules” without having to do it for the entire configuration.

In terms of how to use modules in ASN.1, we see the requirements for a module as the following: For a module, it shall be possible to

- setup, replace and release a module from a configuration

- encode or decode the contents of a module independently from the other configuration

- write the actions for each module operation (i.e. setup, replace, release) in the RRC procedural text

Based on this, modularity could be defined via the use OCTET STRINGs and the parameterized types: Define a parameterized type RRC_Module that allows setup, release and replace actions, and wrap the IE content within OCTET STRINGs. An example of this is shown below.

Proposed design principle: Accommodate parameterized type using OCTET STRING to create “configuration modules” that can be fully replaced by network without. This reduces the usage of fullConfig for configuration when a particular module needs to be refreshed.

| Company Name | Comment on problem |
| --- | --- |
| Nokia | Modularity in the sense of splitting configuration to “pieces” (e.g. using parameterized type) that allows replace of existing config could help to reduce need of fullConfig over the entire configuration. This would allow “full config” for the “modules” without having to do it for the entire configuration. |

### 4.2.x	…

| Company Name | Comment on problem |
| --- | --- |

## 4.3	Relation to between dedicated configuration and MIB/SIB

This section discusses solutions addressing primarily the problems identified in section 3.3, i.e., the following proposals:

Proposal 6: Avoid splitting the connected mode configuration into common- and dedicated branches.

Proposal 7: Discuss whether it is necessary that UEs (re-)acquire parameters from system information. If so, seek for means to specify/configure unambiguously which parameter the UE shall (re-)acquire from system information and which ones it shall take from the dedicated configuration.

### 4.3.1	No common/dedicated branches in dedicated signalling

R2-2508112 (MediaTek) and R2-2508614 (Ericsson) proposed that the 6G configuration structure should not be built around common- and dedicated branches. Based on the discussion in section 3.3 companies seemed to agree to the problem and to the corresponding design principle for 6G’s ASN.1 signalling structure:

Proposed design principle: 6G’s connected mode configuration messages shall not be split into common- and dedicated branches.

| Company Name | Comment on problem |
| --- | --- |
| MediaTek | This obviously means that the same IEs would not be used for common configuration parts (of a BWP or a physical channel) in SIB and in connected mode configuration messages, but instead the connected mode configuration messages should contain IEs used only there to carry the common+dedicated configuration parts truly in single field/IE. In practice, there would be: - SIB -> ... -> PDCCH-ConfigSIB, to carry common part for IDLE mode UEs, and - Connected mode configuration message -> ... -> PDCCH-Config, to carry common+dedicated parts for CONNECTED mode UEs, in a way that PDCCH-Config does not contain embedded PDCCH-ConfigSIB, but the fields for the common part are duplicated in both IEs. We see such extra maintenance burden easily acceptable for cleaner/simpler ASN.1 structure. |
| OPPO | We do not see there is a critical issue here that requires such mandatory requirement. As commented by other companies in Phase-1, the common branch is to carry the parameters that is to be present in both system-information and UE-dedicated configuration, and thus avoid duplication them in the UE-dedicated configuration (if we follow the proposed principle here). There seems no obvious benefit to have this duplication. |
| Apple | A UE in CONNECTED state always follows the configuration in UE dedicated RRC signaling, regardless of whether the configuration is common or dedicated. Given the current NR configuration structure which is built on the common/dedicated branches, the main issue is that many configurations are duplicated, so our most important task is probably to remove the duplicate configurations. |
| ZTE | We do not see the strong need to combine common- and dedicated structures, and we should not mandate the network to always resend the parameters that already obtained by the UE via MIB/SIB, so the “add-on” mechanism should be kept. We can have a principle that any parameters configured via RRC dedicated signalling should not be override due to the reception of MIB/SIB. In addition, if the common- branch is kept, we can consider to remove the restriction that update of common- configuration must require reconfigurationWithSync. |
| InterDigital | We think a common branch to carry IEs that can be configured in both SIB and dedicated configuration saves on redundancy and does not necessarily create a major issue in RRC that requires action. |
| CATT | We also don’t consider this enhancement is necessary, but it can be brought up/checked again when the content of configuration for both connected and idle states are clear. |
| Samsung | Once the UE receives information through a dedicated message, it should ideally skip reading or modifying it based on the common information as defined in SIB. Additionally, there is a need to discuss whether such common information for connected mode UEs is truly necessary. |
| Ericsson | We agree with MediaTek and Apple that a UE in RRC Connected follows the dedicated configuration. The separation into so-called common and dedicated branches complicated the overall structure of the RRCSetup/Reconfiguration significantly without offering any benefit. Except for a tiny part of the configuration (initial BWP and carrier config of the PCell) that the NR UE inherits from SIB1 all other common branches are provided only via dedicated signaling. Hence, we don’t agree with OPPO, ZTE and InterDigital that the structural split into common and dedicated branches saved signaling overhead which would justify this complexity and ambiguity. |
| LGE | Our view is no structural change is needed for this issue. The current common/dedicated branch concept already provides a clear separation between parameters shared across UEs and those specific to individual UEs, avoiding unnecessary duplication. Introducing a new structure without strong justification could increase signaling and maintenance complexity without tangible benefits. We believe the existing approach should be maintained, focusing instead on simplifying parameter management within the current framework. |
| Huawei, HiSilicon | In NR, RAN1 liked to distinguish "common parameters" (i.e., that should be the same for all connected UEs in once cell) and "dedicated parameters" (i.e., that may be different), and sometimes separate structures were introduced in RRC, even for parameters not in SIB. From RRC perspective, this is useless. The only usefulness of "common parameters" in RRC is for parameters in SIB that the connected UE is supposed to also use, and update in case of SIB change. |
| Xiaomi | To our understanding, Common part Only carries the parameters used for IDLE UE, or both IDLE and CONNECTED mode UE. The parameters used for CONNECTED only mode UE, shall not be split into common- and dedicated branches. |
| Nokia | Agree with the principle – this has complicated both LTE and NR RRC unnecessarily. Note that this doesn’t mean we cannot split IEs into multiple parts, some of which are used in e.g. SIB and some in dedicated signalling, but we should aim to define usage of e.g. L1 parameters only once in the specification and no duplicate procedural text in multiple places. It should be possible in the first configuration in a cell, to reuse the SIB content in the initial configuration of the UE, and consider the “common SIB” configuration as the “dedicated” configuration for the specific UE. UE does not need to know that this configuration used by other UE. The initial configuration could use the SIB content in modular way. |

### 4.3.2	Independent by default

In addition to what is captured in 4.3.1, R2-2508614 (Ericsson) proposed 6G’s dedicated configuration should be self-contained, rather than an add-on to the common configuration (MIB/SIB) as in NR. Consequently, the network should provide the complete UE configuration by dedicated signalling by default. However, the network may configure the UE explicitly to acquire selected parameters from system information and to re-acquire them if system information changes. Ericsson outlined that this would be necessary for parameters that may change and that all UEs must apply (almost) simultaneously. The benefit compared to NR would be that it becomes unambiguous whether/where the UE shall use a dedicated or a broadcast parameter.

Proposed design principle: 6G’s dedicated configuration message should be self-contained (not a delta to parameters that the UE acquired from MIB/SIB). In the dedicated configuration the NW may explicitly instruct the UE to (re-)acquire specific parameters from system information. (RAN2 and other WGs should discuss for which parameters this is possible)

| Company Name | Comment on problem |
| --- | --- |
| MediaTek | We agree with this proposal for the part "network should provide the complete UE configuration by dedicated signalling", because one source for interoperability issues in 5G is usage of SIB1->servingCellConfigCommon in RRC_CONNECTED, especially after handover where the UE receives also spCellConfigCommon in RRCReconfiguration. Also as explained by Ericsson, the RRCSetup adding configuration on top of SIB1 instead of being self-contained is unnecessary complex approach and introduces artificial limitations - the only benefit seems to be slightly smaller RRCSetup. For the UE acquiring selective parameters from SIB1 in RRC_CONNECTED, we are more conservative, as it may end up as a feature mandatory for the UE to implement, but difficult to IODT in practice (as networks may not deploy it). Also it seems more an optimization for a corner case rather than essential part of a functionality. As a summary, we'd prefer not to use SIB1 at all for UE configuration in RRC_CONNECTED. |
| Qualcomm | Similar view as MediaTek. |
| OPPO | We're still trying to understand the distinction between "self-contained" and "add-on" parameter delivery mechanisms. Based on our current understanding, we can identify two key scenarios: Q1: System Information-Exclusive Parameters Could certain parameters be delivered exclusively via system information without dedicated configuration? We believe this is possible - for instance, parameters that are only relevant in RRC_IDLE state or vertical-specific information might naturally be delivered solely through system information. For this case, any changes have to rely on system information updates, requiring mechanisms like 5G's short message to notify UEs of system information modifications. [Ericsson] Yes, we agree there may be cases where CONNECTED UEs need to acquire SIBs for other purposes (UTC time, PWS/ETWS, ...). But in the scope of this proposal we only intended to discuss SIB parameters that could determine parameters of the Uu connection, i.e., SIB1-> servingCellConfigCommonSIB. Should the UE care about those while being RRC CONNECTED? If so, how does the UE determine which of those parameters to apply in CONNECTED and whether to acquire updates of those parameters directly from SIB1? Q2: Dual-Delivery Parameters Could some parameters be delivered through both system information and dedicated configuration? Certainly. For parameters that are essential for initial access but also required during subsequent RRC_CONNECTED procedures, both delivery mechanisms should be capable of providing them. For this case, when a UE is in RRC_CONNECTED state, it would be most straightforward to consistently use dedicated signaling for parameter configuration. In such cases, the dedicated configuration should override any corresponding parameters received via system information. In summary, we currently see no compelling need for the network to explicitly instruct UEs in dedicated configuration to (re-)acquire specific parameters from system information. The described approach appears sufficient for handling parameter updates and configuration. |
| Apple | Dedicated signaling should be sufficient to provide the configuration used by UEs in CONNECTED state, and CONNECTED UEs should avoid acquiring the parameters used in CONNECTED mode from SIB by itself. According to this proposal, the UE has to combine parameters from the common config and the dedicated configuration, which increases the UE and contradicts the direction in Section 4.4.1. |
| ZTE | We understand the motivation is to avoid sending RRC signalling to all the UEs when the network wants to update the common configuration. However, the problem is that how to ensure all the RRC_CONNECTED UEs receives updated SIB1 at the same time. (although RAN2 defines SI update boundary, the actual reception can be delayed, e.g. due to gap), so, there could be mismatch between the network and the UE sometimes. We tend to agree with others that once the parameter is sent via RRC dedicated signalling, there is no need to obtain it via MIB/SIB1 during RRC_CONNECTED state. |
| InterDigital | We agree with OPPO that there may be cases where a UE in CONNECTED needs to use parameters provided by SIB, so it is not straightforward to exclude this and rely entirely on dedicated RRC configuration. |
| Samsung | We are aligned with Mediatek and QC view, if dedicated configuration is going to be self-contained then there is no need for the UE to decode common-config and it can be eliminated completely. Having UE decode both the versions and later network control which version to apply is not efficient for the UE. The selective parameters solution from SIB1 may create some ambiguity or inconsistencies |
| Ericsson | An NR UE is required to maintain an up to date copy of SIB1-> servingCellConfigCommonSIB and to “combine” those parameters with those that it obtained in the dedicated part of the RRCSetup/Reconfiguration. Unfortunately, the NR spec does not say clearly how to “combine”, i.e., which parameter from SIB1-> servingCellConfigCommonSIB may be overridden by which parameter in the RRCReconfiguration. In this proposal we assumed that RRC Connected 6G UEs will also be required to acquire/update at least some parameters from SIB1... and for that case suggest to make it less ambiguous than in NR. MediaTek, QC, OPPO and Apple, ZTE, InterDigital and Samsung seem to assume that this is not necessary in 6G at all. We agree that this would be the easiest. However, there are parameters in NR that networks change on-the-fly via SIB1 update and that CONNECTED UEs are required to consider. One example is the ss-PBCH-BlockPower, which, if changed by the NW, must be known to all UEs pretty much at the same time. Therefore, we think it is too early to conclude that CONNECTED UEs don’t need to update any Uu parameters based on SIB1. However, we do agree that it would be good to assume as a default that the network provides all parameters by dedicated signalling. If (and only if) 3GPP identifies parameters that require updates, 3GPP should specify unambiguous means to do that. |
| LGE | The cell common information would not be frequently updated, so the potential benefit of using the common channel for such updates is limited. Considering that this discussion was initiated to reduce implementation complexity rather than to address a logical issue in the NR RRC signalling, we do not see value in introducing a complex solution without clear advantages. |
| Huawei, HiSilicon | If the network cannot trigger connected UEs to update parameters when SIB1 is updated, this may have tremendous signalling impact in certain deployment scenarios. We do not see the need to selectively indicate the fields to update, the fields to update can be fixed in the specification. |
| Xiaomi | Similar view as MediaTek. For CONNECTED UEs, it is easy to obtain all necessary configuration via dedicated signalling. To obtain configuration, part in system information and part in dedicated signalling will increase the complexity and latency, especially if OD-SIB is introduced. |
| Nokia | We agree with the principle combined with avoiding the common/dedicated-branching of IEs: It should be possible for network to unambiguously simply override any SIB configuration via dedicated signalling |

### 4.3.x	…

## 4.4	Other aspects of the ASN.1 structure

This section discusses solutions addressing e.g. the problems identified in section 3.5, i.e., the following proposals:

Proposal 9: Investigate how to use ID-based linking of configuration components while avoiding unfavourable signalling overhead and lack of readability.

### 4.4.1	ID-based linking to improve parallel lists signalling

R2-2508758 (Qualcomm) described in section 2.3, one example of ID-based linking of elements between original list and corresponding extension list(s), instead of always using the so-called “parallel list” where the same number of elements must be included in the same order as original list. For very long original lists with extensions based on parallel lists, there can be a lot of redundancy and encoding overhead even though only ‘empty’ elements are indicated. ID-based linking can be particularly efficient when the original list is very long but only a few elements need to be extended/signalled.

Parameterized Type Macros can be used to achieve this in ASN.1. For example, to enable specific elements in a list to be extended by “Parallel Lists”, by including the list of indices and corresponding extension-elements (much smaller) list, one could define a Parameterized Type Macro called e.g. ParallelList {} (similar to SetupRelease{}) as shown below:

-- TAG-PARALLELLIST-START

ParallelList { INTEGER:maxSize, ElementName } ::= SEQUENCE {

included_indices SEQUENCE (SIZE (1..maxSize)) OF  INTEGER (0..maxSize)  OPTIONAL,

extension_List   SEQUENCE (SIZE (1..maxSize)) OF  ElementName

}

-- TAG-PARALLELLIST-STOP

Then, the following example ASN.1 code from 5G RRC:

BandCombinationList ::=        SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

<<skip>>

BandCombinationList-v1610 ::=  SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1610

BandCombinationList-v1630 ::=  SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1630

BandCombinationList-v1640 ::=  SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination-v1640

would be written as below instead, which would enable including only a limited number of elements if the transmitter chooses to do so.

BandCombinationList ::=             SEQUENCE (SIZE (1..maxBandComb)) OF BandCombination

<<skip>>

BandCombinationList-v1610 ::=       ParallelList {maxBandComb, BandCombination-v1610}

BandCombinationList-v1630 ::=       ParallelList {maxBandComb, BandCombination-v1630}

BandCombinationList-v1640 ::=       ParallelList {maxBandComb, BandCombination-v1640}

On the other hand, the transmitter may also continue to signal the same number of elements as in legacy way and not include included_indices if the transmitter so decides based on the overhead of number of indices to be signalled vs including empty elements to signal in legacy way. I.e., it would be up to the transmitter to chose between the same sized parallel lists (without ID-based linking, as in legacy) or variable sized parallel list (with ID-based linking) in this example.

Proposed design principle: Aim to reduce signalling overhead and redundancy in signalling by using ID-based linking of extension elements in extension lists.

| Company Name | Comment on problem |
| --- | --- |
| Qualcomm | As proponent, we think ID-based linking can be useful to reduce signaling overhead. We give the example for parallel lists above. Specifying it can be facilitated by defining Parameterized Type Macros. Transmitter can decide between which way to signal based on ID-overhead vs empty-elements overhead. |
| MediaTek | We are open to explore this solution idea further and see the benefit. However, we would like to point out that we should also remember simplicity (ideally, single mechanism), so we tentatively think the following part may be unnecessary flexible and could be considered further, if this solution idea will be adopted: "On the other hand, the transmitter may also continue to signal the same number of elements as in legacy way and not include included_indices if the transmitter so decides based on the overhead of number of indices to be signalled vs including empty elements to signal in legacy way. I.e., it would be up to the transmitter to chose between the same sized parallel lists (without ID-based linking, as in legacy) or variable sized parallel list (with ID-based linking) in this example." |
| Toyota | We believe this proposal is worthwhile further consideration, as it significantly improves readability. |
| OPPO | On the other hand, as pointed out by companies in clause 3.5, and also during the offline for capability, “ID” itself is a main source of overhead, e.g., ID for band, ID for FSC and etc. If we take the example above as a reference case, maxBandComb of 65536 would lead to 2-byte consumption for each entry for one extension. That should be taken into account when we explore the ID-related method for UE capability, where signaling overhead is of more concern. |
| Apple | We are open to discuss this direction and see the benefit. However, we do not want the introduction of too many IDs to cause additional signaling burden. |
| ZTE | The solution is only applicable when the IE content needs to be extended. It does not apply to the case where the list length needs to be extended. For ID-based linking, we have the same concern as OPPO, if the ID itself consumes more bits, then signalling it twice results in additional signalling overhead. For the IE structure that most likely to be extended in future, it is better to put non-critical extension mark “…” inside the structure. However, if there is no extension mark “…” and/or if the list length needs to be extended (e.g. 8 -> 16), it is straightforward to define a new list to replace the previous one. On the other hand, we might need different solutions for UE capability and RRC configuration. |
| InterDigital | We think this proposal reduces specification complexity and can be further studied. |
| Samsung | We are open to discuss this method because the parallel list signaling requires a lot of signaling overhead even if many of them are not really needed. But, this details will be discussed later some high level ASN.1 structure is determined. |
| Ericsson | We are open to explore such concepts. Besides potential benefits in signalling overhead we see benefits in readability and consistency if we express this in ParameterizedTypes. If we find ways that simplify also practical product implementation that would be great. For very long lists (like in the specific case of BandCombinations) we agree with OPPO and ZTE that it is important to take the ID overhead into account. However, we disagree with ZTE that for such cases (long lists with many small IEs) an extension marker (“...”) is a good alternative since that could easily consume even more bits that an ID. |
| LGE | We see that proposed ParallelList reduces the encoding overhead as well as reducing the signalling overhead in some case, in which the number of bits for ID is smaller than number of bits for whole number of parameters included in extended IE. However, in case of the above example, proposed ParallelList rather increases the signalling overhead of band combination due to the size of band combination ID. |
| Huawei, HiSilicon | In many cases the overhead of this proposal will not be smaller (even be larger) than that of parallel lists. One possible way to reduce the overhead of parallel lists is for the network to place the entries to be updated in the parallel lists at the beginning of the original list, and specification to allow a different number of entries in the parallel lists compared to the original list. For example: • Original List contains 10 entries and the first 4 entries are extended in the parallel list; • Parallel list only contains the first 4 entries and no any indications/empty elements for the last 6 entries. Moreover, regarding the overhead of each empty element mentioned in Qualcomm's Tdoc (Each empty element in BandCombination-v1610 occupies 7 bits (to indicate ‘absence’ of all 7 subfields in BandCombindationList-v1610)), we think it is easy and reasonable to have a high-level IE to include all the IEs in a parallel list. In this way, for each empty elements, only one OPTIONAL bit is needed instead of 7 bits for each child IEs. |
| Xiaomi | We are open on this. But as mentioned by other companies, the gain depends on the size of the parameters and the required size of ID. We can discuss this once the structure is clear. |
| Nokia | This seems like it could improve readability if we continue to use parallel lists, and likely reduces typical errors. At least we should explore this further by considering the practical overhead e.g. for UE capabilities (where we use a LOT of parallel lists). One question: Perhaps we should also consider if we can make a parameterized type for the parent list in the same way – that might make the use of extensions clearer? The downside is that this may not always save bits as Huawei states. |

### 4.4.x	…

Proposed design principle:

| Company Name | Comment on problem |
| --- | --- |

# 5	Conclusion

<TO BE ADDED WHEN THE EMAIL-DISCUSSION ENDS>

# References

Tdoc Number, Title, Source, Meeting, Date

Spec number, Title, Source, Version, Date