3GPP TSG-SA2 Meeting #143-E	S2-200xxxx

- Elbonia, 24th Feb 2021 – 9th Mar 2021											(Revision of S2-200xxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | - | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service Assistance Information for 3GPP Advanced Interactive Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Huawei(?), CATT, OPPO, Sony(?), Apple(?) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_AIS |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the discussion paper S2-200xxxx, the Service Assistance Information for both AIS and TSC includes the support parameters to allow gNB to more efficiently schedule periodic traffic flows either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants. The TSCAI is defined only for the TSN service and is documented in clause 5.27.2 that is part of clause 5.27 of TSN. So the TSCAI table in the clause 5.27.2 can be merged into this general Service Assistance information for both AIS and TSC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding Service Assistance Information for both AIS and TSC. And provide the functional description on the Service Assistance Information. Merge the TSCAI table into the Service Assistance Information table. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The AIS service can not be supported in 5G system. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x(new), 5.27.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

- 5. x	Service Assistance Information

Service Assistance Information describes service traffic characteristics for use in the 5G System. The knowledge of service traffic pattern is useful for the gNB to allow it to more efficiently schedule periodic, deterministic traffic flows traffic flows either via Configured Grants, Semi-Persistent Scheduling or with dynamic grants. Service Assistance Information, as defined in Table 5.x-1, is provided from SMF to 5G-AN, e.g. upon QoS Flow establishment. The Service Assistance Information parameters are set according to corresponding parameters obtained from the AF. The PCF provides Service Assistance Information for Ethernet and IP PDU Session types to the SMF. The SMF derives Service Assistance Information on a per QoS Flow basis and sends it to 5G-AN, when available. The Service Assistance Information can be used for the media stream (e.g. voice and video) and TSC stream as defined in clause 5.27.

Table 5.x-1: Service Assistance Information

| Assistance Information | Description | Service Type |  |
| --- | --- | --- | --- |
|  |  | TSCAI for TSC | Advanced Interactive Service |
| Flow Direction | The direction of the traffic flow (uplink or downlink). | Required | Required |
| Periodicity | It refers to the time period between start of two bursts. | Required | Required |
| Burst Arrival time | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or egress interface of the UE (uplink flow direction). | Required | N/A |

Second CHANGE

- 5.27.2	TSC Assistance Information (TSCAI)

NOTE 1:	This clause assumes that PSFP information as defined in IEEE Std 802.1Q [98] and Table 5.28.3.1-1is provided by CNC. PSFP information may be provided by CNC if TSN AF has declared PSFP support to CNC. TSN AF indicates the support for PSFP to CNC only if all the DS-TT and NW-TT ports of the 5GS Bridge have indicated support of PSFP. Means to derive TSCAI if PSFP is not supported by 5GS and/or the CNC are beyond the scope of this specification.

TSC assistance information describes TSC traffic characteristics for use in the 5G SystemTSC assistance information, as defined in Table 5.x-1, is provided from SMF to 5G-AN, e.g. upon QoS Flow establishment. The TSCAI parameters are set according to corresponding parameters obtained from the TSN AF, when available. The TSN AF may be able to identify the ingress port and thereby the PDU session as described in clause 5.28.2.

The TSN AF interfaces towards the CNC for the PSFP (IEEE Std 802.1Q [98]) managed objects that correspond to the PSFP functionality implemented by the DS-TT and the NW-TT. Thus, when PSFP information is provided by the CNC, the TSN AF may extract relevant parameters from the PSFP configuration. The TSN AF calculates traffic pattern parameters (such as burst arrival time with reference to the ingress port and periodicity). TSN AF also obtains the flow direction as specified in clause 5.28.2. TSN AF is responsible for forwarding these parameters in TSC Assistance Container to the SMF (via PCF). TSN AF may enable aggregation of TSN streams if the TSN streams belong to the same traffic class, terminate in the same egress port and have the same periodicity and compatible Burst arrival time. One set of parameters and one container are calculated by the TSN AF for multiple TSN streams to enable aggregation of TSN streams to the same QoS Flow.

Annex I describe how the traffic pattern information is determined.

NOTE 2:	Further details of aggregation of TSN streams (including determination of burst arrival times that are compatible so that TSN streams can be aggregated) are left for implementation.

In this case, TSN AF creates one TSC Assistance Container for the aggregated TSN streams. The SMF will bind PCC rules with a TSC Assistance Container as described in clause 6.1.3.2.4 of TS 23.503 [45]. The SMF derives TSCAI on a per QoS Flow basis and sends it to 5G-AN. The Burst Arrival Time and Periodicity component of the TSCAI that the SMF signals to the 5G-AN are specified with respect to the 5G clock. The SMF is responsible for mapping the Burst Arrival Time and Periodicity from a TSN clock to the 5G clock based on the time offset and cumulative rateRatio between TSN time and 5GS time as measured and reported by the UPF.

The TSCAI parameter determination in SMF is done as follows:

-	For traffic in downlink direction, the SMF corrects the Burst Arrival Time in the TSN Assistance Container based on the latest received time offset measurement from the UPF and sets the TSCAI Burst Arrival Time as the sum of the corrected value and CN PDB as described in clause 5.7.3.4, representing the latest possible time when the first packet of the data burst arrives at the AN.

-	For traffic in uplink direction, the SMF corrects the Burst Arrival Time in the TSN Assistance Container based on the latest received time offset measurement from the UPF and sets the TSCAI Burst Arrival Time as the sum of the corrected value and UE-DS-TT Residence Time, representing the latest possible time when the first packet of the data burst arrives at the egress of the UE.

-	The SMF corrects the Periodicity in the TSN Assistance Container by the previously received cumulative rateRatio from the UPF and sets the TSCAI Periodicity as the corrected value.

-	The SMF sets the TSCAI Flow Direction as the Flow Direction in the TSN Assistance Container.

NOTE 3:	In order for the TSN AF to get Burst Arrival Time, Periodicity on a per TSN stream basis, support for IEEE Std 802.1Q [98] (as stated in clause 4.4.8.2) Per-Stream Filtering and Policing (PSFP) with stream gate operation is a prerequisite.

In the case of drift between TSN GM clock and 5G clock, the UPF updates the offset to SMF using the N4 Report Procedure as defined in TS 23.502 [3] clause 4.4.3.4. In the case of change of cumulative rateRatio between TSN time and 5G time, the UPF updates the cumulative rateRatio to SMF using the N4 Report Procedure as defined in TS 23.502 [3] clause 4.4.3.4. The SMF may then trigger a PDU Session Modification as defined in TS 23.502 [3] clause 4.3.3 in order to update the TSCAI to the NG-RAN without requiring AN or N1 specific signalling exchange with the UE.

NOTE 4:	In order to prevent frequent updates from the UPF, the UPF sends the offset or the cumulative rateRatio only when the difference between the current measurement and the previously reported measurement is larger than a threshold as described in TS 23.502 [3] clause 4.4.3.4.

The SMF may update the TSCAI to the NG-RAN as part of handover procedure as defined in TS 23.502 [3] clauses 4.9.1.2.2 and 4.9.1.3.2.

END OF CHANGES