SA WG2 Meeting #143e	S2-210

# Feb 24th – March 9th, 2021 ; Elbonia                   		 	 				(revision of S2-210)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR |  | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MA PDU sessions with connectivity over E-UTRAN/EPC and non-3GPP access to 5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ATSSS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Objectives of ATSSS_Ph2 WID in S P-200977 b) Support for UEs to establish MA PDU Sessions with a 3GPP access leg over EPC and a non-3GPP access leg over 5GC, according to the conclusions in TR 23.700-93, clause 8.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The text that was text currently in R16 23.316 § 4.12.3 and was describing the feature to apply only to 5G RG is moved to 23.501 § 5.32 and adapted to: - apply to any UE and over and non-3GPP access - support Ethernet PDU Session type (for non 5G RG) as documented in TR 23.700-93, clause 8.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No Support for UEs to establish MA PDU Sessions with a 3GPP access leg over EPC and a non-3GPP access leg over 5GC |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501 CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev0 of this CR has been built assuming that the description of a MA PDU Session with multi-access connectivity over E-UTRAN/EPC and W-5GAN can refer to the generic support of the MA PDU session with multi-access connectivity over E-UTRAN/EPC and non-3GPP access over 5GC described now on 23.501 |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 4.12.3	Hybrid Access with multi-access connectivity over E-UTRAN/EPC and W-5GAN

#### 4.12.3.1	General

This clause applies to the case where multi-access connectivity via both EPC and 5GC is supported in the 5G-RG and network. In this case, multi-access connectivity using ATSSS via both EPC and 5GC may be provided as described in this clause.

.

For a 5G-RG, a Multi-Access PDU Session may be extended with user-plane resources via an associated PDN Connection on 3GPP access in EPC. This enables a scenario where a MA PDU Session can simultaneously be associated with user-plane resources on 3GPP access network connected to EPC and W-5GAN connected to 5GC. Such a PDN Connection in EPS would thus be associated with multi-access capability in 5G-RG and PGW-C+SMF.

The feature is supported as defined in 23.501, clause 5.32 with following difference:

-	UE is replaced by 5G-RG.

-	5G-RG is connected to 5GC via a non-3GPP access corresponding to W-5GAN access.

-	MA PDU Sessions of Ethernet PDU Session type where the 3GPP access corresponds to E-UTRAN/EPC are not applicable for 5G-RG.

#### 4.12.3.2	Void

#### 4.12.3.3	Void

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES