SA WG2 Meeting #128bis 	S2-18xxxxx

20 – 24 August 2018, Sophia Antipolis, France	 (revision of S2-18xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	Evaluation of solutions for Key Issue #1

Document for:	Approval

Agenda Item:	6.8

Work Item / Release:	FS_ATSSS

Abstract of the contribution: This paper discusses the pros and cons of solutions addressing Key issue#1 (Overall Architecture for traffic Steering/switching/splitting between Multiple Accesses) based on the criteria described in S2-18xxxxx.

# Proposal

It is proposed to update TS 23.793 as follows.

FIRST CHANGE

# 7	Evaluations and conclusions

## 7.1	Evaluations

### 7.1.1	Evaluations for Key Issue #1

#### 7.1.1.1	Criteria for the evaluation of solutions wrt Key Issue #1

#### 7.1.1.2	Evaluation of the solutions wrt Key Issue #1

Table 7.1.1.2-1: comparison of the solutions using Key Issue #1 criteria

| Criteria/Solution | Solution 1: Proposed architecture framework for ATSSS | Solution 3: TFCP (Traffic Flow Control Protocol) based architecture framework for ATSSS |  |  | Solution 6: Architecture framework with ATSSS rules via NAS and Generic User Plane Reporting Control Protocol (updated per S2-18xxxx) | Solution 7: Traffic Steering using Access Network Performance Measurements | Consolidated ATSSS Solution (per S2-186439) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UL/DL independence | yes | yes |  |  | yes | yes | Yes |
| ATSSS rules wrt USRP | ATSSS rules are extensions of URSP rules. | ATSSS rules are separate from URSP rules. |  |  | ATSSS rules are separate from URSP rules. | Not described | (to be confirmed) ATSSS rules are separate from URSP rules. |
| ATSSS rules: Convergence Methods (multiple methods, single method) | Flexible: any L4 or low layer method. Extendable Null tunneling, GRE tunneling, GMA, L4 Multipath Solutions (MP-TCP, MP-QUIC, SCTP, UDP generic) Existing reordering mechanism in MPTCP and MPQUIC | Only a single method Method based on new TFCP layer. This method is completely new. Not capable of using MPTCP, MPQUIC. Reordering mechanism can reuse the well-known algorithms implemented in computer, e.g. Bubble Sort, Selection Sort, Heap Sort, etc. |  |  | Flexible: any L4 or lower layer method. Extendable Null tunneling, GRE tunneling, GMA, L4 Multipath Solutions (MP-TCP, MP-QUIC, SCTP, UDP generic) Existing reordering mechanism in MPTCP and MPQUIC | Only a single method Convergence method is not described. Not capable of using MPTCP, MPQUIC. | Only a single L4 method (MPTCP), then one lower layer method. Not extendable. MPTCP method, then a brand new single lower layer method for traffic switching. |
| Additional layer between the UE and UPF | Three or two layers for the existing protocols (e.g. GRE/UDP/IP, MP-TCP/TCP/IP, MP-QUIC/UDP/IP, SCTP/IP, UDP/IP generic)), or one layer for GMA, according to the Convergence Methods. | One layer for TFCP |  |  | Three or two layers for the existing protocols (e.g. GRE/UDP/IP, MP-TCP/TCP/IP, MP-QUIC/UDP/IP, SCTP/IP, UDP/IP generic)), or one layer for GMA, according to the Convergence Methods. | No new layer | No new layer |
| ATSSS rules: Rules/criteria for Traffic Steering, Switching | Hot-standby, top up (least cost access), least loaded, best performance | Smallest Delay, Load-Balancing, Active-Standby, Redundancy transmission. |  |  | ATSSS rules separated from URSP rules Hot-standby, priority based (overflow on non-preferred access), lowest RTT, least loaded | Active-Standby (Routing traffic via access X based on application/traffic type and:) Cannot support other steering modes (e.g. Smallest Delay, Load-Balancing) without the reordering mechanism. | Active-Standby, Without the MPTCP function, cannot support other steering modes (e.g. Smallest Delay, Load-Balancing), as missing of the reordering mechanism. |
| ATSSS rules: Rules for Traffic Splitting (distribution) | Load balancing (weight factor) | Load balancing (routing factor) |  |  | Load balancing (weight factor) | Load balancing | MPTCP related only |
| ATSSS rules for UE measurements | list of measurements, decision thresholds, variable periodicity | list of measurements, decision thresholds, variable periodicity |  |  | list of measurements, decision thresholds, variable periodicity | list of measurements, fixed periodicity | list of measurements (called Access Performance Measurements), fixed periodicity |
| Way for providing ATSSS rules | Via NAS for steering rules (load balance, Hot-standby, top up (least cost access), least loaded, best performance) Via user plane for convergence methods: new protocol Multi Access Control Management (MACM) (See Note 6) | via NAS, using PDU session Establishment / modification. ATSSS rules are part of QoS rules or separated parameters (See issues in Note 1) |  |  | via NAS, using PDU session Establishment / modification. | Via NAS at PDU session establishment | Via NAS at PDU session establishment |
| Way to carry measurement reports. Network entity responsible for processing measurement reports (determining the path to send packets). | New MACM protocol: User plane transport UPF | New TFCP protocol: User plane transport, and then to SMF if necessary via N4, SMF or UPF: if it is the SMF, UPF or UE sends measurement reports to SMF. ATSSS rules will be modified based on reports (see Note 2) |  |  | New GRCP protocol: User plane transport UPF | New protocol: User plane transport Performance measurement Function (PMF): standalone or collocated with UPF Standalone PMF: do we need it? benefits questionable: - PMF selection requires to further specify DNS query / NRF discovery service - Measurements PMF->PCF->SMF->UPF: long delay + huge congestion impacts. What added value via PCF (not described)? | New protocol: User plane transport Performance measurement Function (PMF): collocated with UPF |
| UE implementation independent thresholds/measurement reports | Yes For switching methods: - RTT via ping - Standardized Radio thresholds (RSRP, RSRQ, BSS Load, RSSI) For L4 splitting methods: - part of MPTCP/MPQUIC protocols | Yes RTT: via new TFCP Echo Req/Resp Loss ratio and jitter Optional Radio signal strength, available BW thresholds as an option (see issues in Note 3) |  |  | Yes For switching methods: - RTT per IETF RFC 1323 - Standardized Radio thresholds (RSRP, RSRQ, BSS Load, RSSI) For L4 splitting methods: - part of MPTCP/MPQUIC protocols | Yes Delay Throughput Loss rate (Issues in Note 5) Reference to iPerf tool (see https://iperf.fr/) | Yes Delay Throughput Loss rate (Issues in Note 5) |
| Measurements for anticipating bad radio environments | Yes Standardized Radio thresholds (RSRP, RSRQ, RSSI) | Yes Radio signal strength |  |  | Yes Standardized Radio thresholds (RSRP, RSRQ, RSSI) | No | No |
| Measurements for traffic distribution (i.e. for traffic splitting when both paths are available) | Yes Downlink backhaul data rate and the WLAN channel utilization | If for GBR, routing factor can be decided based on guaranteed BW on each access. |  |  | Yes Downlink backhaul data rate and the WLAN channel utilization | Yes Throughput is measured using injection of additional traffic (See issues in Note 5) | Part of MPTCP protocol Throughput is measured using injection of additional traffic (See issues in Note 5) |
| Measurements for delay sensitive traffic | RTT: via new MACM ping message | RTT: via new TFCP Echo Req/Resp message |  |  | RTT per IETF RFC 1323 or Echo Req/Resp | Yes, but not described | Echo Req/Resp |
| Traffic type, Granularity | TCP/IP, UDP/IP, Ethernet packets Depending on the convergence method: per IP flow, per SDF, per packet | TCP/IP, UDP/IP, Ethernet packets. Depending on the ATSSS rules. |  |  | TCP/IP, UDP/IP, Ethernet packets Depending on the convergence method: per IP flow, per SDF, per packet | Not described | Not described |
| Dependency on Operating System and Chipset | Link performance measurement and report based on MACM. Operation System enhanced to support the additional tunnel establishment based on the different protocols according to convergence method. | Link performance measurement and report based on TFCP. Chipset implementation for ATSSS function (i.e. traffic distribution function based on ATSSS rule and additional tunnel function based on TFCP). |  |  | Link performance measurement and report based on GRCP. Operation System enhanced to support the additional tunnel establishment based on the different protocols according to convergence method. | Link performance measurement based on new protocol and Performance Measurement Function. Chipset implementation for ATSSS function (i.e. traffic distribution function based on ATSSS rule). | Link performance measurement based on new protocol and Performance Measurement Function. Chipset implementation for ATSSS function (i.e. traffic distribution function based on ATSSS rule). Operation System for MPTCP function. |

NOTE 1:	Including ATSSS rules in QoS rules may not be practical as a QoS rule may use a TFT that does not correspond to the TFT required for ATSSS rule (e.g. all http/https traffic may use QoS rule A, but what is needed is to distinguish between YouTube with ATSSS rule X, Gmaps with ATSSS rule Y and other http/https traffic with ATSSS rule Z).

NOTE 2:	The UPF is responsible for processing measurement reports when the smallest delay steering mode is applied for the traffic. While the SMF is applied for the case of access specific performance measurement. When the SMF received the access specific performance measurement report, the SMF triggers the traffic switching procedure for the Active-Standby steering mode, similar with the Network-initiated switching procedure as defined in 4G NBIFOM.

NOTE 3:	How could access specific measurements such as radio strength could be optional only? It would be impossible to measure loss rate quick enough: the measurement report will not be available at the time path switch decision is needed. 
Moreover, how could available BW could be measured is not clear.

NOTE 4:	Loss ratio computation takes significant time to measure. For example, BER of 10-6 with 10 samples requires 10 x 106 bits, i.e. 1 second for a traffic of 10 Mbit/s. The measurement report will not be available at the time path switch decision is needed (this is not the case of radio strength measurements RSRP, RSRQ, etc that are available immediately).

NOTE 5:	How delay, throughput, loss rate are computed is not appropriate. In particular:
- Throughput: injecting additional traffic to measure the throughput is impacting the throughput itself. At least it impacts the throughput of the other UEs, which is not acceptable. Furthermore, it is not sufficient to make measurements only when there is no traffic as radio environment varies all the time. Eventually, throughput measurement should be UE vendor independent, which cannot be verified w/o description.
- Loss rate: as for NOTE 4, it is impossible to measure loss rate quick enough: the measurement report will not be available at the time path switch decision is needed.

NOTE 6:	The need for changing convergence method during the life of the PDU session is not clear:   MPTCP already has the ability to reject UE requests for a MPTCP context. This can be added in a further step if needed in the future.

END OF CHANGES