3GPP TSG-SA WG2 Meeting #13x	S2-20XXXXX

Source:	Apple

Title:	Additions of top of NSQ based quota management: Solution for Key Issue #2: Knowledge of slice quota availability for improved UE signalling

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes a solution of how to minimize PDU session establishment rejects by making the UE aware whether certain slices are subject to quota management.

# 1. Introduction

This solution builds upon Solution#10. To reduce PDU session establishment rejects due to quota limits reached, this solution proposes that UE should be made aware and have the knowledge on which slices are subject to quota management, and what is the current quota availability.

START of CHANGE

## 6.X	Solution #X: Knowledge of slice quota availability for improved UE signalling

### 6.X.1	Introduction

This solution builds upon Solution#10. When the maximum quota has been reached for a particular network slice for the maximum number of PDU sessions, UE currently is not aware of this and blindly initiates PDU session establishment procedure. This will then lead to a PDU SESSION ESTABLISHMENT REJECT.

To reduce PDU session establishment rejects due to quota limits reached, this solution proposes that UE should be made aware which slices are subject to quota management, and what is the percentage of current quota availability.

Based on the configured URSP rules, UE may initiate multiple PDU sessions associated with the same S-NSSAI. At a high level, this solution proposes that UE gets the feedback whether a particular network slice is subject to quota management for maximum number of PDU sessions, and if yes, what is the percentage of quota that is still available. With this information, for a new application traffic, UE can then take an informed decision on whether to initiate a new PDU session associated with the S-NSSAI or utilize the existing default PDU session to send the data.

For example, if the quota availability is 85% and current data rate for existing PDU session is close to Session-AMBR, UE can initiate establishment of a new PDU session.

However, if the quota availability is just 5%, instead of establishing new PDU session, UE may continue to reuse existing PDU session assuming current data rate for existing PDU session is not close to Session-AMBR.

## 6.X.3	Procedures

## 6.X.3.1		Knowledge of slice quota availability for improved UE signalling

Figure: 6.X.3.1-1 – Knowledge of Quota slice

1. 	NSQM subscribes to SMF to keep track of the count of PDU sessions established with a S-NSSAI and DNN.

NOTE 1: 	NSQM keeps the count of the PDU sessions for the S-NSSAI that are subject to quota management. NSQM also maintains the percentage of available quota for a S-NSSAI.

2. 	UE initiates a PDU Session Establishment Request associated with S-NSSAI.

3. 	AMF receives the PDU Session Establishment and checks the S-NSSAI if any of the slices have the quota limitation. In case any slice has the quota limitation enabled, AMF looks for NSQM by enquiring NRF.

4. 	NRF provides the NSQM address for AMF to access.

5. 	AMF checks the Established PDU sessions count availability for the S-NSSAI in the NSQM. The step is performed via Nnsqm_PDUCount_AvailabilityCheck_Request message.

6. 	NSQM checks its database for the PDU session establishment quota availability for the S-NSSAI

7. 	Considering this is success case scenario, NSQM has quota available to accept a new PDU session to the S-NSSAI. NSQM sends a success code to the AMF via Nnsqm_PDUCount_AvailabilityCheck_Response message. Additionally, the NSQM could also send two more optional parameters to the AMF. The two parameters are “Subject to Quota” & “Percentage of Quota available”.

8. 	On receiving the success code, AMF requests SMF to setup the PDU session for the UE.

9.	SMF sets up the PDU session for the UE and responds to the AMF.

10.	AMF forwards PDU Session Establishment Accept to the UE. This message may optionally contain two additional fields "Subject to Quota" and "Percentage of Quota available".

NOTE 5: 	How the two parameters are utilised by the UE is left to UE implementation. An example is provided in the introduction section.

NOTE 6: 	Similar quota awareness information (related to PDU Establishment) may also be communicated to UE as part of REGISTRATION PROCEDURE. Each S-NSSAI of Allowed S-NSSAI in REGISTRATION ACCEPT will contain the quota awareness.

### 6.X.4	Impacts on existing entities and interfaces

-	NSQM shall be able to provide the percentage of quota available for the network slice to AMF.

- 	AMF shall be able to provide the percentage of quota available for the network slice to UE.

End of CHANGES