SA WG2 Meeting #13	S2-20xxxxx

Source:	Apple

Title:	Solution of Key Issue #3: Limitation of data rate per network slice in UL and DL per UE

Document for:	            Approval

Agenda Item:	X.X

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes a solution for Key Issue # 3 – Limitation of data rate per network slice in UL and DL per UE in TR 23.700-40.

Key issue #3 is about to study whether and how to limit the data rate of UE for a Network Slice, ensuring that the aggregate of the PDU sessions that use the slice are rate limited to the rate defined for the Network Slice in DL and UL.

START of CHANGE

## 6.X	Solution #X: Limitation of data rate per network slice in UL and DL per UE without RAN involvement

### 6.X.1	Introduction

This solution addresses the Key Issue #3: limitation of data rate per network slice in UL and DL per UE.

It is based on the assumption that the data rate limit per network slice in UL and DL per UE is stored in the UDM and can be retrieved by other NFs when needed.

### 6.X.2	High-level Description

In this solution, the enforcement happens each time when a new PDU Session is established. The data rate limit per network slice in UL and DL per UE is compared with the accumulated per-session AMBR from all the active PDU sessions already established as well as the one new PDU session to be established. If the accumulated AMBR exceeds the data rate limit per network slice in UL and DL per UE, the PDU Session establishment will be either rejected, or its session-AMBR will be capped.

This is a static admission control mechanism based on session-AMBR, without considering the actual flow bit rate in any particular moment. It doesn’t require any RAN involvement.

### 6.X.3	Procedures

Since a network slice may include multiple PDU Sessions, and different SMFs and UPFs may be selected to serve different PDU Sessions involved in a slice, a single UPF alone may not able to enforce the per network slice data rate limit per UE.

This solution proposes to let AMF enforce the rate limit instead. The following changes to the existing procedure are proposed to enforce such a rate limit.

Figure 6.X.3-1: TS 23.502 Figure 4.2.2.2.2-1 Registration procedure

Step 14a-c: In addition to the existing registration procedure defined in 3GPP TS 23.502 [6] Figure 4.2.2.2.2-1, AMF shall retrieve the data rate limit per network slice in UL and DL per UE from UDM in step 14.

Step 4.a

Figure 6.X.3-2: TS 23.502 Figure 4.3.2.2.1-1 UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

Step 4.a: Between step 4 and 5 as defined in 23.502 [6] Figure 4.3.2.2.1-1, SMF shall send the Session-AMBR to AMF. AMF accumulates the per-session AMBR from all the active PDU sessions already established as well as from the new PDU session to be established. AMF should compare the accumulated value with the data rate limit per network slice in UL and DL per UE. If the accumulated AMBR exceeds the data rate limit per network slice in UL and DL per UE, AMF indicates that to SMF. SMF may decide either to reject the PDU Session establishment procedure, or to cap the session AMBR for the PDU Session.

### 6.X.4	Impacts on existing entities and interfaces

AMF:	Interacts with each SMF to indicate whether a new PDU session to be established has exceeded the data rate limit or not.

SMF:     According to the indication from AMF, SMF may decide either to reject the PDU Session establishment procedure, or to cap the session AMBR for the PDU Session.

### 6.X.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.