SA WG2 Meeting #13	S2-20xxxxx

Source:	Apple

Title:	Solution of Key Issue #3: Limitation of data rate per network slice in UL and DL per UE

Document for:	            Approval

Agenda Item:	X.X

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract: This contribution proposes a solution for Key Issue # 3 – Limitation of data rate per network slice in UL and DL per UE in TR 23.700-40.

Key issue #3 is about to study whether and how to limit the data rate of UE for a Network Slice, ensuring that the aggregate of the PDU sessions that use the slice are rate limited to the rate defined for the Network Slice in DL and UL.

START of CHANGE

## 6.Y	Solution #Y: Limitation of data rate per network slice in UL and DL per UE based on RAN enforcement

### 6.Y.1	Introduction

This solution addresses the Key Issue #3: limitation of data rate per network slice in UL and DL per UE.

It is based on the assumption that the data rate limit per network slice in UL and DL per UE is stored in UDM and can be retrieved by other NFs when needed.

### 6.Y.2	High-level Description

In this solution, the data rate limit per network slice in UL and DL per UE is enforced by RAN according to the aggregated bit rate from all the non-GBR QoS flows belonging to that network slice.

### 6.Y.3	Procedures

The following changes to the existing procedure defined in TS 23.502 [6] are proposed:

Figure 6.Y.3-1: TS 23.502 Figure 4.2.2.2.2-1 Registration procedure

Step 14a-c: In addition to the existing registration procedure defined in TS 23.502 [6] Figure 4.2.2.2.2-1, AMF shall retrieve the data rate limit per network slice in UL and DL per UE from UDM in step 14 and provide it to RAN.

Figure 6.X.3-2: TS 23.502 Figure 4.3.2.2.1-1 UE-requested PDU Session Establishment for non-roaming and roaming with local breakout

In the PDU Session establishment procedure (as defined TS 23.502 [6] clause 4.3.2.2), RAN has already received QoS profiles and S-NSSAI according to the existing procedures. In this solution, RAN should further associate the received QoS profiles and the corresponding QFIs with the S-NSSAI at step 11.

RAN shall thereafter enforce the aggregated flow bit rate from all the non-GBR QoS flows associated with a particular S-NSSAI to the data rate limit per network slice in UL and DL per UE received from AMF at any particular measurement window.

### 6.Y.4	Impacts on existing entities and interfaces

AMF:	Interacts with UDM to retrieve the data rate limit per network slice in UL and DL per UE from UDM, and further provide it to RAN.

### 6.Y.5	Evaluation

Editor's note:	This clause provides an evaluation of the solution.