SA WG2 Meeting #139-e	S2-2000xxx

May 25-29 2020, Online	(revision of)

Source:	Nokia, Nokia Shanghai Bell, Telecom Italia

Title:	On support of Multiple PCFs in solution 6.6

Document for:	Discussion and Agreement

Agenda Item:	8.8

Work Item / Release:	FS_eNS_Ph2

Abstract of the contribution: xxx

# Discussion

At SA2#136-AH it was observed that the PCF based solution documented in clause 6.6. lacks the technical description of how to cope with the case multiple PCFs are deployed in a Network slice.

The proposal here is that when multiple PCFs are used, meaning that th then one of these is elected as master for counting purposes in the PLMN and keeps state for the number of counters.

So:

- One PCF is elected to act as master (either by configuration or other means TBD).

- The other PCFs contact this PCF to increment or decrement the counters

- The other PCFs subscribe to notifications of when the counter is overflown.

# Proposal

The following clarifications are proposed to be added to TR 23.700-40

## 6.6	Solution #6: PCF-based counting of PDU Sessions in a Network Slice

### 6.6.1	Introduction

This solution addresses KI#2. It allows an operator to count how many PDU Sessions are in a Network Slice identified by a S-NSSAI in the HPLMN. Then, several actions could be implemented based on this information that is always up to date. For the purposes of this solution, we assume a single global quota exists, i.e. irrespective of roaming. In this Solution description PCF may also refer to PCF set when this is the deployed approach in a PLMN.

### 6.6.2	High-level Description

When a S-NSSAI of HPLMN is subject to PDU Sessions counting, this is indicated in subscription information. This subscription information may also include a quota (e.g. to avoid configuration of PCF especially in roaming LBO case).

If a S-NSSAI of the HPLMN is marked for counting, then:

1)	the SMF shall always invoke the PCF to perform the counting and indicate the counting is required to the PCF (and a quota if included in subscription data).

2)	The PCF increments a counter if the PDU Session is allowed to be established.

3)	If the quota is overflown the PCF returns either the PDU Session is allowed but with an indication the quota is overflown, or, if that is the policy, that PDU Session is rejected e.g. with cause indicating "quota overflown". An optional back-off timer may also be provided. The PCF may also report the information to the charging subsystem for Network Slice Customer (NSC)level charging events collection. This information may also be logged in for OAM Performance / SLA monitoring purposes.

4)	When the PDU Session Establishment fails or an ongoing PDU Session is released for a S-NSSAI subject to counting, the SMF indicates that to the PCF which then decrements the PDU Sessions counter for the S-NSSAI.

5)	In roaming case, only the PCF of the HPLMN performs the counting (involved via VPLMN PCF in LBO case, or via SMF of HPLMN in Home Routed case).

6)	When an SMF receives an indication from the PCF that the PDU Session is allowed but the quota is overflown, then this may trigger the SMF to report this to the Charging Subsystem.

7)	Subscription information may also include the Maximum Value of the counter that defines the quota for the maximum number of connections for the S-NSSAI. This could avoid the need to configure the quotas in the PCFs. This may help to provide quotas to VPLMNs in LBO case. If a quota level is provided over the roaming interface, then the VPLMN shall locally enforce the quota for LBO case only, but may also contact the HPLMN PCF for global quota enforcement.

8) if more than one PCF is used for a S-NSSAI in a PLMN, then

a) One PCF in the Network Slice is elected to act as master (either by configuration or other means).
b) The other PCFs in the Network Slice contact the master PCF to increment or decrement the counters. 
c) In the LBO roaming case the master PCF in the V-PLMN for the handling of the V-PLMN S-NSSAI subject to quota management according the mapping of the V-PLMN S-NSSAI to the H-PLMN S-NSSAI is the one that interacts with the HPLMN PCF for the S-NSSAI.

NOTE:	The per VPLMN quota enforcement may be an operator specific attribute not defined by GSMA NG.116 [3]. For simplicity this is not described in detail in the procedures here below, but it is quite a straightforward extension.

### 6.6.3	Procedures

#### 6.6.3.1	General

The clauses in this clause provide examples of how PCF based counting can work. It is of course possible some PDU Sessions release is triggered by Network-initiated Deregistration or other RM events, like a S-NSSAI no longer being available. However, the conceptual operation is aligned with what is shown in these example procedures here below in clauses 6.6.3.2 and 6.6.3.3. While this is not in scope of this study, the same procedure could also be used to count the number of PDU Sessions per DNN if an operator wants to do so.

#### 6.6.3.2	PDU Session Establishment with S-NSSAIs subject to Quota management/capping

Figure 6.6.3.2-1: PDU Session Establishment from TS 23.502 [6] with S-NSSAI subject to quota management

In step 4: the SMF can inspect the subscription data and detect the PDU Session belongs to an S-NSSAI subject to quota management/capping.

At step 7b (i.e. during SM policy association establishment only (see clause 4.16.4 of TS 23.502 [6]) which must be executed if the SMF detects the PDU session is subject to quota management) the PCF can increment a counter for the PDU Session if it allows the PDU Session (despite the quota is overflown) or the quota is not yet overflown.

If the quota is overflown, the PCF then can provide to the SMF actions including back-off timers, limitations of number of PDU Sessions per UE/S-NSSAI, a cause code for rejection, or just indication the quota is overflown. The PCF can log the overflow events for OAM purposes, and also provide a charging event at Network Slice Customer level.

If quota cap is reached and the PDU Session is rejected by PCF, an optional back-off timer can be applied, i.e. the SMF creates a PDU SESSION ESTABLISHMENT REJECT message including a suitable cause code and, if provided, a back-off timer value (optional) and a 5GSM cause value.

The 5GSM cause value may correspond to #69 "insufficient resources for specific slice" (for S-NSSAI only) or a new cause code may be used. Then the PDU SESSION ESTABLISHMENT REJECT message is sent to the AMF. The AMF sends an DL NAS TRNSPORT message carrying the PDU SESSION ESTABLISHMENT REJECT message.

The PCF may indicate to the SMF and the SMF to the UE that a limitation needs to be applied to the number of PDU Sessions per UE in the S-NSSAI to enforce some fairness across UEs in the Network Slice.

NOTE 1:	In LBO roaming case the VPLMN S-NSSAI mapping to HPLMN S-NSSAI is used to decide which VPLMN S-NSSAI to subject to quota management and then the VPLMN PCF needs to interact with the HPLMN PCF as described below in clause 6.6.3.4.

NOTE 2:	if this procedure is applied at DNN level (outside the scope of this Study) or in addition to S-NSSAI level, then cause codes for rejection may correspond to 26 "insufficient resources" (for DNN only), #67 "insufficient resources for specific slice and DNN".

#### 6.6.3.3	PDU Session Release with S-NSSAIs subject to Quota management/capping

Figure 6.6.3.3-1: PDU Session Release from TS 23.502 [6] with S-NSSAI subject to quota management

When a PDU Session for which a PDU Session per S-NSSAI limitation applies is released, then SMF shall trigger a policy association termination in step 12: this decrements a counter in PCF for the S-NSSAI. for PCF initiated case, the counter is decreased by the PCF and the PDU session Release is triggered at step 1b.

NOTE:	In roaming case the VPLMN S-NSSAI mapping to HPLMN S-NSSAI is used to decide which VPLMN S-NSSAI to subject to quota management and then the VPLMN PCF needs to interact with the HPLMN PCFF as described below in clause 6.6.3.4.

#### 6.6.3.4	V-PCF to H-PCF interaction for S-NSSAIs of HPLMN subject to Quota management/capping

Figure 6.6.3.4-1: V-PCF-H-PCF interactions

1.	The V-PCF detects that the S-NSSAI of HPLMN mapping to V-PLMN S-NSSAI of the PDU Session is subject to quota management/capping and so establishment/release of PDU Sessions needs to be reported to the HPLMN PCF for quota management purposes.

2.	The V-PCF reports the establishment/release of PDU Session to H-PCF. The H-PCF for the S-NSSAI is discovered as in the case of SMF discovery and selection.

3.	The H-PCF updates the counters as necessary and detects whether any action is required.

4.	If any action required, then this is communicated to the V-PCF alongside any back-off timer and/or number of PDU Sessions limitation per (UE+S-NSSAI).

5.	The V-PCF reports to the SMF any quota management related action as per step 4.

#### 6.6.3.5	PCF – Master PCF interactions when multiple PCFs are used in a Network slice

Figure 6.6.3.4-1: PCF -Master PCF interactions

1.	The PCF detects that the S-NSSAI of HPLMN mapping to V-PLMN S-NSSAI of the PDU Session is subject to quota management/capping and so establishment/release of PDU Sessions needs to be reported to the Master PCF for quota management purposes.

2.	The PCF reports the establishment/release of PDU Session to Master-PCF.

3.	The Master-PCF updates the counters as necessary and detects whether any action is required.

4.	If any action required, then this is communicated to the PCF alongside any back-off timer and/or number of PDU Sessions limitation per (UE+S-NSSAI).

5.	The PCF reports to the SMF any quota management related action as per step 4.

### 6.6.4	Impacts on services, entities and interfaces

UE: Handling of new cause codes.

SMF: Handling of quota management as specified above.

PCF: Handling of quota management as specified above.