3GPP TSG SA WG2 Meeting #xxx	S2-200xxxx

- Venue, dd – dd month 2020

Source:	OPPO

Title:	KI#4, Solution 13 update

Spec:	3GPP TR 23.754, v0.2.0

Agenda item:	x.y

Document for:	Approval

# Discussion

This contribution proposes to clarify and remove the following ENs from solution 13.

Editor's note:	It is FFS which entity maps the Geofencing co-ordinates to the PLMN's Tracking Area IDs so that the UTM/USS need not know of the PLMN's Tracking Area layouts.

Editor's note:	It is FFS if the mechanism is applicable when the area required for the geofencing does not map to any area of interest (e.g. the no-fly zone does not map specifically to any combination of tracking areas or cells).

Editor note:	FFS if it is the AMF that does the mapping from geo-coordinates to Tracking Area IDs.

The first EN and the third EN address the same thing that which NF maps the monitoring area received from UTM to the 3GPP network area information. We propose to add a bullet in section 6.13.1 to address this problem.

The second EN can be resolved by the Note in section 6.13.1, it’s clarified in the Note that this solution only applicable when the monitoring areas can be mapped to the 3GPP network area. NEF will do the mapping, in case the monitoring area can be mapped to the 3GPP network area, NEF ask AMF to monitor the UAV presence in the subscribed area.

NOTE:	The solution does not propose that the 5GS concepts of Area of Interest are extended to consider areas of interest defined by geographical coordinates, it can be used if the monitoring areas provided by geographical coordinates can be mapped to Area of interest defined by 3GPP network i.e. TAI(s), Cell id(s) or (R)AN node id(s).

# Proposal

It is proposed to include the following in the TR 23.754, v0.2.0.

* * * Start of Change * * * *

## 6.13	Solution 13: Area of Interest for support of UTM's geofencing

### 6.13.1	Description

This is a solution for Key Issue #4 and 7.

This solution proposes the following principles:

-	Area of Interest mechanism is used for the area monitoring and notification to support UTM/USS's Geofencing needs;

-	UTM/USS subscribes Area of Interest to AMF via NEF, the subscribed Area of Interest is decided by the UTM/USS based on Geofencing area;

-	The subscribed monitoring area could be geographical information (e.g., longitude/latitude, zip code, etc), NEF will map it to 3GPP network area information e.g. a list of TAs/Cell ids before sending it to AMF.

-	AMF performs monitoring of UAV presence in the subscribed area, i.e. UAV moving in or out of a subscribed Area of Interest;

-	AMF reports the UAV presence in the subscribed area to the UTM/USS via NEF. Then UTM/USS takes the decisions and triggers the appropriate actions according to the received area monitoring results.

-	UTM/USS could indicate AMF via NEF to report the area monitoring results to SMF, if UTM/USS preconfigures traffic routing policies at SMF, SMF is able to take the appropriate network layer actions based on the area monitoring result, i.e. without constant or repeated triggers/requests from UTM, the 3GPP network can consider those policies as active and ongoing instructions from UTM.

NOTE:	The solution does not propose that the 5GS concepts of Area of Interest are extended to consider areas of interest defined by geographical coordinates, it can be used if the monitoring areas provided by geographical coordinates can be mapped to Area of interest defined by 3GPP network i.e. TAI(s), Cell id(s) or (R)AN node id(s).

This solution applies to both 5GC and EPC. With reference to Figure 6.13.2-1, in the case of EPC, the NEF is replaced by the SCEF and the AMF is replaced by the MME.

### 6.13.2	Procedures

Figure 6.13.2-1 Geofencing based on Area of Interest

1.	UTM/USS decides the subscribed monitoring area, i.e. the area to be monitored at 3GPP network. The decision is based on the Geofencing area known to the UTM;

2.	UTM/USS provides the subscribed monitoring area to NEF. The subscribed monitoring area could be geographical information (e.g., longitude/latitude, zip code, etc), NEF will map it to 3GPP network area information e.g. a list of TAs/Cell ids before sending it to AMF.

[optional] UTM/USS provides policies for the network layer actions to be performed when the UAV moves in or out of the subscribed monitoring area. E.g. revoke the connectivity between UAV and UAV controller, steering the traffic toward/ from the UAV to UTM/TPAE for further analysis, etc.

3.	NEF reuse the Area of Interest mechanism as specified in TS 23.501 [12] clause 5.3.4.4 to subscribe to the AMF, the monitoring area. If the UTM/USS policies are received at step2, the NEF also provides the received policies to SMF, and indicates AMF to report the area monitoring results to SMF.

4.	Determination of UE presence in Area of Interest is described in TS 23.502 [7] Annex D.1.

5.	AMF reports the UAV presence in the subscribed area, i.e. UAV moving in or out of a subscribed Area of Interest to the NEF.

6.	[Conditional] If triggered by NEF to do so in step 3, the AMF also report the UAV presence in the subscribed area, i.e. UAV moving in or out of a subscribed Area of Interest, to SMF.

7.	[Conditional]If traffic policies have been forwarded by the NEF, the SMF takes the appropriate network layer actions based on the area monitoring result and the polices set out by UTM/USS policies, i.e. without constant or repeated triggers/requests from UTM, the 3GPP network layer actions can consider those policies as active and ongoing instructions from UTM/USS.

8.	NEF forwards the report from AMF of UAV moving in or out of the monitoring area to UTM.

9.	UTM/USS decides the actions according to the received area monitoring results. UTM/USS provides instructions to NEF if the network layer actions are needed.

10.	UTM/USS may also perform application layer control for the UAV.

### 6.13.3	Impacts on services, entities and interfaces

-	Area of Interest mechanism is reused for the area monitoring and notification to support Geofencing.

-	If UTM/USS preconfigures some policies at SMF, SMF is able to take the appropriate network layer actions based on the monitoring report from AMF of Area of Interest. i.e. the policies are considered to be active instructions from UTM.

* * * End of Change * * * *