3GPP TSG-WG SA2 Meeting #139E e-meeting 	S2-200xxxx

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2: evaluation and conclusion

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS / Rel-17

Abstract: This contribution proposes evaluations on key issue 2.

# 1. Discussion

Key Issue #2 “UAV authorization by UTM” has several similar solutions in the TR. Among which, common aspects are observed, and based on those, the following interim agreements are proposed:

- A new Control Plane NF “UAS AF” is introduced in the 5GS for handling interaction between UAV and UTM.

- How UTM authorizes UAV has four options as documented in solution 5.

With regard to Solution 4, it is observed that solution 5 option 2 covers the idea of it. Other solutions #6, #7 and #8 are proposed to be merged to solution #5, as indicated in paper S2-200xxxxx.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.754.

# * * * * First change * * * *

# 7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

## 7.2	Key Issue #2: UAV authorization by UTM

Key issue #2 “UAV authorization by UTM” has several similar solutions in the TR. Among which, common aspects are observed, and based on those, interim agreements are proposed:

-	A new Control Plane NF “UAS AF” is introduced in the 5GS for handling interaction between UAV and UTM.

-	How UTM authorizes UAV has four options as documented in solution 5.

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

## 8.2 Key Issue #2: UAV authorization by UTM

Interim agreements of key issue #2 are as follow:

-	Solution 5 is the way forward to address key issue #2.

-	A separate 5GC NF (UAS AF) is responsible for handling the UAV UE authorization procedure with UTM.

-	UAS AF is responsible for UAV UE contexts management, including the authorization status notified by UTM.

# * * * * End of changes * * * *