3GPP TSG-WG SA2 Meeting #140E e-meeting 	S2-200xxxx

Elbonia, August 19 – September 1, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#6: evaluation and conclusion

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS / Rel-17

Abstract: This contribution provides evaluations and conclusions for KI#6 UAV Controller and UAV association.

# 1. Introduction/Discussion

The solutions of KI#6: UAV Controller and UAV association include solution #2, #5, #20 and #21. The following table provides an analysis and comparison, focusing on aspects of KI#6.

| Solution | Evaluation | Impacts |
| --- | --- | --- |
| Sol#2:3GPP reference architecture for UAV remote identification | Architecture solution, no specific details on how to address KI#6 | none |
| Sol#5:UAV authentication and authorization by USS/UTM based on NAS supplementary and secondary authentication and authorization procedures | UAV/UAVC pairing authentication and authorization is done by USS/UTM during NSSAA-like procedure or PDU session secondary authentication and authorization procedure, the result is notified to SMF or AMF or UFES | - PDU session secondary authentication and authorization procedure enhancements - New secondary authentication and authorization like NSSAA - UAV/UAVC pairing authentication and authorization result is notified to SMF or AMF or UFES |
| Sol#20:UAV and UAVC Association and Connectivity Control Using Secondary Authorization | UAV/UAVC pairing authentication and authorization is done by USS/UTM during PDU session secondary authentication and authorization procedure, the result is notified to SMF | - PDU session secondary authentication and authorization procedure enhancements - UAV/UAVC pairing authentication and authorization result is notified to SMF |
| Sol#21:Connectivity setup for C2 communication and association between UAV and UAV-C | Before PDU session setup, UAV/UAVC pairing authentication and authorization is done by USS/UTM and the result (including a UTID) is notified to the PLMN/UAV/UAVC. During PDU session setup, the PLMN re-authenticates the UAV/UAVC based on the result and the UTID. | - UAV/UAVC pairing authentication and authorization result and UTID is notified to SMF - PDU session establishment procedure enhancement - UTID |

Proposal 1: UAV/UAVC pairing authentication and authorization is done by USS/UTM during PDU session secondary authentication and authorization procedure, the result is notified to SMF.

Proposal 2: UAV/UAVC pairing authentication and authorization is done by USS/UTM during New secondary authentication and authorization procedure like NSSAA, the result is notified to UFES.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.754.

# * * * * First change (all new) * * * *

# 7	Evaluation

## 7.6	Key Issue 6: UAV Controller and UAV association

Sol#2: this solution has no specific details on how to address this key issue.

Sol#5: this solution proposes that UAV/UAVC pairing authentication and authorization is done by USS/UTM during NSSAA-like procedure or PDU session secondary authentication and authorization procedure, the result is notified to SMF or AMF or UFES.

Sol#20: this solution proposes that UAV/UAVC pairing authentication and authorization is done by USS/UTM during PDU session secondary authentication and authorization procedure, the result is notified to SMF.

Sol#21: this solution proposed that UAV/UAVC pairing authentication and authorization is done by USS/UTM and the result (including a UTID) is notified to the PLMN/UAV/UAVC before PDU session setup, hence, the SMF can re-authenticate/authorizes the UAV/UAVC based on the result and the UTID during PDU session setup.

# * * * * Second change (all new) * * * *

# 8	Conclusions

## 8.6	Key Issue 6: UAV Controller and UAV association

When separate PDU Sessions for C2 communication, or when there is a static association between UAV and UAVC then UAV/UAVC pairing authentication and authorization is done by USS/UTM during PDU session secondary authentication and authorization procedure, the result is notified to SMF.

In addition, UAV/UAVC pairing authentication and authorization can also be done by USS/UTM during new secondary authentication and authorization procedure like NSSAA if available, the result is notified to UFES.

# * * * * End of changes * * * *