3GPP TSG-SA3 Meeting #120	S3-25xxxx

# Athens, Greece, 17 - 21 February 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on 3GPP Cryptographic Inventory for PDCP

Document for:	Approval

Agenda item:	5.20

Spec:	3GPP TR 33.938

Version:	0.1.0

Work Item:	FS_CryptoInv

Comments

This contribution provides detailed description for PDCP security protection used in 5G systems for TR 33.938.

* * * First Change * * * *

### 4.4.X PDCP

The PDCP protocol between the UE and the NG-RAN is responsible for the security protection of the following scenarios in 5G system:

RRC integrity and confidentiality protection between UE and gNB (see clause 6.5.1 and 6.5.2 of TS 33.501 [4]).

user plane data integrity and confidentiality protection between UE and gNB (see clause 6.6.3 and 6.6.4 of TS 33.501 [4]).

NOTE: The NAS protocol provides integrity and confidentiality protection for NAS signalling messages between the UE and the AMF. However, from a perspective of the UE, the NAS protocol is above the 5G-AN protocol layer. Therefore, it may be considered that the NAS message is also protected by the PDCP protocol layer.

* * * End of Changes * * * *