3GPP TSG-SA3 Meeting #126																				S3-26xxxx

Goa, India, 9 – 13 February 2026

# Source:	Ericsson

# Title:	Shortlisted subset of MAC-CE risk analysis

# Document for:	Approval

Agenda Item:	x.y.z

# 1	Decision/action requested

Propose to approve the Annexure B skeleton described in this contribution

*** Start of Change ***

# Annex B

# Risk analysis of MAC-CE

Editor’s Note: Structure of annex is FFS.

Editor’s Note: Format of the framework capturing risk analysis is FFS.

Editor’s Note: Methodology for the risk analysis is FFS.

## B.1	General

In LTE and 5GNR, security for Control Plane (CP) and User Plane (UP) traffic between the User Equipment (UE) and the base station is fundamentally anchored at the Packet Data Convergence Protocol (PDCP) layer. The risk of Medium Access Control (MAC) layer needs to be analysed. This Annex captures the security and privacy risk analysis of the MAC-CEs from clause 6.1.3 of TS 38.321.

The MAC Control Element (MAC-CE) is a signaling message used at the MAC layer to manage time-critical control functions. For example, MAC-CEs are used for Layer 2 operations, conveying control information for resource management, scheduling, power control, and link maintenance. MAC-CEs were introduced in Release 8 (LTE) and has been expanded in every subsequent release.

Editor’s Note: The alignment of above paragraph with RAN2 is FFS.

## B.2	Risk Analysis Methodology

Editor’s Note: This clause describes agreed methodology for analyzing risks resulting from potential exploitation of functionality of MAC-CE control messages at the MAC layer.

### B.2.1	Risk Analysis of MAC-CEs

Editor’s Note: This clause contains risk analysis output in a tabular easy to grasp format.

| MAC-CE | Function | Threat |  | Risk | Risk classification |  |
| --- | --- | --- | --- | --- | --- | --- |
| Buffer Status Report MAC CEs | UE's current buffered data volume per QoS classes | Tampering |  | Tampering leads to UE denied resources it needs | Risk level is subjective to context, use case, overall threat landscape, information gathered from reconaissance |  |
|  | gNB determines if, for what QoS class & when to grant UL resources to UE |  |  | Tampering leads to resource depletion at gNB |  |  |
|  |  |  |  | Eavesdropping leads to reconnaisance of UE behaviour |  |  |
| Delay Status Report MAC CE | Tells gNB how much time is left for volume of data buffered at UE | Tampering |  | Tampering leads to gNB prioritising & granting UL resources for forged UEs under attacker control |  |  |
|  | Enables gNB to make decisions based on time-criticality of the data |  |  | Tampering leads to legit UEs denied resources it needs |  |  |
|  | Critical for URLCC applications (XR, IIoT etc.) |  |  |  |  |  |
| Single Entry Power Headroom Report MAC CE | Power management at UE. Spare power of UE (max allowed minus current usage) | 1. Tampering 2. Privacy |  | gNB estimation of UE power capacity leads to inaccurate decisions of resource allocation subsequently impacting spectral usage |  |  |
|  | gNB makes informed scheduling decisions for UL |  |  | Track UE location distance from gNB based on changes in power headroom reporting (triggered by path loss) |  |  |
|  | Enables RF spectral efficiency |  |  |  |  |  |
| Multiple Entry Power Headroom Report MAC CE | Complex (!) matrix of load of information related to UE's RF capabilities across cells, carriers, transmission path, power etc. | 1. Tampering 2. Privacy |  | Miscalculated estimation of UE capabilities at gNB, resulting in inaccurate resourcing, scheduling decisions |  |  |
|  | Enables gNB to make informed decisions taking into account matrix of sheer volume of information |  |  | UE's mobility, its RF environment, power management |  |  |
| Enhanced Single Entry PHR for multiple TRP MAC CE | gNB configures UE via RRC to report via this MAC-CE to the gNB single representative Power Headroom for UE's UL power capacity across multiple "Transmission Reception Points" (TRP) for UE to communicate with the network via multiple antennas/paths. | 1. Tampering 2. Performance degradation |  | Tampering leads to UE getting lower UL resources because gNB thinks UE's power is limited |  |  |
|  | In turn it enables gNB to accurately schedule UE's UL transmissions for multi-TRP scenario. |  |  | Tampering frequently with the objective of causing UL link instability |  |  |
| Enhanced Multiple Entry PHR for multiple TRP MAC CE | UE reports granular TRP link/cell configuration aspects and specific Power Headroom for each configured TRP. | 1. Tampering 2. Privacy |  | gNB estimation of UE power capacity leads to inaccurate decisions of resource allocation subsequently impacting spectral usage |  |  |
|  | Consequently gNB makes highly optimized resource allocation decisions |  |  | Tampering frequently with the objective of causing UL link instability |  |  |
|  |  |  |  | Real time location of UE movement due to Power Headroom per TRP/cell |  |  |
| Enhanced Single Entry PHR MAC CE | Contains more granular separate power values for different TRP for a single cell. | Privacy |  | Precision location tracking of UE based on which TRP UE is closest to and which panel is active |  |  |
|  | UE power reporting for UL. gNB makes decisions for grant of resources accordingly. | Information disclosure |  | Network topology leakage - spatial environment information of active TRPs. |  |  |
|  |  | DoS |  | Tampering leads to gNB allocating resources not reflective of UE's correct power headroom resulting into interference or battery drain. |  |  |
| Enhanced Multiple Entry PHR MAC CE | PHR for multiple cells and multiple TRPs simultaneously. | Privacy |  | Precision location tracking of UE based on which TRP UE is closest to and which panel is active |  |  |
|  | Aggregates data for entire Carrier Aggregation and Dual Connectivity setup. | Information disclosure |  | Network topology leakage - spatial environment information of active TRPs. |  |  |
|  |  | Tampering |  | Tampering leads to gNB allocating resources not reflective of UE's correct power headroom resulting into interference or battery drain for UE across all carriers and secondary cells for the victim UE. Conversely, gNB resource exhaustion to reconfigure UEs at scale, by sending mass RRC reconfiguration |  |  |
| Absolute Timing Advance Command MAC CE | Synchronization of UE - gNB for Tx-Rx in TN & NTN | Privacy |  | Location tracking of UE from distance from the base station. |  |  |
|  | Positioning of the UE in NTN | Tampering |  | In NTN, tampering can lead to satellite thinking UE is in a different location (by 100s Km) as UE positioning is incorrectly derived. |  |  |
|  | Absolute value of the timing for the UE to transmit. gNB tells it to UE in DL and UE replaces its current timing for its transmission slot. |  |  | In NTN, UE transmitted signal arrives outside the window at satellite causing time slot collision or missing the receiver. |  |  |
|  |  |  |  | Combination of LTM Cell Switch and Absolute TA manipulation - UE camps on target cell with incorrect time slot and immediately results into desynchronization. |  |  |
| C-RNTI | Sent UL in message#3 in RACH | Spoofing/Masquerading | Using Software Defined Radio,can be sniffed over the air to impersonate legit user, track meta-data (trafiic pattern, activity levels etc.) |  |  | Risk level is subjective to context, use case, overall threat landscape, information gathered from reconaissance |
|  | Uniquely identifies UE locally within a cell. | Privacy |  |  |  |  |
| UE Content Resolution Identity | Sent DL in message#4 in RACH | Spoofing/Masquerading | Using Software Defined Radio,can be sniffed over the air. FBS impersonate legit gNB and cause DoS to legit user or fool UE to connect to rogue network |  |  |  |
|  | gNB echoes back what it received in message#3 in RACH |  |  |  |  |  |
|  | If doesn't match at UE, it disconnects; thinking of collision or session hijack. | Tampering |  |  |  |  |
| Beam Failure Recovery (BFR) | Used during HO and link-failure scenarios for beam level link recovery. | Spoofing/Masquerading | Using Software Defined Radio,legit UE identities can be sniffed over the air. |  |  |  |
|  | UE includes C-RNTI for BFR in UL message#3 in RACH so gNB can map the RACH attempt to UE's existing security context | Tampering | Resource exhaustion at gNB Legit UE losing connection if fake BFR are injected |  |  |  |
| Scell Activation/Deactivation | In DL, activates specific secondary cells. Since security keys are tied to the cell group and PHY parameters, SCell Index are critical for UE's secure multi-connectivity state to ensure data is transmitted over authenticated correctly configured secondary carrier. | Tampering | False activation/deactivation leads to throughput degradation or battery drain |  |  |  |

### B.2.2	Risk Prioritization

Editor’s Note: This clause contains agreement on risk prioritization.

## B.3	Risk Treatment Principles

Editor’s Note: This clause contains agreed principles taking into account RAN WGs input.

## B.4	Risk Tolerance

Editor’s Note: This clause contains risk tolerance / acceptance criteria.

*** End of Change ***