# 3GPP TSG-SA3 Meeting #125																			S3-25xxxx

# Dallas, US, 17 – 21 November 2025

Source:	Huawei, HiSilicon

Title:	Pseudo-CR on 3GPP consideration for PKI

Document for:	Approval

Agenda item:	5.2.1

Spec:	3GPP TR 33.703

Version:	0.2.0

Work Item:

Comments

This contribution proposes text for subclause of PKI protocol.

* * * First Change * * * *

## 6.5	PKI certificate

### 6.5.1	General

The IETF LAMPS group has introduced multiple Drafts to enable a smooth transition to PQC in PKIX to provide quantum-resistant security for PKIX.

### 6.5.2		Current Work in IETF

#### 6.5.2.1	IETF RFCs

- IETF RFC 9802 [51] has specified algorithm identifiers and ASN.1 encoding format for several stateful Hash-Based Signature (HBS) schemes: Hierarchical Signature System (HSS), eXtended Merkle Signature Scheme (XMSS), and a multi-tree variant of XMSS, XMSS^MT. These schemes are applicable to the Internet X.509 Public Key Infrastructure (PKI) when digital signatures are used to sign certificates and certificate revocation lists (CRLs).

-	IETF RFC 9763 [50] defines a method for requesting and issuing two X.509 end-entity certificates for the same entity, in order to perform two authentications using the two certificates where each certificate corresponds to a distinct digital signature.

#### 6.5.2.2	IETF Adopted Drafts

- IETF Draft draft-ietf-lamps-kyber-certificates-11 "Internet X.509 Public Key Infrastructure - Algorithm Identifiers for the Module-Lattice-Based Key-Encapsulation Mechanism (ML-KEM)" [52] specifies the conventions for using the ML-KEM [21] in X.509 Public Key Infrastructure.

- IETF Draft draft-ietf-lamps-x509-slhdsa-09, "Internet X.509 Public Key Infrastructure: Algorithm Identifiers for SLH-DSA" [53] specifies to the conventions for using the SLH-DSA [23] in X.509 Public Key Infrastructure.

- IETF Draft draft-ietf-lamps-dilithium-certificates-13, "Internet X.509 Public Key Infrastructure - Algorithm Identifiers for the Module-Lattice-Based Digital Signature Algorithm (ML-DSA)" [54] specifies the conventions for using the ML-DSA [22] in X.509 Public Key Infrastructure.

- IETF Draft draft-ietf-lamps-pq-composite-kem-08 "Composite ML-KEM for use in X.509 Public Key Infrastructure" [55] defines a specific instantiation of the PQT Hybrid paradigm called "composite" where multiple cryptographic algorithms (i.e. ML-KEM [21] in hybrid with traditional algorithms RSA-OAEP, ECDH, X25519, and X448) are combined to form a single key encapsulation mechanism (KEM) presenting a single public key and ciphertext such that it can be treated as a single atomic algorithm at the protocol level.

-	IETF Draft draft-ietf-lamps-certdiscovery-01, "A Mechanism for X.509 Certificate Discovery" [56] specifies a method to discover a secondary X.509 certificate associated with an X.509 certificate to enable efficient multi-certificate handling in protocols.

### 6.5.3		3GPP Considerations

Using stateful Hash-Based Signature (HBS) schemes, ML-KEM, SLH-DSA, or ML-DSA in X.509 certificates can be considered in 3GPP based on RFC 9802 [51] and Drafts [52-54], upon approval in IETF.  RFC 9763 [50] with two certificates and Draft [55] for “composite” algorithms can be considered for using hybrid algorithms.

* * * End of Changes * * * *