3GPP TSG-SA3 Meeting #111
S3-23bbbb
Berlin, Germany, 22 -26 May 2023
revision of S3-yyxxxx
Source:
InterDigital 
Title:
Update TR 33.740 conclusion for KI #2
Document for:
Approval

Agenda Item:
5.3
1
Decision/action requested
This contribution proposes to update TR 33.740 conclusion for KI #2. 
2
References

[1]
3GPP TS 23.304 V18.1.0 

[2]
3GPP TS 33.503 V17.3.0 

[3]
3GPP TS 33.536 V17.1.0 

[4]
3GPP TR 33.740 V0.7.0 

3
Rationale

The L2 relay reselection is specified in clause 6.7.4.2 of TS 23.304, with support for exchange of security parameters E2E between the End UEs via the first Relay (steps 3,5) and then via second Relay (step 6):

[image: image1.emf] 

Relay  UE  2   5G ProSe  End UE   5G ProSe  End   UE   Relay  UE  1  

5.  Link Modification Accept message  

Traffic transfer  

6. Connection setup between  5G ProSe   UE s   via new Relay UE  

1. Connection setup between  5G ProSe   UE s   vi a Relay   UE  1  

2. Decide to Relay  UE reselection  

3.  Link Modification Request message  

4. Decide the  new  Relay UE  


Figure 6.7.4.2-1: 5G ProSe Layer-2 UE-to-UE Relay reselection

With the following NOTE:

NOTE 1:
The security information contents and usage will be defined by SA WG3.
In existing mechanisms (TS 33.503 [2], clause 6.3.5 or in TS 33.536 [3]) protection of DCR message is not always possible (e.g., if DUCK, DUIK not provisioned). The availibility of communication keys pre-established during negotiated U2U reselection provides additional means for End UE to protect the DCR sent via the second U2U Relay. 
Based on the above, it is proposed to support the exchange of security parameters for the establishment of new E2E security keys used for the security over the second L2 UE-to-UE Relay, as part of the negotiated UE-to-UE Relay reselection procedure.
4
Proposal

It is proposed to approve the following changes for inclusion in TR 33.740 [4]. 








***
BEGIN OF CHANGES
***

7.2
Key Issue #2: Security of UE-to-UE Relay

For Key Issue #2, the following statements are agreed:
Regarding End-to-End security:

· For L2 relay, the source UE and the target UE can establish a PC5 link over the L2 relay after the per hop link is established.

· For L2 relay reselection security, the source UE and the target UE support the exchange of security parameters to establish new E2E security keys during the negotiated UE-to-UE Relay reselection procedure. The new E2E security keys are used to secure communication over the second relay. Security policy negotiation mechanism as defined in clause 5.3 of TS 33.536 [9] is reused. Solution #34 is used as a baseline.
Editor’s Note: further conclusion is to be updated.
***
END OF CHANGES
***
_1742187117.doc


3. Link Modification Request message







5. Link Modification Accept message







5G ProSe End UE







Relay UE 1







Relay UE 2







5G ProSe End UE







Traffic transfer







6. Connection setup between 5G ProSe UEs via new Relay UE







1. Connection setup between 5G ProSe UEs via Relay UE 1







2. Decide to Relay UE reselection







4. Decide the new Relay UE












