| 3GPP TR 33.745 V0.0.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on security aspects of 5G NR Femto (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Security Architecture and Assumptions	6

5	Gap analysis w.r.t. TS 33.320	7

6	Key issues	8

6.X	Key Issue #X: <Key Issue Name>	8

6.X.1	Key issue details	8

6.X.2	Security threats	8

6.X.3	Potential security requirements	8

7	Solutions	8

7.0	Mapping of solutions to key issues	8

7.Y	Solution #Y: <Solution Name>	8

7.Y.1	Introduction	8

7.Y.2	Solution details	8

7.Y.3	Evaluation	8

8	Conclusions	8

Annex <X> (informative): Change history	9

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

Editor’s Note: This clause contains scope for the study.

The present document …

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Security Architecture and Assumptions

Editor’s Note:  This clause contains security architecture and assumptions to be considered for the study (e.g., per work task/KI).

5	Gap analysis w.r.t. TS 33.320

Table 5-1: Gap analysis table w.r.t TS 33.320

| Clause number in TS 33.320 | Can be re-used with update of 5G terms | Can be re-used partially | Needs to be completely revised | Rationale and relevant KI |
| --- | --- | --- | --- | --- |
| 4.1 |  |  |  |  |
| 4.2 |  |  |  |  |
| 4.3 |  |  |  |  |
| 4.4 |  |  |  |  |
| 5.1 |  |  |  |  |
| 5.2 |  |  |  |  |
| 5.3 |  |  |  |  |
| 5.4 |  |  |  |  |
| 6.1 |  |  |  |  |
| 6.3 |  |  |  |  |
| 7.1 |  |  |  |  |
| 7.2 |  |  |  |  |
| 7.3 |  |  |  |  |
| 7.4 |  |  |  |  |
| 7.5 |  |  |  |  |
| 8.1 |  |  |  |  |
| 8.2 |  |  |  |  |
| 8.3 |  |  |  |  |
| 8.4 |  |  |  |  |
| 8.5 |  |  |  |  |
| 9 |  |  |  |  |
| 10.1 |  |  |  |  |
| 10.2 |  |  |  |  |
| 11.1 |  |  |  |  |
| 11.2 |  |  |  |  |
| Annex A |  |  |  |  |
| Annex B |  |  |  |  |

6	Key issues

Editor’s Note: This clause contains all the key issues identified during the study.

6.X	Key Issue #X: <Key Issue Name>

6.X.1	Key issue details

6.X.2	Security threats

6.X.3	Potential security requirements

7	Solutions

Editor’s Note: This clause contains the proposed solutions addressing the identified key issues.

7.0	Mapping of solutions to key issues

Editor's Note: This clause contains a table mapping between key issues and solutions.

Table 6.0-1: Mapping of solutions to key issues

| Solutions | KI#X | KI#Y | KI#Z |
| --- | --- | --- | --- |

7.Y	Solution #Y: <Solution Name>

7.Y.1	Introduction

Editor’s Note: Each solution should list the key issues being addressed.

7.Y.2	Solution details

7.Y.3	Evaluation

Editor’s Note: Each solution should motivate how the potential security requirements of the key issues being addressed are fulfilled.

8	Conclusions

Editor’s Note: This clause contains the agreed conclusions that will form the basis for any normative work.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |