3GPP TSG-SA3 Meeting #121	S3-251754

# Gothenburg, Sweden, 7 – 11 April 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR | draft | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Living document for AIML_CN_SEC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile, vivo,Huawei, HiSilicon,OPPO,Ericsson,Nokia, Xiaomi,ZTE, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN_SEC |  |  |  |  |  | Date: |  |  | 2025-4-7 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This draft CR is the living document for AIML_CN_SEC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draft CR is the living document based on the approved WID proposal on Security aspects of Core Network Enhanced Support for AIML. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Security aspects of Core Network Enhanced Support for AIML will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.11 (new clause), X.12 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S3-251141, S3-251539, S3-251754 |  |  |  |  |  |  |  |  |

Start of 1st Change

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5G-AN	5G Access Network

5G-RG	5G Residential Gateway

NG-RAN	5G Radio Access Network

5G AV	5G Authentication Vector

5G HE AV	5G Home Environment Authentication Vector

5G NSWO	5G Non-Seamless WLAN Offload

5G SE AV	5G Serving Environment Authentication Vector

ABBA	Anti-Bidding down Between Architectures

AEAD	Authenticated Encryption with Associated Data

AES	Advanced Encryption Standard

AI/ML	Artificial Intelligence/Machine Learning

AKA	Authentication and Key Agreement

AMF	Access and Mobility Management Function

AMF	Authentication Management Field

NOTE:	If necessary, the full word is spelled out to disambiguate the abbreviation.

ARPF	Authentication credential Repository and Processing Function

AUN3	Authenticable Non-3GPP devices

AUSF	Authentication Server Function

AUTN	AUthentication TokeN

AV	Authentication Vector

AV'	transformed Authentication Vector

BAP	Backhaul Adaptation Protocol

BH	Backhaul

CCA	Client Credentials Assertion

Cell-ID	Cell Identity as used in TS 38.331 [22]

CH	Credentials Holder

CHO	Conditional Handover

CIoT	Cellular Internet of Things

cIPX	consumer's IPX

CKSRVCC	Cipher Key for Single Radio Voice Continuity

cNRF	consumer's NRF

CP	Control Plane

CPAC	Conditional PSCell Addition or Change

CPA	Conditional PSCell Addition

CPC	Conditional PSCell Change

cPLMN	consumer's PLMN

cRI	consumer's RI

cSEPP	consumer's SEPP

CTR	Counter (mode)

CU	Central Unit

DCS	Default Credentials Server

DN	Data Network

DNN	Data Network Name

DU	Distributed Unit

EAP	Extensible Authentication Protocol

EDT	Early Data Transmission

EMSK	Extended Master Session Key

EN-DC	E-UTRA-NR Dual Connectivity

ENSI	External Network Slice Information

EPS	Evolved Packet System

FL	Federated Learning

FN-RG	Fixed Network RG

gNB	NR Node B

GUTI	Globally Unique Temporary UE Identity

HFL	Horizontal Federated Learning

HRES	Hash RESponse

HXRES	Hash eXpected RESponse

IAB	Integrated Access and Backhaul

IKE	Internet Key Exchange

IKSRVCC	Integrity Key for Single Radio Voice Continuity

IPUPS	Inter-PLMN UP Security

IPX	IP exchange service

KSI	Key Set Identifier

KSISRVCC	Key Set Identifier for Single Radio Voice Continuity

LI	Lawful Intercept

MBSF	Multicast/Broadcast Service Function

MBSSF	Multicast/Broadcast Service Security Function

MBSTF	Multicast/Broadcast Service Transport Function

MeNB	Master eNB

MN	Master Node

MO-EDT	Mobile Originated Early Data Transmission

MT-EDT	Mobile Terminated Early Data Transmission

MR-DC	Multi-Radio Dual Connectivity

MSK	Master Session Key

N3IWF	Non-3GPP access InterWorking Function

NAI	Network Access Identifier

NAS	Non Access Stratum

NDS	Network Domain Security

NEA	Encryption Algorithm for 5G

NF	Network Function

NG	Next Generation

ng-eNB	Next Generation Evolved Node-B

ngKSI	Key Set Identifier in 5G

N5CW	Non-5G-Capable over WLAN

N5GC	Non-5G-Capable

NIA	Integrity Algorithm for 5G

NR	New Radio

NR-DC	NR-NR Dual Connectivity

NSSAI	Network Slice Selection Assistance Information

NSSAA	Network Slice Specific Authentication and Authorization

NSWO	Non-Seamless WLAN Offload

NSWOF	Non-Seamless WLAN Offload Function

PDN	Packet Data Network

PEI	Permanent Equipment Identifier

pIPX	producer's IPX

pNRF	producer's NRF

pPLMN	producer's PLMN

pRI	producer's RI

PRINS	PRotocol for N32 INterconnect Security

pSEPP	producer's SEPP

PUR	Preconfigured Uplink Resource

QoS	Quality of Service

RES	RESponse

RI	Roaming Intermediary

RH	Roaming Hub

SCG	Secondary Cell Group

SEAF	SEcurity Anchor Function

SCP	Service Communication Proxy

SEPP	Security Edge Protection Proxy

SCPAC	Subsequent Conditional PSCell Addition or Change

SgNB	Secondary gNB

SIDF	Subscription Identifier De-concealing Function

SMC	Security Mode Command

SMF	Session Management Function

SN	Secondary Node

SN Id	Serving Network Identifier

SUCI	Subscription Concealed Identifier

SUPI	Subscription Permanent Identifier

TLS	Transport Layer Security

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TNGF	Trusted Non-3GPP Gateway Function

TWAP	Trusted WLAN Access Point

TWIF	Trusted WLAN Interworking Function

TSC	Time Sensitive Communication

UE	User Equipment

UEA	UMTS Encryption Algorithm

UDM	Unified Data Management

UDR	Unified Data Repository

UIA	UMTS Integrity Algorithm

ULR	Update Location Request

UP	User Plane

UPF	User Plane Function

URLLC	Ultra Reliable Low Latency Communication

USIM	Universal Subscriber Identity Module

VFL	Vertical Federated Learning

XRES	eXpected RESponse

End of 1st Change

Start of 2nd Change

# X.11	Security for UE positioning based on a ML model at the LMF

## X.11.1	General

This clause outlines the security considerations for UE positioning based on a ML model at the LMF. According to TS 23.273 [86], UE positioning based on a ML model at the LMF procedures include data collection procedure and AIML model for positioning retrieval procedure.

Clause X.11.2 outlines the general requirements related to those data collection procedures.

Clause X.11.3 outlines the authorization aspects of LMF retrieval of a trained AI/ML model for positioning.

## X.11.2	Security for data collection for the LMF-based AI/ML positioning

Regarding user consent aspect for LMF-based AI/ML model positioning, Annex V of the present document applies. The LMF is deemed as enforcement point as specified in clause 6.22.3 of TS 23.273 [86].

## X.11.3	Authorization of LMF retrieval of a trained AI/ML model for positioning

For authorization of LMF retrieval of AI/ML model for positioning, the security for AI/ML model storage and sharing as described in clause X.10 shall apply.

The NWDAF containing MTLF shall register ML model interoperability ID per LMF-based AI/ML Positioning indication (supporting model training for LMF-based AI/ML Positioning) if supported.

The LMF shall register at the NRF including its Vendor ID.

NOTE: How to implement the indication of supporting model training for LMF-based AI/ML Positioning in a backward compatible and extensible manner is up to stage 3.

# X.12	Security for Vertical Federated Learning among NWDAFs and AFs

X.12.1	General

This clause outlines the security considerations for vertical federated learning (VFL) where both AFs and NWDAF can act as VFL participants (i.e. either VFL servers or clients), as described in clause 6.2H of TS 23.288 [105]. According to TS 23.288 [105], when an AF acts as VFL server, only NWDAFs can be VFL clients.

The authorization of prospective VFL participants, as described in clause X.12.2, is conducted for the cases that NWDAF serves as the VFL server, external AF acts as the VFL server, and internal AF acts as the VFL server.

Clause X.12.3 outlines the general NEF security requirements related to the privacy aspects of VFL.

Clause X.12.4 addresses the protection of communication data used in the VFL process and will reuse SBA security measures as detailed in clause 13.

X.12.2	Authorization of candidate VFL participants

X.12.2.1 Authorization of candidate VFL participants for VFL when NWDAF or Internal AF is acting as the VFL server

Figure X.12.2.1-1 depicts the authorization mechanism for NWDAF or internal as VFL Server. The authorization is based upon the VFL capability information per supported analytics ID which includes VFL capability type (VFL server and/or VFL client), Analytics ID and VFL Interoperability Indicator per analytics ID provided by VFL members during registration.

Figure X.12.2.1-1: Authorization of VFL participants (NWDAF or Internal AF acts as VFL Server)

1. In case of external AF as VFL Client, the NEF updates its NF profile at the NRF with AF’s VFL capability information per supported analytics ID which includes VFL Capability type i.e. (VFL Client) and VFL interoperability indicator(s) per analytics ID and associated AF ID. In all cases where a NWDAF or Internal AF acts as VFL Client or VFL Server, it registers its NF profile at the NRF as described in clause 5.2 or 6.2H.2.1.1 of TS 23.288[105].

2. The NWDAF or Internal AF acting as VFL Server discovers VFL Client(s) from NRF by invoking the NFDiscovery_Request service operation with VFL capability information per supported Analytics ID that includes VFL capability type (i.e. VFL client), VFL interoperability indicator per analytics ID, Analytics ID(s) as specified in clause 6.2H.2.1.1 of TS 23.288[105]. When an Internal AF is acting as a VFL server, only NWDAF(s) are discovered as VFL clients.

3. The NWDAF or Internal AF acting as VFL Server sends Nnrf_AccessToken_Get request to NRF as specified in clause 13.4.1. For NWDAF as server, the token request(s) includes the Analytics ID, AF ID (for external AF as client) and optionally VFL interoperability indicator per analytics ID For internal AF as server, the token request(s) includes the Analytics ID and optionally VFL interoperability indicator per analytics ID.

4. The NRF checks whether the NWDAF or Internal AF acting as VFL Server is authorized to access the NEF or NWDAF or Internal AF (VFL Client(s)) for VFL service(s) respectively as specified in clause 13.4.1,  optionally verifying the VFL interoperability indicator in the token request is in the VFL interoperability indicator(s) as received from VFL server and VFL client in Step 1. If the server NWDAF or Internal AF acting as server is authorized NRF will issue access token(s) as specified in clause 13.4.1. For external AF as VFL client  the token claim may include VFL interoperability indicator for that particular analytics ID, Analytics ID and AF ID. For NWDAF or Internal AF as VFL Client the token claim may include VFL interoperability indicator for that particular analytics ID and Analytics ID.

5. The NRF sends the access token(s) to the NWDAF or Internal AF acting as VFL Server, or rejects the request in case of failed authorization, as described in clause 13.4.1.

6a. External AF as VFL client: The NWDAF as VFL Server may send VFLService_Request (e.g. VFL Training, VFL Inference) to NEF and the token received in Step 5. The service request includes the Analytics ID, AF ID and optionally VFL interoperability indicator per analytics ID.

6b. NWDAF or internal AF as VFL client: The Internal AF or NWDAF acting as VFL Server may send VFLService_Request to NWDAF or Internal AF as VFL client and the token received in Step 5. The service request includes the Analytics ID and optionally VFL interoperability indicator per analytics ID.

7a. External AF as VFL client: The NEF verifies the access token received from the NWDAF to check whether the NWDAF is allowed to access the external AF for the requested analytics. The NEF extracts the Analytics ID from the access token and check whether it matches the Analytics ID received in step 6a. If the AF ID is included in the token, the NEF also verifies that the AF ID matches the AF ID requested in step 6a.

7b.  NWDAF or internal AF as VFL client: The NWDAF or Internal AF as VFL client verifies whether the access token received from the NWDAF or Internal AF as VFL Server includes Analytics ID and optionally VFL interoperability indicator per analytics ID received in step 6b.

8. External AF as VFL client: The NEF sends the VFLService_Request to the external AF as VFL client that includes the Analytics ID, AF ID and optionally VFL interoperability indicator per analytics ID. The AF performs authorization of VFL service request based on its local policy and implementation. Protection of NEF-AF interface is as specified in clause 12.

9a. External AF as VFL client: The external AF as VFL client sends VFL VFLService response (success or failure) to the NEF.

9b. External AF as VFL client: The NEF forwards the VFLService response to the NWDAF acting as the VFL server.

10. NWDAF or internal AF as VFL client: The NWDAF or Internal AF acting as VFL Client sends the VFLService response to the NWDAF/Internal AF acting as VFL server.

X.12.2.2	Authorization of candidate VFL participants for vertical federated learning when external AF is acting as the VFL server

Figure X.12.2.2-1 depicts the authorization mechanism for NWDAF as VFL Client and external AF as VFL server. The authorization is based upon the VFL capability information per supported analytics ID which includes VFL capability type (VFL server and/or VFL client), Analytics ID and VFL Interoperability Indicator per analytics ID provided by VFL members during registration.

Figure X.12.2.2-1: Authorization of candidate VFL participants for VFL when external AF is acting as the VFL server

1. In case of AF as VFL Server, the NEF updates its NF profile as specified in clause 6.2H.2.1.2 of TS 23.288[105], which including AF’s VFL capability information per supported analytics ID which includes VFL Capability type i.e. (VFL Client) and VFL interoperability indicator(s) per analytics ID and associated AF ID. In case of NWDAF as VFL Client it shall register to NRF with its NF profile as specified in clause 6.2H.2.1.2 of TS 23.288[105].

2. The AF sends a Nnef_NFDiscovery Request to the NEF with information Analytics ID, VFL capability type (VFL Client), VFL interoperability indicator to discover for VFL Client. The NEF discovers NWDAF as VFL client from the NRF by invoking the Nnrf_NFDiscovery Request as specified in 6.2H.2.1.2 of TS 23.288[105]. The NEF checks based on configured policies whether the AF is entitled to discover service to a VFL client as specified in clause 12.

NOTE: Handling of temporary NWDAF instance ID (s) is specified in clause 6.2H.2.1.2 of TS 23.288 [105].

3. The AF acting as VFL server sends VFL Service Request (e.g. VFL Training, VFL Inference)  towards NEF. The Service request shall contain the Analytics ID and optionally VFL interoperability indicator per analytics ID.

4. The NEF requests for an access token to the NRF as specified in clause 13.4.1 to perform VFL service towards the VFL clients (NWDAF). The token request shall contain the Analytics ID, AF ID and optionally VFL interoperability indicator per analytics ID.

5. The NRF checks whether NEF on behalf of VFL Server AF is authorized to access the VFL client NWDAF for the VFL service(s) as specified in clause 13.4.1, optionally verifying the VFL interoperability indicator in the token request is in the VFL interoperability indicator(s) as received from VFL server and VFL client in in Step 1.

6. If the NEF is authorized, the NRF will issue an access token in response. The token claim may also include VFL interoperability indicator for that particular analytics ID, Analytics ID and AF ID.

7. The NEF sends VFL Service Request to NWDAF (VFL Client) and the token includes the Analytics ID, AF ID and optionally VFL interoperability indicator per analytics ID as token received in Step 6.

8. The NWDAF verifies the received access token as specified in clause 13.4.1 and checks for the token claims as received in step 7.

9a. In case of successful access token verification, the NWDAF (VFL as client) sends a VFL Service Response to the NEF.

9b. The NEF sends the VFL Service Response to the AF (VFL as Server). Protection of NEF-AF interface is as specified in clause 12.

X.12.3	NEF security requirements

In case that NEF is involved, NF instance ID shall not be transmitted outside the 3GPP operator domain via NEF.

The NEF can be responsible for performing UE ID mapping, converting internal UE IDs (i.e., SUPI) to external UE IDs in accordance with the security requirement outlined in clause 5.9.2.3 of the present document.

X.12.4	Protection of communication data used in VFL process.

The current SBA mechanisms as defined in clauses 12.2 and 12.3 of  the present document for the NEF-AF interface and clause 13.1 of the present document for communication among VFL participants shall be used.

End of 2nd Change