| 3GPP TR 33.778 V0.0.1 (2025-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on providing Pre-Shared Key(PSK) for Multipath Extension for QUIC (MPQUIC) /Transport Layer Security (TLS) (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Examples for styles	8

4.1	Heading styles	8

4.2	Other common styles	8

"TSG <Name>" on the front page	9

Page setup parameters	9

Proforma copyright release text block	11

X.1	The right to copy	11

Abstract Test Suite (ATS) text block	12

Y	Abstract Test Suite (ATS)	12

Y.1	Introduction	12

Y.2	The TTCN Graphical form (TTCN.GR)	12

Y.3	The TTCN Machine Processable form (TTCN.MP)	12

Annex <A> (normative): <Normative annex for a Technical Specification>	13

Annex <B> (informative): <Informative annex for a Technical Specification>	14

B.1	Heading levels in an annex	14

Annex <C>: <Informative annex title for a Technical Report>	15

Annex <D> (informative): Bibliography	16

Annex <E> (informative): Index	17

Annex <X> (informative): Change history	18

Foreword

This Technical Specification|Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

- Scope

Editor’s Note: This clause is going to capture the scope of this study.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[3]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport".

[4]	IETF RFC 9001: "Using TLS to Secure QUIC".

[5]	draft-ietf-quic-multipath: "Multipath Extension for QUIC".

[6]	IETF RFC 8446: “The Transport Layer Security (TLS) Protocol Version 1.3”.

[7]	3GPP TS 33.210: “Network Domain Security (NDS); IP network layer security”.

[8]		3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[9]		3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[10]	3GPP TS 33.535: "Authentication and key management for applications based on 3GPP credentials in the 5G System (5GS)".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Architecture assumption

Annex AA in TS 33.501[2] is the starting point of this study.

5	Key issues

5.1	Key issue #1: PSK support for MPQUIC TLS

5.1.1	Key issue details

In TS 33.501 [1] Annex AA.2, server authentication for MPQUIC/TLS [2], [3], [5] is specified. The scope of this key issue is to cover the PSK-based option for MPQUIC/TLS. Solutions to this key issue are expected to provide the means for enabling the PSK option for MPQUIC/TLS. More specifically, the PSK option refers to TLS 1.3 PSK with (EC)DHE key establishment (psk_dhe_ke), since MPQUIC/TLS [4] uses TLS 1.3 [6] and TS 33.210 [7] prohibits the use of PSK-only mode (psk_ke) in TLS 1.3.

5.1.2	Security threats

N/A

5.1.3	Potential security requirements

The 5G system shall be able to securely derive, deliver, update, and use the PSK (i.e., TLS 1.3 psk_dhe_ke) between UE and UPF to be used for authentication with MPQUIC/TLS.

5.X	Key Issue #X: key issue names

5.X.1	Key issue details

Editor’s Note: This clause is going to capture the key issue detail of a key issue.

5.X.2	Security threats

Editor’s Note: This clause is going to capture the security threat of a key issue.

5.X.1	Potential security requirements

Editor’s Note: This clause is going to capture the potential security requirements of a key issue.

6	Solutions

6.1	Solution #1: MPQUIC/TLS using PSK derived from KgNB

6.X.1	Introduction

This solution addresses Key issue #1 by enabling a secure UP communication channel between the UE and the UPF. The approach leverages the current KgNB to derive a pre-shared key (UPF_PSK) and a corresponding identifier (UPF_PSK ID). The UPF_PSK/ID is delivered to the UPF and then used for a mutual-authentication and key exchange using TLS 1.3 PSK psk_dhe_ke.

6.1.2	Solution details

Assumptions and scope:

- UE is registered to the 5GS and has established a KgNB with the network.

- Distribution path for UPF_PSK/ID: AMF/SMF → UPF over N2/N4.

Key derivation and identifiers:

- UE and AMF derive UPF_PSK and UPF_PSK ID using current KgNB.

- Input parameters for the KDF include at least the PDU Session ID and a freshness parameter. UPF_PSK derivation can additionally be bound to the selected UPF identity (e.g., FQDN or IP).

Editor’s Note: Motivation for derivation of UPF_PSK using KgNB is FFS.

Editor’s Note: Handling of UPF_PSK derivation in Xn handover scenario is FFS

Setup procedure (PDU Session establishment):

- UE requests a PDU Session indicating support for MPQUIC/TLS using PSK.

- SMF selects a suitable UPF and provides UE with UPF addressing (e.g., IP, port) and obtains UPF_PSK/ID from AMF.

- AMF derives UPF_PSK/ID from current KgNB. SMF forwards UPF_PSK/ID to UPF via N4.

- Upon successful PDU Session Establishment, UE initiates MPQUIC/TLS with the UPF using UPF_PSK, referencing UPF_PSK ID for UPF to locate and use UPF_PSK to perform mutual authentication with the UE.

UPF_PSK update triggers and handling:

- UE CM-IDLE → CM-CONNECTED transition:

- UE and AMF derive new UPF_PSK/ID.

- AMF/SMF updates the UPF with the new UPF_PSK.

- UE initiates MPQUIC/TLS with the UPF using the new UPF_PSK/ID.

Editor’s Note: applicability of the update trigger condition is FFS.

6.1.3	Evaluation

Editor’s Note: Evaluation is FFS

6.2	Solution #2: PSK derivation bound with MA PDU session

6.2.1	Introduction

According to TS 23.501 [8] clause 5.32.6, for steering functionalities based on MPQUIC that apply the QUIC protocol and its multipath extensions, the MPQUIC functionality(ies) in the UE communicates with the associated MPQUIC Proxy functionality(ies) in the UPF. The MPQUIC functionality in the UE and the associated MPQUIC Proxy functionality in the UPF uses the "MPQUIC link-specific multipath" addresses/prefixes for transmitting traffic flows over non-3GPP access and over 3GPP access. The "MPQUIC link-specific multipath" IP addresses/prefixes are allocated by the UPF and provided to the UE via SM NAS signalling. For multiple paths sharing the same TLS tunnel, it is proposed that:

-	On the UE side, the PSK is derived by the UE and used by the MPQUIC functionality in the UE.

-	On the network side, the PSK is used by the associated MPQUIC Proxy functionality in the UPF. The PSK is derived by the SMF or the AMF which holds the root key for PSK derivation and the derived PSK is delivered to the UPF.

-	The PSK is bound with a specific MA PDU session, in which way the old PSK used on authentication for an existing MA PDU session cannot be reused on authentication for a new MA PDU session.

6.2.2	Solution details

To bound the PSK with a specific MA PDU session, it is proposed to use an identity which can uniquely identify the MA PDU session on both the UE side and network side as an input parameter for PSK derivation. It can be the PDU session ID or IP address of the MA PDU session, given that both the UE and the SMF have the PDU session ID and IP address of the MA PDU session.

When deriving a PSK in the SMF or the AMF and the UE, the following parameters are used to form the input S to the KDF:

-	FC = TBD

-	P0 = ID of the MA PDU Session or IP address of the MA PDU Session

-	L0 = Length of P0

-	P1 = SUPI

-	L1 = Length of P1

The input key KEY could be the KAMF or KSEAF or an intermediate key derived from KAMF or KSEAF.

Editor’s Note: The impact on the SMF for key handling is to be captured in the evaluation clause.

The intermediate key derived from KAMF or KSEAF could be the KSMF, which is derived using the following parameters to form the input S to the KDF:

-	FC = TBD

-	P0 = SMF instance ID

-	L0 = Length of P0

The input key KEY could be the KAMF or KSEAF.

Editor’s Note: The use of KSEAF requires the storage of KSEAF. The impact on the legacy handling of KSEAF is to be captured in the evaluation clause.

6.2.3	Evaluation

Editor’s Note: This clause is going to capture the pros and cons of the solution, e.g. whether the threats are addressed totally, how the existing 5G system is impacted, whether there is any leftover issues exists, etc.

6.3	Solution #3: PSK delivery during MA PDU session establishment

6.3.1	Introduction

According to TS 23.502 [9] clause 4.22.2, when receiving the UE requested PDU session establishment request with Request Type as "MA PDU Request", the AMF supporting MA PDU sessions selects an SMF supporting MA PDU sessions. It is proposed that:

-	When selecting an SMF supporting MA PDU, the AMF sends a key to the SMF for PSK derivation.

The SMF determines to use MPQUIC for the new PDU session based on TS 23.502 [9] clause 4.22.2, then selects and configures the selected UPF supporting MPQUIC. It is proposed that:

-	When determining that MPQUIC is to be used for the PDU session, the SMF derives the PSK;

-	When configuring the UPF, the SMF provides the derived PSK to the UPF.

On the UE side, when the UE receives a PDU Session Establishment Accept message indicating that the requested MA PDU session was successfully established, the message will include the ATSSS rules for the MA PDU session derived by SMF. If MPQUIC functionality is supported for the MA PDU Session, the SMF will include the "MPQUIC link-specific multipath" addresses/prefixes of the UE and the MPQUIC proxy information that corresponds to the activated MPQUIC-based steering functionality in the ATSSS rules. It is proposed that:

-	The UE derives the PSK when receiving the ATSSS rules from the SMF containing the "MPQUIC link-specific multipath" addresses/prefixes of the UE and the MPQUIC proxy information.

-	The UE then uses the derived PSK to authenticate with the UPF using MPQUIC/TLS protocol.

6.3.2	Solution details

The detailed procedure is shown in Figure 6.3.2-1.

Figure 6.3.2-1: MPQUIC/TLS Security Establishment during MA PDU session establishment

1.	The UE provides Request Type as "MA PDU Request" in UL NAS Transport message and its ATSSS capabilities in PDU Session Establishment Request message.

2.	Based on Request Type as "MA PDU Request" received from the UE, if the AMF supports MA PDU sessions, the AMF selects an SMF which supports MA PDU sessions. The AMF informs the SMF that the request is for a MA PDU Session by including "MA PDU Request" indication.

In addition, the AMF may send a derived PSK to the SMF or send a root key to the SMF for PSK derivation.

The root key could be the KSMF derived from KAMF or KSEAF.

3.	The SMF retrieves, via Session Management subscription data, the information whether the MA PDU session is allowed or not.

4.	The SMF returns a Nsmf_PDUSession_CreateSMContext Response to the AMF.

5.	The SMF determines the ATSSS capabilities supported for the MA PDU Session taking into consideration the ATSSS capabilities provided by the UE and per DNN configuration on SMF. The SMF initiates an N4 Session Establishment/Modification procedure with the selected UPF. If the MPQUIC functionalities are supported for the MA PDU Session, the SMF instructs the UPF to activate the corresponding functionalities for this MA PDU Session. The SMF receives the UE IP address of the MA PDU session from the UPF.

6.	Upon receiving a positive N4 Session Establishment/Modification Response, the SMF derives the PSK from the root key if received from the AMF.

Alternatively, the SMF can also decide to derive the PSK at step #12 after receiving positive PDU session response from the AMF.

If the AMF does not send a root key in step #2, the SMF sends a key request to the AMF/SEAF to acquire the PSK derived by the AMF/SEAF or retrieve the root key before deriving the PSK.

The PSK derivation refers to solution #2.

7.	The SMF sends the Namf_Communication_N1N2MessageTransfer message to the AMF.

8.	The AMF sends the PDU Session Request message to the gNB.

9.	The gNB issues AN specific signalling exchange with the UE that is related with the NAS information received from SMF.

10a. Upon receiving the ATSSS rule in the NAS message from the AMF, if ATSSS rule contains the "MPQUIC link-specific multipath" addresses/prefixes of the UE and the MPQUIC proxy information, the UE determines to derive the PSK from the root key in the same way as the AMF or SMF.

10b. After AN specific signalling exchange with the UE, the gNB returns the PDU Session Response message to the AMF.

11. The AMF sends the Nsmf_PDUSession_UpdateSMContext Request to forward the N2 SM information received from gNB to the SMF.

12.	The SMF derives the PSK if not received in step #2 or not derived in step #6.

13.	The SMF sends the PSK to the UPF in the N4 Session Modification Request.

14.	The UE and UPF perform authenticate using MPQUIC/TLS based on the PSK.

15.	The UPF returns the N4 Session Modification Response to the SMF.

Editor’s Note: Key update for reauthentication is FFS.

Editor’s Note: Key derivation and delivery from serving network to home network in roaming scenarios is FFS.

6.3.3	Evaluation

Editor’s Note: This clause is going to capture the pros and cons of the solution, e.g. whether the threats are addressed totally, how the existing 5G system is impacted, whether there is any leftover issues exists, etc.

6.4	Solution #4: Using 5G security context to derive authentication pre-shared key for MPQUIC

6.4.1	Introduction

This solution addresses key issue #1 “PSK support for MPQUIC TLS”.

This solution proposes to derive authentication pre-shared key from the 5G security context to establish the security of MPQUIC for UE and UPF.

6.4.2	Solution details

6.4.2.1 The procedure for PSK retrieval

Considering UE and network already generated shared security context during the registration procedure, a sub-level shared key can be generated, and be used as a pre-shared key for MPQUIC.

AMF derives the KUPF from KAMF during the PDU session establishment procedure as shown in the following procedure (Figure 6.4.2.1).

Figure 6.4.2.1 MA PDU session using MPQUIC functionality establishment procedure

1. UE sends PDU session request to AMF which carries an MA PDU request type, PDU session ID and ATSSS capability for the UE as defined in TS 23.502[9].

2-3. AMF selects MA PDU session enabled SMF and forwards PDU session request to SMF as defined in TS 23.502[9].

4. The SMF determines whether the MA PDU session is allowed or not based on operator policy and subscription data, and selects ATSSS enabled UPF as defined in TS 23.502[9]. If the SMF activates MPQUIC functionality, it will derive ATSSS rules and N4 rules for the MA-PDU session as defined in TS 23.502[9].

5. SMF send key request to AMF which carries the UE’s SUPI and PDU session ID,

6. AMF derives KUPF for the UE according to the PDU session ID, generates a KID from PDU session ID and the corresponding UE ID (i.e. SUPI), and sends the KUPF and KID to SMF.

7. Then the SMF initiates the N4 Session Establishment procedure with the selected UPF and sends the KUPF and KID to UPF.

8. The UPF stores the KUPF and the KID for the KUPF.

9. The UPF sends the N4 Session Establishment response message to the SMF.

10-11. Since the UE and the UPF can use certificate or pre-shared key to establish MPQUIC connection. The SMF sends the Using_PSK_indication to the UE in order to inform UE to use PSK for MPQUIC connection establishment.

12. UE derives the key KUPF used for authentication of MPQUIC between UE and UPF according to the Using_PSK_indication and generates KID for KUPF using PDU session ID and its own identifier as defined in clause 6.4.2.4.

13. The UE starts the MPQUIC Establishment procedure to the UPF, and uses KUPF as pre-shared key and KID as the pre-shared key identifier to do the TLS handshake and authentication procedure.

Editor’s Note: roaming scenario is FFS.

Editor’s Note: Key update for reauthentication is FFS.

6.4.2.2 Key hierarchy

The key hierarchy defined in TS 33.501[2] for this scenario can be extended as follows:

Figure 6.4.2.2 Key hierarchy for KUPF retrieval

A new key KUPF is derived from KAMF as depicted in Figure 6. 4.2.2.

6.Y.2.3 KUPF generation

The KUPF is generated by KAMF using the following input parameters.

-	FC = 0xXX

-	P0 = PDU session ID

-	L0 = length of PDU session ID

-	P1 = NAS Uplink COUNT value

-	L1 = length of NAS Uplink COUNT value

The input key KEY is KAMF.

6.4.2.4 Key ID generation

The Key ID is generated from the PDU session ID and UE ID (i.e. SUPI) as follows:

KID = H(SUPI)|| PDU session ID

6.4.3	Evaluation

This solution proposes a solution of deriving authentication pre-shared key from the 5G security context to establish the security of MPQUIC for UE and UPF.

AMF has to derive a key for UPF after SMF determines that MPQUIC functionality will be used and send a request to AMF. UPF has to store the key and the corresponding key identifier in order to use it in the following TLS handshake procedure. For the UE side, KUPF will be derived after the UE receives an Using_PSK_indication indicator from the SMF.

Editor’s Note: further evaluation is FFS.

6.5	Solution #5: two layer PSK generation method

6.5.1	Introduction

This solution proposes a two layer key generation. The AMF will use KAMF generates a Key KSMF and send the KSMF to the selected SMF. The SMF will further generate KUPF using KSMF, and then deliver the key KUPF to the UPF. Meanwhile, the SMF also generates a key ID, and the Key ID is also sent to the UPF together with the KUPF.

6.5.2	Solution details

6.5.2.1 The procedure for PSK retrieval

Figure 6.5.2-1 Procedure to get a PSK between UE and UPF for MPQUIC

1. UE sends PDU Session Establishment Request message to the AMF. The message contains the MAP PDU session information defined in TS 23.502[9] and a PSK capability indication. The PSK capability indication is to indicate that the UE supports to generate a PSK for the MPQUIC/TLS between UE and UPF.

2. The AMF selects a SMF that supports MA PDU as described in TS 23.502[9].

3. The AMF sends  Nsmf_PDUSession_CreateSMContext Request. The message includes the MA PDU session information and the PSK capability indication.

4. The SMF decides MPQUIC may be used based on the decision as defined in TS 23.502[9], and knows the UE supporting to generate a PSK based on the PSK capability indication.

5. The SMF request the KSMF by sending a request message to the AMF. The message includes the SUPI of the UE.

6. The AMF generates the KSMF, and sends the KSMF to the SMF in the response message.

NOTE: this solution will not address the message name in step 5 and step6.

7. The SMF uses the KSMF to generate a KUPF  and a Key ID.

8. the SMF sends a N4 Session Establishment/modification Response to the UPF. In addition to what is defined in TS 23.502[9], the message further includes the KUPF  and a Key ID.

10 – 12. As defined in TS 23.502[9].

13. The UE generates the KSMF, the KUPF  and the Key ID the same way as AMF and SMF before the UE starts to use MPQUIC.

14. The UE sends a Client Hello message to the UPF, the message contains the Key ID.

15. The UPF uses the Key ID to retrieve the KUPF. The KUPF is used as the PSK for MPQUIC/TLS.

16. The UPF replies a Server Hello message to the UE.

17. The rest of MPQUIC procedure.

Editor’s Note: roaming scenario is FFS.

Editor’s Note: Key update for reauthentication is FFS.

6.Y.2.2 Key hierarchy

Figure 6.9.2-2 Key hierarchy for KUPF retrieval

Based on the procedure in clause 6.9.2.1, the AMF generates the KSMF by using the KAMF and deliver it to the SMF, and then the SMF uses the KSMF to generate the KUPF that will be further delivered to the UPF.

6.9.2.3 KSMF generation method

The KSMF is generated by KAMF reusing the method in A.13 of TS 33.501[2] with the following updated:

- Set the P0 input parameter DIRECTION to the value 0x02.

6.5.2.4 KUPF generation method

The KUPF is generated by KSMF using the method in A.13 of TS 33.501[2] with the following updated:

- Set the input KEY  to KSMF.

- Set the P0 DIRECTION to 0x01.

- Set the COUNT value is set to the value of PDU session ID.

6.5.2.5 Key ID generation method

The Key ID is generated by KSMF using the method in A.3 of TS 33.535[10] with the following updated:

- Set the input key KAUSF to KSMF.

- Set the P0 = "A-TID" to  P0 = "UPF Key ID”.

- Set the L0 = length of "A-TID"; (i.e. 0x00 0x05) to  L0 = length of " UPF Key ID "; (i.e. 0x00 0x05).

6.5.3	Evaluation

The solution considers the backward compatible issue to let the SMF knows whether the UE is upgraded to support generating PSK.

In 3GPP system, all PSKs in the key hierarchy are delivered in one hop only. Thus deliver the PSK to the UPF from SMF is not fully comply with the principle. In case that no new interface is introduced directly between AMF and UPF, it is better the AMF generate a middle key for SMF, and then the SMF generates the key for UPF. The less nodes know the PSK, the better.

The key generation method is based on existing method, the solution proposes to reuse the existing key generation as much as possible. If a parameter can be updated to achieve the goal, then no need to introduce a fully new key generation scheme.

A Key ID is used for UPF to find the right PSK.

This solution needs to change SMF to support storage of KSMF and generation of KUPF and a key ID.

Editor’s Note: Further evaluation is FFS.

## 6.6	Solution #6: Key derivation and delivery to UE and UPF

### 6.6.1	Introduction

The following solutions addresses KI#1 by proposing a mechanism to derive the key inside the 5G core and distribute it to both UE and UPF. Additionally, it proposes a mechanism to initiate re-authentication by deriving and delivering new keys to UE and UPF.

### 6.6.2	Solution details

6.6.2.1 Key derivation and distribution

1. A Multi-Access PDU session is established and one or more ATSSS rules require the use of MPQUIC.

2. The UPF request SMF the pre-shared secret for the session with the UE.

3. SMF forwards the Key request to AMF.

4. AMF generates the new key by deriving it from KAMF. The following parameters should be use as input to the KDF:

-	FC= 0xWX

-	P0= Random Number

-	L0= P0 length

5.a. AMF sends a response to SMF containing the generated key.

5.b. AMF send the key and PDU session ID to UE to identify where the correct session to use the key.

6. SMF forwards the response, along with the Key and an identifier of the UE to UPF.

7. UE and UPF authenticate each other and initiate the MPQUIC connection as supported in ATSSS based on the pre-shared secret, i.e., the key.

Editor’s Note: Key derivation and delivery from serving network to home network in roaming scenarios is FFS.

6.6.2.2 Re-Keying mechanism

1. MPQUIC connection has been set up through PSK.

2. Either UE or 5G core requires to renew the pre-shared secret.

3. AMF generates a new key through the same mechanism used during the initial key derivation.

4.a. AMF sends notification of the new Key to UE.

4.b. AMF replies to SMF with the new key.

5. SMF provides the new key to UPF.

6. UE and UPF gracefully terminate the current MPQUIC session.

7. UE and UPF establish a new one based on the pre-shared key.

Editor’s Note: Key update for reauthentication is FFS.

Editor’s Note: The need for a key renewal is FFS.

### 6.Y.3	Evaluation

The solution completely addresses the problem highlighted by KI#1 both for initial authentication of the connection and for update of the key in case of a compromise. The security is achieved by deriving a new dedicated key for each MPQUIC connection, ensuring that each connection is independently secured, and the compromise of one key will not impact the security of the overall system.

The solution impacts AMF by enhancing its key derivation capabilities to support the new use case. Additionally, it defines delivery mechanism which impact SMF, as both initiator of the procedure and intermediate layer between AMF and UPF, and UPF in the 5G core and the connection towards the UE.

The solution relies on AS security to ensure the confidentiality of the PSK, deactivating the AS security will impact the security of the solution.

Editor’s Note: Further eval is FFS.

6.X	Mapping of solutions to key issues

Editor’s Note: This clause is going to capture mapping between key issues and solutions. If there is only one key issue in this study, this clause will be removed.

6.Y	Solution #Y: solution names

6.Y.1	Introduction

Editor’s Note: This clause is going to capture the abstract of the solution to address one or more key issues. Which requirements of the key issue shall be included, and what is the key point of the solution is recommended to be listed here as a guidance for the solution details.

6.Y.2	Solution details

Editor’s Note: This clause is going to capture the details of the whole solution, figures and flows are recommended to be used for better understanding the core of the solution.

6.Y.3	Evaluation

Editor’s Note: This clause is going to capture the pros and cons of the solution, e.g. whether the threats are addressed totally, how the existing 5G system is impacted, whether there is any leftover issues exists, etc.

7	Conclusions

Editor’s Note: This clause is going to capture the conclusions of this study.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025.10 | SA3#124 | S3-253745 |  |  |  | The merger of S3-253753,711,712,713,714,715,717,718,415 |  |