| 3GPP TS 33.502 V1.1.0 (2026-02) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Security related Events Handling (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Overview of Security related Events handling	8

5	Security related events requirements	9

5.1	General Requirements	9

5.2	Requirements on events storage	9

5.3	Requirements on configuration for security related events	9

5.4	Requirements on delivery of security related events	9

6	Security related Events	10

6.1	General	10

6.2	 Common information elements	10

6.3	 Security events related to malformed messages	10

6.4	 Security events related to Authorization Failure	11

6.5	 Security events related to Authentication Failure	11

6.6	 Security events related to massive number of incoming messages	12

6.7	Security events related to SBA parameters configuration	12

6.8	Security events related to Unexpected communication model flow	13

7	Protection of Security related events	13

7.1	Protection for the configuration and enabling/disabling detection of security related events	13

7.2	Protection for the delivery of security related events	13

Annex <X> (informative): Change history	15

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies general requirements for security related events handling and collection as well as the general requirements to transfer or communicate the security related events occurring at the SBA layer of the 5G system. The protection mechanisms to be applied for configuration and delivery of the events are also specified.

In addition, the present document specifies the events that need to be reported, including how the event is detected and the elements that needs to be included in the reporting.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]					3GPP TS 29.500 "Technical Realization of Service Based Architecture; Stage 3"

[3]					3GPP TS 33.501: "Security architecture and procedures for 5G System"

[4]					3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)"

[5]					3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security"

[6]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3”

[7]	3GPP TS 28.570: “Management of Security related Events; Stage 1, Stage 2 and Stage 3”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

# 4	Overview of Security related Events handling

The Service Based Architecture (SBA) is the dominant method for control plane as well as the Service Based Management Architecture (SBMA) which is for management communications. In addition to the many benefits of using SBA (e.g. agility to increase and decrease the number of service instances in coordination with demand), there can be potential attacks to the network, services and/or APIs.

The 5G system includes various Network Functions (NF) deployments, where each and every NF has a specified behaviour according to 3GPP specifications. If any NF runs into unexpected behaviours (e.g. a violation of normal behaviour, abnormal access or unauthorised request), then the NF needs to be evaluated from a security perspective. The collection of security related events need to be performed for the evaluation of the NF behaviour. The related data is transmitted to a security entity which will execute the evaluation.

The following figure shows an example of trust domains in the overall architecture for the collection and transmission of the Security related Events detected by the NFs.

Figure 4-1 Example of trust domains in the Security related Events Handling architecture

The NF gets configuration of security related events from the Management entity and delivers security related events through the Event transmitter. The Event transmitter could be an independent function or part of the NF. The Event transmitter is recommended to be part of an NF. Whether the Event transmitter is inside the NF, outside the NF and inside the 5GC, or outside the 5GC, as depicted in Figure 4-1, is based on operator implementation.

The Event transmitter, Management Entity and Security related event collecting entity are the end points of the related configuration and collection interfaces. The interfaces for collection of security related events are in scope of the present document. Requirements on the configuration of security related events are in scope of the present document. The interfaces between the Event transmitter and the NFs are out of the scope of 3GPP.

NOTE 1: Operators will define the relationship between trust domains.

NOTE 2: The Security related events collecting entity is under operator control (e.g. through business agreements, policy, managed service, directly managed, etc) and it is out of the scope of 3GPP.

NOTE 3: Whether the security collecting entity is the same as the management entity is an operator decision.

# 5	Security related events requirements

## 5.1	General Requirements

The NFs in the 5G system shall support the generation of security related events.

## 5.2	Requirements on events storage

Security related events data shall be securely stored with confidentiality and integrity protection.

Access to security related events data shall be authorized.

## 5.3	Requirements on configuration for security related events

The capability to configure the NFs shall be supported.

The Management entity in charge of configuring the NFs shall support configuring the security related events for event detection and for events delivery.

The Management entity in charge of configuring the NFs shall support activating and deactivating the security related events detection and delivery.

NOTE1: The security related events to be configured are specified in clause 6.

NOTE 2: The structure of the information elements included in the configuration is part of stage 3 protocol design.

It shall be possible to configure the NF to stop sending events in case of overload at the Security related events collection entity.

NOTE 3:	There is a risk that there is a DoS attack to the Security release event collections entity which can be mitigated by throttling or disabling event reporting.

The 5G system shall support mutual authentication between the 5GC NF (for configuration/activation of the functionality) and the Management Entity in charge of the configuration/activation of the events.

Authorization to the Management Entity in charge of the configuration/activation of the events shall be supported.

The 5G system shall support integrity protection, replay protection and confidentiality protection for communication between the 5GC NF and the Management Entity in charge of the configuration/activation of the events.

NOTE 4: The configuration of the security related events is specified in TS 28.570[7]

## 5.4	Requirements on delivery of security related events

The delivery of security related events shall be protected against unauthorized parties. Mutual authentication shall be supported between the end entities of such a delivery.

The delivery of security related events shall be confidentiality, integrity and replay protected.

The delivery of the security related events should be separate from other 5G system traffic.

NOTE: The separation of the delivery depends on regional constraints and/or operator needs. How the separation is achieved is part of the stage 3 work.

## 5.5		Other requirements

The security related events collection entity shall have access to overload related information of the NF.

NOTE: 	Overload related information of the NF can be available through the O&M FM/PM (Fault Management / Performance Management) northbound interfaces.

The security related events collection entity shall have access to configuration related information of the NF.

# 6	Security related Events

## 6.1	General

The security related event consists of two parts: common information elements, and specific information elements. The common information elements are specified in section 6.2, and specific information elements are specified in separated clauses.

## 6.2		Common information elements

The common information elements for all security related events shall consist of the following:

- Event name: The name of the event in a human-readable format; e.g., "malformed message"

- Event code: The name of the event in a machine-readable format; e.g., a machine-readable identifier corresponding to "malformed message"

- Event instance: Identification of the event instance

NOTE 1: The uniqueness of the event instance is left to implementation.

- Event Source: Identification of the NF generating the event;

- Event timestamp.

NOTE 2: The identification of events and the format of the information elements is part of the stage 3 design.

## 6.3		Security events related to malformed messages

The NF collects information on the SBA layer about malformed messages it receives that deviate from the 3GPP specified messages or are considered invalid according to the protocol specification and network state.

In addition to the information elements of clause 6.2, this type of events shall include the following:

- Message: malformed message which triggers event. If the message length exceeds a certain, implementation specific, threshold, the message can be truncated.

NOTE 1: Including the whole malformed message could lead to DoS at the Security related events collection entity if the malformed message is very large.

- NF Consumer (optional): Identification of the NF where such malformed message originated.

NOTE 2: The message source and intermediaries are contained in the 3gpp-Sbi-NF-Peer-Info header (specified in TS 29.500 [2]) when included in the full message. If the 3gpp-Sbi-NF-Peer-Info header is not included, the NF Consumer information is potentially not available. In this case, including the NF Consumer information is left to implementation.

## 6.4		Security events related to Authorization Failure

The NF collects information about failed authorization attempts from inbound connections on the SBA layer.

In addition to the information elements of clause 6.2, this type of events shall include the following:

- Message: message which fails to pass authorization. If the message length exceeds a certain, implementation specific, threshold, the message can be truncated.

NOTE 1: Including the whole unauthorized message could lead to DoS at the Security related events collection entity if the unauthorized message is very large.

- NF Consumer (optional): Identification of the NF where the unauthorized message originated.

NOTE 2: The message source and intermediaries are contained in the 3gpp-Sbi-NF-Peer-Info header (specified in TS 29.500 [2]) when included in the full message. If the 3gpp-Sbi-NF-Peer-Info header is not included, the NF Consumer information is potentially not available. In this case, including the NF Consumer information is left to implementation.

## 6.5		Security events related to Authentication Failure

The NF collects information about failed authentication attempts from inbound connections on the SBA layer.

When the failed authentication attempt is at the TLS layer, in addition to the information elements of clause 6.2, this type of events should include the following:

-	Error details: Additional information about the authentication failure if available, e.g. error message received from the TLS stack or vendor specific information.

When the indirect NF communication mode is used, and when CCA (clause 13.3.8 in TS 33.501[3]) is used, an NF can detect an authentication failure at application layer and can reply with an HTTP status code 403 including the cause "CCA_VERIFICATION_FAILURE" or "TOKEN_CCA_MISMATCH ". Along with the reply, the NF can generate an authentication failure event with the following additional information apart from the common information elements in clause 6.2:

-	Message: message which fails to pass authentication at application layer. If the message length exceeds a certain, implementation specific, threshold, the message can be truncated.

- 	NF Consumer (optional): Identification of the NF where the unauthenticated message originated

NOTE:	The message source and intermediaries are contained in the 3gpp-Sbi-NF-Peer-Info header (specified in TS 29.500 [2]) when included in the full message. If the 3gpp-Sbi-NF-Peer-Info header is not included , the NF Consumer information is potentially not available. In this case, including the NF Consumer information is left to implementation.

## 6.6		Security events related to massive number of incoming messages

The massive number of incoming messages event may be detected when an NF, SCP or SEPP issues a number of HTTP status code 429 or HTTP status code 503 responses to a requesting/notifying NF as described in TS 29.500 [2] clause 6.4.2.1, or includes the optional OCI (Overload Control Information) header defined in TS 29.500 [2] in a response to a requesting or notifying NF.

NOTE 1: The event can be detected after a number of overload conditions have been reported (with HTTP status codes 429 or 503 or including OCI) during a predefined period of time. The behaviour of NF producers on overload condition is implementation specific so, this event generation depends on vendor  implementation and operator policy.

In addition to the information elements of clause 6.2, this type of events shall include the following:

- Message: The incoming request message which triggered status code 429, 503, or OCI. If the message length exceeds a certain, implementation specific, threshold, the message can be truncated.

- Problem details: e.g. status code 429, 503, or OCI.

- NF consumer (optional): Identification of the NF where the message originates.

NOTE 2:	The message source and intermediaries are contained in the 3gpp-Sbi-NF-Peer-Info header (specified in TS 29.500 [2]) when included in the full message. If the 3gpp-Sbi-NF-Peer-Info header is not included, the NF Consumer information is potentially not available. In this case, including the NF Consumer information is left to implementation.

## 6.7	Security events related to SBA parameters configuration

An NF configuration is received in which related SBA level parameters (e.g., the SBA NF certificate) are changed/updated.

The NF shall detect a reconfiguration event. The event shall be reported to the security related event collection entity as described in clause 4 and clause 5.4.

The event should be delivered to the Security related events collection entity via an event transmitter in the operator trust domain.

NOTE: Direct delivery through event transmitters in the 5GC is preferred over using event transmitters outside of the core.

Examples of SBA level parameters include:

-	The certificate, containing NF identifiers like FQDN, Issuer information, subject public key info, etc. If the parameter is changed, it can potentially mean the affected NF is recognized as another NF (see TS 33.310 [4]).

-	The ManagedNFProfile, contains the profile definition of a managed NF. If such parameter is changed, affected NF can potentially be illegally accessed (see TS 28.541 [6]).

-	The commModelList, contains the communication model that a specific NF fulfills. If the parameter is changed, it can potentially mean that affected NF changes its behavior (see TS 28.541 [6]).

-	For SEPPs, SBA level parameter is seppInfo which can impact the PRINS protection (see TS 33.501 [3] and TS 28.541 [6]),

-	For SCPs, supportedFunction, address and scpInfo are essential to provision SCPs (see TS 28.541 [6]).

Editor's Note: Whether the event format is specified or left to implementation is FFS

Editor’s Note: This event including its details is for FFS.

## 6.8	Security events related to Unexpected communication model flow

Unexpected communication model flows can be detected when an NRF, SCP or an NF Producer receive requests not complying with the specified procedures related to communication models.

Editor’s note:	It is for FFS what (additional) events are reported by NF (NRF, SCP or NF producer).

# 7	Protection of Security related events

## 7.1	Protection for the configuration and enabling/disabling detection of security related events

TLS shall be supported and used to provide mutual authentication, integrity protection, replay protection and confidentiality protection for the interface handling the configuration and the enabling/disabling of events collection.

NOTE 1: If the interface is trusted (e.g. physically protected), it is for the PLMN-operator to decide whether to use cryptographic protection.

Security profiles for TLS implementation and usage shall follow the TLS profile given in clause 6.2 of TS 33.210 [5] and the certificate profile given in clause 6.1.3a of TS 33.310 [4]. The identities in the end entity certificates shall be used for authentication and policy checks.

NOTE 2: A PLMN-operator policy can use dedicated certificates for this secure communication.

## 7.2	Protection for the delivery of security related events

The protection mechanism to provide mutual authentication, integrity protection, replay protection and confidentiality protection is to be implemented at transport layer.

When UDP is used as transport protocol for the delivery of security related events, DTLS shall be supported to provide mutual authentication, integrity protection, replay protection and confidentiality protection between the Events transmitter and the Security collecting entity. When TCP is used as transport protocol for the delivery of security related events, TLS shall be supported to provide mutual authentication, integrity protection, replay protection and confidentiality protection between the Events transmitter and the Security collecting entity.

NOTE 1: If the interface is trusted (e.g. physically protected), it is for the PLMN-operator to decide whether to use cryptographic protection.

Security profiles for DTLS and TLS implementations and usage shall follow the TLS profile given in clause 6.2 of TS 33.210 [5] and the certificate profile given in clause 6.1.3a of TS 33.310 [4]. The identities in the end entity certificates shall be used for authentication and policy checks.

NOTE 2: A PLMN-operator policy can use dedicated certificates for this secure communication.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 08/2025 | SA3#123 | S3-252547 |  |  |  | Initial draft | 0.0.1 |
| 08/2025 | SA3#123 | S3-252991 |  |  |  | Includes agreed tdocs S3-252547, S3-252992, S3-252993, S3-252994 and S3-252995 | 0.1.0 |
| 10/2025 | SA3#124 | S3-253792 |  |  |  | Includes agreed tdocs S3-253790, S3-253791, S3-253793, S3-253794, S3-253795 | 0.2.0 |
| 11/2025 | SA3#125 | S3-254021 |  |  |  | Includes agreed tdocs S3-254125, S3-254672, S3-254673, S3-254674, S3-254675, S3-254676, S3-254677, S3-254679, S3-254680, S3-254681, S3-254682 | 0.3.0 |
| 2025-12 | SA#110 | SP-251520 |  |  |  | Presented for information | 1.0.0 |
| 2026-02 | SA3#126 | S3-260777 |  |  |  | Includes agreed tdocs S3-260615, S3-260616, S3-260619, S3-260778, S3-26-260779, S3-260780, S3-260781, S3-260782, S3-260783 | 1.1.0 |