3GPP TSG-SA3 Meeting #104-e 	S3-21xxxx

- e-meeting, 16 - 27 August 2021	Revision of S3-21xxxx

- Source:	InterDigital

- Title:	Analysis of solutions for L3 U2N Relay Authorization and security

- Document for:	Discussion

Agenda Item:	5.9

# 1	Decision/action requested

This contribution proposes an analysis of TR 33.847 solutions for L3 U2N Relay Authorization and security

# 2	References

[1]	TR 33.847

[2]	TS 23.304

[3]	TS 33.536

[4]	S3-211401, LS on Layer-3 UE-to-Network Relay authentication and authorization

# 3	Rationale

This document presents an analysis of the solutions for L3 U2N Relay authorization and security to be used for discussions to progress the conclusions for the related KIs (KI#3, KI#4, KI#9).

# 4	Discussion

The solutions for L3 U2N Relay authorization and security can be classified as user-plane (UP) or controlled-plane (CP) based solutions.

The following table summarizes the analysis of the solutions with respect to the identified evaluation criteria below. The solutions mapping to key issues is available in [1] TR 33.847 clause 6.0.

The solutions (sol#13, #24, #25, #34) for the support of secondary or slice-specific authentication and authorization (under KI#4) are not covered in the table below however such support needs to be considered as part of the selection process for the solutions below.

|  | CP/UP based | UE (ME) impact | Network impact | Security impact | PC5 keys management | Conformance to SA2 architecture (PCF based authorization/provisioning) | Remote UE Out of coverage support | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol#1 | CP | Yes | AMF AUSF | GUTI reallocation/registration update with Remote UE context transfer to relay's AMF FFS Security context transfer not possible if Remote and Relay are served by different PLMNs How does Relay's AMF discover AUSF with latest KAUSF? | Remote UE/AUSF: REAR key = KDF (KAUSF) Remote UE/AMF: PC5 root key = KDF (REAR key) AMF => Relay: PC5 root key | No (uses DDNMF along PCF) | No (Remote UE needs NAS messaging to request key) | KI#9 |
| Sol#6 | UP | Yes | New AF (PKMF) Possibly UDM | Requires GBA or AKMA support for PKMF connection security PC5 key ID in DCR may be exposed to privacy threat. Support for revocation of authorization of Remote UE to use relay is not specified | Relay's PKMF generates PC5 key. Remote UE/PKMF: PC5 root key = KDF (PC5 key) PKMF => Relay: PC5 root key | No (uses new PKMF) | No (UE needs UP connection to get key material from PKMF) | KI#4, #9 Remote and Relay UE belong to the same PLMN |
| Sol #10 | CP | Yes | AMF AUSF | GUTI reallocation/registration update with Remote UE context transfer to relay's AMF FFS (option 2) | Remote UE/AMF: PC5 root key = KDF (KAMF) AMF => Relay: PC5 root key | Yes | Yes (Remote UE signaling/traffic over PC5) | KI#3, #4, #9 |
| Sol #15 | CP | Yes | AMF AUSF UDM | Change of KAUSF requires Remote UE to retrieve new key material from AUSF for PC5 security P-KID in DCR may be exposed to privacy threat. Support for revocation of authorization of Remote UE to use relay is not specified | Remote UE/AUSF: Key id = KDF (KAUSF) AUSF => UDM: Key id Remote UE/AUSF: Pc5 root Key = KDF (KAUSF) AMF => Relay: PC5 root key | Yes | No (Remote UE needs NAS messaging to request key ) | KI#3, #4, #9 |
| Sol #18 | UP | Yes | New AF (PKMF) | Requires GBA or AKMA support for PKMF connection security. Support for AKMA while roaming is FFS PRUK ID in DCR may be exposed to privacy threat. Support for revocation of authorization of Remote UE to use relay is not specified. KNRP derivation during DSMC is not aligned with [3] TS 33.536 (done before DSMC). | Relay's PKMF generates PRUK. Remote UE/PKMF: PC5 root key = KDF (PRUK) PKMF => Relay: PC5 root key | No (uses new PKMF) | No (UE needs UP connection to get key material from PKMF) | KI#3, #4 |
| Sol #19 | UP | Yes | PCF (RSC associated with N3IWF connection) | Authorization for Remote UE to access the Relay PDU Session covered in other solutions for KI#4 | PC5 link security assumed with existing mechanisms and/or other solutions | Yes | Yes (Remote UE signaling/traffic over PC5) | KI#3 Uses N3IWF. |
| Sol #21 | UP | Yes | New AF (PKMF) | Requires GBA or AKMA support for PKMF connection security. Support for AKMA while roaming is FFS. PC5 key ID in DCR may be exposed to privacy threat. PC5 root key derivation during DSMC during DSMC is not aligned with [3] TS 33.536 (done before DSMC). Support for revocation of authorization of Remote UE to use relay is not specified | Relay's PKMF generates PC5 key. Remote UE/PKMF: PC5 root key = KDF (PC5 key) PKMF => Relay: PC5 root key | No (uses new PKMF) | No (Remote UE needs UP connection to get key material from PKMF) | KI#4, #9 |
| Sol #29 | UP | Yes | New AF (PKMF) AUSF DDNMF | Requires GBA or AKMA support for PKMF/DDNMF connection security. Support for AKMA while roaming is FFS. 5GPRUK ID in DCR may be exposed to privacy threat. Support for revocation of authorization of Remote UE to use relay is not specified. | Relay's PKMF generates PRUK. Remote UE/PKMF: PC5 root key = KDF (PRUK) PKMF => Relay: PC5 root key | No (uses new PKMF and DDNMF, new interface relay's PKMK and Remote UE's DDNMF) | No (Remote UE needs UP connection to DDNMF and Relay's PKMF) | KI#3, #4, #9 |
| Sol #30 | CP | Yes | AMF AUSF | GUTI reallocation/registration update with Remote UE context transfer to relay's AMF FFS 5GPRUK ID in DCR may be exposed to privacy threat. Support for revocation of authorization of Remote UE to use relay is not specified | Remote UE/AUSF: PC5 root key = KDF (KAUSF) AMF => Relay: PC5 root key | Yes | Yes (Remote UE signaling/traffic over PC5) | KI#3, #4, #9 |
| Sol #36 | CP | Yes | DDNMF (as AKMA AF) | PC5 security per different RSC is FFS (currently based on single AKMA key i.e., per UE). A-KID in DCR may be exposed to privacy threat. Support for revocation of authorization of Remote UE to use relay is not specified | Remote UE/AAnF: derive AKMA key. Remote UE/DDNMF: PC5 root key = KDF (AKMA key) DDNMF => Relay: PC5 root key | No (uses DDNMF as AKMA AF) | Yes (Remote UE signaling over PC5) | KI#4 |

Based on above analysis the following recommendations are proposed:

- Recommend solutions that are compatible with SA2 architecture i.e.

  - Not introducing a new AF for authorization and PC5 security in the L3 relay scenario, in compliance with [2] TS 23.304 clause 4.2.7.1; and

  - The selected solution(s) must provide support with and without N3IWF to comply with [2] TS 23.304 respectively clause 6.5.1.1 and 6.5.1.2.

- Solution #19 with N3IWF can be considered as a baseline to support Remote UE services with end-to-end security requirements.

  - It builds on SA2 baseline [2] TS 23.304 clause 6.5.1.2, however it should be selected as complementary to baseline solution(s) that address the relay/remote UE authorization and PC5 link security aspects.

- Selected baseline solutions for relay/remote UE authorization and PC5 security need to accommodate and be compatible with complementary solutions for secondary and slice-specific authentication and authorization support.

- Finally, a Reply LS to SA2 ([4] S3-211401) may be necessary for SA3 to be able to reach a final agreement.

# 5	Proposal

It is proposed to use the above analysis and recommendations as input to progress conclusions for TR 33.847 KI#3, KI#4 and KI#9 for the L3 U2N Relay scenario.