| 3GPP TR 28.884 V0.2.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Service Based Management Architecture enhancement phase 4; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Concepts and background	8

4.1	Introduction	8

4.2	Management data streaming based on Message Bus technologies	8

4.3	Message Bus principles integrated in SBMA	9

5	Use cases and potential solutions	10

5.1	Use case #1: Integration of SBMA with 5GC and 5G Access Network architecture	10

5.1.1	Description	10

5.1.2	Potential requirements	10

5.1.3	Potential solutions	10

5.1.4	Evaluation of potential solutions	10

5.2	Use case #2: Management data streaming based on Message Bus	10

5.2.1	Description	10

5.2.2	Potential requirements	11

5.2.3	Potential solutions	11

5.2.4	Evaluation of potential solutions	12

5.3	Use case #3: Historical alarms	12

5.3.1	Description	12

5.3.2	Potential requirements	13

5.3.3	Potential solutions	13

5.4	Use case#4 on software management for 5G	14

5.4.1	Description	14

5.4.2	Potential requirements	16

5.4.3	Potential solutions	16

5.4.4	Evaluation of potential solutions	16

5.5	Use case#5 on inventory management for 5G	17

5.5.1	Description	17

5.5.2	Potential requirements	17

5.5.3	Potential solutions	17

5.5.4	Evaluation of potential solutions	17

5.6	Use case #6: Merge operation	17

5.6.1	Description	17

5.6.2	Potential requirements	19

5.6.3	Potential solutions	19

5.6.4	Evaluation of potential solutions	20

5.7	Use case#7: MnS selection in distributed deployment scenarios for SBMA	20

5.7.1	Description	20

5.7.2	Potential requirements	21

5.X	Use case #<X>: <use case title>	21

5.X.1	Description	21

5.X.2	Potential requirements	21

5.X.3	Potential solutions	21

5.X.4	Evaluation of potential solutions	21

6	Conclusions and recommendations	21

6.X	Use case #<X>: <use case title>	21

Annex A: Change history	23

# Foreword

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

New and updated use cases, as well as potential requirements, are emerging for managing 5G networks and services within the Service Based Management Architecture (SBMA). This document captures the corresponding potential enhancements to the SBMA.

# 1	Scope

The present document  describes use cases, potential requirements, and potential solutions aimed at enhancing the Service Based Management Architecture (SBMA) in the context of 5G Advanced (5GA). It also presents conclusions and recommendations regarding the next steps in the standardization process.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[3]	3GPP TS 28.532: " Management and orchestration; Generic management services".

[4]	3GPP TS 28.537: " Management and orchestration; Management capabilities".

[5]	3GPP TS 28.552: " Management and orchestration; 5G performance measurements".

[6]	3GPP TS 28.554: " Management and orchestration; 5G end to end Key Performance Indicators (KPIs)".

[7]	3GPP TS 32.423: " Telecommunication management; Subscriber and equipment trace: Trace data definition and management".

[8]	https://datatracker.ietf.org/doc/html/rfc6455

[9]	https://websocket.org/guides/websocket-protocol/

[10]	3GPP TS 28.111: "Management and orchestration; Fault management (FM)".

[11]	3GPP TS 32.531: "Telecommunication management; Software management (SwM); Concepts and Integration Reference Point (IRP) Requirements".

[12]	3GPP TS 32.532: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Information Service (IS)".

[13]	3GPP TS 32.533: "Telecommunication management; Software management (SwM); Integration Reference Point (IRP); Common Object Request Broker Architecture (CORBA) Solution Set (SS)".

[14]	3GPP TS 28.631: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements".

[15]	3GPP TS 28.632: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[16]	3GPP TS 28.633: "Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions".

[17]	SP-250863: Study on SBMA enhancement phase 4

[18]	3GPP TS 32.158: " Design rules for REpresentational State Transfer (REST) Solution Sets (SS)"

[19]	RFC 6241 Network Configuration Protocol (NETCONF)

[20]	3GPP TS 32.101: "Telecommunication management;Principles and high level requirements".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Concepts and background

## 4.1	Introduction

SBMA adopts a modular, service-oriented approach that enables flexible, scalable, and interoperable management solutions.

The architecture for the present study builds upon the SBMA framework in TS 28.533 [2]. All potential enhancements and solutions are expected to comply with the principles outlined in TS 28.533 [2], including service abstraction, interface standardization, and functional decoupling.

The present study aims to preserve architectural continuity with the SBMA baseline model while expanding its applicability to address emerging requirements in 5G Advanced networks.

## 4.2	Management data streaming based on Message Bus technologies

3GPP TS 28.532 [3] defines Stage 2 and Stage 3 of the streaming data reporting service. Currently, the 3GPP management system supports WebSocket-based data streaming for PM, tracing, and analytics, establishing a point-to-point transport connection between the streaming data reporting MnS consumer and MnS producer. The producer initiates the connection with the consumer and exchanges metadata (informing the consumer about its identity and the nature of the data to be reported, i.e., streamInfo). After this phase, the producer reports streaming data to the consumer via the established connection.

Using point-to-point WebSocket communication protocol, which operates over a single TCP connection between a client and a server, a separate connection would be established between each consumer and the producer, causing the producer to generate and transmit multiple copies of the same data. The WebSocket protocol is standardized by the IETF in RFC 6455 [8]. It defines WebSocket as a protocol that enables ongoing, full-duplex, bidirectional communication between web servers and web clients over an underlying TCP connection, see [9]. Though a server can maintain multiple WebSocket connections simultaneously and broadcast messages to all connected clients. However, this is not a protocol-level feature, it is an application-level point-to-multipoint behaviour.

Editor's note:	It is FFS to investigates the integration and compatibility with the message bus industry solutions by providing a general scope which covers management data types defined in 3GPP TS 28.537 [4] (e.g. performance data, trace data, etc.). This does not imply that the message bus investigated in the present document are intended to replace the currently used WebSocket.

NOTE:	The management data includes performance measurements or KPIs defined in TS 28.552 [5] and TS 28.554 [6], trace reporting data defined in TS 32.423 [7] and analytic reporting data defined in TS 28.532 [3].

## 4.3	Message Bus principles integrated in SBMA

Message Bus principles

A Message Bus is a middleware architecture that enables different systems or services to communicate through a shared messaging infrastructure in a decoupled way. Key principles include the following:

- Loose Coupling: Services interact without knowing each other’s internal details. Enhances modularity and allows independent evolution of components.

- Asynchronous Messaging: Messages are sent without waiting for immediate responses. Improves system responsiveness and fault tolerance.

- Publish/Subscribe Model: Producers publish messages to a topic; consumers subscribe to topics of interest. Enables one-to-many and many-to-many communication.

- Scalability and Resilience: Supports high-throughput, fault-tolerant communication with buffering, retries, and failover etc. Maintains service continuity under load or failure.

Table 4.3-1 shows how the message bus principles are supported in the SBMA to modernize 3GPP network management.

Table 4.3-1: Message Bus principles supported in the SBMA

| Message Bus principle | SBMA implementation |
| --- | --- |
| Loose Coupling | MnS producers and consumers interact via service exposure. Scaling of one side, e.g. MnS producer, does not affect the other side, e.g. MnS consumer. Adding more number of MnS consumers, does not affect the MnS producer. Error handling and failure recover does not require coupled proprietary implementation between MnS producer and MnS consumer. |
| Asynchronous Messaging | SBMA supports subscription/notification and streaming interfaces for non-blocking data flow (i.e., MnS producer sends the message data and continues its work without delay, while MnS consumer processes the message independently). |
| Publish/Subscribe | MnS consumers can subscribe to events or data streams exposed by MnS producers. |
| Scalability and Resilience | SBMA supports multipoint-to-multipoint streaming, improving efficiency and fault tolerance |

# 5	Use cases and potential solutions

## 5.1	Use case #1: Integration of SBMA with 5GC and 5G Access Network architecture

### 5.1.1	Description

In a 5G Advanced network deployment, a mobile operator adopts an SBMA that leverages a modular reference model as defined in annex A.11 of TS 28.533 [YY]. This model abstracts Management Functions (MnFs) as logical entities, independent of their physical implementation, and enables them to interact exclusively through Management Services (MnSs).

Each MnF registers its MnSs with a service registry, enabling dynamic discovery and selection of services across domains and vendors.

For example:

- A fault management function in the RAN management domain autonomously detects faults using AI/ML models and triggers a healing workflow.

- This MnF can then orchestrate actions across both RAN and Core domains, invoking MnSs such as performance analytics, configuration updates, and resource scaling.

- The orchestration logic is implementation-agnostic, which means it does not rely on where or how the functions are deployed, only on the standardized MnS interfaces.

- The SBMA framework ensures that each MnF clearly declares its role as a producer or consumer of MnSs, supporting modularity and reusability.

In Release-19, a modular reference model has been defined in annex A.11 of TS 28.533 [YY]. Continue evolving the architecture reference model for management and orchestration, to support new requirements such as AI/ML integration, autonomy, and cross-domain orchestration.

### 5.1.2	Potential requirements

REQ-SBMA-ARM-1:	SBMA should provide a consolidated view of management architecture.

### 5.1.3	Potential solutions

### 5.1.4	Evaluation of potential solutions

## 5.2	Use case #2: Management data streaming based on Message Bus

### 5.2.1	Description

The 5G network employs a distributed, service-oriented architecture, leading to scenarios where multiple consumers request the same management data for multiple uses. In a typical network performance data reporting scenario, an MnS producer collects and transmits performance data to multiple MnS consumers within the network management system (e.g., real-time analytics, network optimization engines, visualization dashboards, and audit services).

The WebSocket protocol is standardized by the IETF in RFC 6455 [f]. It defines WebSocket as a protocol that enables ongoing, full-duplex, bidirectional communication between web servers and web clients over an underlying TCP connection, see [g].

Using point-to-point WebSocket communication protocol, which operates over a single TCP connection between a client and a server, a separate connection would be established between each consumer and the producer. For the case of multiple consumers requesting the same performance data, it causes the producer to retrieve and duplicate the relevant performance data from its cache for each connection individually.

By adopting a message bus architecture, the producer establishes connection(s) with the message bus and publishes the performance data only once. Consumers retrieve the data whenever they want (i.e., the consumer either pulls the data from the message bus or subscribes and receives messages pushed by the bus) - hence the principle of loose coupling. No additional connections are initiated by the producer, and no duplicate data reporting occurs on its side for the case that multiple consumers request the same performance data. As a comparison example between WebSocket and Message Bus for the case that multiple consumers request the same performance data as an assumption:

- for the WebSocket option, duplicate data reporting occurs N times on the producer's side for sending the data to the N consumers who request the same performance data.

- for the Message Bus option, depending on the collection job, no duplicate data reporting occurs on the producer's side for sending the data to the N consumers who request the same performance data because the only one copy of data buffered on the message bus broker (i.e., a middleware that facilitates communication between different participants by acting as an intermediary for messages) can be polled by all the consumers.

While the message bus architecture introduces certain operational considerations, such as the need for proper message category segmentation and consumer coordination management to avoid throughput bottlenecks. These aspects are addressed through established internal mechanisms and configuration practices. Such implementation details are outside the scope of this document. The message bus interface is not used on ManagedElement/NF level but only on management function level.

### 5.2.2	Potential requirements

### 5.2.3	Potential solutions

5.2.3.1	Solution #1: Multipoint-to-Multipoint management data streaming based on message broking MnF

This potential solution proposes introduction of a new MnF implementing the message broker(s) that mediates data with the help of new MnS services, namely, message bus data reporting MnS and message bus data retrieval MnS (for data subscribers) to enable multipoint-to-multipoint streaming data reporting. This solution is meant for use at the management level and not at the managedElement/NF level. This solution also does not impact existing MnS services and can coexist beside such services as an additional option.

Figure 5.2.3.1-1: Potential solution for multipoint-to-multipoint management data streaming based on message bus

As shown in Figure 5.2.3.1-1, in this solution, Message bus data reporting MnS producers send management data to Message broking MnF using the message bus data reporting service based on message bus publishing protocol. Message bus data retrieval MnS consumers receive the management data using the message bus data retrieval service based on message bus retrieval protocol. This approach allows the same copy of management data produced by the Message bus data reporting MnS producer(s) to be shared across multiple Message bus data retrieval MnS consumers thus enabling multipoint-to-multipoint communication and a publishing-and-subscribing based communication pattern.

Editor’s note: The following aspects are FFS

- What types of management data (e.g. PM) to be supported.

- What specific message bus-based communication protocol to be supported (e.g. Kafka) and why?

3.    How to do data access control.

4.    How the information related to the message bus (e.g., end-point URIs) is conveyed or configured to the MnS producers and MnS consumers needs to be clarified.

5.    Whether new MnSs is needed is to be studied

### 5.2.4	Evaluation of potential solutions

## 5.3	Use case #3: Historical alarms

### 5.3.1	Description

The "AlarmList" defined in TS 28.111 [10] contains "AlarmRecords" that represent currently active alarms. Alarms, that are cleared and acknowledged (inactive alarms) are removed from the alarm list and cannot be retrieved any more by MnS consumers.

However, old inactive alarms (historical alarms) are a valuable source of information in many use cases:

-	AI/ML training: Historical alarms provide context information for AI/ML training.

-	Security auditing: Historical security alarms provide information for security monitoring and auditing.

- Fault Repair: By analyzing historical alarms associated with previous faults, operators can identify recurring issues, understand root causes more effectively, and accelerate troubleshooting. This reduces mean time to repair (MTTR) and improves network and service reliability.

- Predictive Maintenance: Historical alarm can be used to forecast potential future failures. For example, repeated alarms from a specific network element may indicate impending hardware degradation. Predictive analytics based on alarm history enables proactive maintenance, minimizing service disruption.

- Enhanced analytics: Multiple MDA capabilities utilize alarm information as enabling data (see TS 28.104, clause 8). Currently the enabling data is defined as “Alarm information and notifications as per TS 28.111”, but the MDA capabilities would be more valuable if historical alarm data is also considered.

### 5.3.2	Potential requirements

REQ-1: The 3GPP Management system should have the capability allowing to retrieve alarms that were active in the past (historical alarms).

REQ-2: The 3GPP Management system should have the capability allowing to retrieve alarms that were active at a certain point in time in the past.

REQ-3: The 3GPP Management system should have the capability allowing to retrieve alarms that were active during a certain time window in the past.

REQ-4: The 3GPP Management system should have the capability allowing to retrieve the dates and times when the perceived severity of an alarm changed

REQ-5 The 3GPP management system should have the capability allowing to retrieve the periods when the retrieved historical alarms may not fully represent the real situation in the network at that point in time.

Editor’s Note: The size of the log should not be set by the standard, leaving it to be a vendor decision..

Open issues:

- Which other parameters have to be stored if it changes during the alarm's active lifetime (beside severity)?

- What kind of filter shall be available when retrieving the alarms (beside a timewindow in REQ-3)

- filter based on some(?) alarmRecord parameters

- filter for one subnet as source

- filter for a specific IOC as source

- filter for a given percievedSeverity or

- full JEX filter on all parameters

- How can the consumer retrieve the historical alarms

- as a file

- some other method

### 5.3.3	Potential solutions

#### 5.3.3.1	NRM based approach

The NRMs defined by SA5 are a representation of the current state of the managed system. They do not represent states before the current time. Reading the data node tree always returns its current state.

For retrieving historical alarms two methods can be envisaged:

-	A new retrieval operation can be introduced with the time dimension in the operation input parameters. The easiest case is to add a "time" parameter to the operation input parameters that specifies a time in the past for which the active alarms (as represented by their alarm records) shall be retrieved. Filtering capabilities of the retrieval operation may be used to scope the response to specific managed object instances. The target object of the retrieval operation is the already specified "AlarmList" object. Variations of this approach are possible, for example a time window may be specified in the request (for some managed objects), the response then returns the alarm records for all alarms that were active in this time window.

-	The NRM is extended to allow for the representation of historical alarms and their alarm records. For retrieving these historical alarm record instances, the normal "getMOIAttributes" operation can be used.

The capability to retrieve historical alarms is expected to be available in a typical deployment only on Management Functions and not on Network Functions.

#### 5.3.3.2	Management data based approach

The alarm records are handled as a specific kind of management data, like performance metrics or Trace and MDT data. The MnS allowing to retrieve active alarms (based on the "AlarmList" object) is not modified.

More specifically, the schema of the alarm record is used to specify the value of a new management data category called alarms. The historical alarms are then managed together with other kinds of management data.

## 5.4	Use case#4 on software management for 5G

### 5.4.1	Description

3GPP TS 32.531 [11], TS 32.532 [12], and TS 32.533 [13] define the concepts, requirements, Information Service, and CORBA solution set for the software management of NEs for 4G, encompassing both automated and non-automated software management approaches.

Software management for 5G enhances 5G network operational efficiency. The benefits of software management retain for 5G network management:

-	One benefit of software management is interoperability: standardized procedures make it possible for operators to manage software on heterogeneous network elements in a multi-vendor environment and reduces operational complexity.

-	Another benefit is network service continuity. By supporting mechanisms such as staged upgrades, version control, and fallback strategies, software management capability minimizes downtime and protects user experience.

Currently, TS 28.533 [2] does not include support for software management. As 5G networks evolve in scale and complexity, it is essential to introduce software management capabilities into SBMA. These capabilities will enable operators to maintain software of NEs and NFs.

Editor's note:	To enable software management within SBMA, the associated potential requirements and potential solutions are FFS, building upon the legacy of software management of NEs for LTE while adapting to the principles of SBMA.

Based on 4G definitions in [11], when describing SW management in this document we use the following terminology:

Activation: Activation of software makes it ready to be used and the software starts providing service

Installation: Installation of software puts it into a form suitable for activation or use.  (Activation  may include steps like unpacking, unzipping, checks, but does not include starting to provide service.)

Fallback: The process of activating a previous SW version.  (Fallback may involve moving to a previous configuration content of NRM).

Cancellation: Is the process where a consumer stops an ongoing operation (e.g. download, activation). If the process is stopped by the producer, it is considered a failure of the process not a cancellation.

Use cases for PNF SW management

The following are examples of possible/typical use-cases. (They are presented here as informative examples)

- UC1: Download software on MnS producer: In preparation for a software upgrade, relevant software artefacts are downloaded by suitable means to the MnS producer. After download, other preparatory steps (a.k.a. software installation) may be performed (such as unzipping, integrity validation) to make sure the software artefacts are suitable and ready for subsequent activation. This download/installation activity is typically not traffic-impacting and may happen some time before the software is activated. The activity might be triggered by an MnS consumer, or might be triggered by some other event (perhaps the installation of new hardware on the PNF, requiring a software upgrade). The download/installation is typically long-running, and its progress can be externally observed. At the end of this use case, the software is ready to be activated. In exceptional error situations (such as unsuitable software being installed; checksum errors), activation of this software will not be possible.

- UC2: Activate software: Previously downloaded/installed software is activated. The activation of the software brings the software into service, effectively replacing the previously running software. As part of the software activation, the PNF’s configuration may be automatically updated. Software activation may or may not be traffic-impacting, depending on the particulars of the PNF implementation. The activation activity is usually triggered by an MnS consumer; but may be triggered by an event or a scheduler. The software activation is typically long-running, and its progress can be externally observed. At the end of this use case, the activated software has been brought into service. In exceptional error situations, the software activation procedure is aborted, the previous software brought (back) into service, and any automated updates to the PNFs configuration undone.

- UC3: Cancellation: The software installation and activation are typically long-running operations. In some scenarios (e.g., ongoing emergency, slow/faulty O&M network, a mistake having been made), the ongoing operation is cancelled. A cancellation is typically triggered by an MnS consumer. After the cancellation, the MnS producer will be in the same state as it was before the operation started. It is possible to re-start the same operation.

- UC4: Fallback to previous software: After the PNF’s software has been successfully upgraded, after observing the system's behaviour for some time, a decision may be made that the upgraded software is unsatisfactory (perhaps it is functionally or performance-wise inferior to the previously installed software). In this case, a software fallback may be triggered. The software fallback will re-activate previously activated software and effectively repeat the steps of the software activation (see UC2) for the previously activated software.

- UC5: Remove inactive software (house cleaning): Software residing on the producer may become unneeded. Unneeded and in-active software is removed from the producer. This operation is typically triggered by the MnS consumer. At the end of this use case the in-active software has been removed from the PNF and therefore is not available anymore for fallback or activation. One typical  case when software becomes unneeded is when a PNF’s software is successfully activated, and found to be operating satisfactorily (perhaps after having been observed for a while), other software still present on the PNF, but not in service, may be removed in order to free resources on the PNF.

Implementation considerations for further study

The producer should check the SW intended to be activated. Whether these checks are executed immediately after download, in a separate step, or at the beginning of activation is implementation dependent.

The producer may take additional preparatory steps before activation (e.g. unpacking the SW). Whether these are executed immediately after download, in a separate installation step or at the beginning of activation is implementation dependent.

Open issues for further study:

-Whether fallback shall be initiated by the consumer or by the producer or potentially both

- Whether activation shall be triggered explicitly by the consumer or automatically in the producer (e.g. by a scheduler) or both

Two modes shall be considered:

- a single step activation that includes download, any checks and activation. Activation may be immediate or  delayed, triggered by a scheduler or some other condition.

- a multi-step activation, where after download, activation is executed as a separate step within the producer. The separate activation step shall be initiated by a consumer operation.

### 5.4.2	Potential requirements

#### 5.4.2.1	Potential general requirements

REQ-SBMA-SWM-1: The 3GPP management system should support software management.

#### 5.4.2.2	Potential requirements for PNF

REQ-SBMA-PNFSWM-1: The 3GPP management system shall support the capability to download SW to the MnS producer.

REQ-SBMA-PNFSWM-2 The 3GPP management system shall support the capability to activate the new SW, that is to start running it by bringing it into service

REQ-SBMA-PNFSWM-3: The 3GPP management system shall support the capability to monitor the progress and the result of software download, activation and fallback and any other vendor specific steps e.g. installation

REQ-SBMA-PNFSWM-4: The 3GPP management system should support the capability to allow the consumer to cancel all SW management operations e.g. download.

REQ-SBMA-PNFSWM-5: The 3GPP management system shall support the capability to allow the consumer to retrieve information about the SW on the producer, including its state.

Editor's note: The relation between this requirement and the inventory information is FFS.

REQ-SBMA-PNFSWM-6: The 3GPP management system shall support the capability to remove inactive SW from the producer.

REQ-SBMA-PNFSWM-7: The 3GPP management system shall support the capability to fallback to a previous SW version. Fallback may include returning to the previous configuration.

Editor's note:	Unless the consumer or the producer is explicitly mentioned, the requirements do not imply whether the consumer or the producer initiates an activity. Unless explicitly specified the requirements do not imply whether an activity is executed as a separate step or potentially together with other activities. These issues are FFS.

#### 5.4.2.3	Potential requirements for VNF

### 5.4.3	Potential solutions

### 5.4.4	Evaluation of potential solutions

## 5.5	Use case#5 on inventory management for 5G

### 5.5.1	Description

3GPP TS 28.631 [14], TS 28.632 [15], and TS 28.633 [16] define the requirements, Information Service definitions, and XML solution set for inventory management. These specifications cover inventory information of NEs, including hardware, software, licences, Tower Mounted Amplifiers (TMAs) and antennas.

The benefits of inventory management retain for 5G network management:

Inventory management enables operators to track equipment status, plan capacity expansions, schedule upgrades, and manage decommissioning more effectively, which improves consistency across large, geographically distributed networks

Inventory management shows the links/topology among network elements, network functions and physical network resources, which helps operators for faster fault localization and root-cause analysis when problems occur. The insight into these dependencies also help operators minimize risks during maintenance or upgrades.

Currently, TS 28.533 [2] does not include support for Inventory Management (IM). As 5G networks evolve in scale and complexity, it is essential to introduce IM capabilities into SBMA. These capabilities will enable operators to maintain accurate and dynamic records of static network resources, supporting planning, assurance, and operational efficiency.

Editor's note 1:	To enable inventory management in SBMA, the associated potential requirements, and potential solutions are FFS, building upon the legacy of inventory management for LTE while adapting to the principles of SBMA.

Editor's note 2:	For topology or the relationship among different inventory objects, the difference between IM and CM for 5G is FFS.

### 5.5.2	Potential requirements

REQ-SBMA-IM-1: The 3GPP management system should support inventory management.

REQ-SBMA-IM-2: The 3GPP management system should support inventory management for hardware.

### 5.5.3	Potential solutions

Though the scope of TS 28.631 [14]/TS 28.632 [15] is limited to the communication between IRPAgent and IRPManager, the content of the inventory NRMs defined in TS 28.632 [15] can be reused to support inventory management for hardware. This solution addresses REQ-SBMA-IM-1 and REQ-SBMA-IM-2 in clause 5.5.2 and proposes to:

-	Define the Inventory NRMs, which are static for read only, for 5G which are communicated between MnS consumer and MnS producer;

-	Reuse the content of 4G Inventory NRMs (i.e., InventoryUnit, InventoryUnitNE and InventoryUnitHw defined in TS 28.632 [15]) as a basis to model 5G inventory NRMs.

### 5.5.4	Evaluation of potential solutions

## 5.6	Use case #6: Merge operation

### 5.6.1	Description

The changeMOIs operation and the notifyMOIChanges notification (as described in TS 28.532[12]) both support multiple operation types, defined by the "modifyOperator" in changeMOIs and as "op" in notifyMOIChanges. These operations are optimized for handling individual data nodes e.g. replacing a full object with all its attributes or updating a single attribute (or attribute field).

A common use-case is updating a multiple attributes of multiple MOIs while leaving other attributes of the same MOIs unchanged. This is not properly handle and documented.

#### 5.6.1.1		Problem when updating multiple MOIs

The changeMOIs operation should be used, as using individual createMOI, deleteMOI and modifyMOIAttributes operations would require using multiple operations requiring more processing and network resources. See TS 28.532 clause 11.1.1.4a.

Lets look at the following example where an immediate heartbeat is ordered in one ManagedElement and multiple attributes of a PerfMetricJob are updated in another ManagedElement.

5.6.1.1-1	  Example configuration, modifying two separate objects

As multiple MOIs are to be modified by changeMOIs the

- baseObjectInstance shall identify a common ancestor MOI for all MOIs to-be-modified or to-be-reported, this could potentially be the root ManagedElement or the SubNetwork. As the ManagedElements may belong to different SubNetworks the baseObjectInstance can only be set to a common ancestor Subnetwork:

"baseObjectInstance": "SubNetwork=Buda-1"

The rest of the path to each argument must be defined in the "path" element using long multi-segment strings. Each "path" element needs to specify the path from the baseObjectInstance to the individual attribute or attribute field. E.g. for updating 5 attributes the following 5 "paths" have to be created and parsed:

"path": "/SubNetwork=Lagymanyos/ManagedElement=BudaME1/NtfSubscriptionControl=sub1/HeartbeatControl=hb1/triggerHeartbeatNtf"

"path": "/SubNetwork=Obuda/ManagedElement=BudaME17/PerfMetricJob=pmJob1/administrativeState"

"path": "/SubNetwork=Obuda/ManagedElement=BudaME17/PerfMetricJob=pmJob1/granularityPeriod"

"path": "/SubNetwork=Obuda/ManagedElement=BudaME17/PerfMetricJob=pmJob1/objectInstances"

"path": "/SubNetwork=Obuda/ManagedElement=BudaME17/ PerfMetricJob=pmJob1/scheduler"

- "node Value" shall contain the new value of each attribute(element) e.g. true, UNLOCKED ,30, SubNetwork=SubNetwork=Buda-1,Scheduler=Sched-1

This means that for each updated attribute(element) the consumer has to produce a long "path" element which the consumer has to read/parse.

A computationally cheaper solution would be to use the "path" element to address each modified MOI, and let the "nodeValue" element contain all attribute changes; to have an operation that allows modifying multiple attributes of an MOI while leaving other attributes of the same MOI unchanged. That would allow the consumer to address each MOI only once. In the example above that would mean creating/parsing only 2 "path" elements instead of 5.

In Netconf, JSON Merge patch or the 3GPP JSON Merge patch this is possible using the "merge" operation. While this is describe both for the Netconf_YANG and for the OpenAPI solution sets, it is not described on stage-2.

Note: While the above example is about a SubNetwork level producer, the same problem is valid for ManagedElement level producers as even inside a single ManagedElement there are IOCs that are embedded in the ManagedElement 5 levels deep.

#### 5.6.1.2		Problem use-case when notifying the update of multiple MOIs

When reporting the changes made in the example of the previous clause the notification notifyMOIChanges should be used. The alternative, using some of the notifications notifyMOICreation, NotifyMOIDeletion, notifyMOIAttributeValueChanges, would mean sending multiple notifications instead of one, which requires more processing and network resources.

When using notifyMOIChanges just as described above we will need to set

"objectInstance": "SubNetwork=Buda-1"

5 separate moiChanges values will be needed, where the "path" element will specify the path to each attribute separately. (The replace operation towards the PerfMetricJob=pmJob1 MOI cannot be used as that would delete all not listed attributes.)

It would be good if we could collect all the attribute changes for a single MOI into one moiChanges value, thereby requiring only a single "path" element for each MOI to be created/parsed even if multiple attributes are updated.

An operation similar to "merge" would help. However, for notifyMOIChanges there is no such operation defined on stage-2 or stage-3.

With a proposed "merge" operation only 2 paths would need to be handled:

"path": "/SubNetwork=Lagymanyos/ManagedElement=BudaME1/NtfSubscriptionControl=sub1/HeartbeatControl=hb1"

"path": "/SubNetwork=Obuda/ManagedElement=BudaME17/PerfMetricJob=pmJob1"

### 5.6.2	Potential requirements

REQ-MERGE-1: Add a simple solution for indicating the update of multiple attributes of  multiple MOIs with a single notification while minimizing the need for parsing long "path" parameters.

### 5.6.3	Potential solutions

Two separate solutions are proposed for the problems described in clause 5.6.1.1 and 5.6.1.2.

a) Describe the merge operation for the changeMOIs operation on stage-2 in 28.532[12].

Note: Exact details of merge shall not be described on stage-2 as they are solution set specific and already described in RFC 6241[19] and in 3GPP TS 32.158[18].

b) Add a new "merge" value to the "moiChanges.op" parameter of the notifyMOIChanges notification. This will allow

- the (base)ObjectInstance to identify a common ancestor MOI for all MOIs to-be-modified or to-be-reported.

- the use of  a single "moiChanges" value for each MOI thus avoiding the need to create/read a separate "path" element for each attribute. Within the single moiChanges value

- the "path" shall to the individual MOI

- the "value" element shall contain all modified attribute values (created, deleted, updated)

Editor’s note: How to report a deleted attribute within merge for YANG based schemas is FFS. Maybe null as a value can be used, although that’s not part of RFC 7951..

As attribute (elements) can have a complex structure too, the merge operation shall be applicable for any kind of target: MOI, attribute, attribute field, or attribute element.

The "value" element shall not contain any child MOIs, thereby ensuring that changes to separate MOIs are reported as separate "moiChange" items.

### 5.6.4	Evaluation of potential solutions

TBD

## 5.7	Use case#7: MnS selection in distributed deployment scenarios for SBMA

### 5.7.1	Description

3GPP management system introduce Service-Based Management Architecture in 5G which has improved system flexibility to adapt to different types of deployment scenarios. In SBMA, MnS producer and MnS consumer are two fundamental entities as defined in TS 28.533[2]. According to definitions in and TS 28.537[4] and TS 32.101[20], figure 1 shows the discovery mechanism of MnS. MnS producer registers service to MnS registry. MnS consumer retrieves service from MnS registry. Then MnS consumer invokes service from MnS producer.

Figure 1. Discovery mechanism of MnS

In distributed deployment scenarios, MnS consumers may get multiple MnSs returned when retrieving MnSs from the MnS registry. MnS consumer has to decide which MnS to invoke. Figure 2 depicts the discovery mechanism in the distributed scenario. The current challenge is how an MnS consumer can select the most appropriate MnS from a set of discovered MnS(s) from the MnS registry.

Figure 2. Discovery mechanism in the distributed scenario

### 5.7.2	Potential requirements

REQ-Distributed-FUN-1: The 3GPP management system should have capability to assist the MnS consumer to select the appropriate MnS(s) from multiple discovered MnS(s).

## 5.X	Use case #<X>: <use case title>

### 5.X.1	Description

Editor's note:	This clause provides a description of use case.

### 5.X.2	Potential requirements

Editor's note:	This clause provides potential requirements for the corresponding use case.

### 5.X.3	Potential solutions

Editor's note:	This clause provides one or more solutions. Further (sub-)clause(s) may be added to capture details.

### 5.X.4	Evaluation of potential solutions

Editor's note:	This clause provides evaluation of potential solutions.

# 6	Conclusions and recommendations

## 6.X	Use case #<X>: <use case title>

Editor's note:	This clause provides conclusions and recommendations for the corresponding use case.

# Annex A: Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA5#162 |  |  |  |  | Initial skeleton | V0.0.0 |
| 2025-10 | SA5#163 | S5-254454 |  |  |  | pCR TR 28.884 Add structure for SBMA study report | V0.1.0 |
| 2025-10 | SA5#163 | S5-254676 |  |  |  | pCR TR 28.884 Add introduction for SBMA study report | V0.1.0 |
| 2025-10 | SA5#163 | S5-254677 |  |  |  | pCR TR 28.884 Add scope for SBMA study report | V0.1.0 |
| 2025-10 | SA5#163 | S5-254678 |  |  |  | pCR TR 28.884 Add concepts and background | V0.1.0 |
| 2025-10 | SA5#163 | S5-254681 |  |  |  | pCR TR 28.884 Use case on integration of SBMA with 5GC and 5G Access Network architecture | V0.1.0 |
| 2025-10 | SA5#163 | S5-254682 |  |  |  | pCR TR 28.884 Management data streaming based on message bus | V0.1.0 |
| 2025-10 | SA5#163 | S5-254732 |  |  |  | New KI on Historical alarms | V0.1.0 |
| 2025-10 | SA5#163 | S5-254844 |  |  |  | pCR TR 28.884 Use case on software management | V0.1.0 |
| 2025-10 | SA5#163 | S5-254845 |  |  |  | pCR TR 28.884 Use case on inventory management | V0.1.0 |
| 2025-11 | SA5#164 | S5-255534 |  |  |  | pCR TR 28.884 Update use case on management data streaming based on message bus | v0.2.0 |
| 2025-11 | SA5#164 | S5-255535 |  |  |  | pCR TR 28.884 Message Bus principles integrated in SBMA | v0.2.0 |
| 2025-11 | SA5#164 | S5-255537 |  |  |  | pCR TR 28.884 Management data streaming based on message bus potential solution 2 | v0.2.0 |
| 2025-11 | SA5#164 | S5-255541 |  |  |  | pCR TR 28.884 Add new use case on wide-area distributed deployment scenarios for SBMA | v0.2.0 |
| 2025-11 | SA5#164 | S5-255542 |  |  |  | pCR TR 28.884 SW Management additions | v0.2.0 |
| 2025-11 | SA5#164 | S5-255544 |  |  |  | pCR TR 28.884 Solution for inventory management | v0.2.0 |
| 2025-11 | SA5#164 | S5-255545 |  |  |  | pCR TR 28.884 Historical alarm list requirements | v0.2.0 |
| 2025-11 | SA5#164 | S5-255546 |  |  |  | Rel-20 pCR 28.884 Add NRM based solution for historical alarms | v0.2.0 |
| 2025-11 | SA5#164 | S5-255580 |  |  |  | pCR TR 28.884 Merge operation | v0.2.0 |
| 2025-11 | SA5#164 | S5-255581 |  |  |  | Rel-20 pCR 28.884 Add Management data based solution for historical alarms | v0.2.0 |