3GPP TSG RAN meeting #68

Malmö, Sweden, June 15 - 18, 2015

Title:	Response to the LS on 3GPP RAN Study on regulatory aspects for flexible duplex for E-UTRAN

Source:	ARIB

To:	3GPP TSG RAN

Contact Person:

Name:	Nakamura, Takaharu
Leader of TSG-RAN WG4 Support Adhoc, 3GPP-RAN Support WG, Mobile Partnership Subcommittee, Advanced Wireless Communications Study Committee, ARIB

E-mail Address:	n.takaharu@jp.fujitsu.com

ARIB has reviewed the LS in [1] together with its associated technical document in [2] and would like to inform the following analysis as the consequence of the review.

- Assignable frequencies in Japan are given in “Frequency assignment plan” [3][4]. The Plan is established and disclosed by Minister of Internal Affairs and Communications under the provisions of the item 1 of Article 26 of the Radio Law. In its Annex 10-2  [3][4][5], there is a table providing frequencies for Portable Radiocommunications (FDD system) as shown below:

- In Articles 4-6, 4-12, 4-7-3 of The Regulations for Enforcement of the Radio Law[6], definitions of base stations, land mobile stations and land mobile relay stations are given, where land mobile relay stations are defined as radio stations that relay communications between land mobile stations and base stations. Based on the definitions in these articles, several technical conditions are specified for base stations, land mobile stations and land mobile relay stations in [7].

According to the above mentioned rules, base stations shall use the frequency band as defined in “Frequency assignment plan” [3] and land mobile relay stations relay the communications between land mobile stations and base stations that use the frequencies listed in Annex 10-2 of [3]. As the result it is concluded that;

- Land mobile relay stations are not allowed to use the FDD uplink as its downlink.

- Base stations are not allowed to use the FDD uplink as its downlink.

It should also be noted that the Essential Standards for Establishing Radio Stations other than Broadcasting Stations [8] and other several ordinances or public notices also specify technical conditions for mobile station transmitters/receivers and base station transmitters/receivers according to Annex 10-2 of [3].

- References

- 3GPP TSG-RAN, RP-150509, “LS on 3GPP RAN Study on regulatory aspects for flexible duplex for E-UTRAN,” Source: TSG-RAN, (Mar. 2015).

- 3GPP TSG-RAN, RP-150250, “Further discussion on regulatory aspects related to flexible duplex operation for E-UTRAN,” Source: LG Electronics, China Telecom, Huawei, HiSilicon, IAESI (Mar. 2015).

- “Frequency Assignment Plan,” Minister of Internal Affairs and Communications  (as of January 2013)  http://www.tele.soumu.go.jp/e/adm/freq/search/share/index.htm.

- “Frequency Assignment Plan,” Ministry of Internal Affairs and Communications Notice No. 714 of 2008 (in Japanese)

- Partial revision of “Frequency Assignment Plan,” Ministry of Internal Affairs and Communications Notice No. 172 of 2012 (in Japanese) http://www.soumu.go.jp/main_content/000155957.pdf

- The Regulations for Enforcement of the Radio Law, (In Japanese)
 http://law.e-gov.go.jp/htmldata/S25/S25F30901000014.html.

- The Rules for Regulating Radio Equipment, (In Japanese)
 http://law.e-gov.go.jp/htmldata/S25/S25F30901000018.html.

- The Essential Standards for Establishing Radio Stations other than Broadcasting Stations, (In Japanese) http://law.e-gov.go.jp/htmldata/S25/S25F30901000012.html.