# Comparison of TETRA, MCPTT, and North American LMR Terminology

# March 2016

Prepared by the ATIS LMR/LTE Interworking Ad hoc which is comprised of members of 3GPP Organizational Partners (for LTE), TIA (for P25), and TCCA (for (TErrestrial Trunked Radio (TETRA)) to progress the LMR (Land Mobile Radio) and LTE (Long Term Evolution) systems interworking.  The group developed the following document, which provides an organized list of common nomenclature used in the North American LMR community, the TETRA community and the 3GPP Mission Critical Push To Talk (MCPTT) standards, and identifies correspondences and differences between the three sets of terms.

# Abstract

Enumeration and comparison of nomenclature commonly used within the North American LMR community (particularly within the Project 25 (TIA-102) LMR radio community) TETRA community, and 3GPP MCPTT community.

Table of Contents

1	Introduction	1

2	Nomenclature	1

2.1	Standards Nomenclature	1

2.2	P25 and TETRA Systems Architecture Nomenclature	3

2.3	Entities and Identities	6

2.4	Radio Terminology	11

2.5	Conventional Analog Radio Terminology	14

2.6	Mobility Terminology	15

2.7	Communications Features	17

2.8	Priority Management Nomenclature	23

2.9	Management Terminology	25

2.10	Performance Nomenclature	26

2.11	Security Nomenclature	27

# Introduction

This contribution provides an organized list of common nomenclature used in the North American LMR community, the TETRA community and the 3GPP MCPTT standards, and identifies correspondences and differences between the three sets of terms.

# Nomenclature

## Standards Nomenclature

The following terms refer to North American LMR standards organizations and projects.

| Term | Meaning | Applies to | Cross Reference | Comment |
| --- | --- | --- | --- | --- |
| Project 25 (P25, adjective) | Supporting all or some of the subset of TIA-102 standards approved by the Project 25 steering committee. | P25 |  |  |
| TETRA (adjective) | Supporting the ETSI voice plus data standards (EN 300 392 series), the DMO standards (EN300 396 series) and the relevant TRs and TSs. | TETRA |  |  |
| MCPTT (adjective) | Supporting MCPTT standards (e.g. MCPTT system) developed by 3GPP. | MCPTT |  |  |
| Project 25 (P25, noun) | A cooperative project between APCO and TIA to develop land mobile radio standards to meet the needs of public safety in North America. | - TETRA |  |  |
| TETRA (noun) | TErrestrial Trunked RAdio (TETRA) is a digital trunked mobile radio standard developed to meet the needs of traditional Professional Mobile Radio (PMR) user organizations such as: Public Safety, Transportation, Utilities, Government, Military, PAMR, Commercial & Industry, Oil & Gas | - P25 |  |  |
| MCPTT (noun) | Mission Critical Push-to-Talk: A Push To Talk communication service supporting applications for Mission Critical Organizations and mission critical applications for other businesses and organizations (e.g., utilities, railways) with fast setup times, high availability, reliability and priority handling. | MCPTT |  |  |
| TIA-102 (adjective) | Supporting all or part of the TIA-102 series of standards. | P25 |  |  |
| TIA-102 standards | Digital Land Mobile Radio standards developed by the Telecommunications Industry Association. | P25 |  |  |
| TETRA Standards | Standards developed by ETSI for Digitial trunked mobile radio including EN300 392, 300 394 and 300 396 standards. | Tetra |  |  |
| TIA-603 (adjective) | Supporting TIA-603 standards. | North America |  |  |
| TIA-603 (noun) | (Analog) Land Mobile communications equipment measurement and performance standards | North America |  |  |
| MCPTT standards | Standards developed by 3GPP for MCPTT, e.g 3GPP TS 23.179 and associated specifications, which is the architecture specification developed by 3GPP System Aspects Working Group 6. | MCPTT |  |  |

## P25 and TETRA Systems Architecture Nomenclature

The following are North American LMR terms referring to important architectural elements of the existing standards matched by their understanding in TETRA and 3GPP MCPTT.

| Term | Meaning | Applies to | Cross Reference | Comment |
| --- | --- | --- | --- | --- |
| Base Station | An element of a trunked radio network comprising one or more control channels and one or more traffic channels providing a single shared set of communications channels | TETRA | Trunking Site |  |
| Base Station | A fixed element of a P25 conventional or trunked system that receives from and transmits to mobile units. | P25 |  |  |
| (Simple) Repeater | An element of conventional infrastructure that “repeats” signals received on an uplink to corresponding downlink without modifying the information content of the repeated signals. | P25 & TETRA & MCPTT |  | TETRA supports Trunked Mode Operation and also Direct Mode Operation repeaters. In TETRA and P25, repeater is a sort of base station. For MCPTT: - the 3GPP EPS (Evolved Packet System) supports the repeater functionality for on-network operation. - the ProSe UE-to-UE relay (not supported in Rel-13, should be supported in Rel-14) supports the repeater functionality for direct mode of operation. |
| DMO Gateway | A means of extending network coverage. An extension of the trunked network that is available for remote terminal access via Direct Mode Operation | TETRA | ProSe UE-to-network relay capability used by MCPTT | Not standardized in P25. |
| Gateway | A means of interconnecting TETRA networks to other public or private networks such as PSTN, ISDN etc. | TETRA |  | Not standardized in P25. |
| (Trunking) Site | An element of a trunked radio network comprising one or more control channels and one or more traffic channels providing a single shared set of communications channels | P25 & TETRA & MCPTT |  | For MCPTT, this is part of the EPS. |
| Common Air Interface | The P25 specifications for FMDA or TDMA traffic, the TETRA specification for TDMA signaling and traffic and the 3GPP LTE specification for the MCPTT EPS. | P25 & TETRA & MCPTT |  |  |
| Console | An element of a trunked or conventional radio network that provides access to the network for fixed users, generally dispatchers. | P25 & TETRA& MCPTT |  | In TETRA console also used for the remote head in vehicle applications. For MCPTT, the interconnectivity mechanism with the EPS is not specified, but the function of a dispatcher is supported. |
| Console Subsystem Interface (CSSI) | A standardized SIP/RTP based protocol for interconnecting consoles to conventional or trunked P25 RFSS. | P25 |  | Not standardized in TETRA. Proprietary solutions. MCPTT supports IMS-compliant SIP protocols for interoperability. |
| Conventional Fixed Network Station | An element of a conventional radio network that provides a conventional channel that may be an active participant in communications, or provides connectivity for wireline subsystems such as consoles. Contrast with (Simple) Repeater. | P25 |  |  |
| Inter RF-Subsystem Interface (ISSI) | A standardized SIP/RTP based protocol for interconnecting RF Subsystems in P25 networks | P25 | TETRA ISI | In TETRA ISSI stands for Individual Short Subscriber Identity. GSSI is the group equivalent. |
| Inter-System Interface (ISI) | A standardized set of interface protocols for interconnecting TETRA SwMIs | TETRA | P25 ISSI |  |
| Key Management Centre (KMC) | A subsystem that allows an agency to manage end-to-end cryptographic keys and other security parameters. | TETRA | KMF (P25 & MCPTT) |  |
| Key Management Facility (KMF) | A subsystem that allows an agency to manage end-to-end cryptographic keys and other security parameters. | P25 & MCPTT | TETRA KMC | Key Management Client/Server in MCPTT. |
| RF Subsystem (RFSS) | A basic RF infrastructure bounded by standardized interfaces in a P25 network. Standards are mute on the internals of an RFSS. | P25 |  | TETRA is mute about all interfaces within the one infrastructure |
| On-network MCPTT service | The collection of functions and capabilities required to provide MCPTT via EPS bearers using E-UTRAN to provide the last hop radio bearers. Certain MCPTT functions such as dispatch and administrative functions can be supported using either MCPTT UEs in E-UTRAN or using MCPTT UEs via non-3GPP access networks. | MCPTT |  |  |
| Off-Network MCPTT service | The collection of functions and capabilities required to provide MCPTT using ProSe Discovery and the ProSe Communication path for MCPTT Users using Public Safety ProSe-enabled UEs as a direct communication between UEs using E-UTRA. | MCPTT |  |  |
| UE-to-network relay MCPTT service | The collection of functions and capabilities required to provide MCPTT via a ProSe UE-to-network relay using ProSe direct communication paths to provide the last hop radio bearer(s). | MCPTT | DMO Gateway |  |
| ProSe UE-to-network relay | A UE that provides functionality to support connectivity to the network for remote UE(s). | MCPTT | DMO Gateway |  |

## Entities and Identities

The following are common North American LMR terms referring to entities and identities compared with those used for TETRA and 3GPP MCPTT

| Term | Meaning | Applies To: | Cross Reference | Comment |
| --- | --- | --- | --- | --- |
| Dispatcher | A public safety practitioner who participates in the radio system for command and control purposes. | P25 & TETRA & MCPTT |  |  |
| Group – Announcement | A broadcast group associated with a user’s selected group. Multiple groups can share a common Announcement Group. | P25 & TETRA |  | In TETRA the communication type in the call set up says whether a call to a group address is a point to multi-point group call or broadcast (one way). There is also a broadcast group address which defines an address to which all MS belong. In MCPTT, broadcast group call (i.e.,announcements) can be sent to any group and may or may not be associated with GCSE broadcast capabilities. |
| Group – Scan | A group that is selected by a user to be listened to when higher priority traffic (e.g., his selected group) is not active. | P25, MCPTT & TETRA |  | MCPTT supports multiple groups may be monitored (i.e. listened to). |
| Group – Selected | The talk group associated with the user’s primary current mission, which is the group he “keys-up” when he initiates a PTT call. | P25 & TETRA & MCPTT |  | In practice the same but TETRA permits more flexibility (rarely used). MCPTT allows a single selected group for transmission. |
| Group (aka Talk-group) | Subscribers organized by agency and/or mission to conduct mission critical operations. | P25 & TETRA & MCPTT |  |  |
| Group Short Subscriber Identity (GSSI) | A 24 bit identifier that uniquely identifies a TETRA group within a TETRA network | TETRA | P25 Group ID (GID) |  |
| Group TETRA Subscriber Identity (GTSI) | A 48 bit identifier that uniquely defines a TETRA group. The GTSI comprises the TETRA MCC, TETRA MNC and GISSI of the group. | TETRA | P25 Subscriber ID (SGID) MCPTT Group ID |  |
| Individual Short Subscriber ID (ISSI) | A 24 bit identifier of a TETRA MS that uniquely identifies the MS within a particular TETRA network | TETRA | P25 Unit Identifier |  |
| Individual TETRA Subscriber Identity (ITSI) | ITSI is a 48 bit identifier that uniquely defines an MS. ITSI comprises TETRA Mobile Country Code, MCC, TETRA Mobile Network Code, MNC and the ISSI of the MS assigned an alias when it migrates so no need for uniqueness across the world | TETRA | P25 Subscriber Unit ID (SUID) MCPTT ID |  |
| MCPTT administrator | An individual authorized to control parameters of the MCPTT Service for an organization including, for example, user and group definition, user/group aliases, user priorities, group membership/priorities/hierarchies, security and privacy controls. | MCPTT |  |  |
| MCPTT group identity (MCPTT group ID) | Globally unique identifier within the MCPTT service that represents a set of MCPTT users | MCPTT | Group TETRA Subscriber Identity (GTSI) (TETRA) P25 Subscriber Group ID (SGID) |  |
| MCPTT system | The collection of applications, services, and enabling capabilities required to provide Mission Critical Push To Talk for a Mission Critical Organization. | MCPTT | P25 System TETRA SwMI | 3GPP defines two types of MCPTT systems: - Partner MCPTT System: Allied MCPTT system that provides MCPTT Services to an MCPTT User based on the MCPTT User Profile that is defined in the Primary MCPTT System of that MCPTT User. - Primary MCPTT System: MCPTT system where the MCPTT User Profile of an MCPTT User is defined. |
| MCPTT UE | A UE that enables an MCPTT User to participate in MCPTT Service. | MCPTT | Mobile Station (MS) (TETRA) Subscriber Unit (SU) (P25) |  |
| MCPTT user identity (MCPTT ID) | Globally unique identifier within the MCPTT service that represents the MCPTT user. | MCPTT | P25 Subscriber Unit ID (SUID) Individual TETRA Subscriber Identity (ITSI) |  |
| Mission Critical user identity (MC ID) | Identity that an MCPTT user presents to the identity management server during a user authentication transaction. | MCPTT |  |  |
| Mobile Country code, MCC | A 10-bit country identifier for TETRA managed by ITU. | TETRA | P25 WACN ID |  |
| Mobile Network Code (MNC) | A 14-bit identifier of a mobile network within an MCC | TETRA | P25 System ID |  |
| Mobile Radio (MR aka SU) | A specific instance of an LMR radio. An MR may have multiple “personalities” associated with different system types (e.g., Analog conventional, P25 conventional, P25 trunking), or with different operating modes within a single system. | P25 | Mobile Station (MS) MCPTT UE |  |
| Mobile Stations (MS) | A specific instance of a handheld, vehicle mounted, or fixed radio station. | TETRA | Subscriber Unit (SU) MCPTT UE |  |
| Mobile Network Identity (MNI) | The 24 bit concatenation of the MCC and MNC which uniquely identifies a specific network. | TETRA | EPS PLMN Id |  |
| P25 Group ID (GID) | A 16-bit identifier that uniquely identifies a P25 Group within a P25 System. | P25 | Group Short Subscriber Identity (GSSI) |  |
| P25 Subscriber Group ID (SGID) | A 48-bit identifier that uniquely identifiers a P25 Group. The SGID comprises the P25 WACN, P25 System IDand P25 GID of the group. | P25 | GTSI (Group TETRA Subscriber Identity) MCPTT group ID |  |
| P25 Subscriber Unit (SU) | A subscriber unit of a P25 network. | P25 | Mobile Station (MS) |  |
| P25 Subscriber Unit ID (SUID) | A 56 bit identifier that uniquely identifies an SU amongst a collection of interconnected Project 25 Trunking systems. The SUID comprises the P25 WACN ID, P25 System ID, and P25 Unit Identifier of the SU. | P25 | ITSI (Individual TETRA Subscriber Identity) MCPTT ID |  |
| P25 System | 1) A collection of P25 infrastructure that shares a common set of services and identifiers and may be part of a large P25 Wide Area Communications Network; 2) A P25 System ID. | P25 | TETRA SwMI MCPTT System | TETRA Network or perhaps TETRA SwiMI (Switching and Management Infrastructure) is closest |
| P25 System ID | A 12-bit identifier that uniquely identifies a P25 System within a P25 WACN | P25 | Mobile Network Code (MNC) |  |
| P25 Unit Identifier | 1) A 24-bit identifier of a P25 conventional subscriber unit that uniquely identifies the unit within a specific P25 conventional system. 2) A 24-bit identifier of a P25 subscriber unit that uniquely identifies the unit within a particular P25 trunking system. | P25 | Individual Short Subscriber Identity |  |
| P25 WACN ID | A 20-bit identifier of a P25 WACN. | P25 | MCC |  |
| P25 Wide Area Communications Network (WACN) | 1) A collection of interoperating P25 Systems that share a common set of identifiers discriminated by a WACN ID; 2) A P25 WACN ID. | P25 | TETRA Network with unique MNI |  |
| Subscriber Unit (SU) | A device, usually a mobile radio, which is identified by a system specific identity and is unique within the identified system. | P25 |  | Understood in TETRA (probably would call it an MS) – identified by a 15 character TEI (Terminal Equipment Identifier) |
| TETRA Switching and Management Infrastructure (SwMI) | The abbreviation SwMI is used to classify all of the equipment and sub-systems that comprise a TETRA network, including base stations. | TETRA | P25 System MCPTT System plus EPS |  |
| Unit | A subscriber unit. | P25 | MCPTT client |  |

.

## Radio Terminology

The following are common terms for radio technologies.

| Term | Meaning | Applies To | Cross Reference | Comment |
| --- | --- | --- | --- | --- |
| Call | One or more consecutive talk-spurts with the same group destination (in the case of a group call), or the same pair of participants (in the case of an individual call). | P25 & TETRA & MCPTT |  | Same understanding (although the term talk-spurt is not used in TETRA).Sometimes called, “over” or call transaction. MCPTT supports both private and group calls. |
| Control Channel | In trunking, the channel by which assignment requests and responses and other messages are exchanged between the controlling entity and the subscriber units. Control channels may convey other short information flows in addition to assignments to traffic channels. | P25 & TETRA | MCPTT Signaling Control Plane | In TETRA also deals with assignments to packet data channels |
| Conventional | A radio technology in which a multiplicity of units communicate via “ALOHA” or similar access protocols that do not employ a central arbitrating node. “Radio discipline” is the primary channel access method | P25 | EPS Access Channel | Not used in TETRA although understood and radio discipline has to be used for TETRA DMO calls |
| Direct Mode Operation (DMO) | A conventional radio technology in which units communicate with each other in the absence of infrastructure. | P25 & TETRA | Off-network MCPTT service |  |
| Analog Conventional | A conventional radio technology using analog means of voice conveyance | P25 |  | Same understanding but not used in TETRA |
| Key-up | To press the PTT button to initiate a talk-spurt. | P25 | In TETRA this would be “call set up” or activation of the pressel or ptt button. |  |
| Late Entry | Joining a call “in progress” even though not present for the initiation of the call. | P25 & TETRA & MCPTT |  |  |
| P25 Conventional | A digital conventional radio technology based on approved TIA-102 standards. | P25 |  |  |
| P25 Trunking | A digital trunking technology based on approved TIA-102 standards | P25 | TETRA | A digital trunking technology based on ETSI EN 300 392 and 300 396 series standards |
| Repeated | A radio technology in which units communicate via an uplink to infrastructure which rebroadcasts the communications via a downlink with no modification of the information content of the uplink channel. | P25 & TETRA |  | TETRA includes DMO repeaters where handheld terminals can be flexibly configured to act as DMO repeaters to other handhelds and gateways can also be configured as repeaters. |
| Talk-around | A conventional radio technology in which units communicate with each other using the downlink frequency of a repeating infrastructure | P25 |  |  |
| Talk-spurt | An audio burst, bounded by activation and release of a PTT button from a specific source to a specific group or individual destination. | P25 | Over or call transaction |  |
| Traffic Channel | In trunking, one of a multiplicity of channels to which specific communications (e.g., voice, data or control signaling) are assigned. | P25 & TETRA |  | Theoretically an assigned channel. In TETRA a traffic channel is an assigned channel for voice or circuit mode data or associated signaling. There are also assigned packet data channels for packet data. |
| Trunking | A radio technology in which a collection of RF channels are efficiently shared by a multiplicity of units by virtue of a having a central control element (a “control channel”) that assigns specific communications to one of a multiplicity of “traffic channels”. | P25 & TETRA |  |  |
| Trunking – Message Trunking | A type of trunking where a call is assigned to a traffic channel and the call persists as long as consecutive transmissions are separated by no more than a known “hang-time”. | P25 & TETRA |  |  |
| Trunking – Transmission Trunking | A type of trunking where the units dwell on the traffic channel for a single communications transmission then return to the control channel for arbitration and traffic channel assignment of subsequent transmissions even on the same group. | P25 & TETRA |  |  |
| Unkey (aka dekey) | To release the PTT button, thus ending a talk-spurt. | P25 |  | In TETRA releasing the key ends the call transaction but not the call. This requires an explicit call end or expiration of hang time |
| Uplink | A radio path from a mobile radio to infrastructure. | P25 & TETRA & MCPTT |  |  |

## Conventional Analog Radio Terminology

The following are common North American LMR terms for conventional ANALOG radio technologies.

| Term | Meaning |
| --- | --- |
| Continuous Digital Controlled Squelch System (CDCSS) | A development of CTCSS that uses a slow-speed digital binary data stream passed as sub-audible data along with the transmission. |
| Continuous Tone Controlled Squelch System (CTCSS) | A system in which sub-audible tones are used to filter audio from groups on a shared radio channel that are not of interest to a particular user, or by which repeater access may be controlled. |
| DCS | Digital Coded Squelch, synonym for Continuous Digital Controlled Squelch System |
| Squelch | A circuit or function that suppresses the output of a radio receiver if the signal strength falls below a certain level, or some other condition (e.g., detection of certain codes in sub-audible signaling) is found. |
| Sub-audible Signaling | Any of a variety of means by which analog conventional ANALOG channels carry control and identifying information. See e.g., CTCSS, and CDCSS. |

## Mobility Terminology

The following are terms related to mobility management.

| Term | Meaning | Applies To | Cross Reference | Comment |
| --- | --- | --- | --- | --- |
| Automatic Roaming – Inter-system | Roaming in which the home identity and operating characteristics (e.g., group membership) of the roaming unit is maintained and, when possible, his connectivity to resources in his home system is maintained as well. | P25 | Migration Partner MCPTT system based roaming (aka "migration" |  |
| Migration | When the MS moves to another TETRA network and retains home identity and operating characteristics plus connectivity where possible to home resources.(TETRA specifies Inter System Interfaces to permit this). | TETRA |  |  |
| Partner MCPTT system based roaming (aka "migration" | Obtaining service from a partner MCPTT system where no MCPTT user profile exists for that MCPTT user. | 3GPP MCPTT | Automatic Roaming – Inter-system (P25) Migration (TETRA) | Not supported for Rel-13 MCPTT. |
| Automatic Roaming – Intra-system | Roaming in which the radio selects the best site to operate on based on a combination of signal strength and other factors. | P25 & TETRA | EPS Roaming | Can be geo-fenced to prevent traffic being dragged to a site and causing congestion. Site loading can be one of the selection criteria as well as service support |
| Group Affiliation | The means by which a unit notifies infrastructure of the group that it wishes to participate in. In P25 trunking, group affiliation also provides the means by which the infrastructure informs the unit of the announcement group for the affiliated group, and provides a locally significant identity when the affiliated group is from a different system. | P25 & MCPTT | Group attachment |  |
| Group Attachment | Equivalent to Group Affiliation, but may be initiated by the MS or the SwMI. | TETRA | Group Affiliation |  |
| Home System | The system in which a unit or group normally operates and from which it is managed. For P25 trunking, the system identified by the WACN and System ID fields of the SUID or SGID. For TETRA, the system is identified by MNI of the MS. | P25 & TETRA | Primary MCPTT System |  |
| ITSI Attachment | The means by which a unit notifies infrastructure of its presence and desire to use the communications resources of the infrastructure. | TETRA | P25: Unit Registration |  |
| Group home MCPTT system | The MCPTT system where the MCPTT group is defined. | MCPTT |  |  |
| Manual Roaming | Roaming in which the user manually selects the trunking system or conventional channel to operate on (e,g., a new identity upon entering into the foreign system.) Also manually tuning/selecting the conventional frequency set to operate on. | P25 | EPS System Selection | TETRA only supports automatic roaming (Migration) |
| Roaming (P25) | Moving between infrastructure in order to take advantage of the communications resources of the visited infrastructure, either for communications within the visited infrastructure or as an extension of the roaming unit’s home infrastructure. | P25 | Roaming & Migration EPS Roaming |  |
| Roaming (TETRA) | Moving between infrastructures within a TETRA network. | TETRA | Roaming EPS Roaming |  |
| Unit Registration | The means by which a unit notifies infrastructure of its presence and desire to use the communications resources of the infrastructure. In P25, unit registration provides the means by which the infrastructure provides a locally significant identity. | P25 | TETRA: ITSI Attachment | Combination of EPS Registration, IMS Registration and MCPTT Service Authorization |
| Visited System (aka Foreign system, aka Serving System) | A system in which a roaming unit operates when that system is not its home system. | P25 & TETRA | Partner MCPTT System |  |
| Partner MCPTT system | Allied MCPTT system that provides MCPTT Services to an MCPTT User based on the MCPTT User Profile that is defined in the Primary MCPTT System of that MCPTT User. | MCPTT |  |  |

.

## Communications Features

The following are common terms for communications features.

| Term | Meaning | Applies To | Cross Reference | Comment |
| --- | --- | --- | --- | --- |
| Announcement Group Call | A group call to an announcement group. | P25 | MCPTT Group Broadcast Group |  |
| Group broadcast group | A group call to an announcement group. | MCPTT | Announcement Group Call in P25 |  |
| Broadcast Voice Call | A transmission trunked group call, or any call in a conventional system addressed to a group. | P25 & TETRA | MCPTT Broadcast Group Call | In TETRA a broadcast call is a one- way call addressed to a group address |
| Call Alert | A means for one unit to inform another that the initiator would like a call back from the responder. | P25 | MCPTT Call back request (not supported in Rel-13) | No equivalent in TETRA although a status number has been reserved for this |
| Call back request | A mechanism (i.e., MCPTT Private Call call back request) for the calling party of an MCPTT Private Call to request that the called party (at earliest convenience) place a call to the calling party. | MCPTT | Call Alert (P25) | Not supported in Rel-13 |
| Dynamic Regrouping | A means to merge multiple talk groups (talkgroup regrouping) and individual SUs (individual regrouping) into a single dynamically defined talkgroup. | P25 & MCPTT | In TETRA: Dynamic Group Number Assignment (DGNA) |  |
| Dynamic Group Number Assignment (DGNA) | Allows authorized users to create, modify, delete and interrogate group(s). The affected user may interrogate the group information based on group numbers. Can group all the participants in an ongoing call. Also used to create groups based on affected user identities, which may be individual users or groups. A combination of call related and call unrelated DGNA may be supported. As a network option, call related DGNA may be invoked after the call release within a network dependent predefined | TETRA | Dynamic Regrouping Group regrouping (MCPTT) | One of the TETRA Supplementary Services |
| Enable/disable | A means to inhibit the normal operation temporarily or permanently, or restore to service. | P25 & TETRA | P25: Inhibit/ Un-inhibit | Enable/disable is used informally in the P25 community. A permanently disabled unit cannot be recovered with an enable command, and erases certain security parameters. |
| Emergency Alarm | A unit-based means to indicate an emergency condition outside of the context of an emergency call. | P25 & TETRA & MCPTT |  | TETRA has a defined status number for this function and similar behaviors. In 3GPP MCPTT this is MCPTT emergency alert. |
| Emergency Group Call | A group call with preemptive priority and other special behaviors. | P25 & TETRA & MCPTT |  | In 3GPP MCPTT this is MCPTT emergency group call. |
| Group Call | A call disseminated to a collection of users identified by a group identity. | P25 & TETRA & MCPTT |  |  |
| Group Call – Confirmed (aka Acknowledged) | A group call in which infrastructure attempts to ensure that specific sites (i.e., geographic areas) or users (e.g., units) are included in the call | P25 & TETRA & MCPTT |  | MCPTT group calls can require acknowledgement. |
| Group Regrouping | A means to temporarily combine multiple groups together, forming a newly defined temporary group, so that all of the users participate in the same call when any of the combined groups are called. | P25 & MCPTT | In TETRA: Patch and re-group | Can also provide a temporary group address to achieve the same |
| Immediate Location Request | A means to request the GPS location of a unit. | P25 & TETRA & MCPTT |  | This is MCPTT Location Information Request. |
| Individual Regrouping | A means to temporarily group units into a designated group. | P25 & MCPTT | DGNA | In 3GPP MCPTT this is user regrouping. |
| PSTN-to-Group Call | A call initiated from a PSTN user to participate in a group call. | P25 & TETRA |  |  |
| PSTN-to-Unit Call | A call initiated from a PSTN user to contact an individual unit. | P25 & TETRA |  |  |
| Radio Inhibit/Un-inhibit | A means to inhibit the normal operation of a unit, or restore normal operation | P25 | In TETRA: Enable/ Disable | These facilities are used remotely on lost or stolen MS |
| Transmit Inhibit | A user means to inhibit transmission in areas where Rf transmission could potentially cause a problem | TETRA |  | Used, for example, in hospitals |
| Radio Unit Monitor (aka Ambiance Listening) | A means by which a dispatcher causes an SU to initiate a group or unit-to-unit call without the operator depressing the PTT switch. Radio Unit Monitor allows a dispatcher to covertly listen to activities at the location of the SU. | P25 & TETRA | Ambience listening Ambient listening (MCPTT) |  |
| Ambient listening | Feature that allows an authorized MCPTT User, typically a dispatcher, to cause an MCPTT UE to initiate a call which results in no indication on the MCPTT UE that it is transmitting. Ambient Listening can be initiated by an authorized MCPTT User who wants to be listened to by another remote authorized MCPTT User or can be initiated by a remote authorized MCPTT User who wants to listen to another MCPTT User. The purpose of this feature allows a dispatcher to listen to activities at the Location of the remote MCPTT UE to find out what is happening around that MCPTT UE without providing an indication to the MCPTT User or people around the user (whom the MCPTT User does not want to make aware of this action) that this is happening. | MCPTT |  |  |
| Scanning | A means by which an SU listens to calls from “scan groups” when no higher priority call (e.g., on the selected, announcement, or broadcast groups) are active. Scanning can also refer to searching for trunking sites or conventional channels. | P25 & TETRA |  |  |
| Priority Scanning | A scanning facility where one or more talkgroups are given a higher priority than other groups. | P25, TETRA & MCPTT |  |  |
| Short Data Service (SDS) | A means to convey messages of different sizes to a target address. This includes standardized application level services such as text messaging. | TETRA |  | Three types of SDS allow different amounts of user defined binary data to be send. Additional types allow larger amounts to be transmitted and standardized application formats to be recognized. |
| Short Message | A means for a unit to send a numeric message to a unit or group where the numeric message has a programmed meaning to the units involved. | P25 |  | In P25, also known as Status Messaging. |
| Status Query | A means to query a unit for a status update. | P25 & TETRA |  |  |
| Status Update | A means for a unit to update its status to a designated target address by virtue of pre-defined numeric codes. | P25 & TETRA |  |  |
| System Call (aka All Call) | A transmission trunked call that is preemptively propagated to all units within a specific geographic region (e.g., collection of sites) which may include all sites within a system. In P25 trunking systems, a system call activates the call on all sites within the system. In conventional, a similar function is referred to as “All Call”. | P25 & TETRA & MCPTT |  | In TETRA there is a pre-defined broadcast group address which defines a group to which all MS belong. Does not have to be pre-emptive. |
| Triggered Location Request | A means to cause a unit to send GPS location information in accordance with a specified trigger condition, e.g., distance travelled or time. | P25 & TETRA & MCPTT |  |  |
| Unit-to-PSTN Call | A call initiated by a unit to a PSTN destination | P25 & TETRA |  |  |
| Unit-to-unit Call | A call between two specific units. | P25 | Individual call |  |
| Unit-to-Unit Call with Availability Check | A Unit-to-unit call in which the responding unit acknowledges the call before it proceeds. Can appear to the user like a telephony call, except that it is typically half-duplex. | P25 | TETRA: Similar to Individual Call with Hook Signaling. MCPTT: See Private Call | In MCPTT, similar to MCPTT Private Call in Manual Commencement Mode. |
| MCPTT private call | A call between a pair of MCPTT Users using the MCPTT Service with or without MCPTT Floor control. | MCPTT |  | Call can be in manual commencement mode (the initiation of the private call requires the receiving MCPTT User to perform some action to accept or reject the private call setup) or in automatic commencement mode (the initiation of the private call does not require any action on the part of the receiving MCPTT user). |
| Individual Call with Hook Signaling | Options for either on/off hook signaling which includes an alerting process to the called user and an explicit indication that the called user has answered before the call is set up | TETRA | MCPTT Manual Commence-ment Mode | In DMO there is no alerting process. The options are either call set up without presence check where traffic transmission commences without any acknowledgement from the called unit or call set up with a presence check where no acknowledgement is needed |
| Unit-to-Unit Call without Availability Check | A unit-to-unit “barge” call in which no acknowledgement is required by the destination unit. | P25 | In TETRA: Similar to Direct call MCPTT Private Call, Automatic Commence-ment Mode | In TETRA only the called unit needs to respond –there is no option to go ahead with an individual call without an acknowledgement from the called unit As above for DMO |
| Direct individual call | An individual call sent without hook signaling, and therefore does not require a response by the called user. | TETRA | In P25, Unit-to-Unit call without availability check. In MCPTT: Private Call, Automatic Commence-ment Mode |  |
| Unit-to-unit Emergency Call | A unit-to-unit call with preemptive priority and other special behaviors. | P25 | In TETRA: Individual Emergency call. MCPTT private emergency call |  |
| MCPTT private emergency call | An MCPTT private call started by an MCPTT user while in the MCPTT emergency state. | MCPTT | Unit-to-unit Emergency Call (P25) |  |

## Priority Management Nomenclature

The following are common North American LMR terms related to priority and preemption and are compared to those used in TETRA and 3GPP MCPTT.

| Term | Meaning |  | Applies To | Cross Reference to TETRA | Comment |
| --- | --- | --- | --- | --- | --- |
| Override - Dispatch |  | Audio Preemption by a dispatcher/dispatch console. | P25 | Preemption MCPTT Floor Override |  |
| Override - Supervisor |  | Audio Preemption by a user or SU designated as a “supervisor”. | P25 | Preemptive PTT MCPTT Floor Override |  |
| Override - Administrator |  | The MCPTT service enables MCPTT administrators to create a priority hierarchy for determining what participants, participant types, and urgent transmission types shall be granted a request to override an active MCPTT transmission. | MCPTT |  |  |
| Losing Audio |  | When a call is overridden by a dispatcher or supervisor, or when race conditions between sites result in multiple simultaneous talkers in the same call, talk-spurts other than the one selected for transmission on the downlink are “losing audio” and are made available at dispatch positions. | P25 |  | Withdrawn transmission request (either to be restored in the call or needing a new PTT to re-apply). Lost audio is then not available in TETRA |
| Priority |  | An attribute of a group, service or SU that is used to determine the order in which resources are assigned when queuing is required. | P25 & TETRA & MCPTT |  |  |
| Preemption – Audio (aka “override” aka “takeover”) |  | Switching of the outbound audio source during the call, thereby allowing a talkgroup member with a higher individual priority than the currently assigned talker to become the source of outbound audio. The previously assigned talker may be allowed to continue transmitting, or may be signaled to stop transmitting. | P25 & TETRA & MCPTT | . | Similar but TETRA assumes that the previously assigned talker will be signaled to stop transmitting. In 3GPP MCPTT only the overriding participant can transmit or both the overriding and overridden participant can transmit. |
| Preemption – Ruthless (aka “call preemption”) |  | Replacement of an assigned call with a different call of higher priority. Ruthless Preemption occurs when the infrastructure determines that a resource (i.e. channel) is needed to service a higher priority call. Certain high priority calls (like an emergency) on a site with all resources/channels occupied may trigger this type of preemption scenario | P25 & TETRA |  |  |
| Queuing |  | A means by which, when resources are exhausted, service requests are delayed rather than abandoned and prioritized based on “priority”, so that when resources become available, higher priority service requests are processed first. | P25 & TETRA & MCPTT |  |  |

## Management Terminology

The following are common terms referring to management of mobile radios.

| Term | Meaning | Applies To | Cross Reference to TETRA | Comment |
| --- | --- | --- | --- | --- |
| Over the Air Re-keying (OTAR) | 1) Provisioning of keys and related security parameters to SUs by virtue of protocols transmitted “over-the-air”. 2) Project 25 OTAR. 3) TETRA air interface key management. | P25 & TETRA | MCPTT Key Management and Group Management Client/Server | TETRA uses the term OTAR for air interface encryption rekeying only. End-to-end encryption users OTAK. |
| Over The Air Key Management (OTAK) | - Provisioning of security configuration and encryption keys over-the-air. | TETRA & P25 | P25 OTAR; MCPTT Key Management and Group Management Client/Server | In TETRA, OTAK specifically refers to end-to-end key management. OTAR refers to air interface key management. In P25, the terms are used interchangeably. |
| Over the air programming (OTAP) | Provisioning of aspects of personalities to SUs by virtue of over-the-air protocols. | P25 & TETRA | EPS OTA Service provisioning and OMA DM |  |
| Personality | The totality of management information required by a mobile radio to prepare it for operation in a particular system. A mobile radio may have a multiplicity of personalities selectable by the user for different operational scenarios. “Personality” is generally used to refer to the information set required by the radio, rather than the information set required by infrastructure | P25 & TETRA | MCPTT User Profile | In TETRA this definition may overlap with what is sometimes included in “Customization” |
| Programming | Provisioning of personalities to mobile radios by local wired or wireless means that do not include utilization of the land-mobile radio interface. | P25 & TETRA | MCPTT Configuration Management |  |
| MCPTT User profile | The set of information that allows an MCPTT User to employ the MCPTT Service in a given role and/or from a given MCPTT device. | MCPTT |  |  |

## Performance Nomenclature

The following are common terms related to performance measurement.

| Term | Meaning |
| --- | --- |
| Access Time | The time from the pressing of the PTT button to the grant of channel access for a talk-spurt. |
| End-to-End Call Setup Time | The time from the pressing of the PTT button to the receipt of audio. |
| Latency | The “mouth-to-ear” time for audio to be carried between SUs. |

## Security Nomenclature

The following are common terms related to security. The other security option for TETRA is the ability to remotely stun or kill MS if, for example, an MS is lost or stolen.

| Term | Meaning |
| --- | --- |
| Air Interface encryption | Encryption of signaling and traffic across the air interface as opposed to end-to-end encryption. |
| Authentication | Verification of the identity of an entity by cryptographic means. |
| Authorization | Verification that an entity is allowed to use some service or perform some action. |
| End-to-end Encryption | Confidentiality protection achieved by encrypting information at its source (e.g., a mobile radio), passing the encrypted information without modification to its destination (e.g., another mobile radio) |
| Key-fill | The process of providing keys and other security parameters to a mobile radio. |
| Link Layer Authentication | P25 supports Mutual Authentication through an optional feature known as link layer authentication. |
| Link Layer Encryption | P25 will support Air Interface encryption through a feature called Link Layer Encryption which is in development in the TIA standards body. |
| Message Authentication | Verification by cryptographic means that a message was unchanged during transmission, and was sent by one possessing a known key. |
| Mutual Authentication | Authentication of the two entities (user/UE/MR and base station) by each other to, for example, prevent base station spoofing. |
| Zeroization | The process of erasing keys, usually including multiple rewrites of physical memory, to ensure that they cannot be extracted from the device possessing the keys. |