Recommendations for NGMN KPIs and Requirements for 5G

by NGMN Alliance

| Version: | 1.0 |
| --- | --- |
| Date: | 03-June-2016 |
| Document Type: |  |
| Confidentiality Class: |  |
| Authorised Recipients: (for CR documents only) |  |

| Project: | P1 WS#3 BBTS |
| --- | --- |
| Editor / Submitter: |  |
| Contributors: |  |
| Approved by / Date: | 03-June-2016 |

For all Confidential documents (CN, CL, CR):

This document contains information that is confidential and proprietary to NGMN Ltd. The information may not be used, disclosed or reproduced without the prior written authorisation of NGMN Ltd., and those so authorised may only use this information for the purpose consistent with the authorisation.

For Public documents (P):

© 2016 Next Generation Mobile Networks Ltd. All rights reserved. No part of this document may be reproduced or transmitted in any form or by any means without prior written permission from NGMN Ltd.

The information contained in this document represents the current view held by NGMN Ltd. on the issues discussed as of the date of publication. This document is provided “as is” with no warranties whatsoever including any warranty of merchantability, non-infringement, or fitness for any particular purpose. All liability (including liability for infringement of any property rights) relating to the use of information in this document is disclaimed. No license, express or implied, to any intellectual property rights are granted herein. This document is distributed for informational purposes only and is subject to change without notice. Readers should not design products based on this document.

# Key Performance Indicators

Table 1: General KPIs and their high level evaluation approach

| KPI | Definition | Requirement1 | Evaluation method | Remarks |
| --- | --- | --- | --- | --- |
| Bandwidth | The highest aggregated total system bandwidth. It may be supported by single or multiple RF carriers. | Leave to ITU-R to decide | Inspection |  |
| Bandwidth scalability | The ability of the access technology to operate with different bandwidth allocations. | Not worse than IMT-Advanced | Inspection | This bandwidth may be supported by single or multiple RF carriers. Bandwidth scalability’s relevance includes providing higher occupancy for the various spectrum block sizes encountered internationally, including those not multiples of 5/10/20 MHz. |
| Control plane latency | 1. Latency from most battery efficient state (e.g., IDLE) to continuous transfer of large data volume The time it takes for a mobile device in its most “battery efficient” state (e.g. RRC Idle) to start transmission of a large volume of Mobile Originated application layer data over the radio interface, from the time when data arrives at its radio protocol layer 2/3 SDU ingress point. | [10 ms] | Analytical | Notes: The states for 5G are not yet defined, but this should typically be a state transition time between the idle state and a connected state that supports efficient transfer of large data volumes. |
| User plane latency | The time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point via the radio interface in both uplink and downlink directions, where neither device nor Base Station reception is restricted by DRX. | General: 4 ms Special case: 0.5 ms (the feasibility of 0.25 ms should be studied) Note: These values are average values for the uplink and downlink , e.g. 6 ms UL and 2 ms DL would meet the general requirement | Analytical | In the general case the evaluation should include an assessment of the applicable procedural delays when no resource has been already allocated (e.g. request/grant, contention channel access). Furthermore, a fronthauling delay of 250 microseconds should be assumed to allow for a split of processing functions across NG radio access location. The value should be an average latency value, including the averaged HARQ delay. However in special cases for providing a minimum latency (e.g. for URLLC use case or, in case 0.25 ms is feasible, enabling use of NG access for fronthaul), it can be assumed that resources are already allocated, and that no additional transport delay is involved. Furthermore extra robustness can be assumed to reduce the HARQ probability, although the link efficiency impacts of that should then also be evaluated. FFS: How to obtain reasonable assumptions about retransmissions, HARQ probability, etc. in analytical evaluations. |
| Latency for infrequent small packets | For infrequent application layer small packet/message transfer, the time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point at the mobile device to the radio protocol layer 2/3 SDU egress point in the RAN, when the mobile device starts from its most “battery efficient” state. | X ms for ≤Y bytes application layer packet size For mMTC extreme coverage and extreme battery life: X = [10000], Y = [TBD] For other use cases X = [10], Y = [TBD] | Analytical | This requirement shall be evaluated for at least when the device is operating in a scenario where extreme battery life and extreme coverage requirements also need to be simultaneously met. FFS: Reasonable assumptions about retransmissions, HARQ probability, etc. in analytical evaluations. |
| Mobility interruption time | The shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions. | 0 ms | Analytical | Possibly different requirements for intra-frequency and inter-frequency mobility interruption and for different services. In case multi-connectivity is supported, there could be no mobility interruption time. |
| Inter-system handover interruption time | The shortest time duration supported by the system during which a user terminal cannot exchange user plane packets with any base station during transitions between 5G new radio and another radio access technology (RAT). Other RATs include at least LTE evolution. Additional other RATs, including non-3GPP RATs, are FFS. | TBD | Analytical | Possibly different requirements for handovers between new 5G RAT and different RATs. |
| Support for wide range of services | The ability of the access technology to meet the connectivity requirements of a range of existing and future (as yet unknown) services to be operable on a single continuous block of spectrum in an efficient manner. | Yes | Inspection | The components to inspect are for FFS. |
| Duplexing flexibility | The ability of the access technology to adapt its allocation of resources flexibly for uplink and downlink for both paired and unpaired frequency bands. | Yes | Inspection | Applicable for frequency bands in at least existing and future IMT-bands. This flexibility may be used for a wide range of requirements such as uplink/downlink traffic patterns, latency, load, etc. |
| Network energy efficiency | The capability of a radio access network (RAN) to minimize the RAN energy consumption while providing a much better area traffic capacity. | FFS | Inspection and system-level simulation (FTP traffic) | Note: Definition of better area traffic capacity is to be captured as part of the evaluation. 3GPP is encouraged to study models to help clarify how energy efficiency can be evaluated. Evaluation shall allow to benchmark technology proposals on their energy efficiency potential. Inspection: Introduce design principles taking into account energy consumption, e.g., (if no/limited transmission when there is no data, etc.) System-level simulation: This KPI should be evaluated in terms of Mbit/s/W at the global RAN level. Specifically, this KPI has to be defined for different load levels and for different deployment scenarios. 3GPP is encouraged to evaluate energy efficiency not for peak hour but based on a 24 hour daily traffic profile. The related evaluation methodology needs to be developed within 3GPP relevant studies [5G Technology Study] (ex. ETSI 203 228). For example, base station load levels could be recorded during simulations to be translated into energy consumption by means of a base station power model (e.g., Greentouch) assuming the following energy efficiency (EE) formulation: - al=weight for load level l - bK=weight for deployment scenario K - Vl=aggregated throughput served in the simulated area for load level l (in Mbps), or equivalently served traffic volume divided by the simulation period - ECl=sum of the average power consumption of all RAN nodes in the simulated area during load level l (in Watt) EEscenario=load level lalVlECl EEglobal=scenario KbKEEscenario Other methodology could also be studied. Proposals for the 24 hour daily traffic profiles could be provided by operators. At least, energy efficiency should be evaluated in coverage limited environments (e.g., rural) and capacity limited environments (e.g., urban). Every effort should be made to obtain the energy gain without degrading the performance, but the technology should allow native flexibility for the operator to configure trade-off between energy efficiency versus performance where justified. |
| Peak data rate | The highest theoretical data rate which is the received data bits assuming error-free conditions assignable to a single mobile station, when all available radio resources for the corresponding link direction are utilised (i.e., excluding radio resources that are used for physical layer synchronisation, reference signals or pilots, guard bands and guard times). | N/A | Analytical | NGMN will not define requirements for peak data rate and peak spectral efficiency. Consistent user experience is of a higher priority for NGMN than peak data rate. |
| Peak spectral efficiency | The peak data rate normalized by bandwidth. | N/A | Analytical | See remark for peak data rate (above). |

Note: The different content from the previous LS to RAN AH is marked in red.

Table 2: Deployment-scenario specific KPIs and requirements

| KPI | Definition | Requirement | Evaluation method | Remarks |
| --- | --- | --- | --- | --- |
| Transmission/ Reception Point (TRP) average spectrum efficiency | The aggregate throughput of all users (the number of correctly received bits, i.e. the number of bits contained in the service data units (SDUs) delivered to Layer 3, over a certain period of time) divided by the channel bandwidth divided by the number of TRPs. The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalised appropriately considering the uplink/downlink ratio. The TRP spectral efficiency is measured in bit/s/Hz/TRP. A 3 sector site consists of 3 TRPs. | TBD | System-level simulation (full-buffer) | Assessment for multi-layer and multi-band is FFS. How to evaluate outdoor and indoor users independently needs to be considered. NGMN white paper targets significant improvements in spectrum efficiency. |
| User experienced data rate | The 5%-percentile of the user throughput. User throughput (during active time) of an individual burst is defined as the size of a burst divided by the time between the arrival of the first packet of a burst and the reception of the last packet of the burst. | Indoor DL: 1 Gbit/s UL: 500 Mbit/s Dense urban DL: 300 Mbit/s UL: 50 Mbit/s Urban DL: 50 Mbit/s UL: 25 Mbit/s Rural DL: 50 Mbit/s UL: 25 Mbit/s High speed DL: 50 Mbit/s UL: 25 Mbit/s | System-level simulation (FTP traffic) | This KPI needs to be achieved at the target area traffic capacity for each relevant deployment scenario. The definition of a burst depends on the traffic model and is FFS. The requirements derived from the NGMN white paper may be revised after a careful review of assumptions on connection density and available bandwidth. |
| 5th percentile user spectrum efficiency | The cell edge user spectral efficiency is defined as 5% point of the cumulative distribution function (CDF) of the normalized user throughput. The (normalized) user throughput is defined as the average user throughput (the number of correctly received bits by users, i.e., the number of bits contained in the SDU delivered to Layer 3, over a certain period of time, divided by the channel bandwidth and is measured in bit/s/Hz. The channel bandwidth for this purpose is defined as the effective bandwidth times the frequency reuse factor, where the effective bandwidth is the operating bandwidth normalised appropriately considering the uplink/downlink ratio. | TBD | System-level simulation (full buffer) | This KPI was previously called user experienced spectrum efficiency in (reference LS to RAN #70). Assessment for multi-layer and multi-band is FFS. How to evaluate outdoor and indoor users independently needs to be considered. NGMN white paper targets significant improvements in spectrum efficiency. |
| Connection density | Total number of devices fulfilling specific QoS per unit area (per km2). | 1e6 devices/km2 for mMTC for urban environment | Option 1: Analytical Option 2: Simulation | Foreseen as most relevant for mMTC. QoS definition should take into account the amount of data generated within a time t_gen that can be sent or received within a given time, t_sendrx, with x% probability. Note 1: Evaluation environment is FFS Note 2: Report bandwidth used in evaluation. |
| Area traffic capacity | Full buffer: Total traffic throughput served per geographic area (in Mbit/s/m2). The computation of this metric is based on full buffer traffic. |  | Analytical |  |
|  | Non full buffer: Total traffic throughput served per geographic area (in Mbit/s/m2). Both the user experienced data rate and the area traffic capacity need to be evaluated at the same time using the same traffic model. |  | System level simulations (FTP traffic) | # This KPI is used to derive the burst arrival rate for the FTP simulations |
| Mobility | Maximum user speed at which a defined QoS can be achieved (in km/h). | 500 km/h | Simulation (link and system level) | Mobility classes and QoS will be defined for each scenario. |
| Reliability | The success probability of transmitting a layer 2/3 packet of [x bytes] within a maximum time of [t ms], which is the time it takes to deliver a small data packet from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface at a certain channel quality. | TBD | Link-level simulation | Foreseen as most relevant for URLLC. If channel access is not granted, the reliability and delay of that should be taken into account. The channel quality target and its variation characteristics need to be defined, including the relevant channel model. Mobility aspects can be treated as part of the channel quality to be targeted. |
| Device battery life | The battery life of the device without recharge. For at least mMTC, device battery life in extreme coverage shall be based on the activity of mobile originated data transfer consisting of [200 bytes] UL per day followed by [20 bytes] DL from maximum coupling loss of [164 dB], assuming a stored energy capacity of [5Wh]. | [15 years] | Link-level simulation (Reuse TR 45.820) | Battery life KPI definition of other terminal types than mMTC is FFS. The battery life extension should not be achieved by a relaxation of any of the key parameters - and their corresponding values - indicated in the definition. NGMN white paper targets device battery life of up to 15 years. 3GPP should investigate the feasible operating boundaries (data activity, coupling loss, etc.) under which such a device battery life target could be reached. |
| Extreme Coverage | “Maximum coupling loss” in uplink and downlink between device and Base Station site (antenna connector(s)) for a data rate of [160bps], where the data rate is observed at the egress/ingress point of the radio protocol stack in uplink and downlink. | [164 dB] | Link-level simulation (Reuse TR 45.820) | Foreseen as most relevant for mMTC. |
| UE energy efficiency | The capability of a UE to reduce UE modem energy consumption while sustaining better performance (e.g., mobile broadband data rate). | Significant enhancement compared to LTE | Inspection | Foreseen as most relevant for eMBB. |

# Deployment scenarios – key challenges and evaluation goals

Table 3: Deployment scenarios with associated key challenges and evaluation goals

| Usage scenario | Deployment scenario | Key challenges | Most relevant KPIs | Evaluation goals | Remarks |
| --- | --- | --- | --- | --- | --- |
| eMBB | Indoor hotspot (eMBB-InH) | - high capacity - high density - consistent user experience - poor cell isolation | - TRP spectrum efficiency - 5th percentile user spectrum efficiency - user experienced data rate | - Show that high frequency bands can be used indoor. | Candidate scenario for system level evaluations. |
|  | Dense urban (eMBB-UMx) | - high capacity - high density - consistent user experience - poor cell isolation - users distributed in height | - TRP spectrum efficiency - 5th percentile user spectrum efficiency - user experienced data rate - [network energy efficiency] | - Clarify the feasibility of different outdoor deployment options for serving both outdoor and indoor users: macro only or macro and outdoor small cells; considering different spectrum options: low frequency bands only , high frequency bands only or a combination of both high and low frequency bands. | Candidate scenario for system-level evaluations. |
|  | Urban macro (eMBB-UMa) | - capacity - consistent user experience | - TRP spectrum efficiency - 5th percentile user spectrum efficiency - user experienced data rate - [network energy efficiency] | - Clarify the feasibility of a macro outdoor deployment considering different spectrum options: low frequency bands only, high frequency bands only or a combination of both high and low frequency bands to serve both outdoor and indoor users. | Candidate scenario for system-level evaluations. |
|  | Rural macro (eMBB-RMa) | - consistent user experience over wide area - capacity - high user mobility | - TRP spectrum efficiency - 5th percentile user spectrum efficiency - user experienced data rate - mobility - [network energy efficiency] | - Show that low frequency bands can provide wide area coverage. - Clarify the feasibility of meeting the user experience requirement and the capacity requirement with a reasonable site density. | Candidate scenario for system-level evaluations. |
|  | High speed train (eMBB-HS) | - very high speed - high capacity - high connection density - consistent user experience | - user experienced data rate - mobility | - Show that link performance can be maintained at very high speeds. - Show the capacity requirement can be met with a reasonable site density. | Special test (link level) could be employed, e.g., under a rural macro setting. |
| mMTC |  | - massive number of devices - coverage - energy constraints | - connection density - extreme coverage - device battery life | - Clarify the coverage and capacity impact of offering mMTC service (e.g., by evaluation of link-level performance). | Some relevant KPIs may be evaluated without system level simulations. For KPIs that need system level simulations, eMBB deployment scenarios could be reused, possibly with some parameter changes. The need for additional non-eMBB specific deployment scenarios is FFS. |
| URLLC |  | - high reliability - very low latency - capacity - coverage - mobility | - latency - reliability | - Clarify the coverage and capacity impact of offering URLLC service (e.g., by evaluation of link-level performance). |  |

Table 4: Deployment scenarios and spectrum options

| Deployment scenario | KPI | KPI value | Deployment options | Spectrum options | Bandwidth | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Indoor hotspot (eMBB-InH) | User experienced data rate Area traffic capacity (non-full buffer) for a single operator4 | DL: 1 Gbps UL: 500 Mbps DL: 15 Tbps/km2 UL: 2 Tbps/km2 | Indoor floor, ISD 20m 100% indoors | Above 6 GHz (around 30 GHz or around 70 GHz) | Up to 1000 MHz |  |
|  |  |  |  | Above 6 GHz (around 30 GHz or around 70 GHz) and Below 6 GHz (around 4 GHz) | Up to 1000 MHz and Up to 200 MHz | Evaluation only required if >6GHz cannot meet requirement. |
| Dense urban (eMBB-UMx) | User experienced data rate Area traffic capacity (non-full buffer) | DL: 300 Mbps UL: 50 Mbps DL: 250 Gbps/sq km UL: 42 Gbps/sq km | Macrocell only, ISD 200m 20% outdoor (3 km/h) and 80% indoor (3 km/h) | Below 6 GHz (around 4 GHz) | Up to 200 MHz |  |
|  |  |  |  | Above 6 GHz (around 30 GHz) | Up to 1000 MHz |  |
|  |  |  |  | Below 6 GHz (around 4 GHz) and above 6 GHz (around 30 GHz) combined | Up to 200 MHz and Up to 1000 MHz |  |
|  |  |  | Macro cells with outdoor small cells Macro ISD: 200m Small cell ISD: [3] small cell per macro sector. Users: 20% outdoor (3 km/h) and 80% indoor (3 km/h) | Below 6GHz (around 4 GHz) for both macro and small cells | Up to 200 MHz |  |
|  |  |  |  | Above 6 GHz (around 30 GHz) for both macro and small cells | Up to 1000MHz |  |
|  |  |  |  | Below 6 GHz (around 4 GHz) and above 6 GHz (around 30 GHz) combined for macro and small cells | Up to 200 MHz and Up to 1000 MHz |  |
|  |  |  |  | Below 6 GHz (around 4 GHz) for macro and above 6 GHz (around 30 GHz or around 70 GHz) for small cells | Up to 200 MHz and Up to 1000 MHz |  |
| Urban macro (eMBB-UMa) | User experienced data rate Area traffic capacity (non-full buffer) | DL: 50 Mbps UL: 25 Mbps DL: 33 Gbps/sq km UL: 17 Gbps/sq km | Macro only, hex grid ISD: 500 m Users: 1)20% outdoor (3 km/h) and 80% indoor (3 km/h) 2)100% in cars (30 km/h) | Below 6 GHz (around 2 GHz or around 4 GHz) | Up to: 160 Mhz (2GHz) 200 MHz (4GHz) |  |
|  |  |  |  | Above 6 GHz (around 30 GHz) | Up to 1000 MHz |  |
|  |  |  |  | Below 6 GHz (around 2 GHz or around 4 GHz) and above 6 GHz (around 30 GHz) combined. | Up to 160 Mhz (2GHz) or200 MHz (4GHz) and Up to 1000 MHz (30 Ghz) |  |
| Rural macro (eMBB-RMa) | User experienced data rate Area traffic capacity (non-full buffer) | DL: 50 Mbps UL: 25 Mbps DL: 0.33 Gbps/sq km UL: 0.17 Gbps/sq km | Macro only ISD: 1732 m Users: 1)20% outdoor (3 km/h) and 80% indoor (3 km/h) 2)100% in cars (120 km/h) | Below 6 GHz (around 800 MHz or around 4 GHz). | Up to 200 MHz | Note: Bands above 6 GHz if needed. |
|  |  |  | Macro only ISD: 5 km Users: 1)20% outdoor (3 km/h) and 80% indoor (3 km/h) 2)100% in cars (120 km/h) | Below 6 GHz (around 800 MHz and around 2 GHz combined) | Up to 100 MHz (e.g., 60 MHz @ around 800 MHz 40 MHz @ around 2 GHz) |  |
| High speed (eMBB-HS) Train | User experienced data rate Users/train Traffic/train | DL: 50 Mbps UL: 25 Mbps 1000 /train DL: 15 Gbps/ train UL: 7.5 Gbps/ train | Outdoor base stations to users in trains @ 500 Km/h Use of link level or system level simulation is TBD | Below 6 GHz (Tbc) |  |  |
|  |  |  | Outdoor BS to relay on trains, and then from relay to users in trains @ 500 Km/h Use of link level or system level simulation is TBD | For BS to relay: Below 6 GHz (around 4 GHz) For relay to UE: Above 6 GHz (around 30 GHz or around 70 GHz) and below 6 GHz (around 4 G Hz) | Up to 200 MHz Up to 1000 MHz and Up to 200 MHz | Evaluations will focus on the BS to relay link. |
|  |  |  |  | For BS to relay: Above 6 GHz (around 30 GHz) For relay to UE: Above 6 GHz (around 30 GHz or around 70 GHz) and below 6 GHz (around 4 GHz) | Up to 1000 MHz Up to 1000 MHz and Up to 200 MHz | Evaluations will focus on the BS to relay link. |

# Appendix: Assumptions for deriving Area traffic capacity (non-full buffer) for a single operator

- Downlink:

|  | Overall User density | User density (1 out of 3 operators) | Active user average data rate | Activity factor | Average user busy hour usage rate | Area traffic capacity (non-full buffer) |
| --- | --- | --- | --- | --- | --- | --- |
|  | per sq km | per sq km | Mbps | % | Mbps | Gbps/sq km |
| Indoor | 250,000 | 250,000 | 200 | 30% | 60 | 15000 |
| Dense Urban | 25,000 | 8333 | 300 | 10% | 30 | 250 |
| Urban | 10,000 | 3333 | 50 | 20% | 10 | 33 |
| Rural | 100 | 33 | 50 | 20% | 10 | 0.33 |
| High speed | 1000/train | 1000/train | 50 | 30% | 15 | - bps/train |

- Uplink:

|  | Overall User density | User density (1 out of 3 operators) | Active user average data rate | Activity factor | Average user busy hour usage rate | Area traffic capacity (non-full buffer) |
| --- | --- | --- | --- | --- | --- | --- |
|  | per sq km | per sq km | Mbps | % | Mbps | Gbps/sq km |
| Indoor | 250,000 | 250000 | 26.67 | 30% | 8.0 | 2000 |
| Dense Urban | 25,000 | 8333 | 50 | 10% | 5 | 42 |
| Urban | 10,000 | 3333 | 25 | 20% | 5 | 17 |
| Rural | 100 | 33 | 25 | 20% | 5 | 0.17 |
| High speed | 1000/train | 1000/train | 25 | 30% | 7.5 | 7.5 Gbps/train |

Indicates deployment options that can be studied to identify the option(s) that can meet the requirements. Not all options need to be necessarily evaluated.

The options noted here are for evaluation purpose, and do not mandate the deployment of these options or preclude the study of other spectrum options.

This is not the simulation bandwidth. Simulation bandwidth may be much smaller. This bandwidth denotes an aspirational background to the capacity/user experienced data rate/area traffic capacity requirements and not necessarily what will be available to an operator in a single block. The bandwidth value given is per operator.

The user experienced data rate (non full buffer) and area traffic capacity values are target values and not strict requirements. The user experienced data rate is the target for 5th percentile user data rate at a loading determined by the area traffic capacity (non-full buffer) target for that deployment scenario.

In case the user experienced data rate target is not achieved at the target loading for a particular deployment scenario, the 5th percentile user data rate shall be calculated at a reasonably well utilized loading (e.g. consistent with 50% harmonic mean normalized cell throughput (HM-NCT, see TR 36.814)).

Providing results for different loading levels is useful, but it is not strictly necessary to check that the user experience data rate is met. It is up to 3GPP to decide how many loading levels are simulated. The assumptions for deriving area traffic capacity (non-full buffer) for a single operator are provided in the appendix

A range of bands from 24 GHz – 40 GHz identified for WRC-19 are currently being considered and around 30 GHz is chosen as a proxy for this range.

A range of bands from 66 GHz – 86 GHz identified for WRC-19 are currently being considered and around 70 GHz is chosen as a proxy for this range.

This does not preclude the study of other options (e.g., 4 or 10 small cells).