| ECC PT1 |  | ECC PT1(25)171 Annex 04 |
| --- | --- | --- |
| ECC PT1 #82 |  |  |
| Vilnius, Lithuania and online, 15-19 September 2025 |  |  |
| Date issued: | 17 September 2025 |  |
| Subject: | Parameters for AAS BS operating in bands below 1 GHz |  |
| To: | Dr. Younsun Kim, 3GPP TSG RAN Chair E-mail: younsun@samsung.com Mr. Joern Krause, TSG RAN secretary E-mail: joern.krause@etsi.org Dr. Yang Tang, 3GPP TSG RAN WG4 Chair E-mail: yang.tang@apple.com Ms. Carolyn Taylor, RAN WG4 secretary E-mail: Carolyn.Taylor@etsi.org 3GPP Liaisons Coordinator E-mail: 3GPPLiaison@etsi.org |  |
| Cc: | Dr. Ivica Stevanovic, WG SE Chair E-mail: ivica.stevanovic@bakom.admin.ch Dr. Michael Paynter, SE21 Chair E-mail: michael.paynter@ofcom.org.uk Mr. Holger Butscheidt, ETSI TC ERM Chair E-mail: holger.butscheidt@bnetza.de, ermSupport@etsi.org Mr. Dominique Everaere, ETSI TC MSG/TFES Chair E-mail: dominique.everaere@erisson.com Mr. Marcello Pagnozzi, ETSI Technical Officer TC ERM E-mail: marcello.pagnozzi@etsi.org Mr. Muhammad Tayyeb, ETSI Technical Officer TC MSG/TFES E-mail: MSGTFESsupport@etsi.org |  |

Dear Younsun Kim, dear Yang Tang,

ECC agreed at its 67th plenary meeting in June 2025 to start work on the suitability of Active Antenna Systems (AAS) operation in the frequency bands 694-791 MHz (700 MHz), 791-862 MHz (800 MHz) and 880-960 MHz (900 MHz). The work will be carried out in ECC PT1 and captured in new ECC Reports that would form the basis for the update of the regulatory technical conditions in ECC Decision (15)01 (700 MHz), ECC Decision (09)03 (800 MHz) and ECC Decision (06)13 (900/1800 MHz).

ECC PT1 has in the past conducted compatibility studies with base stations (BSs) using AAS in the 1800 MHz band and other mobile bands at higher frequencies. The characteristics of AAS BSs in those bands are well known.

The situation for AAS BSs in bands below 1 GHz is different. ECC PT1 expects that AAS BSs in the three bands under study would be deployed in existing sites and hence the deployment parameters would be similar to non-AAS. However, ECC PT1 does not have information about the technical characteristics of AAS BSs in these frequency bands.

ECC PT1 notes that 3GPP TS 37.105 and TS 38.104 contain BS RF requirements such as unwanted emissions for AAS BSs and that these TSs do not set a lower frequency limit of validity. With this in mind, ECC PT1 would like to get the information from 3GPP on the following points:

- Unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands.

- Whether 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration.

- A model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, both in-band and out of band for frequency bands under consideration.

ECC PT1 kindly invites 3GPP to provide a response on the above points for the next ECC PT1 meeting scheduled for 12-16 January 2026.

Best regards

Christoph Hildebrand

ECC PT1 Chair

E-mail: Christoph.Hildebrand@BNetzA.de