|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  |  | SG16-TD212/PLEN |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 16 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  |  | 7/16 | Ljubljana, 9-20 July 2018 |  |
| TD |  |  |  |  |  |
| Source: |  |  | Rapporteur Q7/16 |  |  |
| Title: |  |  | Consent: G.722.2 (2003) Annex C (2017) Cor.1 "Corrections to Table C.5" (New) |  |  |
| Purpose: |  |  | Proposal |  |  |
| Contact: |  | Paul Coverdale Huawei Technologies China |  |  | Tel: +1 613 820 6643 Fax: E-mail: coverdale@sympatico.ca |  |

| Keywords: | Audio codec; wideband |
| --- | --- |
| Abstract: | This document proposes a corrigendum to G.722.2 Annex C in order to harmonise with the corresponding 3GPP codec, AMR-WB |

An incoming LS was received from 3GPP SA4 (SG12-TD190/Gen, with attachment SG12-TD190A1) which thanked Q7/16 for pointing out some minor discrepancies in the description of the 3GPP implementation (AMR-WB, TS 26.173 version 14.1.0) of ITU-T G.722.2.

While dealing with these discrepancies, 3GPP SA4 uncovered some additional minor discrepancies in the capitalisation of certain names in Table C.3. A corrigendum to G.722.2 Annex C has been prepared to harmonise the two codec specifications, for Consent at this meeting.

AAP Summary

This Corrigendum details a number of minor corrections to the capitalizations of certain file and table names in the description in Table C.3 used in the C-code in G.722.2 Annex C. These changes harmonise Annex C with the AMR-WB codec specification, TS 26.173 version 14.1.0 (Rel-14).

Draft new ITU-T Recommendation G.722.2 (2003) Annex C (2017) Cor.1

Wideband coding of speech at around 16 kbit/s using Adaptive Multi-
Rate Wideband (AMR-WB)
Annex C: Fixed-point C-code:
Corrections to Table C.3

Summary

This Corrigendum details a number of minor corrections to the capitalizations of certain file and table names in the description in Table C.3 used in the C-code in G.722.2 Annex C. These changes harmonise Annex C with the AMR-WB codec specification, TS 26.173 version 14.1.0 (Rel-14).

...

#### C.1.4.2	Description of fixed tables used in the C-code

Table C.5 before the change:

| Table C.5 – Fixed tables |  |  |  |
| --- | --- | --- | --- |
| File | Table name | Length | Description |
| C4t64fx.c | Tipos | 36 | Starting points of iterations |
| Cod_main.c | HP_gain | 16 | High band gain table for 23.85 kbit/s mode |
| Cod_main.c | Interpol_frac | 4 | LPC interpolation coefficients |
| Cod_main.c | Isp_init | 16 | Isp tables for initialization |
| Cod_main.c | Isf_init | 16 | Isf tables for initialization |
| D_gain2.c | cdown_unusable | 7 | Attenuation factors for codebook gain in lost frames |
| D_gain2.c | cdown_usable | 7 | Attenuation factors for codebook gain in bad frames |
| D_gain2.c | pdown_unusable | 7 | Attenuation factors for adaptive codebook gain in lost frames |
| D_gain2.c | pdown_usable | 7 | Attenuation factors for adaptive codebook gain in bad frames |
| D_gain2.c | Pred | 4 | Algebraic code book gain MA predictor coefficients |
| Dec_main.c | HP_gain | 16 | High band gain table for 23.85 kbit/s mode |
| Dec_main.c | Interpol_frac | 4 | LPC interpolation coefficients |
| Dec_main.c | Isp_init | 16 | Isp tables for initialization |
| Dec_main.c | Isf_init | 16 | Isf tables for initialization |
| Decim54.c | fir_down | 120 | Downsample FIR filter coefficients |
| Decim54.c | fir_up | 120 | Upsample FIR filter coefficients |
| Dtx.c | en_adjust | 9 | Energy scaling factor for each mode during comfort noise |
| Grid100.tab | grid | 101 | This table points to specific Chebyshev polynomials |
| Ham_wind.tab | Window | 384 | LP analysis window |
| Hp400.c | A | 3 | HP filter coefficients (denominator) in higher band energy estimation |
| Hp400.c | B | 3 | HP filter coefficients (numerator) in higher band energy estimation |
| Hp50.c | A | 3 | HP filter coefficients (denominator) in pre-filtering |
| Hp50.c | B | 3 | HP filter coefficients (numerator) in pre-filtering |
| Hp6k.c | Fir_6k_7k | 31 | Bandpass FIR filter coefficients for higher band generation |
| Hp7k.c | Fir_7k | 31 | Bandpass FIR filter coefficients for higher band in 23.85 kbit/s mode |
| Hp_wsp.c | A | 3 | HP filter coefficients (denominator) in open-loop lag gain computation |
| Hp_wsp.c | B | 3 | HP filter coefficients (numerator) in open-loop lag gain computation |
| Isp_isf.tab | slope | 128 | Table to compute cos(x) in Isf_isp() |
| Isp_isf.tab | Table | 129 | Table to compute acos(x) in Isp_isf() |
| Lag_wind.tab | lag_h | 16 | High part of the lag window table |
| Lag_wind.tab | lag_l | 16 | Low part of the lag window table |
| Lp_dec2.c | h_fir | 5 | HP FIR filter coefficients in open-loop lag search |
| Math_op.c | table_isqrt | 49 | Table used in inverse square root computation |
| Math_op.c | table_pow2 | 33 | Table used in power of two computation |
| P_med_ol.tab | Corrweight | 199 | Weighting of the correlation function in open loop LTP search |
| Ph_disp.c | ph_imp_low | 64 | Phase dispersion impulse response |
| Ph_disp.c | ph_imp_mid | 64 | Phase dispersion impulse response |
| Pitch_fr4.c | inter4_1 | 32 | Interpolation filter coefficients |
| Pred_lt4.c | inter4_2 | 128 | Interpolation filter coefficients |
| Q_gain2.c | pred | 4 | Algebraic code book gain MA predictor coefficients |
| Q_gain2.tab | t_qua_gain6b | 2*64 | Gain quantization table for 6-bit gain quantization |
| Q_gain2.tab | t_qua_gain7b | 2*128 | Gain quantization table for 7-bit gain quantization |
| Qisf_ns.tab | dico1_isf_noise | 2*64 | 1st ISF quantizer for comfort noise |
| Qisf_ns.tab | dico2_isf_noise | 3*64 | 2nd ISF quantizer for comfort noise |
| Qisf_ns.tab | dico3_isf_noise | 3*64 | 3rd ISF quantizer for comfort noise |
| Qisf_ns.tab | dico4_isf_noise | 4*32 | 4th ISF quantizer for comfort noise |
| Qisf_ns.tab | dico5_isf_noise | 4*32 | 5th ISF quantizer for comfort noise |
| Qisf_ns.tab | mean_isf_noise | 16 | ISF mean for comfort noise |
| Qpisf_2s.tab | dico1_isf | 9*256 | 1st ISF quantizer of the 1st stage |
| Qpisf_2s.tab | dico2_isf | 7*256 | 2nd ISF quantizer of the 1st stage |
| Qpisf_2s.tab | dico21_isf | 3*64 | 1st ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode) |
| Qpisf_2s.tab | dico21_isf_36b | 5*128 | 1st ISF quantizer of the 2nd stage (the 6.60 kbit/s mode) |
| Qpisf_2s.tab | dico22_isf | 3*128 | 2nd ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode) |
| Qpisf_2s.tab | dico22_isf_36b | 4*128 | 2nd ISF quantizer of the 2nd stage (the 6.60 kbit/s mode) |
| Qpisf_2s.tab | dico23_isf | 3*128 | 3rd ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode) |
| Qpisf_2s.tab | dico23_isf_36b | 7*64 | 3rd ISF quantizer of the 2nd stage (the 6.60 kbit/s mode) |
| Qpisf_2s.tab | dico24_isf | 3*32 | 4th ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode) |
| Qpisf_2s.tab | dico25_isf | 4*32 | 5th ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode) |
| Qpisf_2s.tab | Mean_isf | 16 | ISF mean |

Table C.5 after the change:

| Table C.5 – Fixed tables |  |  |  |
| --- | --- | --- | --- |
| File | Table name | Length | Description |
| c4t64fx.c | tipos | 36 | Starting points of iterations |
| cod_main.c | HP_gain | 16 | High band gain table for 23.85 kbit/s mode |
| cod_main.c | interpol_frac | 4 | LPC interpolation coefficients |
| cod_main.c | isp_init | 16 | Isp tables for initialization |
| cod_main.c | isf_init | 16 | Isf tables for initialization |
| d_gain2.c | cdown_unusable | 7 | Attenuation factors for codebook gain in lost frames |
| d_gain2.c | cdown_usable | 7 | Attenuation factors for codebook gain in bad frames |
| d_gain2.c | pdown_unusable | 7 | Attenuation factors for adaptive codebook gain in lost frames |
| d_gain2.c | pdown_usable | 7 | Attenuation factors for adaptive codebook gain in bad frames |
| d_gain2.c | pred | 4 | Algebraic code book gain MA predictor coefficients |
| dec_main.c | HP_gain | 16 | High band gain table for 23.85 kbit/s mode |
| dec_main.c | interpol_frac | 4 | LPC interpolation coefficients |
| dec_main.c | isp_init | 16 | Isp tables for initialization |
| dec_main.c | isf_init | 16 | Isf tables for initialization |
| decim54.c | fir_down | 120 | Downsample FIR filter coefficients |
| decim54.c | fir_up | 120 | Upsample FIR filter coefficients |
| dtx.c | en_adjust | 9 | Energy scaling factor for each mode during comfort noise |
| grid100.tab | grid | 101 | Grid points of Chebyshev polynomials |
| ham_wind.tab | window | 384 | LP analysis window |
| hp400.c | a | 3 | HP filter coefficients (denominator) in higher band energy estimation |
| hp400.c | b | 3 | HP filter coefficients (numerator) in higher band energy estimation |
| hp50.c | a | 3 | HP filter coefficients (denominator) in pre-filtering |
| hp50.c | b | 3 | HP filter coefficients (numerator) in pre-filtering |
| hp6k.c | fir_6k_7k | 31 | Bandpass FIR filter coefficients for higher band generation |
| hp7k.c | fir_7k | 31 | Bandpass FIR filter coefficients for higher band in 23.85 kbit/s mode |
| hp_wsp.c | a | 3 | HP filter coefficients (denominator) in open-loop lag gain computation |
| hp_wsp.c | b | 3 | HP filter coefficients (numerator) in open-loop lag gain computation |
| isp_isf.tab | slope | 128 | Table to compute cos(x) in Isf_isp() |
| isp_isf.tab | table | 129 | Table to compute acos(x) in Isp_isf() |
| lag_wind.tab | lag_h | 16 | High part of the lag window table |
| lag_wind.tab | lag_l | 16 | Low part of the lag window table |
| lp_dec2.c | h_fir | 5 | HP FIR filter coefficients in open-loop lag search |
| math_op.c | table_isqrt | 49 | Table used in inverse square root computation |
| math_op.c | table_pow2 | 33 | Table used in power of two computation |
| p_med_ol.tab | corrweight | 199 | Weighting of the correlation function in open loop LTP search |
| ph_disp.c | ph_imp_low | 64 | Phase dispersion impulse response |
| ph_disp.c | ph_imp_mid | 64 | Phase dispersion impulse response |
| pitch_fr4.c | inter4_1 | 32 | Interpolation filter coefficients |
| pred_lt4.c | inter4_2 | 128 | Interpolation filter coefficients |
| q_gain2.c | pred | 4 | Algebraic code book gain MA predictor coefficients |
| q_gain2.tab | t_qua_gain6b | 2*64 | Gain quantization table for 6-bit gain quantization |
| q_gain2.tab | t_qua_gain7b | 2*128 | Gain quantization table for 7-bit gain quantization |
| qisf_ns.tab | dico1_isf_noise | 2*64 | 1st ISF quantizer for comfort noise |
| qisf_ns.tab | dico2_isf_noise | 3*64 | 2nd ISF quantizer for comfort noise |
| qisf_ns.tab | dico3_isf_noise | 3*64 | 3rd ISF quantizer for comfort noise |
| qisf_ns.tab | dico4_isf_noise | 4*32 | 4th ISF quantizer for comfort noise |
| qisf_ns.tab | dico5_isf_noise | 4*32 | 5th ISF quantizer for comfort noise |
| qisf_ns.tab | mean_isf_noise | 16 | ISF mean for comfort noise |
| qpisf_2s.tab | dico1_isf | 9*256 | 1st ISF quantizer of the 1st stage |
| qpisf_2s.tab | dico2_isf | 7*256 | 2nd ISF quantizer of the 1st stage |
| qpisf_2s.tab | dico21_isf | 3*64 | 1st ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode) |
| qpisf_2s.tab | dico21_isf_36b | 5*128 | 1st ISF quantizer of the 2nd stage (the 6.60 kbit/s mode) |
| qpisf_2s.tab | dico22_isf | 3*128 | 2nd ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode) |
| qpisf_2s.tab | dico22_isf_36b | 4*128 | 2nd ISF quantizer of the 2nd stage (the 6.60 kbit/s mode) |
| qpisf_2s.tab | dico23_isf | 3*128 | 3rd ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode) |
| qpisf_2s.tab | dico23_isf_36b | 7*64 | 3rd ISF quantizer of the 2nd stage (the 6.60 kbit/s mode) |
| qpisf_2s.tab | dico24_isf | 3*32 | 4th ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode) |
| qpisf_2s.tab | dico25_isf | 4*32 | 5th ISF quantizer of the 2nd stage (not the 6.60 kbit/s mode) |
| qpisf_2s.tab | mean_isf | 16 | ISF mean |

_________________