|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |  |  | COM 16 – LS 172 – E |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  |  |  |  |  |
|  |  |  |  |  | English only Original: English |  |  |
| Question(s): |  | 16/16 |  | Geneva, 19 - 30 July 2010 |  |  |  |
| LIAISON STATEMENT |  |  |  |  |  |  |  |
| Source: |  | ITU-T SG 16 |  |  |  |  |  |
| Title: |  | Reply LS to ETSI TC STQ and 3GPP SA, TC TISPAN, ITU-T SG 12 on procedures for the identification and selection of common modes of de-jitter buffers and echo cancellers |  |  |  |  |  |
| LIAISON STATEMENT |  |  |  |  |  |  |  |
| For action to: |  |  | ETSI TC STQ |  |  |  |  |
| For comment to: |  |  | - |  |  |  |  |
| For information to: |  |  | 3GPP SA, TC TISPAN, ITU-T SG 12 |  |  |  |  |
| Approval: |  |  | ITU-T SG 16 meeting (Geneva, 19-30 July 2010) |  |  |  |  |
| Deadline: |  |  | 31 October 2010 |  |  |  |  |
| Contact: |  | Bob Reeves BT UK |  |  |  | Tel: +44 1473 644654 Fax: +44 1473 649777 Email: bob.reeves@bt.com |  |

ITU-T SG 16 Question 16/16 thanks ETSI STQ for the liaison statement (your STQ(10)0103r3, our TD 287/Gen) informing us of your new work item regarding “Procedures for the identification and selection of common modes of de-jitter buffers and echo cancellers”.

Q16/16 has reviewed V.0.0.4 of this document and we have the following comments:

- It is noted that this is a very early draft and although echo cancellers are within scope, there appears to be no detail concerning their operation in the body of the document.

- It is not clear whether the attachment is a Technical Report (TR) or a Technical Specification (TS) since both are referred to (e.g. Forward and Introduction)

- Given that the document covers both de-jitter buffers and echo cancellers, it would be useful to show the relative location of these two functions in a suitable diagram towards the beginning of the document. It is noted that Figures 2 and 3 currently only show the de-jitter buffer function.

- Section 5.8 contains the following statement: “The adaptive jitter buffer may activate for signal drop-outs higher than 100 ms”. We strongly advise against this requirement, due to its potential effect on non-continuous carrier data modem signals which may be severely degraded by mid-call de-jitter buffer adaptations. Our recommendation would be to fix the de-jitter buffer on detection of the appropriate tone and remain fixed for the duration of the call. The downside of this is accidental fixing of the de-jitter buffer due to speech, but it is considered to be the lesser of the two evils.

Q16/16 request that ETSI TC STQ keep us informed on further developments related to this work item.

______________________