## Introduction

In ETSI TS 103 281 [1], two models for predicting results of P.835 evaluations of the sending speech quality in noise are described.  Due to a desire by 3GPP SA4 to reference a single model in 3GPP TS 26.132 [2], a model consisting of the combination of the predictions from Model A and Model B of [1] was proposed in [2].  Below are described the combination of the models and an analysis of the performance of the combined model on three validation databases of [1].

### 1.1 Description of combination of model predictions

As defined in [2], the predictions from Model A and Model B of [1] are combined as follows:

S-MOS-LQOfb = (S-MOS-LQOfb_modelA + S-MOS-LQOfb_modelB)/2

N-MOS-LQOfb = (1.438*N-MOS-LQOfb_modelA – 1.959 + N-MOS-LQOfb_modelB)/2

G-MOS-LQOfb = (G-MOS-LQOfb_modelA + G-MOS-LQOfb_modelB)/2

Except for a linear post-mapping of N-MOS from Model A, the combination is simply the average of the corresponding predictions from each Model. The linear post-mapping of N-MOS from Model A was derived from validation databases 3 (DES-25) and 4 (DES-26).  Figure 1 shows a scatter plot of unmapped model N-MOS predictions and subjective BAK ratings for both databases separately.  In addition, linear regression lines are also shown separately for each database.

Figure 1 Scatter plot of N-MOS from Model A versus subjective BAK ratings for validation databases DES-25 and DES-26

Note that the scatter plots for both databases are nearly the same, as are the linear regression lines.  From this, it was proposed to combine the results from both databases and compute a single regression line, as shown in Figure 2.

Figure 2 Scatter plot of N-MOS from Model A versus subjective BAK ratings for combined validation databases DES-25 and DES-26

The regression equation obtained from the combined databases was then used as post-mapping for the N-MOS predictions from Model A.  Using the combined model, comparisons to validation databases 3, 4, and 5 are shown in the following sections.

### 1.2 Validation database 3 (DES-25): Results for combined model

Results are shown as scatter plots, comparing instrumental predicted ratings to subjective ratings. Results from the combined model on validation database 3 for each of the three ratings, SIG, BAK, and OVRL, are shown in Figure 3. For each rating (rows), two scatter plots are shown, one before a monotonic mapping is applied (right column) and one after a monotonic 3rd order mapping is applied (left column).

Figure 3: Scatter plots from combined model for validation database 3 (DES-25)

The rmse* and maximum absolute error* (maxabs*) after mapping are shown on all figures, with and orange-colored symbol indicating the condition with the the largest overall maximum absolute error. The mapping polynomial is shown in the upper left corner of each panel. The dashed green lines show error of ± 0.5 MOS. The error bars indicate the 95% confidence interval before mapping (left column) and after mapping (right column).

Additional performance metrics, including Pearson’s ρ correlation coefficient, Spearman’s ρ rank order correlation, and Kendall’s τ are shown in Table 1.

Table 1: Performance metrics for combined model on validation database 3 (DES-25)

| Dimension | Metric | Raw | Mapped | d* | Mapped & d* |
| --- | --- | --- | --- | --- | --- |
| SIG | Rmse | 0,299 | 0,256 | 0,194 | 0,142 |
|  | Max Abs Error | 0,572 | 0,450 | 0,452 | 0,331 |
|  | Pearson’s ρ | 0,941 | 0,942 | 0,972 | 0,980 |
|  | Spearman’s rank order ρ | 0,902 | 0,902 | 0,942 | 0,978 |
|  | Kendall’s τ | 0,740 | 0,740 | 0,821 | 0,869 |
| BAK | Rmse | 0,210 | 0,207 | 0,111 | 0,095 |
|  | Max Abs Error | 0,504 | 0,377 | 0,333 | 0,238 |
|  | Pearson’s ρ | 0,975 | 0,974 | 0,992 | 0,995 |
|  | Spearman’s rank order ρ | 0,968 | 0,968 | 0,987 | 0,988 |
|  | Kendall’s τ | 0,854 | 0,854 | 0,926 | 0,932 |
| OVRL | Rmse | 0,195 | 0,168 | 0,094 | 0,079 |
|  | Max Abs Error | 0,480 | 0,359 | 0,325 | 0,246 |
|  | Pearson’s ρ | 0,972 | 0,972 | 0,991 | 0,993 |
|  | Spearman’s rank order ρ | 0,965 | 0,965 | 0,989 | 0,993 |
|  | Kendall’s τ | 0,854 | 0,854 | 0,940 | 0,952 |

### 1.3 Validation database 4 (DES-26): Results for combined model

Results are shown as scatter plots, comparing instrumental predicted ratings to subjective ratings. Results from the combined model on validation database 4 for each of the three ratings, SIG, BAK, and OVRL, are shown in Figure 4. As in the previous figure, for each rating (rows), two scatter plots are shown, one before a monotonic mapping is applied (right column) and one after a monotonic mapping is applied (left column).

Figure 4: Scatter plots from combined model for validation database 4 (DES-26)

The rmse* and maximum absolute error* (maxabs*) after mapping are shown on all figures, with and orange-colored symbol indicating the condition with the the largest overall maximum absolute error. The mapping polynomial is shown in the upper left corner of each panel. The dashed green lines show error of ± 0.5 MOS. The error bars indicate the 95% confidence interval before mapping (left column) and after mapping (right column).

Additional performance metrics, including Pearson’s ρ correlation coefficient, Spearman’s ρ rank order correlation, and Kendall’s τ are shown in Table 2.

Table 2: Performance metrics for combined model on validation database 4

| Dimension | Metric | Raw | Mapped | d* | Mapped & d* |
| --- | --- | --- | --- | --- | --- |
| SIG | Rmse | 0,345 | 0,276 | 0,224 | 0,144 |
|  | Max Abs Error | 0,858 | 0,488 | 0,669 | 0,345 |
|  | Pearson’s ρ | 0,938 | 0,945 | 0,957 | 0,976 |
|  | Spearman’s rank order ρ | 0,945 | 0,945 | 0,955 | 0,962 |
|  | Kendall’s τ | 0,758 | 0,758 | 0,783 | 0,881 |
| BAK | Rmse | 0,139 | 0,147 | 0,069 | 0,064 |
|  | Max Abs Error | 0,434 | 0,347 | 0,305 | 0,218 |
|  | Pearson’s ρ | 0,985 | 0,985 | 0,996 | 0,997 |
|  | Spearman’s rank order ρ | 0,988 | 0,988 | 0,998 | 0,997 |
|  | Kendall’s τ | 0,917 | 0,917 | 0,982 | 0,973 |
| OVRL | Rmse | 0,183 | 0,188 | 0,102 | 0,080 |
|  | Max Abs Error | 0,538 | 0,363 | 0,405 | 0,232 |
|  | Pearson’s ρ | 0,967 | 0,963 | 0,991 | 0,993 |
|  | Spearman’s rank order ρ | 0,968 | 0,968 | 0,992 | 0,989 |
|  | Kendall’s τ | 0,863 | 0,863 | 0,954 | 0,932 |

### 1.4 Validation database 5 (DES-27): Results for combined model

Results are shown as scatter plots, comparing instrumental predicted ratings to subjective ratings. Results from the combined model on validation database 5 for each of the three ratings, SIG, BAK, and OVRL, are shown in Figure 5. As in the previous figures, for each rating (rows), two scatter plots are shown, one before a monotonic mapping is applied (right column) and one after a monotonic mapping is applied (left column).

Figure 5: Scatter plots from combined model for validation database 5 (DES-26)

The rmse* and maximum absolute error* (maxabs*) after mapping are shown on all figures, with and orange-colored symbol indicating the condition with the the largest overall maximum absolute error. The mapping polynomial is shown in the upper left corner of each panel. The dashed green lines show error of ± 0.5 MOS. The error bars indicate the 95% confidence interval before mapping (left column) and after mapping (right column).

Additional performance metrics, including Pearson’s ρ correlation coefficient, Spearman’s ρ rank order correlation, and Kendall’s τ are shown in Table 3.

Table 3: Performance metrics for combined model on validation database 5

| Dimension | Metric | Raw | Mapped | d* | Mapped & d* |
| --- | --- | --- | --- | --- | --- |
| SIG | Rmse | 0,870 | 0,351 | 0,737 | 0,214 |
|  | Max Abs Error | 1,383 | 0,648 | 1,260 | 0,453 |
|  | Pearson’s ρ | 0,849 | 0,888 | 0,867 | 0,960 |
|  | Spearman’s rank order ρ | 0,849 | 0,849 | 0,855 | 0,955 |
|  | Kendall’s τ | 0,672 | 0,672 | 0,681 | 0,835 |
| BAK | Rmse | 0,358 | 0,242 | 0,248 | 0,130 |
|  | Max Abs Error | 0,713 | 0,388 | 0,616 | 0,244 |
|  | Pearson’s ρ | 0,971 | 0,971 | 0,984 | 0,992 |
|  | Spearman’s rank order ρ | 0,962 | 0,962 | 0,974 | 0,989 |
|  | Kendall’s τ | 0,857 | 0,857 | 0,890 | 0,940 |
| OVRL | Rmse | 0,343 | 0,217 | 0,233 | 0,116 |
|  | Max Abs Error | 0,902 | 0,421 | 0,760 | 0,305 |
|  | Pearson’s ρ | 0,965 | 0,970 | 0,972 | 0,991 |
|  | Spearman’s rank order ρ | 0,975 | 0,975 | 0,973 | 0,994 |
|  | Kendall’s τ | 0,884 | 0,884 | 0,878 | 0,950 |

### 1.5 References

[1] ETSI TS 103 281 (2017-04) V1.1.1. “Speech quality in the presence of background noise: Objective test methods for super-wideband and fullband terminals”

[2] 3GPP SA4-170439 CR to TS 26.132, Addition of SWB and FB noise suppression test methods.  24-28 April 2017, Busan, KR.