3GPP TSG-CT Meeting #91e	CP-210285

# E-Meeting, 18th – 24th March 2021

(revision of CP-aaaaaaa)

# Source:	3GPP TSG CT WG6

# Title:	New WID on CT aspects on UICC-terminal interface testing for UEs with non-removable UICCs

# Document for:	Endorsement

Agenda Item:	17.2 - New Work Items

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	CT aspects on UICC-terminal interface testing for UEs with non-removable UICCs

## Acronym:	nrUICC_UEConTest

## Unique identifier: 	910008

Potential target Release: {Rel-16}.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes | x | x | x | x |  |
| No |  |  |  |  |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
| x | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
|  |  |  | CT aspects on UICC-terminal interface testing for UEs with non-removable UICCs |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

## 3	Justification

With the usage of an ETSI TS 102 671 form factor (MFF1/MFF2) or ETSI TS 103 666-2 (iSSP) access to a physical connection in between UICC and terminal may not be available. This would exclude those UEs from tests that require content and/or availability checks on the APDU based 3GPP application (USIM) as part of the test procedure related to a conformance requirement. Therefore, some tests in accordance to these test specifications will not be applicable to those MEs.

## 4	Objective

Different from explicit verification as used in 3GPP TS 31.121 and 31.124, the introduction of new and additional test methods shall enable an implicit verification of content and requirements specified in TS 31.102 (Characteristics of the USIM application) and TS 31.111 (USIM Application Toolkit) for devices where the physical UICC-terminal interface is not accessible. As there is a variety of UICC/USIM implementations is available supporting different methods to store, access and provide file content and CAT functionality different methods to verify the conformance requirements of each particular test cases may be offered. In addition to verification methods defined in the initial creation phase of these new specifications alternative verification methods can be endorsed by CT6 group decision. E.g. the verification by events when specified and available in ETSI Java API specifications.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| TS | 31.127 | UICC-terminal interaction; non-removable Universal Subscriber Identity Module (nrUSIM) application behavioural test specification | CT#93 | CT#94 | Marquordt, Arne, COMPRION GmbH amarquordt@comprion.com |
| TS | 31.117 | Universal Subscriber Identity Module Application Toolkit (USAT) application behavioural test specification | CT#93 | CT#94 | Marquordt, Arne, COMPRION GmbH amarquordt@comprion.com |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 31.122 | Possible updates on USIM testing and verification methods | March 2022 | depends on ETSI implementations |
| 31.213 | Possible modifications on the USIM Java API due to the handling of newly introduced events | March 2022 | depends on ETSI implementations |

## 6	Work item Rapporteur(s)

Marquordt, Arne, COMPRION GmbH

amarquordt@comprion.com

## 7	Work item leadership

3GPP CT WG6.

## 8	Aspects that involve other WGs

None identified yet.

## 9	Supporting Individual Members

Supporting IM name

COMPRION

Qualcomm

Apple

Keysight

AT&T