GSM_Onboard_0905_Sophia meeting

Sophia Antipolis, 9 September 2005

GSMOB-04

Source:	TeliaSonera

Title:	GSM on air: leaky feeders and interference to GSM900

Agenda item:	4

Document for:	Discussion

# Introduction

The proposed onboard GSM system involves a NCU (Network Control Unit) devised to prevent onboard MS to terrestrial networks (in many bands). This contribution considers the interference from the onboard NCU into terrestrial GSM900 networks. One problem then is to determine the effective EIRP of the feeder as seen from the ground. A number of contributions to CEPT SE7 have discussed this, and the present contribution provides another attempt.

First we discuss a leaky feeder model that has been used by Qualcomm [1] in a GERAN contribution and validate some of the assumptions made therein. Alternative methods for describing the feeder are also briefly discussed. Using this leaky feeder theory we then follow up an earlier interference analysis by TeliaSonera, presented both to 3GPP RAN WG4 and GERAN WG1.

It is concluded that there is a considerable risk that the NCU will interfere with the terrestrial GSM900 network. Further analysis is needed; it seems to be difficult to arrive at a conclusion within the timeframe set by ECC/CEPT.

# Considerations on leaky feeders

Modelling leaky feeders is a delicate problem, and several models exist. A recent paper by Qualcomm, presented at GERAN#26 [1], uses a model based on diffuse radiation from the cable cited from a paper by Morgan [2]. The idea here is to try to derive the results of [2] and then use the same analysis to derive the EIRP and compare with the assumptions made in [1].

Hence our first problem is to determine the power from the feeder when a receiver is located aboard the aircraft at a distance D<<L, where L is the length of the feeder within the fuselage, and then to determine the EIRP from the feeder seen from the ground at a distance D>>L.

Turning to the first problem, the Morgan result assumes that the cable is radiates diffusely according to Lambert’s law. Furthermore, it is supposed that the cable is infinitely long (i.e. D << L) and lossless. The power radiated from a diffusely radiating element of length dl along the cable is

(1)	dp=sinθ(πr)2Π dl,

where  is the angle between the viewing direction and the cable axis,  is the power radiated per unit length of the cable and r is the distance. (Integrating over a sphere, we get the total power dl.) The receiving half-wave dipole is parallel to the cable and has a directivity function in the E-plane

f(θ)=1.64cos((π2)cosθ)sinθ2.

The received power at a distance D from the axis of the (infinite) feeder is then

P(D)=−∞∞f(θ)Aedp

where Ae=λ2(4π) is the effective antenna area of an isotropic antenna (the square brackets is the power received by the dipole from the direction ). Then make the variable substitution l=−Dcotθ, hence dl=−D/sin2θ, and set r=D/sinθ (see figure in [1]) to obtain

P(D)=ΠπDλ4π240πf(θ)sinθ dθ

The integral is of the same type as that used when determining the radiation resistance, and luckily a close form result exists:

40πf(θ)sinθ dθ=2⋅1.64γ+ln2π−Ci(2π)≈8.00

Where  = 0.5772… is Euler’s constant and Ci a Cosine integral (tabulated). Hence we obtain Morgan’s result

(2)	P(D)=8ΠπDλ4π2.

We can now determine the EIRP from the leaky feeder as seen from the ground by following the same analysis that resulted in (2). We wish to compute the radiation intensity when D >> L. The key to this is (1).  Then just integrate along the length of the feeder to obtain the received power by an isotropic antenna

(3)	Pterr(D)=Ae0Ldp≈λ24πsinθ(πD)2 ΠL=4ΠLsinθπλ4πD2,

This result is not surprising: it is well known that any finite sized radiator looks like a point source at sufficient distance. The aircraft (feeder) is assumed to be parallel to the ground and  is the viewing angle. The maximum occurs in a direction normal to the aircraft, where the result 4π⋅ΠL is very close to the Qualcomm assumption.

Hence Qualcomm’s analysis is almost consistent with the theory in [1], the error is negligible.

Turning back briefly to the case in which D << L, there are alternative ways of deriving an expression for the received power from the feeder. Using the same notation, we note that the power intensity at a distance D from the feeder, assumed to be a cylindrical radiator, is

S(D)=Π2πD  .

The power received by an isotropic antenna of gain Ga is then

P(D)=S(D)Gaλ24π=(2πGa)ΠπDλ4π2.

For a half-wave dipole2πGa=10.3 with the maximum gain 1.64, and we end up with a result close to (2). In practice, both of the models are approximations and either of them could be used. Cable attenuation is more easily included if one assumes an ideal isotropic antenna at the receiver end just like in the last approach. The integral for the power above can then be solved explicitly.

# Interference analysis using the feeder theory

An earlier TeliaSonera contribution to RAN4 [3] addressed the problem of concealing the GSM900 network. On the one hand, the NCU output power must be sufficiently high to prevent the onboard MS to attach to the ground network. However, on the other hand, the NCU output must not be too high in order not to interfere with the ground network (MS).

In [3], the interference calculations only deal with average power values, i.e. the impact of fading is not considered and the fuselage attenuation is just assumed to be a constant 2 dB. Summarising the results of [3], it is concluded that the effective EIRP of the NCU as seen from the ground must satisfy, using (3),

(4)	EIRP=ΠL<−18+2 dBm

not to interfere with the ground network at an altitude of 3000 m. We look in a direction perpendicular to the fuselage and ignore the factor 4/; the last term is the fuselage attenuation. On the other hand, the NCU must supply a signal level, using (2),

(5)	RxLev=P(D)>−58−2 −9dBm

in order to prevent an onboard MS at a distance D from the feeder from attaching to the terrestrial GSM900 network.

Following [1], the factor  can be computed by using the specified coupling loss C at a reference distance Dref from the feeder.  We then have

C=PinP(Dref) and P(D)P(Dref)=DrefD.

It then follows from (2) that

(6)	Π=PinCπDref84πλ2

Using the Qualcomm figures C = 69 dB, Dref = 2 m and L = 30 m and substituting (6) into the first inequality (4) we obtain

Pin<7 dBm

The second inequality leads to

RxLev=PinC>−69 dBm.

Hence, if we only consider the average signal values, it follows that the input power of the feeder per GSM 200 kHz channel must be in the range

0<Pin<7 dBm

in order to satisfy both inequalities above. Note that we have not considered any fading effects nor multiple interference, which will in practice make things worse. Just adding a fading margin of a few dB, it will in practice be impossible to find a suitable input power to the feeder. Moreover, cable (feeder) attenuation is not considered. This will necessitate a higher feeder input power to achieve the necessary power for the onboard MS. Hence there is a considerable risk that the onboard GSM900 terrestrial networks will be interfered by the NCU.

# Conclusions

It is concluded that there is a considerable risk that the onboard NCU will interfere with the terrestrial GSM900 networks.

The assumptions used by Qualcomm in another interference analysis [1] are consistent with a published model of leaky feeder radiation.

# References

1. GP-0502233, Qualcomm Europe, 3GPP TSG GERAN#26.

2. S.P. Morgan, “Prediction if indoor wireless coverage by leaky coaxial cable using ray tracing”, IEEE Trans        Veh. Tech., Vol. 48(6), pp. 2005-2014, Nov 1999.

3. R4-050160, TeliaSonera, 3GPP TSG RAN WG4#34.