3GPP TSG CT WG3 Meeting #143	C3-254049

# Sophia-Antipolis, FR, 13th – 17th October, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.507 | CR | 0354 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates and corrections to Energy Consumption information management |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |
| Source to TSG: | CT3 |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  |  | The following issues were identified in the provisions related to the Energy Consumption information management: - The "Energy" fetaure description wrongly hints that only the reporting of the change of the value of the subscribed Energy Saving Indicator is supported, when the provisioning of the subscribed Energy Saving Indicator during the AM policy association establishment is also supported. - A few terminology alignments are needed. |  |  |  |  |  |  |
| Summary of change: |  |  | This CR proposes to: - Address the above-detailed issues. |  |  |  |  |  |  |
| Consequences if not approved: |  |  | - The above-detailed issues are not addressed and the definition of the Energy management functionality related requirements/provisions is not corrected/completed. |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3.1, 5.6.2.3, 5.6.2.4, 5.8 |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |
| affected: |  |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions of the APIs. |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

* * * * Start of changes * * * *

#### 4.2.3.1	General

The procedure in the present clause is applicable when the NF service consumer modifies an existing AM policy association (including the case where the AMF is relocated and the new AMF selects the old PCF to maintain the policy association and to update the Notification URI).

Figure 4.2.3.1-1 illustrates the update of a policy association.

Figure 4.2.3.1-1: Update of a policy association

The AMF as NF service consumer invokes this procedure when a policy control request trigger (see clause 4.2.3.2) occurs. When a policy control request trigger that does not require the subscription as defined in table 5.6.3.3-1 (e.g. Service Area Restriction change trigger) occurs, the NF service consumer (e.g. AMF) shall always invoke the procedure. When a policy control request trigger requires the subscription as defined in table 5.6.3.3-1 (e.g. location change trigger) occurs, the NF service consumer shall only invoke the procedure if the PCF has subscribed to that event trigger.

If an AMF knows by implementation specific means that the UE context has been transferred to an AMF with another GUAMI within the AMF set, it may also invoke this procedure to update the Notification URI and the GUAMI.

NOTE 1:	Either the old or the new AMF can invoke this procedure.

During the AMF relocation, if the new AMF received the resource URI of the individual AM Policy from the old AMF and selects the old PCF, the new AMF shall also invoke this procedure to update the Notification URI and the GUAMI. The new AMF may also update the alternate or backup IP addresses. If the feature "FeatureRenegotiation" is supported, the new AMF may perform feature renegotiation, as described in clause 4.2.3.4.

To request policies from the PCF, to update the Notification URI, to renegotiate features, to update the trace control configuration and/or to request the termination of trace, the NF service consumer (e.g. AMF) shall request the update of the AM Policy Association by providing the relevant parameters about the UE context by sending an HTTP POST request with "{apiRoot}/npcf-am-policy-control/v1/policies/{polAssoId}/update" as Resource URI and the PolicyAssociationUpdateRequest data structure as request body that shall include:

-	at least one of the following:

1.	a new Notification URI encoded in the "notificationUri" attribute;

2.	observed Policy Control Request Trigger(s) (see clause 4.2.3.2) encoded as "triggers" attribute;

3.	if a Service Area restriction change occurred, the Service Area Restrictions (see clause 4.2.2.3.1) as obtained from the UDM encoded as "servAreaRes" attribute;

4.	if a RFSP index change occurred, the RFSP index (see clause 4.2.2.3.2) as obtained from the UDM encoded as "rfsp" attribute;

5.	if a UE location change occurred and the Policy Control Request Trigger "Location change" was provided, the UE location encoded as "userLoc" attribute;

6.	if the Policy Control Request Trigger "Change of UE presence in PRA" was provided, the current presence status of the UE for the presence reporting areas for which reporting was requested, if not previously provided, or the presence reporting areas for which reporting was requested and the status has changed encoded as "praStatuses" attribute;

NOTE 2:	If the PCF included the identifer of a Core Network predefined Presence Reporting Area Set within the "praId" attribute during the subscription to changes of UE presence in PRA, the AMF only provides the presence reporting area information corresponding to the concerned individual Presence Reporting Area Identifier(s) within the Set. The "praId" attribute within each returned "PresenceInfo" data type hence includes the identifier of the concerned individual Presence Reporting Area.

7.	if the trace control configuration needs to be updated, trace control and configuration parameters information encoded as "traceReq" attribute;

8.	if trace needs to be terminated, the "traceReq" attribute set to the Null value;

9.	if the "SliceSupport" feature, the "DNNReplacementControl" feature and/or the "NetSliceRepl" feature is/are supported, the UE is registered via 3GPP access, the Allowed NSSAI changed, and the Policy Control Request Trigger "Change of Allowed NSSAI" was provided, then the Allowed NSSAI within the "allowedSnssais" attribute;

10.	for AMF relocation scenarios, if available, alternate or backup IPv4 Address(es) where to send Notifications encoded as "altNotifIpv4Addrs" attribute;

11.	for AMF relocation scenarios, if available, alternate or backup IPv6 Address(es) where to send Notifications encoded as "altNotifIpv6Addrs" attribute;

12.	for AMF relocation scenarios, if available,	 alternate or backup FQDN(s) where to send Notifications encoded as "altNotifFqdns" attribute;

13.	for AMF relocation scenarios,  the GUAMI encoded as "guami" attribute;

NOTE 3:	An alternate NF service consumer than the one that requested the generation of the subscription resource can send the request. For instance, an AMF as service consumer can change.

14.	if the feature "UE-AMBR_Authorization" is supported, and a subscribed UE-AMBR change occurred, the UE-AMBR (see clause 4.2.2.3.3) as obtained from the UDM encoded as "ueAmbr" attribute;

15.	if the feature "DNNReplacementControl" is supported, DNN replacement applies and the Policy Control Request Trigger "Change of SMF selection information" was provided, the "smfSelInfo" attribute including:

-	the UE requested DNN in the "dnn" attribute; and

-	the UE requested S-NSSAI in the "snssai" attribute and, if available, the corresponding mapped home S-NSSAI in the "mappingSnssai" attribute;

when:

-	the UE requested an unsupported DNN and the "unsuppDnn" attribute is set to "true"; or

-	the UE requested DNN and S-NSSAI matched one of the S-NSSAI and DNN provided in the "candidates" attribute;

16.	if feature "DNNReplacementControl" is supported, the UE is registered via 3GPP access, the Allowed NSSAI changed and/or the mapping of a S-NSSAI of the Allowed NSSAI to the corresponding S-NSSAI of the HPLMN changed, and the Policy Control Request Trigger "Change of allowed NSSAI" was provided, then the mapping of each S-NSSAI of the Allowed NSSAI to the corresponding S-NSSAI of the HPLMN encoded in the "mappingSnssais" attribute;

NOTE 4:	When the feature "DNNReplacementControl" is supported, the AMF applies DNN replacement for non-roaming scenarios and LBO. For a PDU session with home routed roaming, whether to perform DNN replacement is based on operator agreement.

17.	if feature "UE-Slice-MBR_Authorization" is supported, and a subscribed UE-Slice-MBR change occurred, the subscribed UE-Slice-MBR for each subscribed S-NSSAI of the home PLMN mapping to a S-NSSAI of the serving PLMN (see clause 4.2.2.3.5) encoded in the "ueSliceMbrs" attribute;

18.	if the feature "EneNA" is supported and an NWDAF information change occurred, the list of NWDAF instance IDs used for the UE and their associated Analytic ID(s) with the updated values within the "nwdafDatas" attribute;

NOTE 5:	The NF service consumer provides the complete updated list of NWDAF instance IDs and associated Analytic ID(s) used for the UE. If all NWDAF data is deleted an empty list is included.

19. if the feature "TargetNSSAI" is supported, a new Target NSSAI is generated and the Policy Control Request Trigger "Generation of Target NSSAI" is provided, the new generated Target NSSAI encoded in the "targetSnssais" attribute;

20.	if the "NetSliceRepl" feature is supported;

-	if the AMF is aware that one or more S-NSSAI(s) become unavailable but cannot determine the corresponding Alternative S-NSSAI(s) and the Policy Control Request Trigger "SLICE_REPLACE_MGMT" was provided, these unavailable S-NSSAI(s) within the "unavailSnssais" attribute;

-	if the AMF decides to proceed with network slice replacement and is aware of the Alternative S-NSSAI(s) corresponding to those initial S-NSSAI(s) or previously replaced S-NSSAI(s) is available and the Policy Control Request Trigger "SLICE_REPLACE_MGMT" was provided, the AMF provides the updated "snssaiReplInfos" with the updated mapping of (replaced) S-NSSAI(s) with the Alternative S-NSSAI(s) in case of network slice replacement or by removing the mapping of the replaced S-NSSAI(s) with the Alternative S-NSSAI(s) if the replaced S-NSSAI(s) is available;

21.	if "PartNetSliceSupport" feature and/or "NetSliceRepl" feature is/are supported, the UE is registered via 3GPP access, the Partially Allowed NSSAI changed and the Policy Control Request Trigger "Change of the Partially Allowed NSSAI" was subscribed by the PCF, then the updated Partially Allowed NSSAI within the "partAllowedNssai" attribute;

22.	if the "PartNetSliceSupport" feature is supported, the UE is registered via 3GPP access, the Partially Allowed NSSAI changed and/or the mapping of one or more of the S-NSSAI(s) of the Partially Allowed NSSAI to the corresponding HPLMN S-NSSAI(s) changed, and the Policy Control Request Trigger "Change of the Partially Allowed NSSAI" was subscribed by the PCF, then the mapping of each S-NSSAI of the Partially Allowed NSSAI to the corresponding HPLMN S-NSSAI within the "mappingSnssais" attribute;

23.	if the "PartNetSliceSupport" feature is supported, the UE is registered via 3GPP access and:

-	if the list of the S-NSSAI(s) rejected partially in the RA changed and the Policy Control Request Trigger "Change of the S-NSSAI(s) rejected partially in the RA" was subscribed by the PCF, then the updated list of the S-NSSAI(s) rejected partially in the RA within the "snssaisPartRejected" attribute;

-	if the list of the Rejected S-NSSAI(s) in the RA changed and the Policy Control Request Trigger "Change of the Rejected S-NSSAI(s)" was subscribed by the PCF, then the updated list of the Rejected S-NSSAI(s) in the RA within the "rejectedSnssais" attribute;

-	if the Pending NSSAI changed and the Policy Control Request Trigger "Change of the Pending NSSAI" was subscribed by the PCF, then the updated Pending NSSAI within the "pendingNssai" attribute;

24.	if the "RatTypeChange" feature is supported, and the Policy Control Request Trigger "RAT Type Change" was provided, the RAT Type encoded in the "ratTypes" attribute;

25.	if the "AfNetSliceRepl" feature is supported and the "SLICE_REPLACE_OUTCOME" Policy Control Request Trigger was provided by the PCF, then the outcome of the AF requested Network Slice Replacement initiation or the AF requested Network Slice Replacement termination within the "afSliceReplOut" attribute; and

26.	if the "Energy" feature is supported and the "ENERGY_SAV_IND_CH" Policy Control Request Trigger was provided by the PCF, then the updated Energy Saving Indicator within the "enrgSavInd" attribute.

Upon the reception of the HTTP POST request, the PCF shall:

-	update the corresponding individual AM Policy resource based on the information provided by the NF service consumer;

-	determine the applicable policy based on local policy;

-	for the successful case, send a HTTP "200 OK" response with the PolicyUpdate data type as body with possible updates for that applicable policy and Policy Control Request Trigger(s) encoded as described in clause 4.2.3.3 and according to the following provisions:

a)	if the PCF received the "servAreaRes" attribute in the request, Service Area Restrictions encoded as "servAreaRes" attribute;

b)	if the PCF received the "rfsp" attribute in the request, RAT Frequency Selection Priority (RFSP) Index encoded as "rfsp" attribute. If the feature "RFSPValidityTime" is supported and the PCF determines to provide an RFSP index value that indicates EPC/E-UTRAN access is prioritized over 5GS access, the PCF may provide, based on operator policies, a validity time for the RFSP index value within the "rfspValTime" attribute;

c)	if the feature "UE-AMBR_Authorization" is supported and the PCF received the "ueAmbr" attribute in the request, UE-AMBR encoded as "ueAmbr" attribute;

d)	if the PCF received the "smfSelInfo" attribute in the request, the "smfSelInfo" attribute encoding the PCF selected DNN in the "dnn" attribute corresponding to the S-NSSAI received in the "snssai" attribute;

NOTE 6:	A PolicyUpdate data structure with only mandatory attribute(s) is included in the "200 OK" response when the PCF decides not to update the policies.

e)	if the feature "UE-Slice-MBR_Authorization" is supported and the PCF received the "ueSliceMbrs" attribute in the request, the corresponding authorized UE-Slice-MBR(s) encoded as "ueSliceMbrs" attribute;

f)	if the feature "TargetNSSAI" is supported and the PCF received the "targetSnssais" attribute in the request, the RFSP Index associated with the Target NSSAI encoded as "targetRfsp" attribute;

g)	if the "NetSliceUsageCtrl" feature is supported, the updated network slice usage control information (e.g., updated slice deregistration inactivity timer) within the "sliceUsgCtrlInfoSets" attribute for each on-demand S-NSSAI of the UE's Allowed NSSAI;

NOTE 7:	In this release of the specification, network slice usage control information provisioning/update/removal by the PCF is not supported in roaming scenarios.

h)	if the "NetSliceRepl" feature is supported and the PCF received the "unavailSnssais" attribute in the request, the Alternative S-NSSAI(s) associated with the received S-NSSAI(s) within the "snssaiReplInfos" attribute containing these unavailable S-NSSAI(s), and for each unavailable S-NSSAI, the corresponding status information set to "UNAVAILABLE" and the corresponding Alternative S-NSSAI; and/or

i) 	if the "Energy" feature is supported and the PCF received the "enrgSavInd" attribute in the request, the related applicable policy(ies) based on the local policy;

and

-	if errors occur when processing the HTTP POST request, apply error handling procedures as specified in clause 5.7 and according to the following provisions:

a)	if the PCF is, due to incomplete, erroneous or missing information in the request, not able to provision an AM policy decision, the PCF may reject the request and include in an HTTP "400 Bad Request" response message the "cause" attribute of the ProblemDetails data structure set to "ERROR_REQUEST_PARAMETERS"; and

b)	if the "ES3XX" feature is supported and the PCF (service) instance has changed, the PCF may respond with an HTTP 3xx redirect response pointing to a new PCF (service) instance as defined in clause 6.5.3.3 of 3GPP TS 29.500 [5].

If the PCF received a "traceReq" attribute, it shall perform trace procedures as defined in 3GPP TS 32.422 [18].

If the AMF received the request of removal of Service Area Restrictions and/or RFSP Index and/or UE-AMBR and/or UE-Slice-MBR(s) from the UDM, the AMF shall remove the authorized Service Area Restrictions and/or RFSP Index and/or UE-AMBR and/or UE-Slice-MBR(s) provisioned by the PCF and apply the configured Service Area Restrictions and/or RFSP Index and/or UE-AMBR and/or UE-Slice-MBR(s) at the AMF without interacting with the PCF.

If feature "DNNReplacementControl" is supported and the AMF received the update of the SMF selection information within the "smfSelInfo" attribute in the response, the AMF shall apply the updated SMF selection information to the new PDU Sessions only, i.e. already established PDU Sessions are not affected.

If the feature "AMInfluence" is supported, the PCF determines that the access and mobility policies may be influenced by the traffic of a PDU session(s), e.g. based on the received policy control request trigger(s), and local operator policies indicate the PCF for the UE shall subscribe with the PCF for the PDU session for established/terminated PDU session(s) event notifications, the PCF shall provision/update the AMF with the PCF for the UE information within the "pcfUeInfo" attribute and the complete list of S-NSSAI and DNN combinations within the "matchPdus" attribute. The AMF shall then update the affected established PDU sesssion(s), by forwarding the received PCF for the UE information for the PDU session(s) matching the new S-NSSAI and DNN combination(s) and removing the previously provided PCF for the UE information for the PDU session(s) matching the removed S-NSSAI and DNN combination(s) as defined in 3GPP TS 29.502 [31].

When the "AMInfluence" feature is supported, and the SBA binding indication information for the PCF instance changes, the PCF may update the previously provided information in the AMF. The AMF shall apply the updated PCF callback information to the new PDU Sessions only, i.e., already established PDU sessions are not affected.

If the PCF received a new GUAMI, the PCF may subscribe to GUAMI changes using the AMFStatusChange service operation of the Namf_Communication service specified in 3GPP TS 29.518 [14], and it may use the Nnrf_NFDiscovery Service specified in 3GPP TS 29.510 [13] (using the obtained GUAMI and possibly service name) to query the other AMFs within the AMF set.

If the PCF received a "servAreaRes" attribute which resulted to a change of the Service Area Restrictions, it shall send notifications to any NF Service Consumer(s) (e.g. AF) that have subscribed to the related event by using the Npcf_AMPolicyAuthorization service (see 3GPP TS 29.534 [26]) and/or the Npcf_EventExposure service ((see 3GPP TS 29.523 [28]).

If the PCF received a new list of NWDAF instance IDs used for the UE and their associated Analytic IDs within the "nwdafDatas" attribute, the PCF may select those NWDAF instances based on this new list as described in 3GPP TS 29.513 [7].

If the PCF received the notification of the outcome of the AF requested Network Slice Replacement initiation or the AF requested Network Slice Replacement termination within the "afSliceReplOut" attribute, the PCF shall send notifications to any NF Service Consumer(s) (e.g., AF) that have subscribed to the related event by using the Npcf_AMPolicyAuthorization service (see 3GPP TS 29.534 [26]) and/or the Npcf_EventExposure service (see 3GPP TS 29.523 [28]).

* * * * Next changes * * * *

#### 5.6.2.3	Type PolicyAssociationRequest

Table 5.6.2.3-1: Definition of type PolicyAssociationRequest

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| notificationUri |  | Uri |  | M |  | 1 |  | Identifies the recipient of Notifications sent by the PCF. |  |  |  |
| altNotifIpv4Addrs |  | array(Ipv4Addr) |  | O |  | 1..N |  | Alternate or backup IPv4 Address(es) where to send Notifications. |  |  |  |
| altNotifIpv6Addrs |  | array(Ipv6Addr) |  | O |  | 1..N |  | Alternate or backup IPv6 Address(es) where to send Notifications. |  |  |  |
| altNotifFqdns |  | array(Fqdn) |  | O |  | 1..N |  | Alternate or backup FQDN(s) where to send Notifications. |  |  |  |
| supi |  | Supi |  | M |  | 1 |  | Subscription Permanent Identifier. |  |  |  |
| gpsi |  | Gpsi |  | C |  | 0..1 |  | Generic Public Subscription Identifier. Shall be provided when available. |  |  |  |
| accessType |  | AccessType |  | C |  | 0..1 |  | The Access Type where the served UE is camping. Shall be provided when available. |  |  |  |
| accessTypes |  | array(AccessType) |  | C |  | 1..N |  | The Access Types where the served UE is camping. Shall be provided when available. |  | MultipleAccessTypes |  |
| pei |  | Pei |  | C |  | 0..1 |  | The Permanent Equipment Identifier of the served UE. Shall be provided when available. |  |  |  |
| userLoc |  | UserLocation |  | C |  | 0..1 |  | The location of the served UE. Shall be provided when available. |  |  |  |
| timeZone |  | TimeZone |  | C |  | 0..1 |  | The time zone of the network where the served UE is camping. Shall be provided when available. |  |  |  |
| servingPlmn |  | PlmnIdNid |  | C |  | 0..1 |  | The serving network (a PLMN or an SNPN) where the served UE is camping. For the SNPN the NID together with the PLMN ID identifies the SNPN. For Indirect Network Sharing and the interaction only in the hosting operator network case, the identifier of the serving network is set to the PLMN ID of the hosting operator. Shall be provided when available. |  |  |  |
| ratType |  | RatType |  | C |  | 0..1 |  | The 3GPP or non-3GPP RAT Type where the served UE is camping. Shall be provided when available. |  |  |  |
| ratTypes |  | array(RatType) |  | C |  | 1..N |  | The 3GPP and/or non-3GPP RAT Types where the served UE is camping. Shall be provided when available. |  | MultipleAccessTypes |  |
| groupIds |  | array(GroupId) |  | C |  | 1..N |  | List of Internal Group Identifiers of the served UE. Shall be provided when available. |  |  |  |
| servAreaRes |  | ServiceAreaRestriction |  | C |  | 0..1 |  | Service Area Restriction as part of the AMF Access and Mobility Policy. Shall be provided when available. |  |  |  |
| wlServAreaRes |  | WirelineServiceAreaRestriction |  | O |  | 0..1 |  | Wireline Service Area Restriction as part of the AMF Access and Mobility Policy. |  | WirelineWirelessConvergence |  |
| rfsp |  | RfspIndex |  | C |  | 0..1 |  | RFSP Index as part of the AMF Access and Mobility Policy. Shall be provided when available. |  |  |  |
| ueAmbr |  | Ambr |  | C |  | 0..1 |  | UE-AMBR as part of the AMF Access and Mobility Policy. Shall be provided when available. |  | UE-AMBR_Authorization |  |
| ueSliceMbrs |  | array(UeSliceMbr) |  | C |  | 1..N |  | The subscribed UE-Slice-MBR for each subscribed S-NSSAI of the home PLMN mapping to a S-NSSAI of the serving PLMN. Shall be provided when available. (NOTE) |  | UE-Slice-MBR_Authorization |  |
| allowedSnssais |  | array(Snssai) |  | C |  | 1..N |  | Represents the Allowed NSSAI in the 3GPP access and includes the S-NSSAIs values the UE can use in the serving PLMN. It shall be included if the feature "SliceSupport", "NetSliceRepl" and/or "DNNReplacementControl" is supported in the AMF. |  | SliceSupport, DNNReplacementControl, NetSliceRepl |  |
| partAllowedNssai |  | map(PartiallyAllowedSnssai) |  | O |  | 1..N |  | Represents the Partially Allowed NSSAI. The key of the map shall be set to the value of the "snssai" attribute of the corresponding map entry (encoded using the PartiallyAllowedSnssai data structure). |  | PartNetSliceSupport, NetSliceRepl |  |
| snssaisPartRejected |  | map(SnssaiPartRejected) |  | O |  | 1..N |  | Represents the set of S-NSSAI(s) rejected partially in the RA. The key of the map shall be set to the value of the "snssai" attribute of the corresponding map entry (encoded using the SnssaiPartRejected data structure). |  | PartNetSliceSupport |  |
| rejectedSnssais |  | array(Snssai) |  | O |  | 1..N |  | Represents the set of Rejected S-NSSAI(s) in the RA. |  | PartNetSliceSupport |  |
| pendingNssai |  | array(Snssai) |  | O |  | 1..N |  | Represents the Pending NSSAI. |  | PartNetSliceSupport |  |
| targetSnssais |  | array(Snssai) |  | C |  | 1..N |  | Represents the Target NSSAI. It shall be included if available and the feature "TargetNSSAI" is supported. |  | TargetNSSAI |  |
| mappingSnssais |  | array(MappingOfSnssai) |  | C |  | 1..N |  | If the "DNNReplacementControl" feature is supported, this attribute shall contain the mapping of each S-NSSAI of the Allowed NSSAI, and if the "PartNetSliceSupport" feature is also supported, the mapping of each S-NSSAI of the Partially Allowed NSSAI to the corresponding S-NSSAI of the HPLMN. This attribute shall be included if available. If the "MultipleAccessTypes" feature is supported, this attribute contains also the mapping of the Allowed NSSAI in the non-3GPP access to the corresponding S-NSSAI of the HPLMN. |  | DNNReplacementControl, PartNetSliceSupport |  |
| n3gAllowedSnssais |  | array(Snssai) |  | C |  | 1..N |  | Represents the Allowed NSSAI in the non-3GPP access and includes the S-NSSAIs values the UE can use in the serving PLMN. It shall be included if the feature "MultipleAccessTypes" and, the feature "SliceSupport" or "DNNReplacementControl" are supported in the AMF and the UE is registered in the non-3GPP access. |  | SliceSupport, MultipleAccessTypes, DNNReplacementControl |  |
| guami |  | Guami |  | C |  | 0..1 |  | The Globally Unique AMF Identifier (GUAMI) shall be provided by an AMF as service consumer. |  |  |  |
| serviceName |  | ServiceName |  | O |  | 0..1 |  | If the NF service consumer is an AMF, it should provide the name of a service produced by the AMF that makes use of information received within the Npcf_AMPolicyControl_UpdateNotify service operation. |  |  |  |
| suppFeat |  | SupportedFeatures |  | M |  | 1 |  | Indicates the features supported by the service consumer. |  |  |  |
| traceReq |  | TraceData |  | C |  | 0..1 |  | Trace control and configuration parameters information defined in 3GPP TS 32.422 [18] shall be included if trace is required to be activated. |  |  |  |
| nwdafDatas |  | array(NwdafData) |  | O |  | 1..N |  | List of NWDAF Instance IDs and their associated Analytics IDs consumed by the NF service consumer. |  | EneNA |  |
| enrgSavInd |  | EnergySavingIndicator |  | O |  | 0..1 |  | Contains the subscribed Energy Saving Indicator for the UE. |  | Energy |  |
| NOTE: If the serving PLMN is not the HPLMN, then within the "ueSliceMbrs" attribute, there shall not be more than one array item with the same "servingSnssai" attribute's value in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |

* * * * Next changes * * * *

#### 5.6.2.4	Type PolicyAssociationUpdateRequest

Table 5.6.2.4-1: Definition of type PolicyAssociationUpdateRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| notificationUri | Uri | O | 0..1 | Identifies the recipient of Notifications sent by the PCF. |  |
| altNotifIpv4Addrs | array(Ipv4Addr) | O | 1..N | Alternate or backup IPv4 Address(es) where to send Notifications. |  |
| altNotifIpv6Addrs | array(Ipv6Addr) | O | 1..N | Alternate or backup IPv6 Address(es) where to send Notifications. |  |
| altNotifFqdns | array(Fqdn) | O | 1..N | Alternate or backup FQDN(s) where to send Notifications. |  |
| triggers | array(RequestTrigger) | C | 1..N | Request Triggers that the NF service consumer observes. Shall be provided when a policy control request trigger occurs. |  |
| servAreaRes | ServiceAreaRestriction | C | 0..1 | Service Area Restriction as part of the AMF Access and Mobility Policy. Shall be provided for trigger "SERV_AREA_CH". |  |
| wlServAreaRes | WirelineServiceAreaRestriction | C | 0..1 | Wireline Service Area Restriction as part of the AMF Access and Mobility Policy. Shall be provided for trigger "SERV_AREA_CH". | WirelineWirelessConvergence |
| rfsp | RfspIndex | C | 0..1 | RFSP Index as part of the AMF Access and Mobility Policy. Shall be provided for trigger "RFSP_CH". |  |
| smfSelInfo | SmfSelectionData | C | 0..1 | The UE requested S-NSSAI and UE requested DNN. Shall be provided for trigger "SMF_SELECT_CH". | DNNReplacementControl |
| ueAmbr | Ambr | C | 0..1 | UE-AMBR as part of the AMF Access and Mobility Policy. Shall be provided for trigger "UE_AMBR_CH". | UE-AMBR_Authorization |
| ueSliceMbrs | array(UeSliceMbr) | C | 1..N | The subscribed UE-Slice-MBR for each subscribed S-NSSAI of the home PLMN mapping to a S-NSSAI of the serving PLMN. Shall be provided for the "UE_SLICE_MBR_CH" policy control request trigger. (NOTE) | UE-Slice-MBR_Authorization |
| praStatuses | map(PresenceInfo) | C | 1..N | If the Trigger "PRA_CH" is reported, the UE presence status for tracking area for which changes of the UE presence occurred shall be provided. The "praId" attribute within the PresenceInfo data type shall also be the key of the map. The "presenceState" attribute within the PresenceInfo data type shall be supplied. The "additionalPraId" attribute within the PresenceInfo data type shall not be supplied. The "praId" attribute within the PresenceInfo data type shall include the identifier of an individual presence reporting area. |  |
| userLoc | UserLocation | C | 0..1 | The location of the served UE shall be provided for trigger "LOC_CH". |  |
| allowedSnssais | array(Snssai) | C | 1..N | Represents the Allowed NSSAI in the 3GPP access and includes the S-NSSAIs values the UE can use in the serving PLMN. It shall be provided for trigger "ALLOWED_NSSAI_CH". | SliceSupport, DNNReplacementControl, NetSliceRepl |
| partAllowedNssai | map(PartiallyAllowedSnssai) | C | 1..N | Represents the updated Partially Allowed NSSAI. It shall be provided for the trigger "PARTIALLY_ALLOWED_NSSAI_CH". The key of the map shall be set to the value of the "snssai" attribute of the corresponding map entry (encoded using the PartiallyAllowedSnssai data structure). | PartNetSliceSupport, NetSliceRepl |
| snssaisPartRejected | map(SnssaiPartRejected) | C | 1..N | Represents the updated set of S-NSSAI(s) rejected partially in the RA. It shall be provided for the trigger "SNSSAIS_PARTIALLY_REJECTED_CH". The key of the map shall be set to the value of the "snssai" attribute of the corresponding map entry (encoded using the SnssaiPartRejected data structure). | PartNetSliceSupport |
| rejectedSnssais | array(Snssai) | C | 1..N | Represents the updated set of Rejected S-NSSAI(s) in the RA. It shall be provided for the trigger "REJECTED_SNSSAIS_CH". | PartNetSliceSupport |
| pendingNssai | array(Snssai) | C | 1..N | Represents the updated Pending NSSAI. It shall be provided for the trigger "PENDING_NSSAI_CH". | PartNetSliceSupport |
| targetSnssais | array(Snssai) | C | 1..N | Represents the Target NSSAI. It shall be provided for the trigger "TARGET_NSSAI". | TargetNSSAI |
| mappingSnssais | array(MappingOfSnssai) | C | 1..N | The mapping of each S-NSSAI of the Allowed NSSAI and/or the Partially Allowed NSSAI to the corresponding S-NSSAI of the HPLMN. It shall be provided for the trigger "ALLOWED_NSSAI_CH" and/or "PARTIALLY_ALLOWED_NSSAI_CH", if available. If the "MultipleAccessTypes" feature is supported, this attribute contains also the mapping of the Allowed NSSAI in the non-3GPP access to the corresponding S-NSSAI of the HPLMN. | DNNReplacementControl, PartNetSliceSupport |
| n3gAllowedSnssais | array(Snssai) | C | 1..N | Represents the Allowed NSSAI in the non-3GPP access and includes the S-NSSAIs values the UE can use in the serving PLMN. It shall be provided for trigger "ALLOWED_NSSAI_CH" when the feature "MultipleAccessTypes" is supported. | SliceSupport, MultipleAccessTypes, DNNReplacementControl |
| unavailSnssais | array(Snssai) | C | 1..N | Represents the unavailable S-NSSAI(s) that require network slice replacement. It shall be provided for trigger "SLICE_REPLACE_MGMT" when the "NetSliceRepl" feature is supported. | NetSliceRepl |
| snssaiReplInfos | array(SnssaiReplaceInfo) | C | 1..N | Represents the change or removal of mapping of the (replaced) S-NSSAI(s) with the Alternative S-NSSAI(s) for one or more S-NSSAI(s) of the UE's Allowed NSSAI and/or Partially Allowed NSSAI. It shall be provided for trigger "SLICE_REPLACE_MGMT" and the feature "NetSliceRepl" is supported. | NetSliceRepl |
| accessTypes | array(AccessType) | C | 1..N | The Access Types where the served UE is camping. Shall be provided for trigger "ACCESS_TYPE_CH". | MultipleAccessTypes |
| ratTypes | array(RatType) | C | 1..N | The 3GPP RAT Types and/or non-3GPP RAT Types where the served UE is camping. Shall be provided for trigger "ACCESS_TYPE_CH", when the feature "MultipleAccessTypes" is supported, and/or for trigger "RAT_TYPE_CH", when the "RatTypeChange" feature is supported. | MultipleAccessTypes RatTypeChange |
| traceReq | TraceData | C | 0..1 | Trace control and configuration parameters information defined in 3GPP TS 32.422 [18] shall be included if trace is required to be activated, modified or deactivated. For trace modification, it shall contain a complete replacement of trace data. For trace deactivation, it shall contain the Null value. |  |
| guami | Guami | C | 0..1 | The Globally Unique AMF Identifier (GUAMI) shall be provided by an AMF as service consumer during the AMF relocation. |  |
| nwdafDatas | array(NwdafData) | O | 1..N | List of NWDAF Instance IDs and their associated Analytics IDs consumed by the NF service consumer. | EneNA |
| afSliceReplOut | SliceReplOutcomeInfo | C | 0..1 | Contains the AF requested Network Slice Replacement outcome related information. This attribute shall be present only for the trigger "SLICE_REPLACE_OUTCOME". | AfNetSliceRepl |
| enrgSavInd | EnergySavingIndicator | C | 0..1 | Contains the subscribed Energy Saving Indicator for the UE. This attribute shall be present only for the trigger "ENERGY_SAV_IND_CH". | Energy |
| suppFeat | SupportedFeatures | C | 0..1 | Indicates the features supported by the NF service consumer. It shall be included by the target AMF in inter-AMF mobility scenarios for trigger "FEAT_RENEG". | FeatureRenegotiation |
| NOTE: If the serving PLMN is not the HPLMN, then within the "ueSliceMbrs" attribute, there shall not be more than one array item with the same "servingSnssai" attribute's value in this release of the specification. |  |  |  |  |  |

* * * * Next changes * * * *

## 5.8	Feature negotiation

The optional features in table 5.8-1 are defined for the Npcf_AMPolicyControl API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [5].

Table 5.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | SliceSupport | Indicates the support of AM policies differentiation based on the awareness of the allowed NSSAI. |
| 2 | PendingTransaction | This feature indicates support for the race condition handling as defined in 3GPP TS 29.513 [7]. |
| 3 | UE-AMBR_Authorization | Indicates the support of UE-AMBR control by the PCF in the serving network. |
| 4 | DNNReplacementControl | Indicates the support of DNN replacement control. |
| 5 | MultipleAccessTypes | Indicates the support of AM policies for the multiple (i.e. 3GPP and non-3GPP) access and RAT types where the served UE is camping. |
| 6 | WirelineWirelessConvergence | Indicates the support of Wireline and Wireless access convergence. |
| 7 | ImmediateReport | Indicates the support of the current applicable values report corresponding to the policy control request triggers for policy update notification. |
| 8 | ES3XX | Extended Support for 3xx redirections. This feature indicates the support of redirection for any service operation, according to Stateless NF procedures as specified in clauses 6.5.3.2 and 6.5.3.3 of 3GPP TS 29.500 [5] and according to HTTP redirection principles for indirect communication, as specified in clause 6.10.9 of 3GPP TS 29.500 [5]. |
| 9 | UE-Slice-MBR_Authorization | Indicates the support of UE-Slice-MBR control by the PCF in the serving network. |
| 10 | AMInfluence | Indicates the support of the alternative mechanism to support informing the PCF for the UE of PDU session(s) established/terminated events via the delivery of the PCF for the UE information necessary for the PCF for the PDU session to send notifications on PDU session(s) established/terminated events through the AMF and the SMF. |
| 11 | EneNA | This feature indicates the support of NWDAF data reporting. |
| 12 | TargetNSSAI | Indicates the support for RFSP Index associated with the Target NSSAI. |
| 13 | 5GAccessStratumTime | This feature indicates the support of 5G acess stratum time distribution parameters provisioning. |
| 14 | FeatureRenegotiation | This feature indicates the support of feature renegotiation during the update of a policy association triggered by UE mobility with AMF change. |
| 15 | NetSliceRepl | This feature indicates that both UE and AMFsupport the network slice replacement functionality as part of the enhancements of the network slicing functionality. The following functionalities are supported: - Support the network slice replacement information management. |
| 16 | RFSPValidityTime | This feature indicates the support of the provisioning of a validity time for the RFSP Index value that indicates the EPC/E-UTRAN access is prioritized over 5GS access. |
| 17 | NetTimeSyncStatus | This feature indicates the support of network timing synchronization status and reporting. This feature requires the support of the 5GAccessStratumTime feature as well. |
| 18 | NetSliceUsageCtrl | This feature indicates the support of the network slice usage control functionality as part of the enhancements of the network slicing functionality. The following functionalities are supported: - Support the provisioning by the PCF of the network slice usage control information (e.g., slice deregistration inactivity timer value). This feature requires the support of the "SliceSupport" and/or "DNNReplacementControl" features. |
| 19 | PartNetSliceSupport | This feature indicates the partial network slice support in a Registration Area functionality as part of the enhancements of the network slicing functionality. The following functionalities are supported: - Support the reporting of the changes in the Partially Allowed NSSAI, S-NSSAI(s) rejected partially in the RA, Rejected S-NSSAI(s) in the RA and/or the Pending NSSAI to the PCF. |
| 20 | SLAMUP | This feature indicates the support of the provisioning to the AMF of the CHF information of the CHF selected by the PCF. |
| 21 | RatTypeChange | This feature indicates the support of provisioning the AM policies to the UE for the change in the RAT type within the same Access type. |
| 22 | AfNetSliceRepl | This feature indicates the support of the AF requested Network Slice Replacement services. The following functionalities are supported: - Support the provisioning and management of the AF requested Network Slice Replacement requirements. - Support the reporting of the AF requested Network Slice Replacement outcome related event(s). |
| 23 | Energy | This feature indicates the support of the network Energy-based Policy control and management. The following functionalities are supported: - Support of provisioning the subscribed Energy Saving Indicator and reporting its value changes. |
| 24 | CHFGroup | This feature indicates the support of the CHF Group ID handling for the discovery of the CHF. |

* * * * End of changes * * * *