3GPP TSG-CT WG3 Meeting #143	C3-254446

# Sophia-Antipolis, France, 13 - 17 October 2025						(Revision of C3-254109)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.548 | CR | 0062 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multimodal sync policy threshold handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 15-10-2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Sync policy, The first and second multimodaltype can be same. There is no restriction that it should not of the same type. For example, 2 video streams in the same multi modal session can have the sync threshold by which the first video should not be delayed too much compare to the 1st video. As explained in Media Synchronization Requirements - Research Questions Task Force, “studies conducted on speech intelligibility when audio quality is degraded in such a way as to simulate age-related hearing loss have shown that when the audio signal leads the video, intelligibility declines appreciably for even the shortest asynchrony of 40 ms, but when the video signal leads the audio, intelligibility remains relatively stable for onset asynchronies up to 160 - 200 ms (Grant & Greenberg, 2001).” In Sync policy, the multimodal type can have different thresholds between different media handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Sync policy, the restriction of the same multimodal type is removed. The sync policy with two entries to distinguish the synchronization threshold is updated.. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unnecessary extra restriction in the Sync policy which forbid the policy between the same multimodal type. It is not possible to fully meet the media synchornization policy requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.6.2.9, 6.5.6.2.14 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR does not impact the OpenAPI descriptions defined in this specification. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

##### 6.5.6.2.9	Type: MultiModalSealddPolicy

Table 6.5.6.2.9-1: Definition of type MultiModalSealddPolicy

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| syncPol | array(SyncPolicy) | C | 1..N | Contains the multi-modal synchronization policy. (NOTE 1), (NOTE 2) |  |
| alignmentPol | AlignmentPolicy | C | 0..1 | Contains the multi-modal flows alignment policy. (NOTE 1) |  |
| ueToUePol | UEToUEPolicy | C | 0..1 | Contains the multi-modal UE-to-UE policy. (NOTE 1) |  |
| expTime | DateTimeRo | O | 0..1 | Contains the expiration time of the Multi-modal SEALDD policy configuration. This attribute may be present only in a Policy Configuration creation/update response. If this attribute is absent, this means that the the Multi-modal SEALDD policy configuration shall not expire until explicitly deleted by the service consumer. |  |
| NOTE 1: At least one of these attributes shall be present. NOTE 2: This attribute may contain synchronization thresholds in two directions for one or several pairs of multi-modal flows (e.g., multi-modal flow1 delay compared to multi-modal flow2 and multi-modal flow2 delay compared to multi-modal flow1). In such case, this is achieved by including two array elements within this attribute (i.e., one for each direction) for each concerned pair of multi-modal flows. |  |  |  |  |  |

# * * * * Next changes * * * *

##### 6.5.6.2.14	Type: SyncPolicy

Table 6.5.6.2.14-1: Definition of type SyncPolicy

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| flowType1 | MultiModalFlowType | M | 1 | Contains the first multi-modal flow type. |  |
| flowType2 | MultiModalFlowType | M | 1 | Contains the second multi-modal flow type. |  |
| syncThreshold | DurationMilliSec | M | 1 | Contains the maximum tolerable time delay, expressed in milliseconds, by which the multi-modal flow type identified by the "flowType1" attribute can be delayed compared to the multi-modal flow type identified by the "flowType2" attribute of the application, such that they can be perceived as being synchronous. |  |

# * * * * End of Changes * * * *