3GPP TSG-CT WG3 Meeting #143	C3-254506

# Sophia-Antipolis, France, 13 - 17 October 2025						(Revision of C3-254110)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 29.548 | CR | 0063 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DurationMillisec Common OpenAPI update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C3 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2_App |  |  |  |  |  | Date: |  |  | 2025-10-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The common API reference for DurationMillisec is agreed in TS 29.122 clause A.2. The corresponding update has to performed in this TS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | DurationMillisec is updated with TS 29.122 openAPI reference. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The common datatype openAPI reference is not updated. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.6.1, A.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backward compatible correction to the following APIs: TS29548_SDD_PolicyConfiguration.yaml TS29122_CommonData.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First Change * * * *

#### 6.5.6.1	General

This clause specifies the application data model supported by the API.

Table 6.5.6.1-1 specifies the data types defined for the SDD_PolicyConfiguration API.

Table 6.5.6.1-1: SDD_PolicyConfiguration API specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| AlignmentPolicy | 6.5.6.2.15 | Represents the multi-modal flows alignment policy. | XRMApp |
| BdwCtrlPolicy | 6.5.6.3.3 | Represents the bandwidth control policy. |  |
| GeofencingArea | 6.5.6.2.8 | Represents the geofencing area. | SEALDD_2 |
| GeofencingPolicy | 6.5.6.2.7 | Represents the geofencing policy. | SEALDD_2 |
| GeoPolAction | 6.5.6.3.4 | Represents the geofencing policy action. | SEALDD_2 |
| MultiModalFlowType | 6.5.6.2.20 | Represents the Muti-modal flow type. | XRMApp |
| MultiModalSealddPolicy | 6.5.6.2.9 | Represents the Multi-modal SEALDD Policy. | XRMApp |
| Non3gppAccessMeasPol | 6.5.6.2.11 | Represents the non-3GPP access measurement policy. | SEALDD_2 |
| QualGuarPolicy | 6.5.6.2.5 | Represents the quality guarantee policy. |  |
| QualGuarThresh | 6.5.6.2.6 | Represents the quality guarantee related thresholds. |  |
| PolicyConfig | 6.5.6.2.2 | Represents a SEALDD Policy Configuration. |  |
| PolicyConfigPatch | 6.5.6.2.3 | Represents the parameters to request the modification of a SEALDD Policy Configuration. |  |
| ProximityThresholds | 6.5.6.2.17 | Represents the proximity thresholds for entering/leaving the UE-to-UE direct communication mode. | XRMApp |
| QoEThresholds | 6.5.6.2.19 | Represents the QoE thresholds for entering/leaving the UE-to-UE direct communication mode. | XRMApp |
| QoSThresholds | 6.5.6.2.18 | Represents the QoS thresholds for entering/leaving the UE-to-UE direct communication mode. | XRMApp |
| SealddPolicy | 6.5.6.2.4 | Represents a SEALDD Policy. |  |
| SignalStrength | 6.5.6.2.12 | Represents the signal strength value. | SEALDD_2 |
| SignalStrengthThreshold | 6.5.6.2.13 | Represents the signal strength threshold. | SEALDD_2 |
| SyncPolicy | 6.5.6.2.14 | Represents the multi-modal synchronization policy. | XRMApp |
| TempPolicy | 6.5.6.2.10 | Represents the temporal policy. | SEALDD_2 |
| UEToUEPolicy | 6.5.6.2.16 | Represents the multi-modal UE-to-UE policy. | XRMApp |

Table 6.5.6.1-2 specifies data types re-used by the SDD_PolicyConfiguration API from other specifications, including a reference to their respective specifications, and when needed, a short description of their use within the SDD_PolicyConfiguration API.

Table 6.5.6.1-2: SDD_PolicyConfiguration API re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| BitRate | 3GPP TS 29.571 [18] | Represents a bit rate. | XRMApp |
| DateTimeRo | 3GPP TS 29.122 [2] | Represents a date and a time with the "read-only" property. |  |
| DurationMilliSec | 3GPP TS 29.122 [2] | Represents a time duration in units of milliseconds. | XRMApp |
| GeographicArea | 3GPP TS 29.572 [22] | Represents a geographical area. | SEALDD_2 |
| MatchingDirection | 3GPP TS 29.520 [20] | Represents a threshold matching direction. | SEALDD_2 |
| MeasurementId | Clause 6.4.6.3.3 | Represents the transmission quality measurement type. |  |
| MultiModalId | 3GPP TS 29.514 [19] | Represents the identifier of a multi-modal service. | XRMApp |
| PacketErrRate | 3GPP TS 29.571 [18] | Represents the packet error rate. | XRMApp |
| PacketLossRate | 3GPP TS 29.571 [18] | Represents the packet loss rate. | XRMApp |
| ScheduledCommunicationTime | 3GPP TS 29.122 [2] | Represents a time schedule. | SEALDD_2 |
| Snssai | 3GPP TS 29.571 [18] | Represents a S-NSSAI. | XRMApp |
| SupportedFeatures | 3GPP TS 29.571 [18] | Represents the list of supported feature(s) and used to negotiate the applicability of the optional features. |  |
| ThresholdHandlingMode | 3GPP TS 29.549 [15] | Represents threshold handling mode. | SEALDD_2 |
| TransQualMeasCriteria | Clause 6.4.6.2.7 | Represents the transmission quality measurement reporting criteria. |  |
| TimeWindow | 3GPP TS 29.122 [2] | Represents a time window. | SEALDD_2 |
| Uint32 | 3GPP TS 29.571 [18] | Represents an unsigned 32-bit integer. | XRMApp |
| Uinteger | 3GPP TS 29.571 [18] | Represents an unsigned integer. | XRMApp |
| Uri | 3GPP TS 29.122 [2] | Represents a URI. |  |
| ValTargetUe | 3GPP TS 29.549 [15] | Represents the identifier of the targeted VAL UE or VAL user. |  |

# * * * * Next changes * * * *

# A.6	SDD_PolicyConfiguration API

openapi: 3.0.0

info:

title: SEALDD Server Policy Configuration Service

version: 1.1.0-alpha.3

description: |

SEALDD Server Policy Configuration Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 29.548 V19.4.0; Service Enabler Architecture Layer for Verticals (SEAL);

SEAL Data Delivery (SEALDD) Server Services; Stage 3.

url: https://www.3gpp.org/ftp/Specs/archive/29_series/29.548/

servers:

- url: '{apiRoot}/sdd-pc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 6.5 of 3GPP TS 29.549

security:

- {}

- oAuth2ClientCredentials: []

paths:

/configurations:

post:

summary: Request the creation of a Policy Configuration.

operationId: CreatePolicyConfig

tags:

- Policy Configurations (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

responses:

'201':

description: >

Created. The Policy Configuration is successfully created and a representation of

the created Individual Policy Configuration resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

headers:

Location:

description: >

Contains the URI of the created Individual Policy Configuration resource.

required: true

schema:

type: string

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

/configurations/{configId}:

parameters:

- name: configId

in: path

description: >

Represents the identifier of the Individual Policy Configuration resource.

required: true

schema:

type: string

get:

summary: Retrieve an existing Individual Policy Configuration resource.

operationId: GetIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

responses:

'200':

description: >

OK. The requested Individual Policy Configuration resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29122_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

put:

summary: Request the update of an existing Individual Policy Configuration resource.

operationId: UpdateIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

responses:

'200':

description: >

OK. The Individual Policy Configuration resource is successfully updated and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully updated

and no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

patch:

summary: Request the modification of an existing Individual Policy Configuration resource.

operationId: ModifyIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

requestBody:

required: true

content:

application/merge-patch+json:

schema:

$ref: '#/components/schemas/PolicyConfigPatch'

responses:

'200':

description: >

OK. The Individual Policy Configuration resource is successfully modified and a

representation of the updated resource shall be returned in the response body.

content:

application/json:

schema:

$ref: '#/components/schemas/PolicyConfig'

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully modified and

no content is returned in the response body.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

delete:

summary: Request the deletion of an existing Individual Policy Configuration resource.

operationId: DeleteIndPolicyConfig

tags:

- Individual Policy Configuration (Document)

responses:

'204':

description: >

No Content. The Individual Policy Configuration resource is successfully deleted.

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

#

# STRUCTURED DATA TYPES

#

PolicyConfig:

description: >

Represents a SEALDD Policy Configuration.

type: object

properties:

appTrafficIds:

type: array

items:

type: string

minItems: 1

valTargetId:

$ref: 'TS29549_SS_UserProfileRetrieval.yaml#/components/schemas/ValTargetUe'

sliceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

sealddPol:

$ref: '#/components/schemas/SealddPolicy'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRo'

multimodsddPol:

$ref: '#/components/schemas/MultiModalSealddPolicy'

suppFeat:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- appTrafficIds

anyOf:

- required: [sealddPol]

- required: [multimodsddPol]

PolicyConfigPatch:

description: >

Represents the requested modifications to a SEALDD Policy Configuration.

type: object

properties:

sealddPol:

$ref: '#/components/schemas/SealddPolicy'

multimodsddPol:

$ref: '#/components/schemas/MultiModalSealddPolicy'

SealddPolicy:

description: >

Represents a SEALDD Policy.

type: object

properties:

qualGuarPol:

$ref: '#/components/schemas/QualGuarPolicy'

qualOptPol:

$ref: '#/components/schemas/QualGuarPolicy'

bdwCtrlSets:

type: array

items:

$ref: '#/components/schemas/BdwCtrlPolicy'

minItems: 1

geoPol:

type: array

items:

$ref: '#/components/schemas/GeofencingPolicy'

minItems: 1

tempPol:

$ref: '#/components/schemas/TempPolicy'

non3gppAccMeasPol:

$ref: '#/components/schemas/Non3gppAccessMeasPol'

anyOf:

- required: [qualGuarPol]

- required: [qualOptPol]

- required: [bdwCtrlSets]

- required: [geoPol]

- required: [tempPol]

- required: [non3gppAccMeasPol]

QualGuarPolicy:

description: >

Represents the quality guarantee policy.

type: object

properties:

thresholds:

$ref: '#/components/schemas/QualGuarThresh'

required:

- thresholds

QualGuarThresh:

description: >

Represents the quality guarantee related thresholds.

type: object

properties:

measId:

type: array

items:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/MeasurementId'

minItems: 1

measThesh:

$ref: 'TS29548_SDD_TransmissionQualityMeasurement.yaml#/components/schemas/TransQualMeasCriteria'

required:

- measId

- measThesh

GeofencingPolicy:

description: Represents a geofencing policy.

type: object

properties:

valServiceId:

type: string

area:

$ref: '#/components/schemas/GeofencingArea'

action:

$ref: '#/components/schemas/GeoPolAction'

required:

- valServiceId

- area

- action

GeofencingArea:

description: Represents a geofencing area.

type: object

properties:

geoAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

minItems: 1

anyOf:

- required: [geoAreas]

MultiModalSealddPolicy:

description: >

Represents a Multi-modal SEALDD Policy.

type: object

properties:

syncPol:

type: array

items:

$ref: '#/components/schemas/SyncPolicy'

minItems: 1

description: >

Contains the synchronization policy.

The key of the map shall be set to the multi-modal flow type provided within the

"flowType" attribute of the corresponding map value encoded using the SyncPolicy data

structure.

alignmentPol:

$ref: '#/components/schemas/AlignmentPolicy'

ueToUePol:

$ref: '#/components/schemas/UEToUEPolicy'

expTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DateTimeRo'

anyOf:

- required: [syncPol]

- required: [alignmentPol]

- required: [ueToUePol]

TempPolicy:

description: >

Represents a temporal policy.

type: object

properties:

schedule:

type: array

items:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

minItems: 1

timeWindow:

$ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow'

oneOf:

- required: [schedule]

- required: [timeWindow]

Non3gppAccessMeasPol:

description: >

Represents the non-3GPP access measurement policy.

type: object

properties:

accessIds:

type: array

items:

type: string

minItems: 1

thresholds:

type: array

items:

$ref: '#/components/schemas/SignalStrengthThreshold'

minItems: 1

thrHandling:

$ref: 'TS29549_SS_NetworkResourceMonitoring.yaml#/components/schemas/ThresholdHandlingMode'

timeThr:

$ref: '#/components/schemas/TempPolicy'

anyOf:

- required: [accessIds]

- required: [thresholds]

- required: [timeThr]

SignalStrength:

description: >

Represents the signal strength value.

type: object

properties:

rssi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

anyOf:

- required: [rssi]

SignalStrengthThreshold:

description: >

Represents the signal strength threshold.

type: object

properties:

value:

$ref: '#/components/schemas/SignalStrength'

direction:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/MatchingDirection'

required:

- value

- direction

SyncPolicy:

description: >

Represents the synchronization policy.

type: object

properties:

syncThreshold:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationMilliSec'

required:

- syncThreshold

AlignmentPolicy:

description: >

Represents the multi-modal flows alignment policy.

type: object

properties:

multiModalId:

$ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/MultiModalId'

maxAlignTime:

$ref: 'TS29122_CommonData.yaml#/components/schemas/DurationMilliSec'

anyOf:

- required: [multiModalId]

- required: [maxAlignTime]

UEToUEPolicy:

description: >

Represents the multi-modal UE-to-UE policy.

type: object

properties:

proxThresh:

$ref: '#/components/schemas/ProximityThresholds'

qoSThresh:

$ref: '#/components/schemas/QoSThresholds'

qoEThresh:

$ref: '#/components/schemas/QoEThresholds'

anyOf:

- required: [proxThresh]

- required: [qoSThresh]

- required: [qoEThresh]

ProximityThresholds:

description: >

Represents the proximity thresholds for entering/leaving the UE-to-UE direct communication

mode.

type: object

properties:

minUeToUedist:

type: number

format: double

minimum: 0

avgUeToUedist:

type: number

format: double

minimum: 0

maxUeToUedist:

type: number

format: double

minimum: 0

anyOf:

- required: [minUeToUedist]

- required: [avgUeToUedist]

- required: [maxUeToUedist]

QoSThresholds:

description: >

Represents the QoS thresholds for entering/leaving the UE-to-UE direct communication mode.

type: object

properties:

minLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

avgLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

maxLatency:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

minBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

avgBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

maxBitRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate'

minPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

avgPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

maxPackLossRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketLossRate'

minPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

avgPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

maxPackErrRate:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PacketErrRate'

minJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

avgJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

maxJitter:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

anyOf:

- required: [minLatency]

- required: [avgLatency]

- required: [maxLatency]

- required: [minBitRate]

- required: [avgBitRate]

- required: [maxBitRate]

- required: [minPackLossRate]

- required: [avgPackLossRate]

- required: [maxPackLossRate]

- required: [minPackErrRate]

- required: [avgPackErrRate]

- required: [maxPackErrRate]

- required: [minJitter]

- required: [avgJitter]

- required: [maxJitter]

QoEThresholds:

description: >

Represents the QoE thresholds for entering/leaving the UE-to-UE direct communication mode.

type: object

properties:

mos:

type: integer

minimum: 1

maximum: 5

anyOf:

- required: [mos]

# SIMPLE DATA TYPES

#

#

# ENUMERATIONS

#

BdwCtrlPolicy:

anyOf:

- type: string

enum:

- REALLOCATE_DL

- REALLOCATE_UL

- NOT_REALLOCATE_DL

- NOT_REALLOCATE_UL

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the bandwidth control policy.

Possible values are:

- REALLOCATE_DL: Indicates that the bandwidth control action is to reallocate the

bandwidth limit between different VAL users for DL traffic.

- REALLOCATE_UL: Indicates that the bandwidth control action is to reallocate the

bandwidth limit between different VAL users for UL traffic.

- NOT_REALLOCATE_DL: Indicates that the bandwidth control action is to not reallocate

the bandwidth limit between different VAL users for DL traffic.

- NOT_REALLOCATE_UL: Indicates that the bandwidth control action is to not reallocate

the bandwidth limit between different VAL users for UL traffic.

GeoPolAction:

anyOf:

- type: string

enum:

- ALLOW

- BLOCK

- type: string

description: >

This string provides forward-compatibility with future extensions to the enumeration

and is not used to encode content defined in the present version of this API.

description: |

Represents the geofencing policy action.

Possible values are:

- ALLOW: Indicates that the geofencing policy is to allow.

- BLOCK: Indicates that the geofencing policy is to block.

# Data types describing alternative data types or combinations of data types:

#

# * * * * End of Changes * * * *