3GPP TSG-CT WG1 Meeting #157	C1-256025

# Sophia Antipolis, France, 13-17 October 2025

Source:	vivo

Title:	Discussion on 6G modular NAS

Agenda item:	20.2

Document for:	DISCUSSION

Abstract of the contribution: This paper analyses and proposes to study whether and how to introduce modular NAS into 6G NAS protocol framework design.

1 Introduction

6G will serve a wide range of devices, from smartphones and AR glasses to robots and IoT sensors. These devices have different capabilities and requirements. Therefore, 6G NAS protocol needs to evolve to support not only enhanced communication services but also beyond communication services, such as sensing, computing, data framework and AI services. How to build a more efficient and flexible 6G NAS protocol framework is one of the key motivations of the CT1 6G studies, and one possible way to achieve the goal is to enable NAS modularization.

2 Discussion

2.1 Why need support modular NAS

The reasons why modular NAS (which can be also called as NAS modularization) is needed can be seen as following:

- Pain points learned from 5G NAS protocol

There are several pain points identified from the 5G NAS protocol design and corresponding drawbacks are shown as below:

- strong coupling between AMF and other NFs:

  - Inefficient network upgrade and maintenance: AMF is the sole anchor for all other NFs. Scale-up/down and scale-in/out of other NFs will impact the AMF, and vice versa. This increases complexity and difficulty of network upgrade and maintenance.

  - Signalling detour and congestion due to the central deployment: All UL/DL 5G NAS signalling are anchored at the central deployed AMF, which caused signalling detour to the 5G NFs (e.g., SMF) who can be deployed close to the network edge. The central deployed AMF has a large serving area which can serve millions of UEs. This creates a signalling bottleneck for entire UE-CN communication and increases the risk of signalling congestion and AMF failures.

- strong coupling between NAS protocols:

  - Payload container type value run out issue: A series of service NAS messages, such as SMS, UE policy, and LCS, are piggybacked in a container of the 5GMM message and are identified by the payload container type IE. The encoding values of payload container type IE are limited (i.e., 4 bits) and only 3 values are left, making it difficult to piggyback more new service NAS messages.

  - NAS specification is overly bulky: MM protocol, SM protocol, and policy delivery protocol are all described in a single TS 24.501. This document is overly large, slow to open, and hard to read.

- strong coupling within single NAS protocol:

  - Type 6 IEI run out issue: MM functionalities (such as registration, NAS capability negotiation, and so on) are carried out via 5GMM protocol and many parameters need to be exchanged via MM procedure. Some parameters have large size and need to be transferred using a type 6 IE (maximum length of 65535 octets), but the reserved values for Type 6 IEIs are limited (only 16), making it insufficient to support a large amount of Type 6 IEs. In R18, CT1 had ran out of type 6 IEI in registration accept message. The solution is not optimal, as it does not avoid further IEI run out issue and results in additional encoding overhead (which introduces additional IEI and length field).

  - inefficient NAS protocol upgrade: 5GMM protocol supports many interdependent functionalities, for example, slice updates, UE NAS capabilities updates, UE radio capability updates, registration area updates, synchronization of PDU session status and so on. All these functionalities are all integrated into the mobility registration procedure. Each new trigger of mobility registration procedure requires modifications to the existing registration procedure, message encoding, processing logic and testing. It brings difficulty of iteration, upgrade, and evolution at both UE and network.

  - Inefficient design for capability negotiation between the UE and network: The UE is required to include the 5GMM capability IE towards the AMF every registration even if no capability has been changed. Even when the UE and the network don’t support certain feature, but the UE and network still need to transmit MM message containing many capability bits for the unsupported feature and related sub features.

- New requirement to 6G NAS brought by 6G services

SA1 requirements on diversity of UE types in TR 22.870 can be found as below:

- It is envisioned that a next generation system will continue to support a population of UEs with varying capabilities that would support different use cases. For example, simple UEs with limited capabilities would be supported by the 6G network, alongside more sophisticated UEs that support more advanced features and capabilities.

- It is also important to highlight that while there is expected diversity in the types of UEs, each UE type should have at its core a common set of basic capabilities, to maximise the commonalities and thus reducing market fragmentation. (TR 22.870)

SA1 requirement highlights that diverse UE types have different capabilities while some capabilities (e.g., MM, SM) are common for many UE types and should achieve the on-demand combination of functionalities. 5G NAS does not support the on-demand combination of functionalities.

In addition, 6G new use cases introduce beyond communication services, which poses greater challenges to 6G NAS protocol design.

Observation 1: 5G NAS design is strongly coupled, which brings difficulty and complexity in the upgrade, evolution, and customization for the UE and network, making it tough challengeable to meet the 6G new requirements.

2.2 What is modular NAS

Decoupled design of NAS function and protocol have become a new approach to address the pain points of 5G NAS and meet the new service requirements of 6G, which makes modular NAS an important topic for 6G NAS study.

Modular NAS is a modularized way to design NAS function and NAS protocol, wherein each NAS module is self-contained functional module designed to support certain NAS functionalities or set of NAS functionalities.

UEs or NFs can support several NAS modules on demand based on different capabilities and service requirements.

Figure 1: An example of modular NAS design for different UE types

Observation 2: Modular NAS design enables customized protocol stacks tailored to specific device types and services, providing an effective method to achieve NAS functionalities decoupling, address the pain points of 5G NAS and meet the new service requirements of 6G.

2.3 How to support modular NAS

Modular NAS is a concept to support NAS functionalities decoupling, and there are lots of aspects for CT1 to study to realize such concept into 6G NAS protocol design.

2.3.1 Principles of modular NAS design

Defining the clear principles of modular NAS design is the critical step, as it ensures consistency and scalability of subsequent study. Here are some thought for the principles of modular NAS design:

- Modular NAS should be flexible and unsymmetrical:

- some NAS modules are supported by both the UE side and the network side. For example, if MM protocol is defined as a NAS module, then the MM module shall be supported by both the UE and the network.

- some NAS modules are only supported by the UE side, and some NAS modules are only supported by the network side. For example, if UAC functionality is defined as a NAS module, then the UAC module is only supported by the UE but not supported by the network.

- Modular NAS should be highly correlated and loosely coupled.

- The functionalities within single NAS module should be highly correlated and each NAS module should support only one category of related functionalities, which enables independent evolution, reuse, and update.

- The dependencies between NAS modules should be minimized to prevent changes in one NAS module from causing related impacts on others, which enhances flexibility and scalability.

2.3.2 Granularity of modular NAS design

To accommodate different needs of NAS protocols, different granularity needs to be applied to the design of different NAS module and NAS protocol.

- Figure 2 gives an example of mobility registration flow with coarse-grained modular NAS design.

Figure 2: An example of mobility registration flow with coarse-grained modular NAS design

If the MM protocol is defined as a MM module, some pros and cons can be observed from the mobility registration procedure (which is as an example). The mobility registration procedure has multiple triggers, such as location update, NSSAI update, MM capability update and so on.

- When the UE needs to update its location and update NSSAI simultaneously, only one message is sent by the MM module to update the parameters of two features, which reduces the signalling exchanges and improves update efficiency.

- When the UE only wants to update its location, the unchanged NSSAI and MM capability parameters are also included in the registration message, which increases signalling overhead. Moreover, each new trigger of mobility registration procedure requires modifications to the existing registration procedure, which brings difficulty of iteration, upgrading, and evolution at both UE and network.

- Figure 3 gives an example of mobility registration flow with fine-grained modular NAS design.

If the MM protocol is divided into multiple feature-specific modules, such as an initial registration module, MM capability module, slice module, mobility module, UE radio capability module, CP data transport module, and so on.

Figure 3: An example of mobility registration flow with fine-grained modular NAS design

Taking mobility registration procedure as an example, that means it can be divided into multiple sub flows (procedures). When the UE needs to update its location, the mobility module sends a mobility update message to the network; when the slice information requires update, the slicing module sends an NSSAI update message; and when the UE’s MM capability requires update, the MM capability module sends an MM capability update message.

Some pros and cons can be observed from fine-grained modular NAS design:

- A corresponding new NAS module and associated message(s) will also be introduced when a new feature is introduced. Such design does not impact the existing NAS modules, thereby making it easy for the upgrade and evolution. It also reduces the need to containing multiple types of information within a single message, thus reducing the consumption of IEIs.

- As the number of supported features increase, the number of NAS modules will also grow. Consequently, the number of associated procedures and messages may increase significantly, leading to higher signalling interaction overhead.

From the above analysis, it can be observed that it requires careful consideration to determine the granularity of modular NAS, as different options have distinct trade-offs. Each NAS protocol has distinct characteristics, such as interaction frequency, message size, dependency on network function and UE needs. Therefore, each module requires different trade-off considerations.

Observation 3: Different NAS protocol may require different granularity in the modular NAS design. The granularity should be tailored based on the characteristics of each NAS protocol.

According to the two examples discussed above, it can be observed that determining the appropriate granularity for modular NAS is a critical issue, requiring consideration trade-offs between system flexibility, signalling complexity, functionalities reuse, and module independence. Here are some preliminary considerations regarding the choice of modularization granularity.

- For the communication services:

Since communication services are already deployed, the modularization of these services shall take into account multiple factors, such as compatibility with existing NAS functionalities, interactions with other modules, potential upgrade and evolution paths and so on.

Hence analysis and study are needed when designing modular NAS for communication services, for example, whether to define the MM function as a single NAS module or as multiple fine-grained modules. Modular NAS can be organized on below basis, for example:

  - per protocol: MM module, SM module, location module, UE policy module…

  - per NAS functionality(ies): UAC module, registration management module, mobility management module, connection management module…

  - per feature: UAC module, registration module, mobility module, slice module, SMS module, CP data transport module, capability negotiation module, service request module…

Note: Registration management is a high-level NAS functionality that can be further divided into multiple fine-grained NAS functionalities. Therefore, modular NAS can be organized on a per-feature basis.

- For the beyond communication services:

Beyond communication services are new for both UE and network, it is seemly easier to reach consensus on that beyond communication services shall be designed as independent modules from the beginning. This method enables separate introduction, management, and evolution of these services without impacting existing functionalities, supporting flexibility and diverse 6G use case requirements. It would be appropriate to consider whether to divide such new service module into finer granularity after SA2 conclusion is stable.

2.3.3 Corresponding NAS impact on modular NAS

In addition to study how to define the granularity for modular NAS, there are several aspects to be studied for modular NAS, including:

  - Inter-module coordination: How to achieve effective coordination and interaction among different NAS modules to ensure correct and consistent overall system behaviour.

  - Impact on L3 protocol design principles: Whether modular NAS introduces new requirements or impacts the design principles of the L3 protocol.

  - Message encoding: How to encode and decode the messages for each NAS module to ensure the independence and extensibility of NAS modules.

  - NAS security support: How to support NAS security, including encryption, integrity protection, and replay protection, within a modular NAS design.

2.4 Summary

From the above analysis, the study on modular NAS includes a wide range of topics, including modular NAS design principles, modular granularity, inter-module coordination, message encoding, and NAS security.

3 Proposal

The 6G NAS protocol needs to support efficient design to overcome the limitations of 5G NAS and support both communication and beyond communication services. Modular NAS enables on-demand design to meet the requirements of diverse UE types and supports function decoupling for easier network upgrades, making it a promising direction for 6G NAS evolution.

3.1 Suggested timeline

The following provides our preliminary considerations on the timeline of this work:

- The study on modular NAS for MM and SM protocol can be started immediately. As MM and SM protocol have relatively limited dependency on SA2 and some associated pain points have been identified, the study can focus on addressing the 5G NAS pain points.

- The study on modular NAS for beyond communication services can be started only after SA2 requirements for beyond communication services is stable. As beyond communication services has strong dependency on SA2 and their functional boundaries are not defined.

3.2 Suggested scope

The following impacts and areas of work are expected to be covered in the CT aspects of modular NAS:

CT1:

-	Study on whether and how to define 6G NAS protocol in a modularized way.

-	Study on how to support modular NAS design.

  - Study on the principle to design modular NAS.

  - Study on how to define the NAS module with different granularity to accommodate different needs of NAS protocols.

  - Study on procedure, message, NAS parameters to support modular NAS.

  - Study on potential impact on L3 NAS protocol (e.g., message coding, IE coding).

  - Study on how to coordination among different NAS modules.