3GPP TSG-CT WG1 Meeting #157	C1-256028

Sophia-Antipolis, France, 13-17 October 2025

Source:	Samsung

Title:	Service operation on spatial anchors notification

Spec:	3GPP TS 24.550 v1.1.0

Agenda item:	19.42

Document for:	Agreement

1. Introduction

This p-CR proposes spatial anchors notification to 3GPP TS 24.550.

2. Reason for Change

In the current TS 24.550v1.1.0, the spatial anchor notification service operation is yet to be defined. This pCR defines service operation for spatial anchors notification as per clause 8.3.5.2.2 of 3GPP TS 23.437, which enables the spatial anchor server to share the notification information about the updated spatial anchor(s), to those spatial anchor client(s) subscribed for these updates.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 24.550 v1.1.0.

* * * First Change * * * *

##### 5.2.1.2.1	Introduction

The service operation defined for SS_SAnManagement API is shown in the table 5.2.1.2.1-1.

Table 5.2.1.2.1-1: Operations of the SS_SAnManagement API

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| SS_SAnManagement_Create | This service operation is used by the SAn client to create the spatial anchor on SAn server. | SAn client, VAL Server |
| SS_SAnManagement_Update | This service operation is used by the SAn client to update the spatial anchor on SAn server. | SAn client |
| SS_SAnManagement_Delete | This service operation is used by the SAn client to delete the spatial anchor resource on SAn server. | SAn client |
| SS_SAnManagement_Subscribe | This service operation is used by the SAn client to subscribe for the spatial anchor related notifications on SAn server. | SAn client |
| SS_SAnManagement_UpdateSubscription | This service operation is used by the SAn client to update a subscription for the spatial anchor related notifications on SAn server. | SAn client |
| SS_SAnManagement_Unsubscribe | This service operation is used by the SAn client to unsubscribe for the spatial anchor related notifications on SAn server. | SAn client |
| SS_SAnManagement_Retrieve | This service operation is used by SAn client to retrieve to spatial anchor information from the SAn server. | SAn client |
| SS_SAnManagement_Notify | This service operation is used by the spatial anchor server to notify the spatial anchor(s) related notifications to the spatial anchor client. | SAn server |

* * * Next Change * * * *

##### 5.2.1.2.y	SS_SAnManagement_Notify

###### 5.2.1.2.y.1	General

This service operation is used by the spatial anchor server to notify the spatial anchor(s) related notifications to the spatial anchor client.

###### 5.2.1.2.y.2	SAn server notifying the spatial anchor notification to SAn client

The SAn server shall send an HTTP POST request to the SAn client with the request URI set to the "{notifUri}" URI as specified in 3GPP TS 29.437 [6] in clause 6.1.1.5.2.2. The body of the HTTP POST request shall include the "SpatialAnchorsNotif" data structure as specified in 3GPP TS 29.437 [6] clause 6.1.1.5.2.3.1.

Upon reception of the HTTP POST request from the SAn server, the SAn client shall:

a)	validate if there is an active spatial anchor subscription with SAn server; in case of none the HTTP "406 Not Acceptable" response is sent;

b)	process the "SpatialAnchorsNotif" data structure as specified in 3GPP TS 29.437 [6] clause 6.1.1.6.2.13 shared in the POST request body and send the HTTP "204 No Content" response to the SAn server; and

c)	notify the data spatial anchor notification to the VAL client.

* * * End of Changes * * * *