3GPP TSG-CT WG1 Meeting #157	C1-256049

Sophia Antipolis, France, 13-17 October 2025

Source:	vivo

Title:	Handling on collision of inventory procedures

Spec:	3GPP TS 24.369-100

Agenda item:	19.70

Document for:	Decision

1. Introduction

<Introduction part (optional)>

2. Reason for Change

This CR proposes to modify “the collision of AIoT paging” case in clause 5.2.5 of TS 24.369, to capture the agreements in RAN2 and SA3.

In R2-2504935 (LS on parallel service request), RAN2 gives agreements on support of parallel service requests for overlapping reader scenario:

Agreements on parallel service request

- Rel-19 devices are not expected to receive parallel service request for overlapping reader scenario based on network implementation. Capture this in RAN2 stage 2 specification.

- The Rel-19 device always responds to the new service indicated by the received paging message applicable for that device. Capture this in RAN2 stage 3 specification.

Besides, SA3 also points out in clause 5.2.2 of TS 33.369 that:

NOTE 1: An active attack may send a new paging request to the device while there is an ongoing procedure in device. The device will abort the ongoing procedure and respond to the new paging. The security measure to such denial-of-service attack is not specified in present document.

Both RAN2 and SA3 have agreed that if a new paging is received as an ongoing procedure in progress, the AIoT device will abort the ongoing procedure and respond to the new paging.

However, in clause 5.2.2 of TS 24.369, CT1 has agreed that if a new paging is received as an ongoing procedure in progress, the AIoT device will proceed with the ongoing procedure and then respond to the new paging later. The conclusion of CT1 is opposite to that of RAN2 and SA3.

Related DP is in C1-256048.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 V1.0.0.

* * * First Change * * * *

5.2.5	Abnormal cases in the AIoT device

The following abnormal cases can be identified:

a)	Failure of authentication.

If an AIoT device fails the authentication procedure as specified in 3GPP TS 33.369 [6], the AIoT device shall not respond to the AIoT paging message.

b)	Collision of AIoT paging.

If a new AIoT paging is received while an inventory procedure is in progress, the AIoT device shall abort the ongoing inventory procedure and shall process the new AIoT paging.

* * * End of Changes * * * *