3GPP TSG-CT WG1 Meeting #157	C1-256071

Sophia Antipolis, France, 13-17 October 2025

SA WG2 Meeting #S2-170	S2-2508102

25 - 29 August, 2025, Goteborg, Sweden

# Title:	 Reply LS on the path information in 5G ProSe multi-hop discovery procedures

# Response to:	(S2-2506119/ C1-253724) LS on the path information in 5G ProSe multi-hop discovery procedures

# Release:	Release 19

# Work Item:	5G_ProSe_Ph3

Source:	 SA2

To:	CT1

Cc:	-

Contact Person:

Name:	Naman Gupta

E-mail Address:	nmn DOT gupta AT samsung DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	23.304 CR 0558

1. Overall Description:

SA2 would like to thank CT1 for the LS on path information in 5G ProSe multi-hop discovery procedures. SA2 discussed the issue raised by CT1 and offers the following clarifications.

For Question 1 CT1 asked: For both model A and model B discovery, given that in the path to a 5G ProSe multi-hop layer-3 end UE, the 5G ProSe multi-hop layer-3 end UE's first 5G ProSe layer-3 multi-hop UE-to-UE relay inherently stores the source layer-2 ID of the 5G ProSe multi-hop layer-3 end UE, why does the path information need to include the source layer-2 ID of the 5G ProSe multi-hop layer-3 end UE?

SA2 Answer:

For Model A discovery, it is rightly pointed out that parameter was included incorrectly. SA2 has thus rectified this error in its specifications.

For Model B discovery, similar to the single hop 5G ProSe UE-to-UE Relay operations (as specified in TS 23.304 clause 5.8.4), the User Info of the 5G ProSe End UE is expected to be protected in the Direct Discovery set. Thus, as the User Info ID of the source end UE would not be visible to the U2U relays, source end UE’s first 5G Prose layer-3 multi-hop UE-to-UE relay need the source-layer 2 ID in order to direct the response message to that particular source end UE in a unicast manner. This was discussed and approved in 23.304 CR 0508 S2-2412612.

For Question 2 CT1 asked: In the 5G ProSe UE-to-UE relay discovery announcement message (for model A discovery), it specifies the path information includes the "source layer-2 ID of the discoverer ProSe end UE (yellow highlighted texts as below)". However, the term "discoverer ProSe end UE" pertains to model B discovery. Why does the announcing relay UE indicate this source layer-2 ID of the discoverer ProSe end UE in the path information, given that model A does not involve a "discoverer" end UE?

SA2 Answer:

SA2 acknowledges and appreciates CT1 for correctly identifying the issue. The source layer-2 ID of the discoverer was mistakenly included in the Model A discovery, but SA2 has rectified this error in its specifications.

For Question 3 CT1 asked: Why are the Layer-2 IDs included in the Multi-hop path info IE, together with the list of the User info IDs?

SA2 Answer:

SA2 discussed this issue and also identified the inconsistency in SA2 specs at few places. SA2 came to the conclusion that in the Multi-hop U2N Model B discovery, the Intermediate Relay(s) does not need to store any state and thus the Layer-2 IDs are included to facilitate the Response message transmission as in step 8-9 of clause 6.3.2.5.3. SA2 has also included the Layer-2 IDs for the link establishment of Prose U2U relays using Model B.

Based on the above reply SA2 has updated their specification in TS 23.304 CR 0558

2. Actions:

To CT WG1 group.

ACTION: 	SA2 kindly asks CT1 to take the above information into account.

3. Date of Next SA2 Meetings:

SA2#171                           13th October- 17th October 2025			Wuhan, China

SA2#171                           17th November- 21st November 2025			Dallas, US