3GPP TSG-CT WG1 Meeting #157	C1-256113

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.554 | CR | 0834 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolving the EN related to remote UE context in the PROSE DIRECT LINK MODIFICATION REQUEST message for multi-hop UE-to-network relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The following EN: Editor's note (CR#0682, 5G_ProSe_Ph3): The information element for remote UE context in the PROSE DIRECT LINK MODIFICATION REQUEST message is FFS. …needs to be resolved. It is worth to note that the PROSE DIRECT LINK MODIFICATION REQUEST message already now includes a "List of multi-hop remote UE parameters IE" which can deliver Remote UE context of multiple MH Remote UEs to the parent intermediate U2N relay UEs. Hence the same IE can be used by the MH Remote UE towards its first intermediate U2N relay UE, after setting the Number of remote UE parameters to 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Resolving the EN related to remote UE context in the PROSE DIRECT LINK MODIFICATION REQUEST message for multi-hop UE-to-network relay by reusing the List of multi-hop remote UE parameters IE after doing any needed adjustments. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN stays unresolved and incomplete procedure stays. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8b.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** First change *****

#### 8b.2.2.3	5G ProSe direct link modification procedure for multi-hop UE-to-network relay

The 5G ProSe direct link modification procedure for multi-hop UE-to-network relay reuses the procedure for 5G ProSe direct link modification procedure for UE-to-network relay communication described in clause 7.2.3, with considering the same roles defined in bullet a) and b) of clause 8b.2.2.2.1.

When the 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay is based on model A discovery:

a)	after each of the 5G ProSe multi-hop child UE-to-network relay UE establishes the 5G ProSe direct link with its 5G ProSe intermediate UE-to-network relay UE, the 5G ProSe intermediate UE-to-network relay UE shall trigger a 5G ProSe direct link modification procedure for multi-hop UE-to-network relay towards its 5G ProSe multi-hop parent UE-to-network relay UE and include the User info ID of the 5G ProSe multi-hop child UE-to-network relay UE in the Child UEs multi-hop user info IE in the PROSE DIRECT LINK MODIFICATION REQUEST message. Consequently, every 5G ProSe multi-hop parent UE-to-network relay UE, acting as a 5G ProSe multi-hop child UE-to-network relay UE, shall trigger a 5G ProSe direct link modification procedure for multi-hop UE-to-network relay towards its 5G ProSe multi-hop parent UE-to-network relay UE and include the received Child UEs multi-hop user info IE in the PROSE DIRECT LINK MODIFICATION REQUEST message; and

b)	after the 5G ProSe multi-hop remote UE establishes the 5G ProSe direct link with the 5G ProSe intermediate UE-to-network relay UE:

1)	the 5G ProSe multi-hop remote UE may initiate a 5G ProSe direct link modification procedure for multi-hop UE-to-network relay toward the 5G ProSe intermediate UE-to-network relay UE to inform its remote UE context after obtaining IP address/prefix from the 5G ProSe multi-hop UE-to-network relay UE, where the PROSE DIRECT LINK MODIFICATION REQUEST message created by the 5G ProSe multi-hop remote UE

i)	shall additionally include the List of remote UE parameters IE with the Number of remote UE parameters set to 1 and the Remote UE parameters set to the identity, address information, total hop count and associated PC5 QoS flow of the 5G ProSe multi-hop remote UE; and

2)	then, each 5G ProSe intermediate UE-to-network relay UE, starting from the 5G ProSe intermediate UE-to-network relay UE connected to the 5G ProSe multi-hop remote UE, shall initiate a 5G ProSe direct link modification procedure for multi-hop UE-to-network relay towards its 5G ProSe multi-hop parent UE-to-network relay UE, where the PROSE DIRECT LINK MODIFICATION REQUEST message created by each 5G ProSe intermediate UE-to-network relay UE towards its 5G ProSe multi-hop parent UE-to-network relay UE:

i)	shall additionally include the List of multi-hop remote UE parameters IE to indicate the identity, address information, total hop count and associated PC5 QoS flow of the 5G ProSe multi-hop remote UE(s); and

c)	each 5G ProSe intermediate UE-to-network relay UE shall complete the 5G ProSe direct link modification procedure for multi-hop UE-to-network relay with its 5G ProSe multi-hop child UE-to-network relay UE or its the 5G ProSe multi-hop remote UE before triggering the same procedure for the next hop.

When the 5G ProSe direct link establishment procedure for multi-hop UE-to-network relay is based on model B discovery, after the 5G ProSe multi-hop remote UE establishes the 5G ProSe direct link with the 5G ProSe intermediate UE-to-network relay UE:

a)	the 5G ProSe multi-hop remote UE may trigger the 5G ProSe direct link modification procedure for multi-hop UE-to-network relay towards the 5G ProSe intermediate UE-to-network relay UE to provide PC5 QoS Info and PC5 QoS rule(s);

Editor's note (CR 0670, 5G_ProSe_Ph3): The details of that step including any potential changes in the PROSE DIRECT LINK MODIFICATION REQUEST message to deliver the PC5 QoS Info and PC5 QoS rule(s) are FFS.

b)	then, each 5G ProSe intermediate UE-to-network relay UE, starting from the 5G ProSe intermediate UE-to-network relay UE connected to the 5G ProSe multi-hop remote UE, may initiate a 5G ProSe direct link modification procedure for multi-hop UE-to-network relay towards its 5G ProSe multi-hop parent UE-to-network relay UE, where the PROSE DIRECT LINK MODIFICATION REQUEST message created by each 5G ProSe intermediate UE-to-network relay UE towards its 5G ProSe multi-hop parent UE-to-network relay UE:

1)	shall additionally include the split the PC5 QoS; and

Editor's note (CR 0670, 5G_ProSe_Ph3): The details of the split PC5 QoS hop-by-hop and any corresponding updates in the PROSE DIRECT LINK MODIFICATION REQUEST message are FFS.

b)	each 5G ProSe multi-hop parent UE-to-network relay UE shall complete the 5G ProSe direct link modification procedure for multi-hop UE-to-network relay with its 5G ProSe multi-hop child UE-to-network relay UE before triggering the same procedure for the next hop.

***** End of changes *****