3GPP TSG-CT WG1 Meeting #157	C1-256145

# Sophia-Antipolis, France, 13-17 October 2025

Source:	Ericsson

Title:	AIoT device not supporting read or write command

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Decision

1. Introduction

2. Reason for Change

Observation-1: Stage-2 requires the AIoT device to inform network when an unsupported command is received.

Observation-2: Current stage-3 does not satisfy stage-2 requirement.

Proposal-1: If AIoT device receives unsupported message, AIoT device responds with AIOT STATUS message with cause "message type non-existent or not implemented".

Observation-3: Sending a command to the AIoT device which does not support the command, results into waste of radio resource by paging message (if paging for individual device is used), INVENTORY REPORT message, the command, and AIOT STATUS message.

Proposal-2: If the AIOTF sends a command to the AIoT device and receives AIOT STATUS message with cause "message type non-existent or not implemented", then in order to prevent waste of radio resource by further command procedures of the same command type, the AIOTF refrans from initiating of further command procedures of the same command type towards the AIoT device.

Proposal-3: If the AIOTF sends a command to the AIoT device and receives AIOT STATUS message with cause "message type non-existent or not implemented", then in order to prevent waste of radio resource by command procedures of the command types other than the command type of the sent command, the AIoT device includes its AIOT capabilities, if any, in the AIOT STATUS message and the AIOTF refrans from initiating of command procedures of command types not supported in the AIOT capabilities, towards the AIoT device.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369.

***** change *****

#### 5.3.2.5	Read command procedure not accepted by AIoT device

Upon receiving a READ COMMAND message, if the command type specific parameters (e.g. the offset to read application data from the AIoT Device user memory, the length of the application data to read.) is invalid, the AIoT device shall send a READ COMMAND REJECT message with a cause set to #xx "TBD".

***** change *****

#### 5.3.2.x	Abnormal cases in the network

The following abnormal cases can be identified:

a)	Read command procedure not supported by the AIoT device.

Upon receiving an AIOT STATUS message with cause #97 "message type non-existent or not implemented", the AIOTF shall abort the read command procedure, determine that the AIoT device does not support the read command procedure, and shall refrain from initiating further read command procedures towards the AIoT device. If the AIoT device capability IE is not included in the AIOT STATUS message or the WCPI bit of the AIoT device capability IE of the AIOT STATUS message is set to "Write command procedure not supported", then the AIOTF shall refrain from initiating the write command procedure to the AIoT device.

***** change *****

#### 5.3.3.5	Write command procedure not accepted by AIoT device

Upon receiving a WRITE COMMAND message, if the command type specific parameters (e.g. the offset where to write the application data, the length of the application data to write) is invalid, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

Upon receiving a WRITE COMMAND message, if the write command procedure cannot be implemented because the energy will be run out, the AIoT device shall send a WRITE COMMAND REJECT message with a cause set to #xx "TBD".

***** change *****

#### 5.3.3.x	Abnormal cases in the network

The following abnormal cases can be identified:

a)	Write command procedure not supported by the AIoT device.

Upon receiving an AIOT STATUS message with cause #97 "message type non-existent or not implemented", the AIOTF shall abort the write command procedure, determine that the AIoT device does not support the write command procedure, and shall refrain from initiating further write command procedures towards the AIoT device. If the AIoT device capability IE is not included in the AIOT STATUS message or the RCPI bit of the AIoT device capability IE of the AIOT STATUS message is set to "Read command procedure not supported", then the AIOTF shall refrain from initiating the read command procedure to the AIoT device.

***** change *****

### 5.3.x	AIoT status procedure

#### 5.3.x.1	General

The purpose of the AIoT status procedure is for the AIoT device to report certain error conditions detected in a received AIoT NAS message (see example in figure 5.3.x.1-1).

Figure 5.3.x.1-1: AIoT status procedure

#### 5.3.x.2	AIoT status procedure initiation

In order to initiate an AIoT status procedure, the AIoT device shall send an AIOT STATUS message to the AIOTF.

If a message type of a received NAS message is not defined for the AIoT NAS protocol or not implemented by the AIoT device, the AIoT device shall set the cause IE of the the AIOT STATUS message to cause #97 "message type non-existent or not implemented", set the RCPI bit of the AIoT device capability IE of the AIOT STATUS message to "Read command procedure supported", if the AIoT device supports the read command procedure, and set the WCPI bit of the AIoT device capability IE of the AIOT STATUS message to "Write command procedure supported", if the AIoT device supports the write command procedure.

#### 5.3.x.3	AIoT status procedure completion

On receipt of an AIOT STATUS message in the AIOTF, no specific action shall be taken as seen from the radio interface, i.e. local actions are possible. The local actions to be taken by the AIOTF on receipt of an AIOTF STATUS message are implementation dependent, with exception of those specified in subclause 5.3.2.x and subclause 5.3.3.x.

***** change *****

## 6.3	Unknown or unforeseen message type

If the AIoT device receives an AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the AIoT device, the AIoT device shall perform the AIoT status procedure as specified in subclause 5.3.x.

If the network receives a AIoT NAS message with message type not defined for the AIoT NAS protocol or not implemented by the network, it shall ignore the AIoT NAS message.

NOTE:	A message type not defined for the AIoT NAS protocol in the given direction is regarded by the receiver as a message type not defined for the AIoT NAS protocol, see 3GPP TS 24.007 [3].

***** change *****

### 7.1.x	AIoT status

#### 7.1.x.1	Message definition

The AIOT STATUS message is sent by the AIoT device to report certain error conditions detected in a received AIoT NAS message .

See table 7.1.x.1-1.

Message type:	AIOT STATUS

Significance:	local

Direction:	AIoT device to AIOTF

Table 7.1.x.1-1: AIOT STATUS message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | AIoT status message identity | Message type 7.2.2 | M | V | 1 |
|  | Cause | Cause 7.2.8 | M | V | 1 |
| YY | AIoT device capability | AIoT device capability 7.2.y | O | TLV | 3-4 |

***** change *****

##### 7.1.2.y	AIoT device capability

The AIoT device shall include the AIoT device capability IE if the AIoT device supports one or more optional capabilities.

***** change *****

### 7.2.2	Message type

The Message type IE and its use are defined in 3GPP TS 24.007 [3]. Table 7.2.2.1 defines the value part of the Message type IE used in the AIoT NAS protocol.

Table 7.2.2-1: Message types for AIoT NAS

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | Inventory report |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | Read command |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | Read complete |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | Read command reject |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | Write command |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  | Write complete |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |  | Write command reject |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 |  | Permanent disable command |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 |  | Permanent disable complete |
| x | x | x | x | x | x | x | x |  | AIoT status |

Editor's note:	Whether the range for message type of 8 bits is needed or a shorter range is sufficient is FFS. If a shorter range is sufficient, remaining bits in the IE can be used for other mandatory parameters, e.g. part of the security parameters.

***** change *****

### 7.2.8	Cause

The purpose of the cause information element is to indicate the cause of the failure of the read or write operation.

The cause information element is coded as shown in figure 7.2.8-1 and table 7.2.8-1.

The cause is a type 3 information element with length of 2 octets.

Editor's note:	the possible cause values are FFS.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

Figure 7.2.8-1: Cause information element

Table 7.2.8-1: Cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Error, unspecified |
| 1 | 0 | 0 | 1 | 0 | 1 | 1 | 1 |  | Message type non-existent or not implemented |
| All other values are spare and if received shall be interpreted as "error, unspecified". |  |  |  |  |  |  |  |  |  |

***** change *****

### 7.2.y	AIoT device capability

The purpose of the AIoT device capability information element is to provide the network with information concerning aspects of the AIoT device related to the AIoT NAS protocol. The contents might affect the manner in which the network handles the operation of the AIoT device.

The AIoT device capability is a type 4 information element with a minimum length of 3 octets and maximum length of 4 octets.

The AIoT device capability information element is coded as shown in figure 7.2.y-1 and table 7.2.y-1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AIoT device capability IEI |  |  |  |  |  |  |  | octet 1 |
| Length of AIoT device capability contents |  |  |  |  |  |  |  | octet 2 |
| 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | WCPI | RCPI | octet 3 |
| 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | 0 spare | octet 4* |

Figure 7.2.y-1: AIoT device capability information element

Table 7.2.y-1: AIoT device capability information element

| Read command procedure indicator (RCPI) (octet 3, bit 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 |  |  |  | Read command procedure not supported |
| 1 |  |  |  | Read command procedure supported |
| Write command procedure indicator (WCPI) (octet 3, bit 2) |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 |  |  |  | Write command procedure not supported |
| 1 |  |  |  | Write command procedure supported |
| Bits 3 to 8 of octet 3 and bits of octet 4 are spare and shall be coded as zero. |  |  |  |  |

***** change *****

# A.1	Causes related to command procedure

Cause #x2 – TBD

This cause is used to indicate that the command type specific parameters (e.g. the offset to read application data from the AIoT device user memory, the length of the application data to read, the offset where to write the application data, or the length of the application data to write) is invalid.

Cause #x3 – TBD

This cause is used to indicate that the requested read or write command was not executed successfully.

Cause #x4 – TBD

This cause is used to indicate that the application data to be read cannot be found by the AIoT device in the requested place of the user memory.

Cause #x5 – TBD

This cause is used to indicate that the write command procedure cannot be implemented because the energy will be run out.

Cause #97 – Message type non-existent or not implemented

This cause indicates that the equipment sending this cause has received a message with a message type it does not recognize either because this is a message not defined, or defined but not implemented by the equipment sending this cause.