3GPP TSG-CT WG1 Meeting #157	C1-256278

# Sophia Antipolis, France 13 – 17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.229 | CR | 6757 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to syntax of extended P-Access-Network-Info header field |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, 5GSAT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Both "access-type" and "access-class" take the form of a token as defined in section 5.4 of RFC 7315. The ABNF rule of "token" is defined in section 3.2.6 of RFC 7230 quoted as below. " token = 1*tchar tchar = "!" / "#" / "$" / "%" / "&" / "'" / "*"/ "+" / "-" / "." / "^" / "_" / "`" / "¦" / "~"/ DIGIT / ALPHA; any VCHAR, except delimiters " However, the current definitions for access-class and access-type in Table 7.2A.4 include numerous values containing parentheses (), e.g., "3GPP-NR(LEO)" which are explicitly prohibited within a token as mentioned in RFC 7230. Inclusion of parentheses in a token will lead to the formal syntax unparseable by a standards-compliant processor. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace parentheses () with hyphens - in all relevant access-class and access-type values (e.g. "3GPP-NR(LEO)" is changed to "3GPP-NR-LEO") to comply with RFC 7230 token syntax. Backward compatibility analysis: This change is NOT backward compatible. A receiver compliant with this CR will fail to parse and process the received access-class or access-type set with the values containing parentheses (). A receiver compliant with the previous release will treat the access-class or access-type set with values compliant with this CR as unknown values. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementations may handle these illegal values inconsistently (e.g., rejection vs. acceptance), leading to interoperability failures between network elements from different vendors. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2A.4.2, 7.2A,4.3, 7.7, 8.1.1, 8.1.2, 8.2.2, A.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

7.2A.4.2	Syntax

The syntax of the P-Access-Network-Info header field is described in RFC 7315 [52] and RFC 7913 [234]. There are additional coding rules for this header field depending on the type of IP-CAN, according to access technology specific descriptions.

Table 7.2A.4 describes the 3GPP-specific extended syntax of the P-Access-Network-Info header field defined in RFC 7315 [52] and RFC 7913 [234].

Table 7.2A.4: Syntax of extended P-Access-Network-Info header field

daylight-saving-time   = "daylight-saving-time" EQUAL quoted-string

UE-local-IP-address = "UE-local-IP-address" EQUAL DQUOTE ( IPv4address / IPv6reference ) DQUOTE

UDP-source-port = "UDP-source-port" EQUAL port

TCP-source-port = "TCP-source-port" EQUAL port

ePDG-IP-address = "ePDG-IP-address" EQUAL DQUOTE ( IPv4address / IPv6reference ) DQUOTE

U2N-relay-ID    = "U2N-relay-ID" EQUAL quoted-string

access-class    =/ "untrusted-non-3GPP-VIRTUAL-EPC" / "VIRTUAL-no-PS" / "WLAN-no-PS" /

"3GPP-NR" / "3GPP-NR-U" / "3GPP-NR-SAT" / "3GPP-NR-LEO" / "3GPP-NR-MEO" / "3GPP-NR-GEO" / "3GPP-NR-OTHERSAT" / "3GPP-WB-E-UTRAN-LEO" / "3GPP-WB-E-UTRAN-MEO" / "3GPP-WB-E-UTRAN-GEO" / "3GPP-WB-E-UTRAN-OTHERSAT" / "3GPP-NB-IoT-LEO" / "3GPP-NB-IoT-MEO" / "3GPP-NB-IoT-GEO" / "3GPP-NB-IoT-OTHERSAT" / "3GPP-LTE-M-LEO" / "3GPP-LTE-M-MEO" / "3GPP-LTE-M-GEO" / "3GPP-LTE-M-OTHERSAT" / "3GPP-NR-REDCAP"

access-type     =/ "3GPP-E-UTRAN-ProSe-UNR" / "xDSL" / "3GPP-NR-FDD" / "3GPP-NR-TDD" /

"IEEE-802.11ac" / "3GPP-NR-U-FDD" / "3GPP-NR-U-TDD" / "3GPP-NR-SAT" / "3GPP-NR-LEO" / "3GPP-NR-MEO" / "3GPP-NR-GEO" / "3GPP-NR-OTHERSAT" / "3GPP-WB-E-UTRAN-LEO" / "3GPP-WB-E-UTRAN-MEO / "3GPP-WB-E-UTRAN-GEO" / "3GPP-WB-E-UTRAN-OTHERSAT" / "3GPP-NB-IoT-LEO" / "3GPP-NB-IoT-MEO" / "3GPP-NB-IoT-GEO" / "3GPP-NB-IoT-OTHERSAT" / "3GPP-LTE-M-LEO" / "3GPP-LTE-M-MEO" / "3GPP-LTE-M-GEO" / "3GPP-LTE-M-OTHERSAT" / "3GPP-NR-ProSe-L2UNR" / "3GPP-NR-ProSe-L3UNR" / "3GPP-NR-REDCAP"

eps-fb          = "eps-fallback" EQUAL "0" / "1"

redcap

satellite-id    = "satellite-id" EQUAL 8BIT

Editor' Note:	(WI: ARCH_NR_REDCAP, CR #6656) Possible interactions with "3GPP-NR-REDCAP" are FFS.

The daylight-saving-time and the UE-local-IP-address are instances of generic-param from the current extension-access-info component of the P-Access-Network-Info header field defined in RFC 7315 [52] and RFC 7913 [234].

The presence of the "network-provided" header field parameter defined in RFC 7315 [52] indicates a P-Access-Network-Info header field is provided by the P-CSCF, S-CSCF, the AS, the MSC server enhanced for ICS, the MSC server enhanced for SRVCC using SIP interface, the MSC server enhanced for DRVCC using SIP interface or by the MGCF. The content can differ from a P-Access-Network-Info header field without this parameter which is provided by the UE.

The "network-provided" header field parameter can be used with both "access-type" and "access-class" constructs. The "access-class" construct is provided for use where the value is not known to be specific to a particular "access-type" value, e.g. in the case of some values delivered from the PCRF. The "access-class" field can be set only by the P-CSCF, the MSC server enhanced for ICS, the MSC server enhanced for SRVCC using SIP interface, the MSC server enhanced for DRVCC using SIP interface or by the AS. The "network-provided" header field parameter can be set only by the P-CSCF, S-CSCF, the AS, the MSC server enhanced for ICS, the MSC server enhanced for SRVCC using SIP interface, the MSC server enhanced for DRVCC using SIP interface or by the MGCF. The "local-time-zone" parameter, the "daylight-saving-time" parameter, the "gstn-location" parameter, the "GSTN" value of access-type field and the "untrusted-non-3GPP-VIRTUAL-EPC" value of access-class field shall not be inserted by the UE.

The "local-time-zone" parameter defined in RFC 7315 [52] indicates the time difference between local time and UTC of day. For 3GPP accesses, the "local-time-zone" parameter represents the time zone allocated to the routing area or traffic area which the UE is currently using. As the edge of such areas may overlap, there can be some discrepancy with the actual time zone of the UE where the UE is in the near proximity to a time zone boundary.

The "daylight-saving-time" parameter indicates by how much the local time of the UE has been adjusted due to the use of daylight saving time. Providing the "daylight-saving-time" parameter is optional.

The "UE-local-IP-address" parameter indicates the UE local IP address.

NOTE:	The UE local IP address is the source address on the outer header of the IPsec tunnel packets received by the ePDG on the S2b interface.

The "UDP-source-port" parameter indicates that the IKEv2 messages exchanged between the UE and the ePDG are encapsulated in the UDP messages according to IETF RFC 3948 [63A]. The value of the "UDP-source-port" parameter is the UDP source port of the UDP messages:

-	received by the ePDG; and

-	encapsulating the IKEv2 messages.

The "TCP-source-port" parameter indicates that the IKEv2 messages exchanged between the UE and the ePDG are transported using the firewall traversal tunnel as described in 3GPP TS 24.302 [8U]. The value of the "TCP-source-port" parameter is the TCP source port of the TCP messages:

-	received by the ePDG; and

-	of the firewall traversal tunnel transporting the IKEv2 messages.

The "ePDG-IP-address" parameter indicates the ePDG IP address used as IKEv2 tunnel endpoint with the UE.

The "U2N-relay-ID" parameter indicates the IMSI as described in 3GPP TS 23.003 [3], of 5G ProSe UE-to-network relay, when the call request is triggered from the 5G ProSe remote UE and relayed by the 5G ProSe UE-to-network relay. The IMSI is encoded as specified in 3GPP TS 29.228 [14]. The "U2N-relay-ID" parameter can be set only by the P-CSCF.

The "eps-fallback" header field parameter is used to indicate that the current access technology is used as a result of EPS fallback. The value "1" indicates that EPS fallback has occurred, the value "0" that EPS fallback has not occurred. The parameter can be set only by the P-CSCF.

The "satellite-id" parameter indicates the ID of satellite as described in 3GPP TS 36.331 [19F], when the call request is triggered for UE-Satellite-UE communication as described in Annex Y. The satellite-id is coded as a binary coded integer value from 0 to 255 as defined for the SatelliteId IE in 3GPP TS 36.331 [19F]. The "satellite-id" parameter can be set only by the P-CSCF.

* * * Next Change * * * *

7.2A.4.3	Additional coding rules for P-Access-Network-Info header field

The P-Access-Network-Info header field is populated with the following contents:

1)	the access-type field set to one of "3GPP-GERAN","3GPP-UTRAN-FDD", "3GPP-UTRAN-TDD", "3GPP-E-UTRAN-FDD", "3GPP-E-UTRAN-TDD", "3GPP-E-UTRAN-ProSe-UNR", "3GPP-NR-FDD", "3GPP-NR-TDD", "3GPP-NR-U-FDD", "3GPP-NR-U-TDD", "3GPP-NR-SAT", "3GPP-NR-LEO", "3GPP-NR-MEO", "3GPP-NR-GEO", "3GPP-NR-OTHERSAT", "3GPP-WB-E-UTRAN-LEO", "3GPP-WB-E-UTRAN-MEO, "3GPP-WB-E-UTRAN-GEO", "3GPP-WB-E-UTRAN-OTHERSAT", "3GPP-NB-IoT-LEO", "3GPP-NB-IoT-MEO", "3GPP-NB-IoT-GEO", "3GPP-NB-IoT-OTHERSAT", "3GPP-LTE-M-LEO", "3GPP-LTE-M-MEO", "3GPP-LTE-M-GEO", "3GPP-LTE-M-OTHERSAT", "3GPP-NR-ProSe-L2UNR", "3GPP-NR-ProSe-L3UNR", "3GPP-NR-REDCAP", "3GPP2-1X", "3GPP2-1X-HRPD", "3GPP2-UMB", "3GPP2-1X-Femto", "IEEE-802.11", "IEEE-802.11a", "IEEE-802.11b", "IEEE-802.11g", "IEEE-802.11n", "IEEE-802.11ac", "ADSL", "ADSL2", "ADSL2+", "RADSL", "SDSL", "HDSL", "HDSL2", "G.SHDSL", "VDSL", "IDSL", "xDSL", "DOCSIS", "IEEE-802.3", "IEEE-802.3a", "IEEE-802.3e", "IEEE-802.3i", "IEEE-802.3j", "IEEE-802.3u", "IEEE-802.3ab", "IEEE-802.3ae", "IEEE-802.3ah", "IEEE-802.3ak", "IEEE-802.3aq", "IEEE-802.3an", "IEEE-802.3y", "IEEE-802.3z", or "DVB-RCS2" as appropriate to the access technology in use.

1A)	the access-class field set to one of "3GPP-GERAN", "3GPP-UTRAN", "3GPP-E-UTRAN", "3GPP-NR", "3GPP-NR-U", "3GPP-NR-SAT", "3GPP-NR-LEO", "3GPP-NR-MEO", "3GPP-NR-GEO", "3GPP-NR-OTHERSAT", "3GPP-WB-E-UTRAN-LEO", "3GPP-WB-E-UTRAN-MEO", "3GPP-WB-E-UTRAN-GEO", "3GPP-WB-E-UTRAN-OTHERSAT", "3GPP-NB-IoT-LEO", "3GPP-NB-IoT-MEO", "3GPP-NB-IoT-GEO", "3GPP-NB-IoT-OTHERSAT", "3GPP-LTE-M-LEO", "3GPP-LTE-M-MEO", "3GPP-LTE-M-GEO", "3GPP-LTE-M-OTHERSAT", "3GPP-NR-REDCAP", "3GPP-WLAN", "3GPP-GAN", "3GPP-HSPA", "3GPP2", "untrusted-non-3GPP-VIRTUAL-EPC", "VIRTUAL-no-PS", or "WLAN-no-PS" as appropriate to the technology in use. The access-class field set to "untrusted-non-3GPP-VIRTUAL-EPC" indicates the IP-CAN associated with an EPC based untrusted non-3GPP access with unknown radio access technology. The access-class field set to "VIRTUAL-no-PS" indicates an IP-CAN associated with an unknown radio access technology, such that the IP-CAN is not provided by the packet switched domain of the PLMN of the P-CSCF. The access-class field set to "WLAN-no-PS" indicates an IP-CAN associated with WLAN, such that the IP-CAN is not provided by the packet switched domain of the PLMN of the P-CSCF. The access-class field set to "3GPP-NR-SAT", "3GPP-NR-LEO", "3GPP-NR-MEO", "3GPP-NR-GEO" or "3GPP-NR-OTHERSAT" indicates an IP-CAN associated with satellite NG-RAN. The access-class field set to "3GPP-WB-E-UTRAN-LEO", "3GPP-WB-E-UTRAN-MEO", "3GPP-WB-E-UTRAN-GEO", "3GPP-WB-E-UTRAN-OTHERSAT", "3GPP-NB-IoT-LEO", "3GPP-NB-IoT-MEO", "3GPP-NB-IoT-GEO", "3GPP-NB-IoT-OTHERSAT", "3GPP-LTE-M-LEO", "3GPP-LTE-M-MEO", "3GPP-LTE-M-GEO" or "3GPP-LTE-M-OTHERSAT" indicates an IP-CAN associated with satellite E-UTRAN. The access-class field set to "3GPP-NR-REDCAP" if the UE has provided the NR RedCap indication to the IP-CAN as described in 3GPP TS 23.501 [257].

2)	if the access-type field or the access-class field is set to "3GPP-GERAN", a cgi-3gpp parameter set to the Cell Global Identity obtained from lower layers of the UE. The Cell Global Identity is a concatenation of MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), LAC (4 hexadeciaml digits) and CI (as described in 3GPP TS 23.003 [3]. The "cgi-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

3)	if the access-type field is equal to "3GPP-UTRAN-FDD", or "3GPP-UTRAN-TDD", and a UE provides the P-Acces-Network-Info header field, a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), LAC (4 hexadecimal digits) as described in 3GPP TS 23.003 [3] and the UMTS Cell Identity (7 hexadecimal digits) as described in 3GPP TS 25.331 [9A]), obtained from lower layers of the UE. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

3A)	if the access-type field is equal to "3GPP-UTRAN-FDD", or "3GPP-UTRAN-TDD", and an entitiy that can use the "network-provided" header field parameter provides the P-Access-Network-Info header field, if available a "utran-sai-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), LAC (4 hexadecimal digits) as described in 3GPP TS 23.003 [3] and SAC (4 hexadecimal digits) as described in 3GPP TS 23.003 [3]. The "utran-sai-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

3B)	if the access-class field is equal to "3GPP-UTRAN", or "3GPP-HSPA", if available a "utran-sai-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), LAC (4 hexadecimal digits) as described in 3GPP TS 23.003 [3] and SAC (4 hexadecimal digits) as described in 3GPP TS 23.003 [3]. The "utran-sai-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

4)	void

5)	if the access-type field is set to "3GPP2-1X", a ci-3gpp2 parameter set to the ASCII representation of the hexadecimal value of the string obtained by the concatenation of SID (16 bits), NID (16 bits), PZID (8 bits) and BASE_ID (16 bits) (see 3GPP2 C.S0005-D [85]) in the specified order. The length of the ci-3gpp2 parameter shall be 14 hexadecimal characters. The hexadecimal characters (A through F) shall be coded using the uppercase ASCII characters. If the UE does not know the values for any of the above parameters, the UE shall use the value of 0 for that parameter. For example, if the SID is unknown, the UE shall represent the SID as 0x0000;

NOTE 1:	The SID value is represented using 16 bits as supposed to 15 bits as specified in 3GPP2 C.S0005-D [85].

EXAMPLE:	If SID = 0x1234, NID = 0x5678, PZID = 0x12, BASE_ID = 0xFFFF, the ci-3gpp2 value is set to the string "1234567812FFFF".

6)	if the access-type field is set to "3GPP2-1X-HRPD", a ci-3gpp2 parameter set to the ASCII representation of the hexadecimal value of the string obtained by the concatenation of Sector ID (128 bits) and Subnet length (8 bits) (see 3GPP2 C.S0024-B [86]) and Carrier-ID, if available, (see 3GPP2 X.S0060 [86B])in the specified order. The length of the ci-3gpp2 parameter shall be 34 or 40 hexadecimal characters depending on whether the Carrier-ID is included. The hexadecimal characters (A through F) shall be coded using the uppercase ASCII characters;

EXAMPLE:	If the Sector ID = 0x12341234123412341234123412341234, Subnet length = 0x11, and the Carrier-ID=0x555444, the ci-3gpp2 value is set to the string "1234123412341234123412341234123411555444".

7)	if the access-type field is set to "3GPP2-UMB" 3GPP2 C.S0084-000 [86A], a ci-3gpp2 parameter is set to the ASCII representation of the hexadecimal value of the Sector ID (128 bits) defined in 3GPP2 C.S0084-000 [86A]. The length of the ci-3gpp2 parameter shall be 32 hexadecimal characters. The hexadecimal characters (A through F) shall be coded using the uppercase ASCII characters;

EXAMPLE:	If the Sector ID = 0x12341234123412341234123412341234, the ci-3gpp2 value is set to the string "12341234123412341234123412341234".

8)	if the access-type field set to one of "IEEE-802.11", "IEEE-802.11a", "IEEE-802.11b", "IEEE-802.11g", "IEEE-802.11n", or "IEEE-802.11ac", an "i-wlan-node-id" parameter is set to the ASCII representation of the hexadecimal value of the AP's MAC address without any delimiting characters;

NOTE 2:	The AP's MAC address is provided in the BSSID information element.

EXAMPLE:	If the AP's MAC address = 00-0C-F1-12-60-28, then i-wlan-node-id is set to the string "000cf1126028".

NOTE 3:	"i-wlan-node-id" parameter is not restricted to I-WLAN. "i-wlan-node-id" parameter can be inserted for a WLAN which is not an I-WLAN.

9)	if the access-type field is set to "3GPP2-1X-Femto", a ci-3gpp2-femto parameter set to the ASCII representation of the hexadecimal value of the string obtained by the concatenation of femto MSCID (24 bit), femto CellID (16 bit), FEID (64bit), macro MSCID (24 bits) and macro CellID (16 bits) (3GPP2 X.P0059-200 [86E]) in the specified order. The length of the ci-3gpp2-femto parameter is 36 hexadecimal characters. The hexadecimal characters (A through F) are coded using the uppercase ASCII characters.

10)	if the access-type field is set to one of "ADSL", "ADSL2", "ADSL2+", "RADSL", "SDSL", "HDSL", "HDSL2", "G.SHDSL", "VDSL", "IDSL", or "xDSL", the access-info field shall contain a dsl-location parameter obtained from the CLF (see NASS functional architecture);

11)	if the access-type field set to "DOCSIS", the access info parameter is not inserted. This release of this specification does not define values for use in this parameter;

12)	if the access-type field is equal to "3GPP-E-UTRAN-FDD" or "3GPP-E-UTRAN-TDD", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value) which should be obtained from the E-UTRAN Cell Global Identifier (ECGI), Tracking Area Code (4 hexadecimal digits when accessing to EPC and 6 hexadecimal digits when accessing to 5GCN) as described in 3GPP TS 23.003 [3] and the E-UTRAN Cell Identity (ECI) (7 hexadecimal digits) as described in 3GPP TS 23.003 [3]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

EXAMPLE:	If MCC is 111, MNC is 22, TAC is 33C4 and ECI is 76B4321, then P-Access-Network-Info header field looks like follows: P-Access-Network-Info: 3GPP-E-UTRAN-FDD;utran-cell-id-3gpp=1112233C476B4321;network-provided

NOTE 4:	The total length of the "utran-cell-id-3gpp" parameter depends on the various combinations of MNC and TAC possible sizes. The actual length of MNC and TAC parts can be unambiguously deduced from the total length.

NOTE 5:	The P-CSCF obtains the ECGI in the 3GPP-User-Location-Info AVP received from the PCRF, while the UE obtains the ECGI from RAN. In roaming scenarios with P-GW in the HPLMN, the MCC-MNC contained in the ECGI retrieved by the P-CSCF can differ from that contained in the ECGI retrieved by the UE. Using MNC and MCC from a different source than ECGI can lead to collision between cell-id values which makes the determination of the UE location not possible or incorrect and disables routing of emergency calls based on location information.

12A)	if the access-class field is equal to "3GPP-E-UTRAN", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value) which should be obtained from the E-UTRAN Cell Global Identifier (ECGI), Tracking Area Code (4 hexadecimal digits when accessing to EPC and 6 hexadecimal digits when accessing to 5GCN) as described in 3GPP TS 23.003 [3] and the E-UTRAN Cell Identity (ECI) (7 hexadecimal digits) as described in 3GPP TS 23.003 [3]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

12B)	if the access-type field is equal to "3GPP-E-UTRAN-ProSe-UNR", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value) which should be obtained from the E-UTRAN Cell Global Identifier (ECGI) and the E-UTRAN Cell Identity (ECI) (7 hexadecimal digits) as described in 3GPP TS 23.003 [3] obtained from the ProSe-UE-to-network relay that the UE is connected to as specified in 3GPP TS 24.334 [8ZD]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in in RFC 20 [212];

EXAMPLE:	If MCC is 111, MNC is 22 and ECI is 76B4321, then P-Access-Network-Info header field looks like follows: P-Access-Network-Info: 3GPP-E-UTRAN-ProSe-UNR;utran-cell-id-3gpp=1112276B4321.

12C)	if the access-type field is equal to "3GPP-E-UTRAN-FDD" or "3GPP-E-UTRAN-TDD", an "eps-fallback" header field parameter set to an appropriate value;

12D)	if the access-class field is equal to "3GPP-E-UTRAN", an "eps-fallback" header field parameter set to an appropriate value;

13)	if the access-type field is set to one of "IEEE-802.3", "IEEE-802.3a", "IEEE-802.3e", "IEEE-802.3i", "IEEE-802.3j", "IEEE-802.3u", "IEEE-802.3ab", "IEEE-802.3ae", IEEE-802.3ak", IEEE-802.3aq", IEEE-802.3an", "IEEE-802.3y" or "IEEE-802.3z" and NASS subsystem is used, the access-info field shall contain an eth-location parameter obtained from the CLF (see NASS functional architecture);

14)	if the access-type field is set to one of "GPON", "XGPON1" or "IEEE-802.3ah" and NASS is used, the access-info field shall contain an fiber-location parameter obtained from the CLF (see NASS functional architecture);

15)	if the access-type field is set to "GSTN", the access-info field may contain a gstn-location parameter if received from the GSTN;

NOTE 6:	The "cgi-3gpp", the "utran-cell-id-3gpp", the "ci-3gpp2", the "ci-3gpp2-femto", the "i-wlan-node-id", eth-location, and the "dsl-location" parameters described above among other usage also constitute the location identifiers that are used for emergency services.

16)	if the access-type field is set to "DVB-RCS2", the access-info field shall contain a "dvb-rcs2-node-id" parameter which consists of comma-separated list consisting of NCC_ID, satellite_ID, beam_ID, and SVN-MAC as specified in ETSI TS 101 545-2 [194], ETSI TS 101 545-3 [195]; the NCC_ID shall be represented as two digit hexadecimal value, the satellite_ID shall be represented as a two digit hexadecimal value, the beam_ID shall be respresented as a four digit hexadecimal value, and the SVN-MAC shall be represented as six digit hexadecimal value;

EXAMPLE:	If the (8 bit) NCC_ID = 0x3A, the (8 bit) satellite_ID = 0xF5, the (16 bit) beam_ID = 0xEA23, and the (24 bit) SVN-MAC = 0xE40AB9, then the "dvb-rcs2-node-id" is set to the string "3A,F5,EA23,E40AB9".

17)	the "local-time-zone" parameter in the access-info field is coded as a text string as follows:

UTC±[hh]:[mm]. [hh] is two digits, and [mm] is two digits from four values: "00", "15", "30" or "45", see ISO 8601 [203];

EXAMPLE:	"UTC+01:00" indicates that the time difference between local time and UTC of day is one hour.

18)	the "daylight-saving-time" parameter in the access-info field is coded as a text string as follows:

[hh]. [hh] is a two digits value from three values "00", "01" or "02" indicating the positive adjustment in hours;

19)	void;

20)	the operator-specific-GI in the access-info field is coded as a text string and conveys an operator-specifc geographical identifier;

21)	if

a)	the access-class field is set to "untrusted-non-3GPP-VIRTUAL-EPC"; or

b)	the access-class field is set to "3GPP-WLAN" and the WLAN is an untrusted WLAN;

then:

a)	if a UE local IP address is available, then a "UE-local-IP-address" parameter set to the UE local IP address;

b)	if the IKEv2 messages exchanged between the UE and the ePDG are encapsulated in the UDP messages according to IETF RFC 3948 [63A] and the UDP source port of the UDP messages received by ePDG is available, then a "UDP-source-port" parameter set to the UDP source port of the UDP messages:

-	received by the ePDG; and

-	encapsulating the IKEv2 messages;

c)	if the IKEv2 messages exchanged between the UE and the ePDG are transported using the firewall traversal tunnel as described in 3GPP TS 24.302 [8U] and the TCP source port of the TCP messages of the firewall traversal tunnel received by ePDG is available, then a "TCP-source-port" parameter set to the TCP source port of the TCP messages:

-	received by the ePDG; and

-	of the firewall traversal tunnel transporting the IKEv2 messages; and

d)	if an ePDG IP address used as IKEv2 tunnel endpoint with the UE is available, then an "ePDG-IP-address" parameter set to the ePDG IP address used as IKEv2 tunnel endpoint with the UE;

22)	if the access-type field is equal to "3GPP-NR-FDD" or "3GPP-NR-TDD", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), Tracking Area Code (6 hexadecimal digits) as described in 3GPP TS 23.003 [3], the NR Cell Identity (NCI) (9 hexadecimal digits) and optionally, the Network Identifier (NID) (11 hexadecimal digits) as specified in 3GPP TS 23.003 [3]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212]; and

NOTE 7:	NID is included only if a serving network is a Stand-alone Non-Public Network (SNPN) identified by a combination of NID, MCC and MNC. The serving network type can be unambiguously deduced from the total length of the "utran-cell-id-3gpp" parameter.

22A)	if the access-class field is equal to "3GPP-NR", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), Tracking Area Code (6 hexadecimal digits) as described in 3GPP TS 23.003 [3], the NR Cell Identity (NCI) (9 hexadecimal digits) and optionally, the NID (11 hexadecimal digits) as specified in 3GPP TS 23.003 [3]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212].

23)	if the access-type field is equal to "3GPP-NR-U-FDD" or "3GPP-NR-U-TDD", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), Tracking Area Code (6 hexadecimal digits) as described in 3GPP TS 23.003 [3], the NR Cell Identity (NCI) (9 hexadecimal digits) and optionally, the NID (11 hexadecimal digits) as specified in 3GPP TS 23.003 [3]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

23A)	if the access-class field is equal to "3GPP-NR-U", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), Tracking Area Code (6 hexadecimal digits) as described in 3GPP TS 23.003 [3], the NR Cell Identity (NCI) (9 hexadecimal digits) and optionally, the NID (11 hexadecimal digits) as specified in 3GPP TS 23.003 [3]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212 ];

24)	if the access-type field is equal to "3GPP-NR-SAT", "3GPP-NR-LEO", "3GPP-NR-MEO", "3GPP-NR-GEO", "3GPP-NR-OTHERSAT", "3GPP-WB-E-UTRAN-LEO", "3GPP-WB-E-UTRAN-MEO", "3GPP-WB-E-UTRAN-GEO", "3GPP-WB-E-UTRAN-OTHERSAT", "3GPP-NB-IoT-LEO", "3GPP-NB-IoT-MEO", "3GPP-NB-IoT-GEO", "3GPP-NB-IoT-OTHERSAT", "3GPP-LTE-M-LEO", "3GPP-LTE-M-MEO", "3GPP-LTE-M-GEO", or "3GPP-LTE-M-OTHERSAT", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), Tracking Area Code (6 hexadecimal digits) as described in 3GPP TS 23.003 [3], the NR Cell Identity (NCI) (9 hexadecimal digits) and optionally, the NID (11 hexadecimal digits) as specified in 3GPP TS 23.003 [3]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

24A)	if the access-class field is equal to "3GPP-NR-SAT", "3GPP-NR-LEO", "3GPP-NR-MEO", "3GPP-NR-GEO", "3GPP-NR-OTHERSAT", "3GPP-WB-E-UTRAN-LEO", "3GPP-WB-E-UTRAN-MEO", "3GPP-WB-E-UTRAN-GEO", "3GPP-WB-E-UTRAN-OTHERSAT", "3GPP-NB-IoT-LEO", "3GPP-NB-IoT-MEO", "3GPP-NB-IoT-GEO", "3GPP-NB-IoT-OTHERSAT", "3GPP-LTE-M-LEO", "3GPP-LTE-M-MEO", "3GPP-LTE-M-GEO", or "3GPP-LTE-M-OTHERSAT", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), Tracking Area Code (6 hexadecimal digits) as described in 3GPP TS 23.003 [3], the NR Cell Identity (NCI) (9 hexadecimal digits) and optionally, the NID (11 hexadecimal digits) as specified in 3GPP TS 23.003 [3]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

25)	 if the access-type field is equal to "3GPP-NR-ProSe-L2UNR" or "3GPP-NR-ProSe-L3UNR", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), Tracking Area Code (6 hexadecimal digits) as described in 3GPP TS 23.003 [3], and the NR Cell Identity (NCI) (9 hexadecimal digits) obtained from the 5G ProSe UE-to-network relay UE that the UE is connected to as specified in 3GPP TS 24.554 [8ZI]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

26)	if the access-class field is equal to "3GPP-NR-REDCAP", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), Tracking Area Code (6 hexadecimal digits) as described in 3GPP TS 23.003 [3], the NR Cell Identity (NCI) (9 hexadecimal digits) and optionally, the NID (11 hexadecimal digits) as specified in 3GPP TS 23.003 [3]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212];

26A)	if the access-type field is equal to "3GPP-NR-REDCAP", a "utran-cell-id-3gpp" parameter set to a concatenation of the MCC (3 decimal digits), MNC (2 or 3 decimal digits depending on MCC value), Tracking Area Code (6 hexadecimal digits) as described in 3GPP TS 23.003 [3], the NR Cell Identity (NCI) (9 hexadecimal digits) and optionally, the NID (11 hexadecimal digits) as specified in 3GPP TS 23.003 [3]. The "utran-cell-id-3gpp" parameter is encoded in ASCII as defined in RFC 20 [212]; and

27)	if the access-type field of the P-Access-Network-Info header field not containing "network-provided" parameter is equal to "3GPP-NR-ProSe-L3UNR" and the access-class field of the P-Access-Network-Info header field containing "network-provided" parameter is equal to "3GPP-NR", then an "U2N-relay-ID" parameter of the P-Access-Network-Info header field containing the "network-provided" parameter set to the IMSI of 5G ProSe UE-to-network relay UE.

28)	the satellite-id field set to a binary coded integer value from 0 to 255 as defined for the SatelliteId IE in 3GPP TS 36.331 [19F] to indicate the satellite identifier of the P-CSCF.

* * * Next Change * * * *

7.7	SIP timers

The timers T1, T2, T4 A, B, C, D, E, F, G, H and I (defined in RFC 3261 [26]), timers L and M (defined in RFC 6026 [163]), and timer N (defined in RFC 6665 [28]) need modification in some cases to accommodate the delays introduced by the air interface processing and transmission delays. Table 7.7.1 shows recommended values for IM CN subsystem.

Table 7.7.1 lists in the first column, titled "SIP Timer" the timer names as defined in RFC 3261 [26] and RFC 6026 [163].

The second column, titled "value to be applied between IM CN subsystem elements" lists the values recommended for network elements e.g. P-CSCF, S-CSCF, MGCF, when communicating with each other i.e. when no air interface leg is included. These values are identical to those recommended by RFC 3261 [26], RFC 6026 [163], and RFC 6665 [28].

The third column, titled "value to be applied at the UE" lists the values recommended for the UE, when in normal operation the UE generates requests or responses containing a P-Access-Network-Info header field which included a value of "3GPP-GERAN","3GPP-UTRAN-FDD", "3GPP-UTRAN-TDD", "3GPP-E-UTRAN-FDD", "3GPP-E-UTRAN-TDD", "3GPP-E-UTRAN-ProSe-UNR", "3GPP-NR-FDD", "3GPP-NR-TDD", "3GPP-NR-U-FDD", "3GPP-NR-U-TDD", "3GPP-NR-SAT", "3GPP-NR-LEO", "3GPP-NR-MEO", "3GPP-NR-GEO", "3GPP-NR-OTHERSAT", "3GPP-WB-E-UTRAN-LEO", "3GPP-WB-E-UTRAN-MEO", "3GPP-WB-E-UTRAN-GEO", "3GPP-WB-E-UTRAN-OTHERSAT", "3GPP-NB-IoT-LEO", "3GPP-NB-IoT-MEO", "3GPP-NB-IoT-GEO", "3GPP-NB-IoT-OTHERSAT", "3GPP-LTE-M-LEO", "3GPP-LTE-M-MEO", "3GPP-LTE-M-GEO", "3GPP-LTE-M-OTHERSAT", "3GPP-NR-ProSe-L2UNR", "3GPP-NR-ProSe-L3UNR", "3GPP-NR-REDCAP", "3GPP2-1X", "3GPP2-1X-HRPD", "3GPP2-UMB", "IEEE-802.11", "IEEE-802.11a", "IEEE-802.11b", "IEEE-802.11g", "IEEE-802.11n", "IEEE-802.11ac", or "DVB-RCS2". These are modified when compared to RFC 3261 [26] and RFC 6026 [163] to accommodate the air interface delays. In all other cases, the UE should use the values specified in RFC 3261 [26] or RFC 6026 [163] as indicated in the second column of table 7.7.1.

The fourth column, titled "value to be applied at the P-CSCF toward a UE" lists the values recommended for the P-CSCF when an air interface leg is traversed, and which are used on all SIP transactions on a specific security association where the security association was established using a REGISTER request containing a P-Access-Network-Info header field provided by the UE which included a value of "3GPP-GERAN","3GPP-UTRAN-FDD", "3GPP-UTRAN-TDD", "3GPP-E-UTRAN-FDD", "3GPP-E-UTRAN-TDD", "3GPP-E-UTRAN-ProSe-UNR", "3GPP-NR-FDD", "3GPP-NR-TDD", "3GPP-NR-U-FDD", "3GPP-NR-U-TDD", "3GPP-NR-SAT", "3GPP-NR-LEO", "3GPP-NR-MEO", "3GPP-NR-GEO", "3GPP-NR-OTHERSAT", "3GPP-WB-E-UTRAN-LEO", "3GPP-WB-E-UTRAN-MEO, "3GPP-WB-E-UTRAN-GEO", "3GPP-WB-E-UTRAN-OTHERSAT", "3GPP-NB-IoT-LEO", "3GPP-NB-IoT-MEO", "3GPP-NB-IoT-GEO", "3GPP-NB-IoT-OTHERSAT", "3GPP-LTE-M-LEO", "3GPP-LTE-M-MEO", "3GPP-LTE-M-GEO", "3GPP-LTE-M-OTHERSAT", "3GPP-NR-ProSe-L2UNR", "3GPP-NR-ProSe-L3UNR", "3GPP-NR-REDCAP", "3GPP2-1X", "3GPP2-1X-HRPD", "3GPP2-UMB", "IEEE-802.11", "IEEE-802.11a", "IEEE-802.11b", "IEEE-802.11g", "IEEE-802.11n", "IEEE-802.11ac", or "DVB-RCS2". These are modified when compared to RFC 3261 [26] and RFC 6026 [163]. In all other cases, the P-CSCF should use the values specified in RFC 3261 [26] and RFC 6026 [163] as indicated in the second column of table 7.7.1.

The final column reflects the timer meaning as defined in RFC 3261 [26], RFC 6026 [163] or RFC 6665 [28].

Table 7.7.1: SIP timers

| SIP Timer | Value to be applied between IM CN subsystem elements | Value to be applied at the UE | Value to be applied at the P-CSCF toward a UE | Meaning |
| --- | --- | --- | --- | --- |
| T1 | 500ms default (see NOTE) | 2s default | 2s default | RTT estimate |
| T2 | 4s (see NOTE) | 16s | 16s | The maximum retransmit interval for non-INVITE requests and INVITE responses |
| T4 | 5s (see NOTE) | 17s | 17s | Maximum duration a message will remain in the network |
| Timer A | initially T1 | initially T1 | initially T1 | INVITE request retransmit interval, for UDP only |
| Timer B | 64*T1 | 64*T1 | 64*T1 | INVITE transaction timeout timer |
| Timer C | > 3min | > 3 min | > 3 min | proxy INVITE transaction timeout |
| Timer D | > 32s for UDP | >128s | >128s | Wait time for response retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer E | initially T1 | initially T1 | initially T1 | non-INVITE request retransmit interval, UDP only |
| Timer F | 64*T1 | 64*T1 | 64*T1 | non-INVITE transaction timeout timer |
| Timer G | initially T1 | initially T1 | initially T1 | INVITE response retransmit interval |
| Timer H | 64*T1 | 64*T1 | 64*T1 | Wait time for ACK receipt. |
| Timer I | T4 for UDP | T4 for UDP | T4 for UDP | Wait time for ACK retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer J | 64*T1 for UDP | 64*T1 for UDP | 64*T1 for UDP | Wait time for non-INVITE request retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer K | T4 for UDP | T4 for UDP | T4 for UDP | Wait time for response retransmits |
|  | 0s for TCP/SCTP | 0s for TCP/SCTP | 0s for TCP/SCTP |  |
| Timer L | 64*T1 | 64*T1 | 64*T1 | Wait time for accepted INVITE request retransmits |
| Timer M | 64*T1 | 64*T1 | 64*T1 | Wait time for retransmission of 2xx to INVITE or additional 2xx from other branches of a forked INVITE |
| Timer N | 64*T1 | 64*T1 | 64*T1 | Wait time for receipt of a NOTIFY request upon sending SUBSCRIBE |
| NOTE: As a network option, SIP T1 Timer's value can be extended, along with the necessary modifications of T2 and T4 Timers' values, to take into account the specificities of the supported services when the MRFC and the controlling AS are under the control of the same operator and the controlling AS knows, based on local configuration, that the MRFC implements a longer value of SIP T1 Timer. |  |  |  |  |

* * * Next Change * * * *

8.1.1	SIP compression

If in normal operation the UE generates requests or responses containing a P-Access-Network-Info header field which included a value of "3GPP-GERAN","3GPP-UTRAN-FDD", "3GPP-UTRAN-TDD", "3GPP-E-UTRAN-FDD", "3GPP-E-UTRAN-TDD", "3GPP-E-UTRAN-ProSe-UNR", "3GPP-NR-FDD", "3GPP-NR-TDD", "3GPP-NR-U-FDD", "3GPP-NR-U-TDD", "3GPP-NR-SAT", "3GPP-NR-LEO", "3GPP-NR-MEO", "3GPP-NR-GEO", "3GPP-NR-OTHERSAT", "3GPP-WB-E-UTRAN-LEO", "3GPP-WB-E-UTRAN-MEO, "3GPP-WB-E-UTRAN-GEO", "3GPP-WB-E-UTRAN-OTHERSAT", "3GPP-NB-IoT-LEO", "3GPP-NB-IoT-MEO", "3GPP-NB-IoT-GEO", "3GPP-NB-IoT-OTHERSAT", "3GPP-LTE-M-LEO", "3GPP-LTE-M-MEO", "3GPP-LTE-M-GEO", "3GPP-LTE-M-OTHERSAT", "3GPP-NR-ProSe-L2UNR", "3GPP-NR-ProSe-L3UNR","3GPP-NR-REDCAP", "3GPP2-1X", "3GPP2-1X-HRPD", "3GPP2-UMB", "IEEE-802.11", "IEEE-802.11a", "IEEE-802.11b", "IEEE-802.11g", "IEEE-802.11n", "IEEE-802.11ac", or "DVB-RCS2", then the UE shall support:

-	SigComp as specified in RFC 3320 [32] and as updated by RFC 4896 [118]; and

-	the additional requirements specified in RFC 5049 [79], with the exception that the UE shall take a State Memory Size of at least 4096 bytes as a minimum value.

If in normal operation the UE generates requests or responses containing a P-Access-Network-Info header field which included a value of "3GPP-GERAN","3GPP-UTRAN-FDD", "3GPP-UTRAN-TDD", "3GPP-E-UTRAN-FDD", "3GPP-E-UTRAN-TDD", "3GPP-E-UTRAN-ProSe-UNR", "3GPP-NR-FDD", "3GPP-NR-TDD", "3GPP-NR-U-FDD", "3GPP-NR-U-TDD", "3GPP-NR-SAT", "3GPP-NR-LEO", "3GPP-NR-MEO", "3GPP-NR-GEO", "3GPP-NR-OTHERSAT", "3GPP-WB-E-UTRAN-LEO", "3GPP-WB-E-UTRAN-MEO", "3GPP-WB-E-UTRAN-GEO", "3GPP-WB-E-UTRAN-OTHERSAT", "3GPP-NB-IoT-LEO", "3GPP-NB-IoT-MEO", "3GPP-NB-IoT-GEO", "3GPP-NB-IoT-OTHERSAT", "3GPP-LTE-M-LEO", "3GPP-LTE-M-MEO", "3GPP-LTE-M-GEO", "3GPP-LTE-M-OTHERSAT", "3GPP-NR-ProSe-L2UNR", "3GPP-NR-ProSe-L3UNR","3GPP-NR-REDCAP", "3GPP2-1X", "3GPP2-1X-HRPD", "3GPP2-UMB", "IEEE-802.11", "IEEE-802.11a", "IEEE-802.11b", "IEEE-802.11g", "IEEE-802.11n", "IEEE-802.11ac", or "DVB-RCS2", then the UE may support:

-	the negative acknowledgement mechanism specified in RFC 4077 [65A].

When using SigComp the UE shall send compressed SIP messages in accordance with RFC 3486 [55]. When the UE will create the compartment is implementation specific, but the compartment shall not be created until a set of security associations or a TLS session is set up if signalling security is in use. The UE shall finish the compartment when the UE is deregistered. The UE shall alow state creations and announcements only for messages received in a security association.

NOTE:	Exchange of bytecodes during registration will prevent unnecessary delays during session setup.

If the UE supports SigComp:

-	the UE shall support the SIP dictionary specified in RFC 3485 [42] and as updated by RFC 4896 [118]. If compression is enabled, the UE shall use the dictionary to compress the first message; and

-	if the UE supports the presence user agent or watcher roles as specified in table A.3A/2 and table A.3A/4, the UE may support the presence specific dictionary specified in RFC 5112 [119].

The use of SigComp is not re-negotiated between initial registration and deregistration.

* * * Next Change * * * *

8.1.2	Compression of SIP requests and responses transmitted to the P-CSCF

In normal operation the UE should send the generated requests and responses transmitted to the P-CSCF:

-	compressed according to subclause 8.1.1, if the P-Access-Network-Info header field of the initial registration message includes a value of "3GPP-GERAN","3GPP-UTRAN-FDD", "3GPP-UTRAN-TDD",  "3GPP2-1X", "3GPP2-1X-HRPD", "3GPP2-UMB", "IEEE-802.11", "IEEE-802.11a", "IEEE-802.11b", IEEE-802.11g", "IEEE-802.11n", "IEEE-802.11ac", or "DVB-RCS2";

-	uncompressed, if the P-Access-Network-Info header field of the initial registration message includes a value of "3GPP-E-UTRAN-FDD", "3GPP-E-UTRAN-TDD", "3GPP-E-UTRAN-ProSe-UNR", "3GPP-NR-FDD", "3GPP-NR-TDD","3GPP-NR-U-FDD", "3GPP-NR-U-TDD", "3GPP-NR-SAT", "3GPP-NR-LEO", "3GPP-NR-MEO", "3GPP-NR-GEO", "3GPP-NR-OTHERSAT", "3GPP-NR-ProSe-L2UNR", "3GPP-NR-ProSe-L3UNR", "3GPP-NR-REDCAP".

In other cases where SigComp is supported, the UE need not compress the requests and responses.

NOTE 1:	Compression of SIP messages is an implementation option. However, compression is strongly recommended.

NOTE 2:	In an IP-CAN where compression support is mandatory the UE can send even the first message compressed. Sigcomp provides mechanisms to allow the UE to know if state has been created in the P-CSCF or not.

* * * Next Change * * * *

8.2.2	Compression of SIP requests and responses transmitted to the UE

For all SIP transactions on a specific security association where the security association was established using a REGISTER request from the UE containing a P-Access-Network-Info header field which included a value of "3GPP-GERAN","3GPP-UTRAN-FDD", "3GPP-UTRAN-TDD", "3GPP-E-UTRAN-ProSe-UNR", "3GPP-NR-FDD", "3GPP-NR-TDD","3GPP-NR-U-FDD", "3GPP-NR-U-TDD", "3GPP-NR-SAT", "3GPP-NR-LEO", "3GPP-NR-MEO", "3GPP-NR-GEO", "3GPP-NR-OTHERSAT", "3GPP-WB-E-UTRAN-LEO", "3GPP-WB-E-UTRAN-MEO", "3GPP-WB-E-UTRAN-GEO", "3GPP-WB-E-UTRAN-OTHERSAT", "3GPP-NB-IoT-LEO", "3GPP-NB-IoT-MEO", "3GPP-NB-IoT-GEO", "3GPP-NB-IoT-OTHERSAT", "3GPP-LTE-M-LEO", "3GPP-LTE-M-MEO", "3GPP-LTE-M-GEO", "3GPP-LTE-M-OTHERSAT", "3GPP-NR-ProSe-L2UNR", "3GPP-NR-ProSe-L3UNR", "3GPP-E-UTRAN-FDD", "3GPP-E-UTRAN-TDD", "3GPP2-1X", "3GPP2-1X-HRPD", "3GPP2-UMB", "IEEE-802.11", "IEEE-802.11a", "IEEE-802.11b", "IEEE-802.11g", "IEEE-802.11n", "IEEE-802.11ac", or "DVB-RCS2", and the UE has indicated that it supports SigComp and is willing to receive compressed messages in accordance with RFC 3486 [55], then the P-CSCF should compress the requests and responses transmitted to the UE according to subclause 8.2.1. In other cases where SigComp is supported, it need not.

NOTE:	Compression of SIP messages is an implementation option. However, compression is strongly recommended.

* * * Next Change * * * *

A.1.3	Roles

Table A.2: Roles

| Item | Roles | Reference | RFC status | Profile status |
| --- | --- | --- | --- | --- |
| 1 | User agent | [26] | o.1 | o.1 |
| 2 | Proxy | [26] | o.1 | o.1 |
| o.1: It is mandatory to support exactly one of these items. |  |  |  |  |
| NOTE: For the purposes of the present document it has been chosen to keep the specification simple by the tables specifying only one role at a time. This does not preclude implementations providing two roles, but an entirely separate assessment of the tables shall be made for each role. |  |  |  |  |

Table A.3: Roles specific to this profile

| Item | Roles | Reference | RFC status | Profile status |
| --- | --- | --- | --- | --- |
| 1 | UE | 5.1 | n/a | o.1 |
| 1A | UE containing UICC | 5.1 | n/a | c5 |
| 1B | UE without UICC | 5.1 | n/a | c5 |
| 2 | P-CSCF | 5.2 | n/a | o.1 |
| 2A | P-CSCF (IMS-ALG) | [7] | n/a | c6 |
| 3 | I-CSCF | 5.3 | n/a | o.1 |
| 3A | void |  |  |  |
| 4 | S-CSCF | 5.4 | n/a | o.1 |
| 5 | BGCF | 5.6 | n/a | o.1 |
| 6 | MGCF | 5.5 | n/a | o.1 |
| 7 | AS | 5.7 | n/a | o.1 |
| 7A | AS acting as terminating UA, or redirect server | 5.7.2 | n/a | c2 |
| 7B | AS acting as originating UA | 5.7.3 | n/a | c2 |
| 7C | AS acting as a SIP proxy | 5.7.4 | n/a | c2 |
| 7D | AS performing 3rd party call control | 5.7.5 | n/a | c2 |
| 8 | MRFC | 5.8 | n/a | o.1 |
| 8A | MRB | 5.8A | n/a | o.1 |
| 9 | IBCF | 5.10 | n/a | o.1 |
| 9A | IBCF (THIG) | 5.10.4 | n/a | c4 |
| 9B | IBCF (IMS-ALG) | 5.10.5, 5.10.7 | n/a | c4 |
| 9C | IBCF (Screening of SIP signalling) | 5.10.6 | n/a | c4 |
| 9D | IBCF (Privacy protection) | 5.10.8 | n/a | c4 |
| 10 | Additional routeing functionality | Annex I | n/a | c3 |
| 11 | E-CSCF | 5.11 | n/a | o.1 |
| 11A | E-CSCF acting as UA | 5.11.1, 5.11.2, 5.11.3 | n/a | c7 |
| 11B | E-CSCF acting as a SIP Proxy | 5.11.1, 5.11.2 | n/a | c7 |
| 12 | LRF | 5.12 | n/a | o.1 |
| 13 | ISC gateway function | 5.13 | n/a | o.1 |
| 13A | ISC gateway function (THIG) | 5.13.4 | n/a | c8 |
| 13B | ISC gateway function (IMS-ALG) | 5.13.5 | n/a | c8 |
| 13C | ISC gateway function (Screening of SIP signalling) | 5.13.6 | n/a | c8 |
| 14 | Gm based WIC | [8Z] | n/a | o.1 |
| 15 | Transit function | I.3 | n/a | c9 |
| c2: IF A.3/7 THEN o.2 ELSE n/a - - AS. c3: IF A.3/3 OR A.3/4 OR A.3/5 OR A.3/6 OR A.3/9 THEN o ELSE o.1 - - I-CSCF, S-CSCF, BGCF, MGCF, IBCF. c4: IF A.3/9 THEN o.3 ELSE n/a - - IBCF. c5: IF A.3/1 THEN o.4 ELSE n/a - - UE. c6: IF A.3/2 THEN o ELSE n/a - - P-CSCF. c7: IF A.3/11 THEN o.5 ELSE n/a - - E-CSCF. c8: IF A.3/13 THEN o ELSE n/a - - ISC gateway function. c9 IF A.3/3 OR A.3/4 OR A.3/5 OR A.3/6 OR A.3/9 THEN o ELSE o.1 - - I-CSCF, S-CSCF, BGCF, MGCF, IBCF. o.1: It is mandatory to support exactly one of these items. o.2: It is mandatory to support at least one of these items. o.3: It is mandatory to support at least one of these items. o.4 It is mandatory to support exactly one of these items. o.5: It is mandatory to support exactly one of these items. |  |  |  |  |
| NOTE: For the purposes of the present document it has been chosen to keep the specification simple by the tables specifying only one role at a time. This does not preclude implementations providing two roles, but an entirely separate assessment of the tables shall be made for each role. |  |  |  |  |

Table A.3A: Roles specific to additional capabilities

| Item | Roles | Reference | RFC status | Profile status |
| --- | --- | --- | --- | --- |
| 1 | Presence server | 3GPP TS 24.141 [8A] | n/a | c1 |
| 2 | Presence user agent | 3GPP TS 24.141 [8A] | n/a | c2 |
| 3 | Resource list server | 3GPP TS 24.141 [8A] | n/a | c3 |
| 4 | Watcher | 3GPP TS 24.141 [8A] | n/a | c4 |
| 11 | Conference focus | 3GPP TS 24.147 [8B] | n/a | c11 |
| 12 | Conference participant | 3GPP TS 24.147 [8B] | n/a | c6 |
| 21 | CSI user agent | 3GPP TS 24.279 [8E] | n/a | c7 |
| 22 | CSI application server | 3GPP TS 24.279 [8E] | n/a | c8 |
| 31 | Messaging application server | 3GPP TS 24.247 [8F] | n/a | c5 |
| 32 | Messaging list server | 3GPP TS 24.247 [8F] | n/a | c5 |
| 33 | Messaging participant | 3GPP TS 24.247 [8F] | n/a | c2 |
| 33A | Page-mode messaging participant | 3GPP TS 24.247 [8F] | n/a | c2 |
| 33B | Session-mode messaging participant | 3GPP TS 24.247 [8F] | n/a | c2 |
| 34 | Session-mode messaging intermediate node | 3GPP TS 24.247 [8F] | n/a | c5 |
| 50 | Multimedia telephony service participant | 3GPP TS 24.173 [8H] | n/a | c2 |
| 50A | Multimedia telephony service application server | 3GPP TS 24.173 [8H] | n/a | c9 |
| 51 | Message waiting indication subscriber UA | 3GPP TS 24.606 [8I] | n/a | c2 |
| 52 | Message waiting indication notifier UA | 3GPP TS 24.606 [8I] | n/a | c3 |
| 53 | Advice of charge application server | 3GPP TS 24.647 [8N] | n/a | c8 |
| 54 | Advice of charge UA client | 3GPP TS 24.647 [8N] | n/a | c2 |
| 55 | Ut reference point XCAP server for supplementary services | 3GPP TS 24.623 [8P] | n/a | c3 |
| 56 | Ut reference point XCAP client for supplementary services | 3GPP TS 24.623 [8P] | n/a | c2 |
| 57 | Customized alerting tones application server | 3GPP TS 24.182 [8Q] | n/a | c8 |
| 58 | Customized alerting tones UA client | 3GPP TS 24.182 [8Q] | n/a | c2 |
| 59 | Customized ringing signal application server | 3GPP TS 24.183 [8R] | n/a | c8 |
| 60 | Customized ringing signal UA client | 3GPP TS 24.183 [8R] | n/a | c2 |
| 61 | SM-over-IP sender | 3GPP TS 24.341 [8L] | n/a | c2 |
| 62 | SM-over-IP receiver | 3GPP TS 24.341 [8L] | n/a | c2 |
| 63 | IP-SM-GW | 3GPP TS 24.341 [8L] | n/a | c1 |
| 71 | IP-SM-GW | 3GPP TS 29.311 [15A] | n/a | c10 |
| 81 | MSC Server enhanced for ICS | 3GPP TS 24.292 [8O] | n/a | c12 |
| 81A | MSC server enhanced for SRVCC using SIP interface | 3GPP TS 24.237 [8M] | n/a | c12 |
| 81B | MSC server enhanced for DRVCC using SIP interface | 3GPP TS 24.237 [8M] | n/a | c12 |
| 82 | ICS user agent | 3GPP TS 24.292 [8O] | n/a | c2 |
| 83 | SCC application server | 3GPP TS 24.292 [8O] | n/a | c9 |
| 84 | EATF | 3GPP TS 24.237 [8M] | n/a | c12 |
| 85 | In-dialog overlap signalling application server | Annex N.2, Annex N.3.3 | n/a | c9 |
| 86 | In-dialog overlap signalling UA client | Annex N.2, Annex N.3.3 | n/a | c2 |
| 87 | Session continuity controller UE | 3GPP TS 24.337 [8ZC] | n/a | c2 |
| 88 | ATCF (proxy) | 3GPP TS 24.237 [8M] | n/a | c13 (note 4) |
| 89 | ATCF (UA) | 3GPP TS 24.237 [8M] | n/a | c12 (note 4) |
| 91 | Malicious communication identification application server | 3GPP TS 24.616 [8S] | n/a | c9 |
| 92 | USSI UE | 3GPP TS 24.390 [8W] | n/a | c2 |
| 92A | USSI UE supporting user-initiated USSD operations | 3GPP TS 24.390 [8W] | n/a | c17 |
| 92B | USSI UE supporting network-initiated USSD operations | 3GPP TS 24.390 [8W] | n/a | c17 |
| 93 | USSI AS | 3GPP TS 24.390 [8W] | n/a | c3 |
| 93A | USSI AS supporting user-initiated USSD operations | 3GPP TS 24.390 [8W] | n/a | c18 |
| 93B | USSI AS supporting network-initiated USSD operations | 3GPP TS 24.390 [8W] | n/a | c18 |
| 94 | TP UE | 3GPP TS 24.103 [7G] | n/a | c14 |
| 95 | eP-CSCF (P-CSCF enhanced for WebRTC) | 3GPP TS 24.371 [8Z] | n/a | c15 |
| 101 | Business trunking in static mode of operation application server | 3GPP TS 24.525 [8ZA] | n/a | c16 |
| 102 | MCPTT client | 3GPP TS 24.379 [8ZE] | n/a | c19 |
| 103 | MCPTT server | 3GPP TS 24.379 [8ZE] | n/a | c20 |
| c1: IF A.3/7A AND A.3/7B THEN o ELSE n/a - - AS acting as terminating UA, or redirect server and AS acting as originating UA. c2: IF A.3/1 THEN o ELSE n/a - - UE. c3: IF A.3/7A THEN o ELSE n/a - - AS acting as terminating UA, or redirect server. c4: IF A.3/1 OR A.3/7B THEN o ELSE n/a - - UE or AS acting as originating UA. c5: IF A.3/7D AND A.3/8 THEN o ELSE n/a - - AS performing 3rd party call control and MRFC (note 2). c6: IF A.3/1 OR A.3A/11 THEN o ELSE n/a - - UE or conference focus. c7: IF A.3/1 THEN o ELSE n/a - - UE. c8: IF A.3/7D THEN o ELSE n/a - - AS performing 3rd party call control. c9: IF A.3/7A OR A.3/7B OR A.3/7C OR A.3/7D THEN o ELSE n/a - - AS acting as terminating UA, or redirect server, AS acting as originating UA, AS acting as a SIP proxy, AS performing 3rd party call control. c10: IF A.3/7A OR A.3/7B OR A.3/7D THEN o ELSE n/a - - AS acting as terminating UA, or redirect server, AS acting as originating UA, AS performing 3rd party call control. c11: IF A.3/7D THEN o ELSE n/a - - AS performing 3rd party call control. c12: IF A.2/1 THEN o ELSE n/a - - UA. c13: IF A.2/2 THEN o ELSE n/a - - proxy. c14: IF A.3/1 OR A.3A/11 THEN o ELSE n/a - - UE or conference focus. c15: IF A.3/2A THEN o ELSE n/a - - P-CSCF (IMS-ALG). c16: IF A.3/7A OR A.3/7B THEN o ELSE n/a - - AS acting as terminating UA, or redirect server, AS acting as originating UA. c17: IF A.3A/92 THEN o.1 ELSE n/a - - USSI UE. c18: IF A.3A/93 THEN o.2 ELSE n/a - - USSI AS. c19: IF A.3/1 THEN o ELSE n/a - - UE. c20: IF A.3/7 THEN o ELSE n/a - - AS. o.1: It is mandatory to support at least one of these items. o.2: It is mandatory to support at least one of these items. |  |  |  |  |
| NOTE 1: For the purposes of the present document it has been chosen to keep the specification simple by the tables specifying only one role at a time. This does not preclude implementations providing two roles, but an entirely separate assessment of the tables shall be made for each role. NOTE 2: The functional split between the MRFC and the AS for page-mode messaging is out of scope of this document and they are assumed to be collocated. NOTE 3: A.3A/63 is an AS providing the IP-SM-GW role to support the transport level interworking defined in 3GPP TS 24.341 [8L]. A.3A/71 is an AS providing the IP-SM-GW role to support the service level interworking for messaging as defined in 3GPP TS 29.311 [15A]. NOTE 4: An ATCF shall support both the ATCF (proxy) role and the ATCF (UA) role. |  |  |  |  |

Table A.3B: Roles with respect to access technology

| Item | Value used in P-Access-Network-Info header field |  | Reference |  | RFC status |  | Profile status |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3GPP-GERAN |  | [52] 4.4 |  | o |  | c1 |  |  |
| 2 | 3GPP-UTRAN-FDD |  | [52] 4.4 |  | o |  | c1 |  |  |
| 3 | 3GPP-UTRAN-TDD |  | [52] 4.4 |  | o |  | c1 |  |  |
| 4 | 3GPP2-1X |  | [52] 4.4 |  | o |  | c1 |  |  |
| 5 | 3GPP2-1X-HRPD |  | [52] 4.4 |  | o |  | c1 |  |  |
| 6 | 3GPP2-UMB |  | [52] 4.4 |  | o |  | c1 |  |  |
| 7 | 3GPP-E-UTRAN-FDD |  | [52] 4.4 |  | o |  | c1 |  |  |
| 8 | 3GPP-E-UTRAN-TDD |  | [52] 4.4 |  | o |  | c1 |  |  |
| 8A | 3GPP-E-UTRAN-ProSe-UNR |  | subclause 7.2A.4 |  | n/a |  | c1 |  |  |
| 8B | 3GPP-NR-FDD |  | subclause 7.2A.4 |  | n/a |  | c1 |  |  |
| 8C | 3GPP-NR-TDD |  | subclause 7.2A.4 |  | n/a |  | c1 |  |  |
| 8D | 3GPP-NR-U-FDD |  | subclause 7.2A.4 |  | n/a |  | c1 |  |  |
| 8E | 3GPP-NR-U-TDD |  | subclause 7.2A.4 |  | n/a |  | c1 |  |  |
| 8F | 3GPP-NR-ProSe-L2UNR |  | subclause 7.2A.4 |  | n/a |  | c1 |  |  |
| 8G | 3GPP-NR-ProSe-L3UNR |  | subclause 7.2A.4 |  | n/a |  | c1 |  |  |
| 8W | 3GPP-NR-SAT |  | subclause 7.2A.4 |  | n/a |  | c1 |  |  |
| 8WA |  | 3GPP-NR-LEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WB |  | 3GPP-NR-MEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WC |  | 3GPP-NR-GEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WD |  | 3GPP-NR-OTHERSAT |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WE |  | 3GPP-WB-E-UTRAN-LEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WF |  | 3GPP-WB-E-UTRAN-MEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WG |  | 3GPP-WB-E-UTRAN-GEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WH |  | 3GPP-WB-E-UTRAN-OTHERSAT |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WI |  | 3GPP-NB-IoT-LEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WJ |  | 3GPP-NB-IoT-MEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WK |  | 3GPP-NB-IoT-GEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WL |  | 3GPP-NB-IoT-OTHERSAT |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WM |  | 3GPP-LTE-M-LEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WN |  | 3GPP-LTE-M-MEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WO |  | 3GPP-LTE-M-GEO |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8WP |  | 3GPP-LTE-M-OTHERSAT |  | subclause 7.2A.4 |  | n/a |  | c1 |
| 8X | 3GPP-NR-REDCAP |  | subclause 7.2A.4 |  | n/a |  | c1 |  |  |
| 9 | 3GPP2-1X-Femto |  | [52] 4.4 |  | o |  | c1 |  |  |
| 11 | IEEE-802.11 |  | [52] 4.4 |  | o |  | c1 |  |  |
| 12 | IEEE-802.11a |  | [52] 4.4 |  | o |  | c1 |  |  |
| 13 | IEEE-802.11b |  | [52] 4.4 |  | o |  | c1 |  |  |
| 14 | IEEE-802.11g |  | [52] 4.4 |  | o |  | c1 |  |  |
| 15 | IEEE-802.11n |  | [52] 4.4 |  | o |  | c1 |  |  |
| 16 | IEEE-802.11ac |  | [52] 4.4 |  | o |  | c1 |  |  |
| 21 | ADSL |  | [52] 4.4 |  | o |  | c1 |  |  |
| 22 | ADSL2 |  | [52] 4.4 |  | o |  | c1 |  |  |
| 23 | ADSL2+ |  | [52] 4.4 |  | o |  | c1 |  |  |
| 24 | RADSL |  | [52] 4.4 |  | o |  | c1 |  |  |
| 25 | SDSL |  | [52] 4.4 |  | o |  | c1 |  |  |
| 26 | HDSL |  | [52] 4.4 |  | o |  | c1 |  |  |
| 27 | HDSL2 |  | [52] 4.4 |  | o |  | c1 |  |  |
| 28 | G.SHDSL |  | [52] 4.4 |  | o |  | c1 |  |  |
| 29 | VDSL |  | [52] 4.4 |  | o |  | c1 |  |  |
| 30 | IDSL |  | [52] 4.4 |  | o |  | c1 |  |  |
| 31 | xDSL |  | subclause 7.2A.4 |  | o |  | c1 |  |  |
| 41 | DOCSIS |  | [52] 4.4 |  | o |  | c1 |  |  |
| 51 | DVB-RCS2 |  | [52] 4.4 |  | o |  | c1 |  |  |
| 52 | 3GPP-UTRAN |  | [52] 4.4 |  | o |  | c2 |  |  |
| 53 | 3GPP-E-UTRAN |  | [52] 4.4 |  | o |  | c2 |  |  |
| 54 | 3GPP-WLAN |  | [52] 4.4 |  | o |  | c2 |  |  |
| 55 | 3GPP-GAN |  | [52] 4.4 |  | o |  | c2 |  |  |
| 56 | 3GPP-HSPA |  | [52] 4.4 |  | o |  | c2 |  |  |
| 57 | 3GPP2 |  | [52] 4.4 |  | o |  | c2 |  |  |
| 58 | untrusted-non-3GPP-VIRTUAL-EPC |  | subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 59 | VIRTUAL-no-PS |  | subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 60 | WLAN-no-PS |  | subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 61 | 3GPP-NR |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 62 | 3GPP-NR-U |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63 | 3GPP-NR-SAT |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63A |  | 3GPP-NR-LEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63B |  | 3GPP-NR-MEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63C |  | 3GPP-NR-GEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63D |  | 3GPP-NR-OTHERSAT |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63E |  | 3GPP-WB-E-UTRAN-LEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63F |  | 3GPP-WB-E-UTRAN-MEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63G |  | 3GPP-WB-E-UTRAN-GEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63H |  | 3GPP-WB-E-UTRAN-OTHERSAT |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63I |  | 3GPP-NB-IoT-LEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63J |  | 3GPP-NB-IoT-MEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63K |  | 3GPP-NB-IoT-GEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63L |  | 3GPP-NB-IoT-OTHERSAT |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63M |  | 3GPP-LTE-M-LEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63N |  | 3GPP-LTE-M-MEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63O |  | 3GPP-LTE-M-GEO |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| 63P |  | 3GPP-LTE-M-OTHERSAT |  | Subclause 7.2A.4 |  | n/a |  | c2 |  |  |
| c1: If A.3/1 OR A.3/2 THEN o.1 ELSE n/a - - UE or P-CSCF. c2: If A.3/2 THEN o.1 ELSE n/a - - P-CSCF. o.1: It is mandatory to support at least one of these items. |  |  |  |  |  |  |  |  |  |

Table A.3C: Modifying roles

| Item | Roles | Reference | RFC status | Profile status |
| --- | --- | --- | --- | --- |
| 1 | UE performing the functions of an external attached network | 4.1 |  |  |
| 2 | UE performing the functions of an external attached network operating in static mode | 4.1 |  |  |
| NOTE: This table identifies areas where the behaviour is modified from that of the underlying role. Subclause 4.1 indicates which underlying roles are modified for this behaviour. |  |  |  |  |

Table A.3D: Roles with respect to security mechanism

| Item | Security mechanism | Reference | RFC status | Profile status |
| --- | --- | --- | --- | --- |
| 1 | IMS AKA plus IPsec ESP | clause 4.2B.1 | n/a | c1 |
| 2 | SIP digest plus check of IP association | clause 4.2B.1 | n/a | c2 |
| 3 | SIP digest plus Proxy Authentication | clause 4.2B.1 | n/a | c2 |
| 4 | SIP digest with TLS | clause 4.2B.1 | n/a | c2 |
| 5 | NASS-IMS bundled authentication | clause 4.2B.1 | n/a | c2 |
| 6 | GPRS-IMS-Bundled authentication | clause 4.2B.1 | n/a | c2 |
| 7 | Trusted node authentication | clause 4.2B.1 | n/a | c3 |
| 8 | SIP over TLS with client certificate authentication | clause 4.2B.1 | n/a | c6 |
| 20 | End-to-end media security using SDES | clause 4.2B.2 | o | c5 |
| 20A | End-to-access-edge media security for MSRP using TLS and certificate fingerprints | clause 4.2B.2 | n/a | c4 |
| 20B | End-to-access-edge media security for BFCP using TLS and certificate fingerprints | clause 4.2B.2 | n/a | c4 |
| 20C | End-to-access-edge media security for UDPTL using DTLS and certificate fingerprints | clause 4.2B.2 | n/a | c4 |
| 21 | End-to-end media security using KMS | clause 4.2B.2 | o | c5 |
| 22 | End-to-end media security for MSRP using TLS and KMS | clause 4.2B.2 | o | c5 |
| 30 | End-to-access-edge media security using SDES | clause 4.2B.2 | n/a | c4 |
| 31 | End-to-access-edge media security for RTP media using DTLS-SRTP and certificate fingerprints | clause 4.2B.2 | n/a | c7 |
| c1: IF (A.3/1A OR A.3/2 OR A.3/3 OR A.3/4) THEN m ELSE IF A.3/1B THEN o ELSE n/a - - UE containing UICC or P-CSCF or I-CSCF or S-CSCF, UE without UICC. c2: IF (A.3/1 OR A.3/2 OR A.3/3 OR A.3/4) THEN o ELSE n/a - - UE or P-CSCF or I-CSCF or S-CSCF. c3: IF (A.3/3 OR A.3/4) THEN o ELSE n/a - - I-CSCF or S-CSCF. c4: IF (A.3/1 OR A.3/2A) THEN o ELSE n/a - - UE or P-CSCF (IMS-ALG). c5: IF A.3/1 THEN o - - UE. c6: IF A.3C/2 THEN m ELSE o - - UE performing the functions of an external attached network operating in static mode. c7: IF (A.3/14 OR A.3A/95) THEN m ELSE IF (A.3/1 OR A.3/2A) THEN o ELSE n/a - - Gm based WIC or eP-CSCF or UE or P-CSCF (IMS-ALG). |  |  |  |  |  |

* * * End of Changes * * * *