3GPP TSG-CT WG1 Meeting #157	C1-256293

# Sophia Antipolis, France 13 – 17 October 2025

Source:	ZTE

Title:	Pseudo-CR on clarifying ciphering of AIoT NAS signalling messages

Spec:	3GPP TS 24.369 v1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This p-CR resolves the ENs in clause 4.2.5 based on TS 33.369.

2. Reason for Change

"4.2.2.2	Requirements on Communication Protection

… …

Integrity protection of AIoT NAS Command request and response between the AIoT device and the AIOTF is mandatory to use.

The AIOTF shall support selection of confidentiality and integrity algorithms for protecting AIoT NAS Command request and response between the AIoT device and the AIOTF based on operator’s local policy.

"

"5.3.2	Security procedure on information protection during command procedure

……

3.	The AIOTF shall construct a AIOT NAS Command Request and protect the message based on the KCommand_enc, KCommand_int, the confidentiality and integrity algorithms for the AIoT device. The AIOTF shall send the protected Command Request containing an indication on whether cyphering is activated to NG-RAN.

NOTE 1: The whole AIOT NAS Command Request message is integrity protected. If confidentiality algorithm is not null-scheme, the AIOT NAS Command Request message is partly ciphered with the exception that the selected protection algorithms are in clear text.

"

Based on the quoted texts in TS 33.369 above,

- "Editor's note: It is FFS whether other AIoT NAS messages than INVENTORY REPORT message can be sent unciphered." can be directly remove.

- Propose to clarify that the AIOTF indicates the selected ciphering algorithm in clear text to the AIoT device in command procedures as specified in clause 5.3.

- It is clear that the whole AIoT NAS message is regarded as ciphered regardless of whether ciphering is configured, except for "the selected ciphering algorithm" in command request message. Therefore, "Editor's note:	The part of the AIoT NAS messages subject to confidentiality protection is FFS." can be removed.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TS 24.369 v1.0.0.

* * * First Change * * * *

### 4.2.5	Ciphering of AIoT NAS signalling messages

The use of ciphering in a network is an operator option subject to AIOTF configuration. When operation of the network without ciphering is configured, the AIOTF shall indicate the use of "null ciphering algorithm" NEA0.

All AIoT NAS messages, except the INVENTORY REPORT message, shall be sent ciphered.

The AIOTF indicates the selected ciphering algorithm in clear text to the AIoT device in command procedures as specified in clause 5.3.

When operation of the network without ciphering is configured, the AIOTF shall indicate the use of "null ciphering algorithm" NEA0. If the "null ciphering algorithm" NEA0 has been selected as a ciphering algorithm, the AIoT NAS message is regarded as ciphered.

The algorithm to calculate the ciphering of AIoT NAS messages is described in annex D.2 in 3GPP TS 33.501 [9]. In addition to the data that is to be ciphered, the constant BEARER, DIRECTION bit, COUNT, KEY and LENGTH are input to the ciphering algorithm. These parameters are described in 3GPP TS 33.369 [6].

* * * End of Changes * * * *