3GPP TSG-CT WG1 Meeting #157	C1-256328

Sophia-Antipolis, France 13-17 October 2025

Source:	Your COMPANY NAME

Title:	Discussion on MA PDU session cross releases

Agenda item:	16

Document for:	INFORMATION/APPROVAL/DISCUSSION

- Introduction

In CT1#153, it was decided that for rel-18 onwards, to create a new set of steering functionalities and steering modes for MA PDU session in addition to the already existing ATSSS-ST rules in rel-16 and rel-17 of 3GPP TS 24.193 [1]. Table 6.1.6.2-1 in 3GPP TS 24.193 [1] which defines ATSSS request PCO parameter container contents, is now according to:

| Supported ATSSS steering functionalities and steering modes (ATSSS-ST) (octet 1, bits 1, 2, 3 and 4) (NOTE 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| This field indicates the 5GSM capability of ATSSS steering functionalities and steering modes. |  |  |  |  |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | ATSSS-LL functionality not supported (NOTE 3) |
| 0 | 0 | 0 | 1 | ATSSS Low-Layer functionality with any steering mode allowed for ATSSS-LL supported |
| 0 | 0 | 1 | 0 | MPTCP functionality with any steering mode and ATSSS-LL functionality with only active-standby steering mode supported |
| 0 | 0 | 1 | 1 | MPTCP functionality with any steering mode and ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported |
| All other values are reserved. |  |  |  |  |
| ATSSS-LL functionality support (ATSSS-LL) (octet 1, bits 5 to 6) (NOTE 1) |  |  |  |  |
| This field indicates the support for ATSSS-LL functionality. |  |  |  |  |
| Bits |  |  |  |  |
| 6 | 5 |  |  |  |
| 0 | 0 | ATSSS-LL functionality not supported |  |  |
| 0 | 1 | ATSSS-LL functionality with only active-standby steering mode supported |  |  |
| 1 | 0 | ATSSS-LL functionality with any steering mode allowed for ATSSS-LL supported |  |  |
| 1 | 1 | Spare |  |  |
| Supporting MPTCP functionality with any steering mode (MPTCP) (octet 1, bit 7) (NOTE 1) |  |  |  |  |
| This bit indicates whether the MPTCP functionality with any steering mode is supported. (NOTE 2) |  |  |  |  |
| Bit |  |  |  |  |
| 7 |  |  |  |  |
| 0 | MPTCP functionality with any steering mode not supported |  |  |  |
| 1 | MPTCP functionality with any steering mode supported |  |  |  |
| Supporting MPQUIC-UDP functionality with any steering mode (MPQUIC-UDP) (octet 1, bit 8) (NOTE 1) |  |  |  |  |
| This bit indicates whether the MPQUIC-UDP functionality with any steering mode is supported. (NOTE 2) |  |  |  |  |
| Bit |  |  |  |  |
| 8 |  |  |  |  |
| 0 | MPQUIC-UDP functionality with any steering mode not supported |  |  |  |
| 1 | MPQUIC-UDP functionality with any steering mode supported |  |  |  |
| Supporting MPQUIC-IP functionality with any steering mode (MPQUIC-IP) (octet 2, bit 1) (NOTE 1) |  |  |  |  |
| This bit indicates whether the MPQUIC-IP functionality with any steering mode is supported. |  |  |  |  |
| Bit |  |  |  |  |
| 1 |  |  |  |  |
| 0 | MPQUIC-IP functionality with any steering mode not supported |  |  |  |
| 1 | MPQUIC-IP functionality with any steering mode supported |  |  |  |
| Supporting MPQUIC-E functionality with any steering mode (MPQUIC-E) (octet 2, bit 2) (NOTE 1) |  |  |  |  |
| This bit indicates whether the MPQUIC-UDP functionality with any steering mode is supported. (NOTE 2) |  |  |  |  |
| Bit |  |  |  |  |
| 2 |  |  |  |  |
| 0 | MPQUIC-E functionality with any steering mode not supported |  |  |  |
| 1 | MPQUIC-E functionality with any steering mode supported |  |  |  |
| ATSSS rules provisioning support indicator (ARPSI) (octet 2, bit 3) |  |  |  |  |  |
| Bit |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 0 |  | ATSSS rules provisioning via 3GPP access connected to EPS not supported |  |  |  |
| 1 |  | ATSSS rules provisioning via 3GPP access connected to EPS supported |  |  |  |
| NOTE 1: If the ATSSS request PCO parameter is included in the PDN CONNECTIVITY REQUEST message with the request type information element set to "handover", the ATSSS-ST field, ATSSS-LL field, MPTCP bit, MPQUIC-UDP bit, MPQUIC-IP bit and MPQUIC-E bit are ignored. |  |  |  |  |
| NOTE 2: If ATSSS-LL is set to 00 then this bit shall be set to 0. |  |  |  |  |
| NOTE 3: This value will trigger error handling in the network compliant with the previous releases of the specification. |  |  |  |  |

where,

1-	the green color represents the support by the UE and the network in release 16 onwards;

2-	the red color represents the support by the UE and the network in release 18 onwards; and

3-	the blue color represents the support by the UE and the network in release 19 onwards.

- Analysis

According to 3GPP TS 23.501 [2], release 18 UE may choose

-	MPQUIC-UDP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode; and

release 19 UE may choose:

-	MPQUIC-UDP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode;

-	MPQUIC IP functionality with any steering mode without any ATSSS-LL functionality;

-	MPQUIC IP functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode; or

-	MPQUIC-E functionality with any steering mode and ATSSS-LL functionality with only Active-Standby steering mode

at the time of MA PDU session establishment. Any of the above steering functionalities cannot be encoded in the way that the release 16 and release 17 UPF understand.

In addition, release 19 UE may choose

-	MPQUIC IP functionality with any steering mode without any ATSSS-LL functionality;

at the time of MA PDU session establishment. However, even release 18 UPF cannot understand the encoding and interprets the UE not being capable of any ATSSS elated steering functionality.

Table 1 shows the lack of compatibility between releases of the UE and the UPF.

Table 8.2.5.1: Non-compatible steering functionalities when UE and UPF are from different releases

|  | UPF (Rel-16) | UPF (Rel-17) | UPF (Rel-18) | UPF (Rel-19) |
| --- | --- | --- | --- | --- |
| UE (Rel-16) | None | None | None | None |
| UE (Rel-17) | None | None | None | None |
| UE (Rel-18) | MPQUIC-UDP and ATSSS-LL with only Active-Standby steering mode | MPQUIC-UDP and ATSSS-LL with only Active-Standby steering mode | None | None |
| UE (Rel-19) | MPQUIC-UDP and ATSSS-LL with only Active-Standby steering mode MPQUIC-IP without ATSSS-LL MPQUIC-IP and ATSSS-LL with only Active-Standby steering mode MPQUIC-E and ATSSS-LL with only Active-Standby steering mode | MPQUIC-UDP and ATSSS-LL with only Active-Standby steering mode MPQUIC-IP without ATSSS-LL MPQUIC-IP and ATSSS-LL with only Active-Standby steering mode MPQUIC-E and ATSSS-LL with only Active-Standby steering mode | MPQUIC-IP without ATSSS-LL | None |

- Solutions

5GS

The incompatibilities of the steering functionalities between the UE and the UPF were resolved by a set of documents i.e., C1-252532 for release 16, C1-252105 for release 17, C1-252565 for release18, and C1-252564 for release19 of 3GPP TS 24.501 [3] to add this as a new abnormal case in clause 6.4.2.6. In this solution the 5GSM cause #95 "Semantically incorrect message" is used by the UPF to indicate to the UE that the requested steering functionalities are not understood.

E-UTRAN

3GPP TS 24.193 [1] and 3GPP TS 23.316 [4] identify that the Multiple Access PDU connectivity service may be established in the EPS going back to release 16. Therefore, any solution should be applicable back to release 16. Although 3GPP TS 24.302 [5] contains the ESM cause #95 "Semantically incorrect message", table C.1 in 3GPP TS 29.274 [6] does not map any S11/S4 to NAS ESM cause #95. In fact, clause 7.7.8 in 3GPP TS 29.274 [6] states that if a information element is semantically incorrect, the cause shall be set to #69 "Mandatory IE incorrect". Furthermore, according to table C.1 in 3GPP TS 29.274 [6] cause #69 "Mandatory IE incorrect" is either mapped to ESM cause #30 "Request rejected by Serving GW or PDN GW" or ESM cause #38 "Network failure".

Figure 1: Suggested cause values for unknown steering functionality

CRs C1-256329 for release 16, C1-256330 for release 17, C1-256331 for release 18, and C1-256332 for release 19 are based on Figure 1.

Untrusted non-3GPP

3GPP TS 24.193 [1] and 3GPP TS 24.302 [5] identify that the MA PDU connectivity service with a connection via the untrusted non-3GPP access goes back to release 18. Therefore, the solution for the untrusted non-3GPP should be applicable for release 18 onwards. Table 8.1.4.10-1 in 3GPP TS 24.302 [5] lists the cause values, thus, cause #69 "Mandatory IE incorrect" in table C.1 in 3GPP TS 29.274 [6] can either be mapped to cause #30 "Request rejected by PDN GW" or cause #38 "Network failure" in table 8.1.4.10-1 in 3GPP TS 24.302 [5]. The related CRs are in C1-256331 for release 18 and C1-256332 for release 19.

# Reference

[1]	3GPP TS 24.193: "Access Traffic Steering, Switching and Splitting (ATSSS); Stage 3".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[4]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)"

[5]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3"

[6]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".