3GPP TSG-CT WG1 Meeting #157	C1-256350

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.501 | CR | 7035 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Deletion of acess technology utilization when UE supported access technology is restricted |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | ECRATU |  |  |  |  |  | Date: |  |  | 2025-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The UE maintain "PLMNs with associated access technology restrictions" across the power cycle. Currently, the network (e.g., MME/AMF) sends the access technology restriction based on subscription information as specified in 3GPP TS 29.503 [20AB] or serving network local configuration if the subscription information is not available. So, the network is not considering the UE capability to restrict the access. Consider the below scenarios: UE supports E-UTRAN, NG-RAN 1) UE is on PLMN-1, send NAS message and received access technology utilization control information IE with E-UTRAN restricted. UE add current PLMN (PLMN-1) to list of PLMNs with associated access technology restrictions. After this step PLMNs with associated access technology restrictions contains [PLMN-1, E-UTRAN restricted] 2) UE move to PLMN-2, send NAS message (e.g registrtaion request) and received access technology utilization control information IE with E-UTRAN restricted,NG-RAN restricted UE add current PLMN-2 to list of PLMNs with associated access technology restrictions. After this step PLMNs with associated access technology restrictions contains [PLMN-1, E-UTRAN restricted] [PLMN-2, E-UTRAN restricted, NG-RAN restricted] As the UE support E-UTRAN, NG-RAN so UE will not be able to get the service on PLMN-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The UE shall delete any PLMN entry from the list where all UE supported access technologies are restricted.when the UE powers off or periodically |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE cannot obtain service on a PLMN if all of its supported access technologies are restricted |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*****start change *****

### 4.2A.2	List of PLMNs with associated access technology restrictions

The UE shall replace the previously stored entry associated with the current PLMN, if any, with the newly received access technology utilization control information together with the PLMN identity of the current PLMN in the list of "PLMNs with associated access technology restrictions" (see 3GPP TS 23.122 [5]). Additionally:

a)	if the type of access technology utilization control indicates "current PLMN and its equivalent PLMN(s)", the UE shall delete all previously stored entries associated with PLMNs included in the list of equivalent PLMNs, excluding the entry associated with the current PLMN; and

b)	if the type of access technology utilization control indicates "current PLMN", the UE shall delete all previously stored entries associated with PLMNs included in the list of equivalent PLMNs and indicating "current PLMN and its equivalent PLMN(s)".

The UE shall delete all entries in the list of "PLMNs with associated access technology restrictions" in which only GSM, only UTRAN, or only GSM and UTRAN access technologies are marked as not restricted when the UE is switched off or when the UICC containing the USIM is removed, and periodically (with a period in the range 12 to 24 hours).

NOTE:	The deletion of the entries in which only GSM, only UTRAN, or only GSM and UTRAN access technologies are marked as not restricted is done as in A/Gb and Iu mode the network has no mean to provide the access technology utilization control information to the UE via NAS signalling.

The UE shall delete all entries in the list of "PLMNs with associated access technology restrictions" in which all the access technologies supported by UE is marked as restricted when the UE is switched off or when the UICC containing the USIM is removed, and periodically (with a period in the range 12 to 24 hours).

The same list is used by 5GMM, EMM, GMM and MM, regardless whether the UE is operating in single-registration mode or dual-registration mode. When the UE is switched off, it shall keep the stored list so that it can be used after switch on. The UE shall delete the stored list if the USIM is removed. The number of entries that the ME can store in the list of "PLMNs with associated access technology restrictions" is implementation specific, but it shall be at least one. When the list is full and a new entry has to be inserted, the oldest entry shall be deleted.

*****End change *****