3GPP TSG-CT WG1 Meeting #157	C1-256386

Sophia Antipolis, France, 13-17 October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.193 | CR | 0240 | rev |  | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | The link-specific IP address(es) when using MPQUIC functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MASSS |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MPQUIC based functionalities are enabled, then “two” “link specific IP address/prefix are allocated to UE. In case of IPv4v6, the wording “two” means “two sets” of IP address/prefixes.Therefore, the clarification is needed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarified that when PDU Session is IPv4v6, then two IP address/prefixes per access are allocated to the UE (i.e. total of four address/prefixes). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If it is not approved, the specification remains with unclear texts, which can lead to misinterpretation and faulty implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 6.1.4.1.1	Steering information for different steering functionalities

When the network accepts to enable only the ATSSS-LL functionality with only the active-standby steering mode for an MA PDU session in the UPF as specified in the clause 5.32.2 of 3GPP TS 23.501 [2], then the network shall provide to the UE one or more ATSSS rules including an "ATSSS-LL functionality" steering functionality and an "active-standby" steering mode.

When the UE indicates support for ATSSS-LL functionality with any steering mode allowed for ATSSS-LL and the network accepts to enable this functionality for an MA PDU session of any supported type, then the network shall enable ATSSS-LL functionality with any steering mode allowed for ATSSS-LL in the UPF as specified in the clause 5.32.2 of 3GPP TS 23.501 [2] and provide one or more ATSSS rules to the UE.

When the UE indicates support for one or more of MPTCP functionality with any steering mode, MPQUIC-UDP functionality with any steering mode, MPQUIC-IP functionality with any steering mode, and MPQUIC-E functionality with any steering mode, as well as the ATSSS-LL functionality with only the active-standby steering mode and the network accepts to enable one or more of the indicated functionalities for an MA PDU session in the UPF as specified in the clause 5.32.2 of 3GPP TS 23.501 [2], then the network shall provide the following information to the UE:

NOTE 0:	MPQUIC-E functionality and ATSSS-LL functionality cannot be enabled for the same Ethernet type MA PDU Session (see clause 5.32.6.1 of 3GPP TS 23.501 [2]).

a)	two "link-specific multipath" IP addresses/prefixes used by the indicated steering functionality in the UE, one associated with the 3GPP access and another associated with the non-3GPP access;

b)	the IP address, port number and the type of one or more proxies in the UPF; and

NOTE 1:	It is possible that the network provides the "link-specific multipath" IP addresses/prefixes and the proxy information that are not routable via N6 (e.g. IPv6 link local address).

c)	one or more ATSSS rules including an ATSSS rule which is composed of a precedence with value "255", a "match-all type" traffic descriptor, an "ATSSS-LL functionality" steering functionality and an "active-standby" steering mode.

The UE shall use the "link-specific multipath" addresses/prefixes to establish subflows over non-3GPP access and over 3GPP access.

When the MA PDU session is IP type, the network shall not enable the MPQUIC-E functionality.

When the MA PDU session is IP type, if the network supports indicating IPv4v6 in the IP address type, the network shall:

a)	set the IP address type of the "link-specific multipath" IP address/prefix associated with the 3GPP access to "IPv4v6" and indicate two IP addresses in the corresponding IP address field (an IPv4 address and an IPv6 address);

b)	set the IP address type of the "link-specific multipath" IP address/prefix associated with the non-3GPP access to "IPv4v6" and indicate two IP addresses in the corresponding IP address field (an IPv4 address and an IPv6 address); and

c)	set the proxy IP address type to "IPv4v6" and indicate two IP addresses in the corresponding IP address field (an IPv4 address and an IPv6 address).

When the MA PDU session is Ethernet type, the network shall not enable the MPTCP functionality, the MPQUIC-UDP functionality nor the MPQUIC-IP functionality.

When the MA PDU session is Ethernet type and the MPQUIC-E functionality is enabled, if the network supports indicating IPv4v6 in the IP address type, the network shall:

a)	set the IP address type of the "link-specific multipath" IP address/prefix associated with the 3GPP access to "IPv4v6" and indicate two IP addresses in the corresponding IP address field (an IPv4 address and an IPv6 address);

b)	set the IP address type of the "link-specific multipath" IP address/prefix associated with the non-3GPP access to "IPv4v6" and indicate two IP addresses in the corresponding IP address field (an IPv4 address and an IPv6 address); and

c)	set the proxy IP address type to "IPv4v6" and indicate two IP addresses in the corresponding IP address field (an IPv4 address and an IPv6 address).

NOTE 2:	When the MPQUIC-E functionality is enabled, if the network provides the "link-specific multipath" IP addresses/prefixes and the proxy information for both IP versions (IPv4 and IPv6), the UE can select one version of the IP addresses (either IPv4 or IPv6) based on its capabilities.

* * * End of Change * * * *