3GPP TSG-CT WG1 Meeting #157	C1-256399

Sophia Antipolis, France, 13-17 October August 2025	was C1-255303

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.572 | CR | 0117 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Send the source LMF routing information to UE for LMF relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19, 5G_eLCS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2 discussed and agreed to support the multiple LCS-UPP connections per UE in Rel19. Since there can be more than one LCS secured user plane connections in the UE, when the UE receives an UPP-CM message, the UE needs to identify the UPP-CM message is for which LCS secured user plane connection. Considering that when a LCS secured user plane connection is established, the routing information of the LMF which is for the UPP-CM message is always included in the Additional information IE of the UL NAS TRANSPORT message and DL NAS TRANSPORT message, and since only one LCS secured user plane connection is supported per LMF, the routing information can be used for UE to identify the LCS secured user plane connection. For LMF relocation scenario, if there is an existing user plane connection exists between the UE and the target LMF, it is proposed to introduce the user plane connection modification procedure on the control plane for the target LMF to notify the UE about the source LMF routing information for the user plane connection to be moved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include the user plane connection modification procedure on the control plane for the target LMF to notify the UE about the source LMF routing information for the user plane connection to be moved for the LMF relocation scenario. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE has no clue about which user plane connection to replace for the LMF relocation scenario if there is an existing user plane connection between the UE and the target LMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2, 6.2.1.x (new), 6.2.1.x.1 (new), 6.2.1.x.2 (new), 6.2.1.x.3 (new), 6.2.1.x.4 (new), 6.2.1.x.5 (new), 10.3.y (new), 10.3.y.1 (new), 10.3.y.2 (new), 10.3.z (new), 10.3.z.1 (new), 10.3.z.2 (new), 10.3.zz (new), 11.1.3, 11.3.3, 12.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 6.1.2	Types of UPP-CM procedures

Three types of UPP-CM procedures can be distinguished:

a)	Procedures related to establishing the LCS secured user plane connection for LCS-UPP:

1)	Initiated by the network:

i)	network initiated user plane connection establishment procedure.

2)	Initiated by the UE:

i)	UE requested user plane connection establishment procedure.

b)	Procedures related to releasing the LCS secured user plane connection for LCS-UPP:

1)	Initiated by the network:

i)	network initiated user plane connection release procedure.

2)	Initiated by the UE:

i)	UE requested user plane connection release procedure.

c)	Procedure related to modifying the LCS secured user plane connection for LCS-UPP:

1)	Initiated by the network:

i)	network initiated user plane connection modification procedure.

* * * Next Change * * * *

#### 6.2.1.x	Network initiated user plane connection modification procedure

##### 6.2.1.x.1	General

The purpose of the network initiated user plane connection modification procedure is to enable the network to modify the LCS secured user plane connection between the UE and the LMF via the control plane. The USER PLANE CONNECTION MODIFICATION COMMAND message is encapsulated in the UPP-CMI container of the DL NAS TRANSPORT message, and the USER PLANE CONNECTION MODIFICATION COMPLETE message is encapsulated in the UPP-CMI container of the UL NAS TRANSPORT message.

Figure 6.2.1.x.1.1 illustrates an example of the signalling transport for the network initiated user plane connection modification procedure.

Figure 6.2.1.x.1.1: Signalling transport for network initiated user plane connection modification procedure

* * * Next Change * * * *

##### 6.2.1.x.2	Network initiated user plane connection modification procedure initiation by LMF

The LMF initiates the network initiated user plane connection modification procedure by sending the USER PLANE CONNECTION MODIFICATION COMMAND message to the UE, as shown in figure 6.2.1.x.2.1, the LMF shall:

a)	generate the USER PLANE CONNECTION MODIFICATION COMMAND message according to subclause 10.3.y;

b)	send the USER PLANE CONNECTION MODIFICATION COMMAND message to the UE; and

c)	start a timer Txxxx upon sending the USER PLANE CONNECTION MODIFICATION COMMAND message.

When the network initiated user plane connection modification procedure is initiated for the LMF relocation as specified in clause 6.18.3 of 3GPP TS 23.273 [2], the LMF shall include the Source LMF routing information IE in the USER PLANE CONNECTION MODIFICATION COMMAND message.

Figure 6.2.1.x.2.1: Network initiated user plane connection modification procedure

* * * Next Change * * * *

##### 6.2.1.x.3	Network initiated user plane connection modification procedure accepted by UE

Upon receipt of a USER PLANE CONNECTION MODIFICATION COMMAND message, if the USER PLANE CONNECTION MODIFICATION COMMAND message can be accepted, the UE shall:

a)	identify the LCS secured user plane connection to be modified by using the routing information provided by the lower layer if the multiple LCS secured user plane connections is supported;

b)	if the source LMF routing information IE is included:

1)	identify the user plane connection using the source LMF routing information;

2)	associate all location sessions associated with the source LMF's user plane connection to the modified user plane connection, if any;

3)	stop sending the LCS-UPP messages via the source LMF's the user plane connection; and

4)	start sending LCS-UPP messages for the location sessions found in 2) via the to be modified user plane connection;

c)	create a USER PLANE CONNECTION MODIFICATION COMPLETE message according to clause 10.3.z; and

d)	send the USER PLANE CONNECTION MODIFICATION COMPLETE message to the LMF.

##### Upon reception of a USER PLANE CONNECTION MODIFICATION COMPLETE message from the UE, the LMF shall stop the timer Txxxx and shall consider the LCS secured user plane connection between the UE and the LMF as modified.

* * * Next Change * * * *

##### 6.2.1.x.4	Network initiated user plane connection modification procedure not accepted by UE

If the USER PLANE CONNECTION MODIFICATION COMMAND cannot be accepted, e.g., due to the source LCS secured user plane connection not found, the UE:

a)	shall include the Failure cause IE with the cause value set to:

1)	#mm "Source LCS secured user plane connection not found" if the source LMF LCS-UP address IE is included in the USER PLANE CONNECTION MODIFICATION COMMAND message and the LCS secured user plane connection can not be found with the source LMF LCS-UP address; or

2)	#111 "Protocol error, unspecified" if the modification of the LCS secured user plane connection failed for an unspecified reason;

b)	shall generate the USER PLANE CONNECTION MODIFICATION REJECT message according to clause 10.3.zz; and

c)	shall send the USER PLANE CONNECTION MODIFICATION REJECT message to the LMF.

Upon reception of the USER PLANE CONNECTION MODIFICATION REJECT message from the UE, the LMF shall stop timer Txxxx, consider the network initiated user plane connection modification procedure is failed.

If the Failure cause IE is included in the USER PLANE CONNECTION ESTABLISHMENT REJECT message with the cause value set to #mm "Source LCS secured user plane connection not found", the LMF may re-initiate to establish the secured LCS user plane connection from the source LMF to the UE, or choose the other location solutions, e.g control plane location solution.

* * * Next Change * * * *

##### 6.2.1.x.5	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Expiry of the timer Txxxx

The LMF shall, on the first expiry of the timer Txxxx, retransmit the USER PLANE CONNECTION MODIFICATION COMMAND message and shall reset and start timer Txxxx. This retransmission is repeated up to four times, i.e. on the fifth expiry of timer Txxxx, the LMF shall abort the ongoing network initiated user plane connection modification procedure on the LCS secured user plane connection.

* * * Next Change * * * *

### 10.3.y	User plane connection modification command

#### 10.3.y.1	Message definition

The USER PLANE CONNECTION MODIFICATION COMMAND message is sent by the LMF to the UE to modify the LCS secured user plane connection between the UE and the LMF. See table 10.3.y.1.1.

Message type:	USER PLANE CONNECTION MODIFICATION COMMAND

Significance:	dual

Direction:	network to UE

Table 10.3.y.1.1: USER PLANE CONNECTION MODIFICATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION MODIFICATION COMMAND message identity | Message type 11.1.3 | M | V | 1 |
| yy | Source LMF routing information | Source LMF routing information 11.3.xx | O | TLV | 18 |

* * * Next Change * * * *

#### 10.3.y.2	Source LMF routing information

The LMF shall include this information element to the UE for modification of user plane connection when the network initiated user plane connection modification procedure is initiated for the LMF relocation as specified in subclause 6.18.3 in 3GPP TS 23.273 [2].

* * * Next Change * * * *

### 10.3.z	User plane connection modification complete

#### 10.3.z.1	Message definition

The USER PLANE CONNECTION MODIFICATION COMPLETE message is sent by the UE to the LMF to accept a USER PLANE CONNECTION MODIFICATION COMMAND message and indicates confirmation for the modification of the LCS secured user plane connectiosn between the UE and the LMF. See table 10.3.z.1.1.

Message type:	USER PLANE CONNECTION MODIFICATION COMPLETE

Significance:	dual

Direction:	UE to network

Table 10.3.z.1.1: USER PLANE CONNECTION MODIFICATION COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION MODIFICATION COMPLETE message identity | Message type 11.1.3 | M | V | 1 |

* * * Next Change * * * *

### 10.3.zz	User plane connection modification reject

#### 10.3.zz.1	Message definition

The USER PLANE CONNECTION MODIFICATION REJECT message is sent by the UE to the LMF to reject a USER PLANE CONNECTION MODIFICATION COMMAND message and indicates confirmation for the modification of the LCS secured user plane connection between the UE and the LMF. See table 10.3.zz.1.1.

Message type:	USER PLANE CONNECTION MODIFICATION REJECT

Significance:	dual

Direction:	UE to network

Table 10.3.zz.1.1: USER PLANE CONNECTION MODIFICATION REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | USER PLANE CONNECTION MODIFICATION REJECT message identity | Message type 11.1.3 | M | V | 1 |
| XYZ | Failure cause | Failure cause 11.3.3 | M | V | 1 |

* * * Next Change * * * *

### 11.1.3	Message type

The Message type information element and its use are defined in 3GPP TS 24.007 [11]. Table 11.1.3.1 defines the value part of the message type information element used in the LCS-UPP and table 11.1.3.2 defines the value part of the message type information element used in the UPP-CM.

The Message type is a type 3 information element, with the length of 1 octet.

Table 11.1.3.1: Message type for LCS-UPP

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 1 | - | - | - | - | - | - |  | LCS-UPP messages |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | UL LCS-UP transport |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | DL LCS-UP transport |
| 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | LCS-UP connection binding request |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | LCS-UP connection binding accept |
| 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | LCS-UP connection binding reject |

Table 11.1.3.2: Message type for UPP-CM

| Bits |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 1 | 1 | - | - | - | - | - | - |  | UPP-CM messages |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 |  | User plane connection establishment command |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 |  | User plane connection establishment complete |
| 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 |  | User plane connection establishment failure |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 0 |  | User plane connection establishment request |
| 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |  | User plane connection establishment reject |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 0 |  | User plane connection release command |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |  | User plane connection release complete |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 0 |  | User plane connection release request |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |  | User plane connection modification command |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |  | User plane connection modification complete |
| 1 | 1 | 0 | 0 | 1 | 0 | 1 | 1 |  | User plane connection modification reject |

* * * Next Change * * * *

### 11.3.3	Failure cause

The purpose of the Failure cause information element is to indicate the reason of the failure of a UPP-CM procedure.

The Failure cause is a type 3 information element with a length of 2 octets.

The Failure cause information element is coded as shown in figure 11.3.3.1 and table 11.3.3.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Failure cause IEI |  |  |  |  |  |  |  | octet 1 |
| Failure cause value |  |  |  |  |  |  |  | octet 2 |

Figure 11.3.3.1: Failure cause information element

Table 11.3.3.1: Failure cause information element

| Failure cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | PDU session failure |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  | TLS connection failure |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  | LCS-UP connection binding failure |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  | User plane not available |
| x | x | x | x | x | x | x | x |  | Source LCS secured user plane connection not found |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Protocol error, unspecified |
| All other values are spare, and shall be ignored if received. |  |  |  |  |  |  |  |  |  |

* * * Next Change * * * *

## 12.3	Timers of UPP-CM

Timers of UPP-CM are shown in table 12.3.1 and table 12.3.2.

Table 12.3.1: Timers of UPP-CM – UE side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY (NOTE 1) |
| --- | --- | --- | --- | --- |
| T5011 | 16s | Transmission of USER PLANE CONNECTION ESTABLISHMENT REQUEST message | USER PLANE CONNECTION ESTABLISHMENT COMMAND message received USER PLANE CONNECTION ESTABLISHMENT REJECT message received | Retransmission of USER PLANE CONNECTION ESTABLISHMENT REQUEST message |
| T5013 | 16s | Transmission of USER PLANE CONNECTION RELEASE REQUEST message | USER PLANE CONNECTION RELEASE COMMAND message received | Retransmission of USER PLANE CONNECTION RELEASE REQUEST message |
| T5014 | NOTE 2 | USER PLANE CONNECTION ESTABLISHMENT REJECT received with a back-off timer value as specified in subclause 6.2.2.1.4. USER PLANE CONNECTION RELEASE COMMAND received with a back-off timer value and USER PLANE CONNECTION RELEASE COMPLETE sent as specified in subclause 6.2.1.2.3. | USER PLANE CONNECTION ESTABLISHMENT COMMAND message received | None |
| NOTE 1: Typically, the procedures are aborted on the fifth expiry of the relevant timer. Exceptions are described in the corresponding procedure description. NOTE 2: The value of this timer is provided by the network. |  |  |  |  |

Table 12.3.2: Timers of UPP-CM – LMF side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY (NOTE 1) |
| --- | --- | --- | --- | --- |
| T5012 | NOTE 2 | Transmission of USER PLANE CONNECTION ESTABLISHMENT COMMAND message | USER PLANE CONNECTION ESTABLISHMENT COMPLETE message received USER PLANE CONNECTION ESTABLISHMENT FAILURE message received | Retransmission of USER PLANE CONNECTION ESTABLISHMENT COMMAND message |
| T5010 | 16s | Transmission of USER PLANE CONNECTION RELEASE COMMAND message | A USER PLANE CONNECTION RELEASE COMPLETE message received | Retransmission of USER PLANE CONNECTION RELEASE COMMAND message |
| Txxxx | 16s | Transmission of USER PLANE CONNECTION MODIFICATION COMMAND message | USER PLANE CONNECTION MODIFICATION COMPLETE message received | Retransmission of USER PLANE CONNECTION MODIFICATION COMMAND message |
| NOTE 1: Typically, the procedures are aborted on the fifth expiry of the relevant timer. Exceptions are described in the corresponding procedure description. NOTE 2: The timer value of T5012 is implementation specific and set to be inclusive NAS procedure timer handling (e.g., for PDU session establishment), TLS connection establishment, and user plane connection binding procedure. |  |  |  |  |

* * * End of Changes * * * *