3GPP TSG-CT WG1 Meeting #157	C1-256476

# Gothenburg, Sweden, August 25 – 29, 2025

Source:	Lenovo

Title:	Pseudo-CR on AIoT device authentication updates

Spec:	3GPP TS 24.369 V1.0.0

Agenda item:	19.70

Document for:	Agreement

1. Introduction

This PCR completes the AIoT device authentication clauses as specified in 3GPP TS 33.369.

2. Reason for Change

The current version of TS 24.369 does specify the details of the authentication procedure as part of the command procedure. This PCR adds text to support mutual authentication during the command procedure as defined in 3GPP TS 33.369.

3. Conclusions

This PCR adds the necessary text to support mutual authentication as part of the command procedure.

4. Proposal

It is proposed to adopt the following changes to 3GPP TS 24.369 V1.0.0.

*** First Change ***

### 4.2.2	Authentication

Authentication of the AIoT device is done in the inventory procedure. A network generated RANDAIOT_n is provided by the AIOTF and received by the AIoT device in the paging request. The AIoT device which is target of the paging request (see clause 5.2.3), uses the RANDAIOT_n and its long-term KAIOT to derive a RESAIOT and includes the RESAIOT and a RANDAIOT_d in the INVENTORY REPORT message to the AIOTF. The network verifies the RESAIOT to authenticate the AIoT device.

Upon successful verification, and when the AIOTF needs to initiate a command procedure as specified in clause 5.3 the AIOTF derives the session key KAIOTF and the integrity key KCommand_int to protect the AIoT NAS command message as described in 3GPP TS 33.369 [6]. When the AIoT device receives the AIoT NAS command message, it also derives the session key KAIOTF and the integrity key KCommand_int to verify the MAC and thereby implicitly authenticate the network, as described in 3GPP TS 33.369 [6].

The requirements for the AIoT device to derive RESAIOT at receiving RANDAIOT_n from lower layers are described in 3GPP TS 33.369 [6].

The requirements for parameters and algorithms used at AIoT authentication are specified in 3GPP TS 33.369 [6].

*** Next Change ***

### 4.3.1	General

The primary authentication and key agreement procedure is to enable mutual authentication between the AIoT device and the network.

The authentication procedure is initiated during the inventory procedure and results in mutual authentication between the AIoT device and the network. It also enables the derivation of session keys used for protecting subsequent AIoT NAS messages. The detailed procedures and key derivation functions are described in clause 4.2.2 and specified in 3GPP TS 33.369 [6].

*** End Changes ***