3GPP TSG-CT WG1 Meeting #159	C1-260143

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.282 | CR | 0485 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to ad hoc group emergency alert and ad hoc group communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Kontron Transportation France |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCProtoc19, MC_AHGC |  |  |  |  |  | Date: |  |  | 2026-01-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | The current specification does not correctly determine the CF for an ad hoc group SDS, ad hoc group emergency alert and ad hoc group session initiation based on criteria. This CR correct this and allows, in case the same criteria is used, to suppress an unwanted duplicate ad hoc group session and instead enable the user to join the ongoing ad hoc group session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 9.2.2.2.1: - corrections to include all elements for ad hoc group standalone SDS message 9.2.2.3.1: - corrections in the determination of the PSI of the CF 9.2.2.4.2: - adding missing text required to determine the receipiants of an ad hoc group standalone SDS 16.2A.2.1: - corrections in the determination of the PSI of the 12.1A.3.1: - add handling to cover an ongoing ad hoc group emergency alert to adhoc group emergency alert notification 24.3.2.1.1: - corrections in the determination of the determination of the PSI of the CF to take into account a possible ongoing ad hoc group call using the same criteria 24.4.2.2: - add handling to cover joining of an ongoing ad hoc group session - editorial corrections 17.4.7.1.1: - defining new CF procedure to handle joining user Backward compatibility analysis: This CR does not create any new backward compatibility issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current incorrect determination of the CF for an ad hoc group emergency alert and ad hoc group call based on criteria would remain. This has seroious impact if the same criteria are used in an already ongoing ad hoc group session, a second (duplicate) ad hoc group session will be created on a different CF if users from different MCData systems initiate the hoc group session. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.2.2.1, 9.2.2.3.1, 9.2.2.4.2, 16.2A.2.1, 16.2A.3.1, 24.3.2.1.1, 24.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

##### 9.2.2.2.1	MCData client originating procedures

The MCData client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6] with the clarifications given below.

The MCData client:

1)	shall build the SIP MESSAGE request as specified in clause 6.2.4.1;

2)	if a one-to-one standalone SDS message is to be sent, shall insert in the SIP MESSAGE request:

a)	an application/resource-lists+xml MIME body with the MCData ID of the target MCData user or the functional alias to be called in the "uri" attribute of an <entry> element of a <list> element of the <resource-lists> element, according to rules and procedures of IETF RFC 4826 [9];

b)	an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	a <request-type> element set to a value of "one-to-one-sds"; and

ii)	an <anyExt> element containing:

A)	the <call-to-functional-alias-ind> element set to "true" if the functional alias is used as a target of the call request;

B)	if the MCData client is aware of active functional aliases and if an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> element set to the URI of the used functional alias; and

C)	if the MCData user has requested an application priority, the <user-requested-priority> element set to the user provided value; and

c)	if end-to-end security is required and the security context does not exist or if the existing security context has expired, an application/mikey MIME body with the MIKEY-SAKKE I_MESSAGE as specified in 3GPP TS 33.180 [26]. The MCData client:

i)	if necessary, shall instruct the key management client to request keying material from the key management server as described in 3GPP TS 33.180 [26];

ii)	shall use the keying material to generate a PCK as described in 3GPP TS 33.180 [26];

iii)	shall use the PCK to generate a PCK-ID with the four most significant bits set to "0001" to indicate that the purpose of the PCK is to protect one-to-one communications and with the remaining twenty-eight bits being randomly generated as described in 3GPP TS 33.180 [26];

iv)	shall encrypt the PCK to a UID associated to the MCData client using the MCData ID of the invited user and a time related parameter as described in 3GPP TS 33.180 [26];

v)	shall generate a MIKEY-SAKKE I_MESSAGE using the encapsulated PCK and PCK-ID as specified in 3GPP TS 33.180 [26]; and

vi)	shall add the MCData ID of the originating MCData to the initiator field (IDRi) of the I_MESSAGE as described in 3GPP TS 33.180 [26];

vii)	shall sign the MIKEY-SAKKE I_MESSAGE using the originating MCData user's signing key provided in the keying material together with a time related parameter; and

viii)	shall include the MIKEY-SAKKE I_MESSAGE in an application/mikey MIME body as specified in 3GPP TS 33.180 [26];

3)	if a group standalone SDS message is to be sent:

a)	if the <AllowedSDS> element present in the group document of the requested MCData group as specified in 3GPP TS 24.484 [12] is set to "false", shall reject the request to send SDS and not continue with the rest of the steps in this clause; and

NOTE 1:	The group document can either be known by the group management client in case of a normal group or of a temporary, or be known by the MCData client from the group regroup based on a preconfigured group procedures in case of a group regroup based on a preconfigured group.

b)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "group-sds";

ii)	the <mcdata-request-uri> element set to the MCData group identity;

iii)	if the group identity identifies a temporary group or a group regroup based on a preconfigured group, the <anyExt> element with the <associated-group-id> element set to the MCData group ID of a constituent group the MCData client is member of;

NOTE 2:	The MCData client is informed about temporary groups regrouping MCData groups that the user is a member of as specified in 3GPP TS 24.481 [11]. The MCData client is informed about regroups based on a preconfigured group of MCData groups that the user is member of and affiliated to as specified in clause 23.

NOTE 3:	If the MCData user selected a TGI or the identity of a group regroup based on a preconfigured group where there are several constituent MCData groups where the MCData user is a member, the MCData client selects one of those MCData groups.

iv)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client; and

v)	an <anyExt> element containing:

A)	if the MCData client is aware of active functional aliases, and an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> element set to the URI of the used functional alias; and

B)	if the MCData user has requested an application priority, the <user-requested-priority> element set to the user provided value;

3A)	if an ad hoc group standalone SDS message is to be sent:

a)	shall insert in the SIP MESSAGE request an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <request-type> element set to a value of "ad-hoc-group-sds";

ii)	the <mcdata-client-id> element set to the MCData client ID of the originating MCData client;

iii)	if the MCData client needs to include an identity of adhoc group, the <mcdata-request-uri> element set to the identity of the ad hoc group; and

NOTE 4:	If the call setup request follows an emergency alert for an ad hoc group then this element is included and the identity of adhoc group learned during an ad hoc group emergency alert procedures is used.

NOTE 5:	The MCData client can optionally include an identity of adhoc group if it learns by any other means or generated by the MCData client using required parameters.

iv)	an <anyExt> element containing:

A)	if a stored preconfigured MCData group ID is available according to clause 24.2.7.2, the <preconfigured-group> element set to the preconfigured MCData group ID; and

B)	if the MCData client is aware of active functional aliases, and an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> element set to the URI of the used functional alias;

b)	if the MCData user has requested to include the list of MCData users to be invited for data communication and the data communication setup request does not follow an adhoc group for emergency alert, shall insert in the SIP MESSAGE request an application/resource-lists+xml MIME body with the MCData ID or functional alias of the invited MCData users to be called, according to rules and procedures of IETF RFC 5366 [18]; and

c)	if the MCData user has requested to include the criteria for determining the list of MCData users to be invited for data communication and the data communication setup request does not follow an adhoc group for emergency alert, shall insert a <comn-participants-criteria> with one or more criteria as a comma separated list into <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the SIP MESSAGE request;

NOTE 6:	The MCData client can include either a list of MCData users, functional alias or the criteria for determining the list of MCData users to be invited. These two information elements are not included if the data communication setup request follows an adhoc group for emergency alerts.

4)	shall generate a standalone SDS message as specified in clause 6.2.2.1; and

5)	shall send the SIP MESSAGE request according to rules and procedures of 3GPP TS 24.229 [5].

Upon receiving a SIP 300 (Multiple Choices) response to the SIP MESSAGE request the MCData client shall use the MCData ID contained in the <mcdata-request-uri> element of the received application/vnd.3gpp.mcdata-info MIME body as the MCData ID of the invited MCData user and shall generate a new SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6], with the clarifications given in this clause and with the following additional clarifications:

1)	shall insert in the newly generated SIP MESSAGE request an application/resource-lists+xml MIME body with the MCData ID of the invited MCData user in the "uri" attribute of the <entry> element of the <list> element of the <resource-lists> element of the application/resource-lists+xml MIME body where the MCData ID is found in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info MIME body in the received SIP 300 (Multiple Choices) response;

2)	shall not include a <call-to-functional-alias-ind> element into the <anyExt> element of the <mcdata-Params> element of the <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body; and

3)	shall include a <called-functional-alias-URI> element into the <anyExt> element of the <mcdata-Params> element of the <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body with the target functional alias used in the initial SIP MESSAGE request for for sending one-to-one standalone SDS message <Contents of the first change, including the text of the whole of the affected clause.>

==============Next change==============

##### 9.2.2.3.1	Originating participating MCData function procedures

Upon receipt of a "SIP MESSAGE request for standalone SDS for originating participating MCData function", the participating MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The participating MCData function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4] and skip the rest of the steps;

2)	shall determine the MCData ID of the originating user from the public user identity in the P-Asserted-Identity header field of the SIP MESSAGE request, and shall authorise the calling user;

NOTE 1:	The MCData ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in clause 7.3.

3)	if the participating MCData function cannot find a binding between the public user identity and an MCData ID or if the validity period of an existing binding has expired, then the participating MCData function shall reject the SIP MESSAGE request with a SIP 404 (Not Found) response with the warning text set to "141 user unknown to the participating function" in a Warning header field as specified in clause 4.9, and shall not continue with any of the remaining steps;

3A)	may, based on local policy modify the content of the <comn-participants-criteria> element of the <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mdata-info+xml MIME body of the incoming SIP MESSAGE request;

NOTE 2: 	Based on application requirements it can be necessary to modify the value of the <comn-participants-criteria> element. If and how it is modified is outside the scope of of the present document.

4)	if the <request-type> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP MESSAGE request is:

a)	set to a value of "group-sds", shall determine the public service identity of the controlling MCData function associated with:

i)	if present, the MCData group identity contained in the <associated-group-is> element in an <anyExt> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the SIP MESSAGE request; or

NOTE 3:	If the incoming SIP MESSAGE request contains an <associated-group-id> element in an <anyExt> element of the application/vnd.3gpp.mcdata-info+xml MIME body, then the group identity contained in the <mcdata-request-uri> element is expected to be a TGI or the identity of a group regroup based on a preconfigured group and the participating MCData function forwards the request to the non-controlling function serving the constituent MCData group identity contained in the <associated-group-id> element.

ii)	the MCData group identity contained in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the SIP MESSAGE request;

b)	set to a value of "one-to-one-sds", shall determine the public service identity of the controlling MCData function hosting the one-to-one standalone SDS service for the calling user;

c)	set to a value of "ad-hoc-group-sds", shall determine the public service identity of the controlling MCData function:

i)	 if the incoming SIP MESSAGE request contains an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element containing the <anyExt> element with the <adhoc-grp-emg-alert-grp-ind> element set to "true", then the public service identity of the controlling MCData function associated with the adhoc group identity contained in the <mcdata-request-uri> element shall be used;

ii)	if the incoming SIP MESSAGE request contains an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element containing the <anyExt> element with a <comn-participants-criteria> element, then the public service identity of the controlling MCData function associated with the criteria shall be used; or

iii)	if the incoming SIP MESSAGE request contains an application/resource-lists+xml MIME body, then the public service identity of the controlling MCData function associated with the calling user's MCData ID shall be used;

5)	if unable to identify the controlling MCData function for standalone SDS, it shall reject the SIP MESSAGE request with a SIP 404 (Not Found) response with the warning text "142 unable to determine the controlling function" in a Warning header field as specified in clause 4.9, and shall not continue with any of the remaining steps;

6)	shall determine whether the MCData user identified by the MCData ID is authorised for MCData communications by following the procedures in clause 11.1;

7)	if the procedures in clause 11.1 indicate that the user identified by the MCData ID:

a)	is not allowed to send MCData communications as determined by step 1) of clause 11.1, shall reject the "SIP MESSAGE request for standalone SDS for originating participating MCData function" with a SIP 403 (Forbidden) response to the SIP MESSAGE request, with warning text set to "200 user not authorised to transmit data" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps in this clause;

b)	is not allowed to initiate one-to-one MCData communications due to exceeding the maximum amount of data that can be sent in a single request as determined by step 7) of clause 11.1, shall reject the "SIP MESSAGE request for standalone SDS for originating participating MCData function" with a SIP 403 (Forbidden) response to the SIP MESSAGE request, with warning text set to "202 user not authorised for one-to-one MCData communications due to exceeding the maximum amount of data that can be sent in a single request" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps in this clause; and

c)	is not allowed to initiate one-to-one MCData communications to the targeted user as determined by step 1a) of clause 11.1, shall reject the "SIP MESSAGE request for standalone SDS for originating participating MCData function" with a SIP 403 (Forbidden) response including warning text set to "229 one-to-one MCData communication not authorised to the targeted user" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

8)	if the payload size of the message is larger than the value contained in the <max-payload-size-sds-cplane-bytes> element in the MCData service configuration document as specified in 3GPP TS 24.484 [12], shall reject the "SIP MESSAGE request for standalone SDS for originating participating MCData function" with a SIP 403 (Forbidden) response to the SIP MESSAGE request, with warning text set to "203 message too large to send over signalling control plane" in a Warning header field as specified in clause 4.9;

NOTE 4:	The term "payload size" refers to the "Length of Payload contents" of the payload IE of the DATA PAYLOAD message transported in the SIP MESSAGE request, minus 1 (to account for the added "Payload content type" field).

9)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6];

10)	shall set the Request-URI of the outgoing SIP MESSAGE request to the public service identity of the controlling MCData function as determined by step 4) in this clause;

NOTE 5:	The public service identity can identify the controlling MCData function in the local MCData system or in an interconnected MCData system.

NOTE 6:	If the controlling MCData function is in an interconnected MCData system in a different trust domain, then the public service identity can identify the MCData gateway server that acts as an entry point in the interconnected MCData system from the local MCData system.

NOTE 7:	If the controlling MCData function is in an interconnected MCData system in a different trust domain, then the local MCData system can route the SIP request through an MCData gateway server that acts as an exit point from the local MCData system to the interconnected MCData system.

NOTE 8:	How the participating MCData function determines the public service identity of the controlling MCData function serving the target MCData ID or of the MCData gateway server in the interconnected MCData system is out of the scope of the present document.

NOTE 9:	How the local MCData system routes the SIP request through an exit MCData gateway server is out of the scope of the present document.

11)	shall copy all MIME bodies included in the incoming SIP MESSAGE request to the outgoing SIP MESSAGE request;

12)	shall include the MCData ID of the originating user in the <mcdata-calling-user-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the outgoing SIP MESSAGE request;

12A)	if the incoming SIP MESSAGE request contains an application/vnd.3gpp.mcdata-info+xml MIME body that contains a <functional-alias-URI> element, shall check if the status of the functional alias is activated for the MCData ID. If the functional alias status is activated, then the participating MCData function shall set the <functional-alias-URI> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the outgoing SIP INVITE request to the received value, otherwise shall not include a <functional-alias-URI> element;

12B)	if the <comn-participants-criteria> element of the <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SIP MESSAGE request was modified in step 3A, shall include the modified value of the <comn-participants-criteria> element of the <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the outgoing SIP MESSAGE request;

13)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]), into the P-Asserted-Service header field of the outgoing SIP MESSAGE request;

14) shall include a P-Asserted-Identity header field in the outgoing SIP MESSAGE request set to the public service identity of the participating MCData function; and

15)	shall send the SIP MESSAGE request as specified in 3GPP TS 24.229 [5].

Upon receipt of a SIP 202 (Accepted) response in response to the SIP MESSAGE request in step 15):

1)	shall generate a SIP 202 (Accepted) response as specified in 3GPP TS 24.229 [5]; and

2)	shall send the SIP 202 (Accepted) response to the MCData client according to 3GPP TS 24.229 [5].

Upon receipt of a SIP 200 (OK) response in response to the SIP MESSAGE request in step 15):

1)	shall generate a SIP 200 (OK) response as specified in 3GPP TS 24.229 [5]; and

2)	shall send the SIP 200 (OK) response to the MCData client according to 3GPP TS 24.229 [5].

Upon receipt of a SIP 4xx, 5xx or 6xx response to the SIP MESSAGE request in step 15) the participating MCData function:

1)	shall generate a SIP response according to 3GPP TS 24.229 [5];

2)	shall include Warning header field(s) that were received in the incoming SIP response; and

3)	shall forward the SIP response to the MCData client according to 3GPP TS 24.229 [5].

==============Next change==============

##### 9.2.2.4.2	Terminating controlling MCData function procedures

Upon receipt of a "SIP MESSAGE request for standalone SDS for controlling MCData function", the controlling MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The controlling MCData function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4]. Otherwise, continue with the rest of the steps;

2)	if the SIP MESSAGE does not contain:

a)	an application/vnd.3gpp.mcdata-info+xml MIME body;

b)	an application/vnd.3gpp.mcdata-signalling MIME body; and

c)	an application/vnd.3gpp.mcdata-payload MIME body;

shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response, with warning text set to "199 expected MIME bodies not in the request" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps in this clause;

3)	shall decode the contents of the application/vnd.3gpp.mcdata-signalling MIME body contained in the SIP MESSAGE;

4)	if the application/vnd.3gpp.mcdata-signalling MIME body contains a SDS SIGNALLING PAYLOAD message with a SDS disposition request type IE, shall store the value of the Conversation ID IE and the value of the Message ID IE in the SDS SIGNALLING PAYLOAD message;

NOTE 1:	The controlling MCData function uses the Conversation ID and Message ID for correlation with disposition notifications.

5)	if the <request-type> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP MESSAGE request is set to a value of "one-to-one-sds" and:

a)	the conditions in clause 11.1 indicate that the MCData user is not allowed to SDS communications due to message size as determined by step 3) of clause 11.1, shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response to the SIP MESSAGE request, with warning text set to "218 user not authorised for one-to-one SDS communications due to message size" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps in this clause; and

b)	the SIP MESSAGE request:

i)	does not contain an application/resource-lists+xml MIME body or contains an application/resource-lists MIME body with more than one <entry> element in the set of <list> elements in the <resource-lists> element, shall return a SIP 403 (Forbidden) response with the warning text set to "204 unable to determine targeted user for one-to-one SDS" in a Warning header field as specified in clause 4.9, and skip the rest of the steps below;

ii)	if the <anyExt> element of the <mcdata-Params> element of the <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body contains a <call-to-functional-alias-ind> element set to a value of "true":

A)	shall identify the MCData ID(s) of the MCData user(s) that have activated the called functional alias received in the "uri" attribute of an <entry> element of a <list> element of the <resource-lists> element of the application/resource-lists+xml MIME body of the SIP MESSAGE request by performing the actions specified in clause 22.2.2.2.8;

I)	if unable to determine any MCData ID that has activated the called functional alias received in the "uri" attribute of an <entry> element of a <list> element of the <resource-lists> element of the application/resource-lists+xml MIME body of the SIP MESSAGE, shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response including a warning text set to "145 unable to determine called party" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps; and

II)	selects one of the identified MCData IDs, and shall send a SIP 300 (Multiple Choices) response to the SIP MESSAGE request with an application/vnd.3gpp.mcdata-info MIME body containing an <mcdata-request-uri> element set to the selected MCData ID and shall not continue with the rest of the steps in this clause; and

NOTE 2:	How the controlling MCData function selects the MCData ID is implementation-specific.

iii)	contains an application/resource-lists+xml MIME body with exactly one <entry> element in the set of <list> elements of the <resource-lists> element, shall send a SIP MESSAGE request to the MCData user identified in the "uri" attribute of the <entry> element of the <list> element of the <resource-lists> element of the application/resource-lists+xml MIME body, as specified in clause 9.2.2.4.1.1;

6)	if the <request-type> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP MESSAGE request is set to a value of "group-sds":

a)	if the group identity is associated with a group document maintained by the GMS:

NOTE 3:	How the MCData server determines that a group identity represents a group for which a group document is stored in the GMS is an implementation detail.

i)	shall retrieve the group document associated with the group identity in the SIP MESSAGE request by following the procedures in clause 6.3.3, and shall continue with the remaining steps if the procedures in clause 6.3.3 were successful; or

b)	if the group identity is associated with a user or group regroup based on a preconfigured group:

i)	shall retrieve the stored information for the group identity; and

ii)	if there is no stored information for the group identity, the controlling MCData function:

A)	shall return a SIP 404 (Not Found) response with the warning text set to "163 the group identity indicated in the request does not exist" as specified in clause 4.4 "Warning header field" and shall not continue with the rest of the steps;

NOTE 4:	The user or group regroup can have been removed very recently and the client has sent the group call request prior to receiving the removal notification.

c)	if the group document contains a <list-service> element that contains a <preconfigured-group-use-only> element that is set to the value "true", shall reject the SIP request with a SIP 403 (Forbidden) response with the warning text set to "167 call is not allowed on the preconfigured group" as specified in clause 4.9 "Warning header field" and shall skip the rest of this procedure;

d)	if the <on-network-disabled> element is present in the group document, shall send a SIP 403 (Forbidden) response with the warning text set to "115 group is disabled" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

e)	if the <entry> element of the <list> element of the <list-service> element in the group document does not contain an <mcdata-mcdata-id> element with a "uri" attribute matching the MCData ID of the originating user contained in the <mcdata-calling-user-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the SIP MESSAGE request, shall send a SIP 403 (Forbidden) response with the warning text set to "116 user is not part of the MCData group" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

f)	if the <list-service> element contains a <mcdata-allow-short-data-service> element in the group document set to a value of "false", shall send a SIP 403 (Forbidden) response with the warning text set to "206 short data service not allowed for this group" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

g)	if the <supported-services> element is not present in the group document or is present and contains a <service> element containing an "enabler" attribute which is not set to the value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds", shall send a SIP 488 (Not Acceptable) response with the warning text set to "207 SDS services not supported for this group" in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

h)	if the group referred to by the group identity has been regrouped:

i)	send a SIP 403 (Forbidden) response with the warning text set to "148 group is regrouped " in a Warning header field as specified in clause 4.9 and shall not continue with the rest of the steps;

ii)	if the group referred to by the group identity has been regrouped based on a preconfigured group, shall send a copy of the notifying SIP MESSAGE that was generated and sent per clause 23.2.4.1 to the participating function for the MCData ID of the incoming SIP MESSAGE request; and

iii)	skip the rest of the steps;

i)	if the MCData server group SDS procedures in clause 11.1 indicate that the user identified by the MCData ID:

i)	is not allowed to send group MCData communications on this group identity as determined by step 2) of clause 11.1, shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response, with warning text set to "201 user not authorised to transmit data on this group identity" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps in this clause;

ii)	is not allowed to send group MCData communications on this group identity due to exceeding the maximum amount of data that can be sent in a single request as determined by step 8) of clause 11.1, shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response to the SIP MESSAGE request, with warning text set to "208 user not authorised for MCData communications on this group identity due to exceeding the maximum amount of data that can be sent in a single request" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps in this clause; and

iii)	is not allowed to send SDS communications on this group identity due to message size as determined by step 5) of clause 11.1, shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response to the SIP MESSAGE request, with warning text set to "217 user not authorised for SDS communications on this group identity due to message size" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps in this clause;

j)	if:

i)	the originating user identified by the MCData ID is not affiliated to the group identity contained in the SIP MESSAGE request, as specified in clause 6.3.5;

ii)	the group identity contained in the SIP MESSAGE resquest refers to a user regroup based on a preconfigured group and the originating user is not a member of that user regroup; or

ii)	the group identity contained in an <mcdata-calling-group-id> element of the SIP MESSAGE request is not a constituent group of the group identity contained in the SIP MESSAGE request;

NOTE 5:	If the SIP MESSAGE is for a temporary group or a group regroup based on preconfigured group, the affiliation of the calling user to the constituent group has been assured by the non-controlling MCData function of the constituent group before forwarding this SIP MESSAGE to the controlling function of the regroup.

shall return a SIP 403 (Forbidden) response with the warning text set to "120 user is not affiliated to this group" in a Warning header field as specified in clause 4.9, and skip the rest of the steps below;

k)	if the group identity in the SIP MESSAGE request for standalone SDS for controlling MCData function is not a TGI nor the identity of a group regroup based on a preconfigured group:

i)	shall determine the targeted group members for the MCData standalone SDS by following the procedures in clause 6.3.4;

ii)	if the procedures in clause 6.3.4 result in no affiliated members found in the selected MCData group, shall return a SIP 403 (Forbidden) response with the warning text set to "198 no users are affiliated to this group" in a Warning header field as specified in clause 4.9, and skip the rest of the steps below; and

iii)	shall send SIP MESSAGE requests to the targeted group members identified in step h) above by following the procedure in clause 9.2.2.4.1.1; and

l)	if the group identity in the SIP MESSAGE request for standalone SDS for controlling MCData function is a TGI or the identity of a group regroup based on a preconfigured group:

i)	shall, for each of the constituent MCData groups except for the calling MCData group identified in the <mcdata-calling-group-id> element of the incoming SIP MESSAGE, generate a SIP INVITE request towards the MCData server that owns the constituent MCData group identity by following the procedures in clause 9.2.2.4.1.2; and

NOTE 6:	The MCData server that the SIP MESSAGE request is sent to acts as a non-controlling MCData function;

6A)	if the <request-type> element in the application/vnd.3gpp.mcdata-info+xml MIME body of the SIP MESSAGE request is set to a value of "ad-hoc-group-sds":

a)	if the <adhoc-grp-emg-alert-grp-ind> element set to a value of "false" or does not exists, shall create the adhoc group and generate the group identity to be associated with the adhoc group if the identity of adhoc group included in the <mcdata-request-uri> element of the <mcdata-Params> element of the <mcdatainfo> element containing in an application/vnd.3gpp.mcdata-info+xml MIME body received in the SIP INVITE request is not acceptable or not included;

b)	shall determine the MCData users receiving the adhoc group SDS:

i)	if the application/resource-lists+xml MIME body with the MCData ID of the invited MCData users to be called exists in the incoming SIP MESSAGE request, shall consider each entry of the MCData users to receive the adhoc group SDS;

ii)	if the application/resource-lists+xml MIME body includes entries that are determined to be functional aliases:

NOTE 7:	How the controlling function determines that an entry is a functional alias is out of scope of this specification.

A)	shall identify the MCData ID(s) of the MCData user(s) that have activated the invited functional aliases by performing the actions specified in clause 22.2.2.2.8;

B)	if unable to determine any MCData ID that has activated any received functional alias of the MCData user to be invited to the call, shall reject the "SIP INVITE request for controlling MCData function of a adhoc group call" with a SIP 403 (Forbidden) response including a warning text set to "145 unable to determine called party" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps;

C)	if the number of identified MCData IDs is not within the configured limit as specified in the <max-no-participants> element of the <adhoc-group-call> element of the <anyExt> element contained in the <OnNetwork> element of the MCData service configuration document (see the service configuration document in 3GPP TS 24.484 [12]), shall send a SIP 403 (Forbidden) response including warning text set to "189 maximum number of allowed adhoc group participants exceeded" in a Warning header field as specified in clause 4.9 and shall skip the rest of the steps;

D)	shall include a <called-functional-alias-URI> element into the <anyExt> element of the <mcdata-Params> element of the <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body with the functional alias used to identify the MCData ID; and

E)	shall consider all identified MCData IDs to receive the adhoc group SDS, if the same MCData IDs appear multiple times, only one occurrence shall be considered;

NOTE 8:	How the controlling MCData function selects the appropriate MCData ID and associated functional alias is outside the scope of 3GPP;

iii)	if the application/vnd.3gpp.mcdata-info+xml MIME body with the criteria for determining the list of MCData users to be called exists in the incoming SIP MESSAGE request included in the <comn-participants-criteria> element of the <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body, shall determine the users as specified in the clause 24.4.6 and consider each of the determined MCData users meeting the specified criteria and also based on the local policy to receive the adhoc group SDS. If unable to determine any users meeting the specified criteria within the data communication setup duration, shall send a SIP 403 (Forbidden) response including warning text set to "240 can't determine the adhoc group participants" in a Warning header field as specified in clause 4.9 and shall skip the rest of the steps; or

iv)	if the application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdata-request-uri> element set to an identity of the adhoc group and <adhoc-grp-emg-alert-grp-ind> element with the value set to "true" exists in the incoming SIP INVITE request, shall consider each member of the adhoc group to receive the adhoc group SDS;

c) shall send SIP MESSAGE requests to the targeted group members identified in step b) above by following the procedure in clause 9.2.2.4.1.1

7)	shall generate a SIP 202 (Accepted) response in response to the "SIP MESSAGE request for standalone SDS for controlling MCData function"; and

8)	shall send the SIP 202 (Accepted) response towards the originating participating or non-controlling MCData function according to 3GPP TS 24.229 [5].

==============Next change==============

#### 16.2A.2.1	Receipt of a SIP MESSAGE request for adhoc group emergency notification from the served MCData client

Upon receipt of a "SIP MESSAGE request for adhoc emergency notification for originating participating MCData function", the participating MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The participating MCData function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4] and skip the rest of the steps;

NOTE 1:	If the SIP MESSAGE request contains an alert indication set to a value of "true", the participating MCData function can, according to local policy, choose to accept the request.

2)	shall determine the MCData ID of the calling user from the public user identity in the P-Asserted-Identity header field of the SIP MESSAGE request;

NOTE 2:	The MCData ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in clause 7.3.

3)	if the participating MCData function cannot find a binding between the public user identity and an MCData ID or if the validity period of an existing binding has expired, then the participating MCData function shall reject the SIP MESSAGE request with a SIP 404 (Not Found) response with the warning text set to "141 user unknown to the participating function" in a Warning header field as specified in clause 4.9, and shall not continue with any of the remaining steps;

3A)	may, based on local policy modify the content of the <comn-participants-criteria> element of the <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SIP MESSAGE request;

NOTE 3: 	Based on application requirements it can be necessary to modify the value of the <comn-participants-criteria> element. If and how it is modified is outside the scope of of the present document.

4)	shall determine the public service identity of the controlling MCData function for the adhoc group emergency alert service associated with the criteria;

NOTE 4:	How the participating MCData function determines the public service identity of the controlling MCData function for the adhoc group emergency alert service associated with the criteriais out of the scope of the present document.

5)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [5] and IETF RFC 3428 [6];

6)	shall set the Request-URI of the outgoing SIP MESSAGE request to the public service identity of the controlling MCData function determined in step 4) above;

7)	shall copy the contents of the application/vnd.3gpp.mcdata-info+xml MIME body in the received SIP MESSAGE request into an application/vnd.3gpp.mcdata-info+xml MIME body as specified in Annex D.1 included in the outgoing SIP MESSAGE request;

7A)	if the <comn-participants-criteria> element of the <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SIP MESSAGE request was modified in step 3A, shall include the modified value in the <comn-participants-criteria> element of the <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the outgoing SIP MESSAGE request;

8)	if the received SIP MESSAGE request contains a <functional-alias-URI> element of the application/vnd.3gpp.mcdata-info+xml MIME body, shall check the status of the functional alias for the MCData ID. If the functional alias status is activated, then the participating MCData function shall set the <functional-alias-URI> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the outgoing SIP MESSAGE request to the received value, otherwise it shall not include a <functional-alias-URI> element;

9)	shall set the <mcdata-calling-user-id> contained in <mcdata-Params> element of the application/vnd.3gpp.mcdata-info+xml MIME body to the MCData ID determined in step 2) above;

10)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcdata-location-info+xml MIME body as specified in clause D.4 shall copy the contents of the application/vnd.3gpp.mcdata-location-info+xml MIME body in the received SIP MESSAGE request into an application/vnd.3gpp.mcdata-location-info+xml MIME body included in the outgoing SIP MESSAGE request;

11)	shall include a P-Asserted-Identity header field in the outgoing SIP MESSAGE request set to the public service identity of the participating MCData function; and

12)	shall send the SIP MESSAGE request as specified to 3GPP TS 24.229 [5].

Upon receipt of a SIP 2xx response in response to the SIP MESSAGE request sent in step 12):

1)	shall generate a SIP 200 (OK) response as specified in 3GPP TS 24.229 [5] with the follow clarifications:

a)	shall include a P-Asserted-Identity header field in the outgoing SIP 200 (OK) response set to the public service identity of the participating MCData function;

2)	shall send the SIP 200 (OK) response to the MCData client according to 3GPP TS 24.229 [5].

==============Next change==============

#### 16.2A.3.1	Handling of adhoc group emergency alert notification

Upon receipt of a "SIP MESSAGE request for adhoc emergency notification for controlling MCData function", the controlling MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP MESSAGE request with a SIP 500 (Server Internal Error) response. The controlling MCData function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4]. Otherwise, continue with the rest of the steps;

NOTE 1:	If the SIP MESSAGE request contains an alert indication set to a value of "true", the controlling MCData function can, according to local policy, choose to accept the request.

2)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata";

2a)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcdata-info+xml MIME body with the <adhoc-alert-ind> element set to a value of "false", shall perform the procedures specified in clause 16.2A.3.2 and skip the rest of the steps;

3)	if the received SIP MESSAGE request contains an application/vnd.3gpp.mcdata-info+xml MIME body with the <adhoc-alert-ind> element set to a value of "true":

a)	if the received SIP MESSAGE request is an unauthorised request for an adhoc group emergency alert as specified in clause 6.3.7.2.8, shall reject the SIP MESSAGE request with a SIP 403 (Forbidden) response to the SIP MESSAGE request as specified in 3GPP TS 24.229 [5] with the following clarifications:

i)	shall include in the SIP 403 (Forbidden) response an application/vnd.3gpp.mcdata-info+xml MIME body as specified in Annex D.1 with the <mcdatainfo> element containing the <mcdata-Params> element containing the <anyExt> element with the <adhoc-alert-ind> element set to a value of "false"; and

ii)	shall send the SIP 403 (Forbidden) response as specified in 3GPP TS 24.229 [5] and skip the rest of the steps;

b)	if no ad hoc group emergency alert using the same criteria as received in the incoming SIP MESSAGE request is ongoing, or, based on local policy:

i)	shall create the adhoc group and generate the group identity to be associated with the adhoc group if the identity of adhoc group included in the <mcdata-request-uri> element of the <mcdata-Params> element of the <mcdatainfo> element containing in an application/vnd.3gpp.mcdata-info+xml MIME body received in the SIP MESSAGE request is not acceptable or not included;

ii)	shall determine the participants of the adhoc group emergency alert and send the adhoc group emergency alert, as specified in clause 24.4.6.1;

iii)	shall cache the information that the MCData user has initiated an adhoc group emergency alert;

c)	shall generate a SIP 200 (OK) response to the received SIP MESSAGE request as specified in 3GPP TS 24.229 [5];

d)	shall send the SIP 200 (OK) response to the received SIP MESSAGE according to rules and procedures of 3GPP TS 24.229 [5];

e)	shall generate a SIP MESSAGE request as described in clause 6.3.7.1.5 to indicate successful receipt of an adhoc group emergency alert to the initiator, and shall include in the application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element containing:

i)	the <mcdata-client-id> element with the MCData client ID that was included in the incoming SIP MESSAGE request;

ii)	the <mcdata-calling-group-id> element set to the adhoc group identity as determined in this clause; and

iii)	an <anyExt> element containing:

A)	an <adhoc-alert-ind> element set to a value of "true";

B)	an <adhoc-alert-ind-rcvd> element set to a value of "true"; and

C)	a <comn-participants-criteria> element as determined in step 5) b) in clause 24.4.6.1; and

f)	shall send the SIP MESSAGE request towards the originating participating MCData function according to rules and procedures of 3GPP TS 24.229 [5]; and

4)	shall continue to include the MCData users meeting or to remove MCData users no longer meeting the criteria as specified in the clause 16.2A.3.3.

==============Next change==============

##### 24.3.2.1.1	Originating procedures

Upon receipt of a "SIP INVITE request for MCData adhoc group session for originating participating MCData function" containing an application/vnd.3gpp.mcdata-info+xml MIME body with the <request-type> element set to a value of "adhoc-group-sds-session" or a value of "adhoc-group-fd-session", the participating MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The participating MCData function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4]. Otherwise, continue with the rest of the steps;

2)	shall determine the MCData ID of the calling user from public user identity in the P-Asserted-Identity header field of the SIP INVITE request;

NOTE 1:	The MCData ID of the calling user is bound to the public user identity at the time of service authorisation, as documented in clause 7.3.

3)	if the participating MCData function cannot find a binding between the public user identity and an MCData ID or if the validity period of an existing binding has expired, then the participating MCData function shall reject the SIP INVITE request with a SIP 404 (Not Found) response with the warning text set to "141 user unknown to the participating function" in a Warning header field as specified in clause 4.9, and shall not continue with any of the remaining steps;

4)	if through local policy in the participating MCData function, the user identified by the MCData ID is not authorised to initiate adhoc group data communications, shall reject the "SIP INVITE request for MCData adhoc group session for originating participating MCData function" with a SIP 403 (Forbidden) response to the SIP INVITE request, with warning text set to "237 user not authorised to make adhoc group data communications" in a Warning header field as specified in clause 4.9;

4A)	if the SIP INVITE request contains an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element containing the <anyExt> element with a <comn-participants-criteria> element, may modify based on local policy the content of the <comn-participants-criteria> element of the <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body;

NOTE 2:	Based on application requirements it can be necessary to modify the value of the <comn-participants-criteria> element. If and how it is modified is outside the scope of of the present document.

5)	shall determine the public service identity of the controlling MCData function for the adhoc group data communication service according to one of the following conditions:

a)	 if the incoming SIP MESSAGE request contains an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element containing the <anyExt> element with the <adhoc-grp-emg-alert-grp-ind> element set to "true", then the public service identity of the controlling MCData function associated with the adhoc group identity contained in the <mcdata-request-uri> element shall be used;

b)	if the incoming SIP MESSAGE request contains an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element containing the <anyExt> element with a <comn-participants-criteria> element, then the public service identity of the controlling MCData function associated with the criteria shall be used;

c)	if the incoming SIP MESSAGE request contains an application/resource-lists+xml MIME body, then the public service identity of the controlling MCData function associated with the calling user's MCData ID shall be used; or

d)	in case of rejoining an ongoing adhoc group call, the public service identity of the controlling MCData function associated with the session identity of the ongoing adhoc group session shall be used;

NOTE 3:	The public service identity can identify the controlling MCData function in the primary MCData system or in a partner MCData system.

NOTE 4:	If the controlling MCData function is in a partner MCData system in a different trust domain, then the public service identity can identify the MCData gateway server that acts as an entry point in the partner MCData system from the primary MCData system.

NOTE 5:	If the controlling MCData function is in a partner MCData system in a different trust domain, then the primary MCData system can route the SIP request through an MCData gateway server that acts as an exit point from the primary MCData system to the partner MCData system.

NOTE 6:	How the participating MCData function determines the public service identity of the controlling MCData function for the adhoc group data communication service associated with the originating user or of the MCData gateway server in the partner MCData system is out of the scope of the present document.

NOTE 7:	How the primary MCData system routes the SIP request through an exit MCData gateway server is out of the scope of the present document.

6)	if the participating MCData function is unable to identify the controlling MCData function for the adhoc group data communication service for SDS if standalone SDS and SDS session is requested or FD if FD is requested associated with the originating user's MCData ID identity, it shall reject the SIP INVITE request with a SIP 404 (Not Found) response with the warning text "142 unable to determine the controlling function" in a Warning header field as specified in clause 4.9, and shall not continue with any of the remaining steps;

7)	if the <allow-adhoc-group-data-comn> element of the <ruleset> element is not present in the MCData user profile document on the participating MCData function or is present with the value "false" (see the MCData user profile document in 3GPP TS 24.484 [12]), indicating that the user identified by the MCData ID is not authorised to initiate adhoc group data communications, shall reject the "SIP INVITE request for MCData adhoc group session for originating participating MCData function" with a SIP 403 (Forbidden) response, with warning text set to "238 user not authorised to initiate the adhoc group data communication" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps;

8)	shall generate a SIP INVITE request in accordance with 3GPP TS 24.229 [5];

9)	shall set the Request-URI to the public service identity of the controlling MCData function as determined in step 5;

10)	shall include the option tag "timer" in the Supported header field;

11)	should include the Session-Expires header field according to IETF RFC 4028 [38]. It is recommended that the "refresher" header field parameter is omitted. If included, the "refresher" header field parameter shall be set to "uac";

12)	shall copy the application/vnd.3gpp.mcdata-info+xml MIME body from the incoming SIP INVITE request to the outgoing SIP INVITE request;

13)	shall include the MCData ID of the originating user in the <mcdata-calling-user-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the outgoing SIP INVITE request;

14)	if the incoming SIP INVITE request contains an application/vnd.3gpp.mcdata-info+xml MIME body that contains a <functional-alias-URI> element, shall check if the status of the functional alias is activated for the MCData ID. If the functional alias status is activated, then the participating MCData function shall set the <functional-alias-URI> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the outgoing SIP INVITE request to the received value, otherwise shall not include a <functional-alias-URI> element;

NOTE 8:	The participating MCData server learns the functional alias state for an MCData ID from procedures specified in clause 22.2.2.2.7.

15)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" (coded as specified in 3GPP TS 24.229 [5]) if SDS requested or value "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" (coded as specified in 3GPP TS 24.229 [5]) if FD requested, into the P-Asserted-Service header field of the outgoing SIP INVITE request;

16)	shall include a P-Asserted-Identity header field in the outgoing SIP INVITE request set to the public service identity of the participating MCData function;

17)	shall include in the SIP INVITE request an SDP offer based on the SDP offer in the received SIP INVITE request from the MCData client as specified in clause 9.2.3.3.1 for standalone SDS or clause 9.2.4.3.1 for SDS session or clause 10.2.5.3.1 for FD;

18)	shall include a Resource-Priority header field according to rules and procedures of 3GPP TS 24.229 [5] set to the value indicated in the Resource-Priority header field, if included in the SIP INVITE request from the MCData client;

NOTE 9:	The participating MCData function will leave the verification of the Resource-Priority header field to the controlling MCData function.

18A)	if the incoming SIP INVITE request contained an application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element containing the <anyExt> element with a <comn-participants-criteria> element, and the value is modified in step 4A, shall include modified the value in the <comn-participants-criteria> element of the <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the outgoing SIP INVITE request; and

19)	shall send the SIP INVITE request as specified to 3GPP TS 24.229 [5].

Upon receipt of a SIP 2xx response in response to the above SIP INVITE request, the participating MCData function:

1)	shall generate a SIP 200 (OK) response as specified in 3GPP TS 24.229 [5];

2)	shall include in the SIP 200 (OK) response an SDP answer as specified in the clause 9.2.3.3.2 for standalone SDS or clause 9.2.4.3.2 for SDS session or clause 10.2.5.3.2 for FD;

3)	shall include Warning header field(s) that were received in the incoming SIP 200 (OK) response;

4)	shall include the following in the Contact header field:

a)	the g.3gpp.mcdata.sds media feature tag for standalone SDS and SDS session;

b)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" for standalone SDS and SDS session;

c)	the g.3gpp.mcdata.fd media feature tag for FD;

d)	the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" for FD; and

e)	the isfocus media feature tag;

5)	shall include a P-Asserted-Identity header field in the outgoing SIP 200 (OK) response set to the public service identity of the participating MCData function;

6)	shall include an MCData session identity mapped to the MCData session identity provided in the Contact header field of the received SIP 200 (OK) response;

7)	if the incoming SIP 200 (OK) response contained an application/vnd.3gpp.mcdata-info+xml MIME body, shall copy the application/vnd.3gpp.mcdata-info+xml MIME body to the outgoing SIP 200 (OK) response;

8)	shall send the SIP 200 (OK) response to the MCData client according to 3GPP TS 24.229 [5];

9)	shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 6.2.1.4 for standalone SDS or clause 6.2.2.4 for SDS session or clause 7.2.1 for FD; and

10)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38].

Upon receipt of a SIP 4xx, 5xx or 6xx response to the above SIP INVITE request, the participating MCData function:

1)	shall generate a SIP response according to 3GPP TS 24.229 [5];

2)	shall include Warning header field(s) that were received in the incoming SIP response; and

3)	shall forward the SIP response to the MCData client according to 3GPP TS 24.229 [5].

==============Next change==============

#### 24.4.2.2	Terminating Procedures

In the procedures in this clause:

1)	MCData ID in an incoming SIP INVITE request refers to the MCData ID of the originating user from the <mcdata-calling-user-id> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SIP INVITE request;

2)	adhoc group identity in an incoming SIP INVITE request if included refers to the adhoc group identity from the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the incoming SIP INVITE request; and

3)	MCData ID in an outgoing SIP INVITE request refers to the MCData ID of the called user in the <mcdata-request-uri> element of the application/vnd.3gpp.mcdata-info+xml MIME body of the outgoing SIP INVITE request.

Upon receipt of a "SIP INVITE request for MCData adhoc group session for controlling MCData function", the controlling MCData function:

1)	if unable to process the request due to a lack of resources or a risk of congestion exists, may reject the SIP INVITE request with a SIP 500 (Server Internal Error) response. The controlling MCData function may include a Retry-After header field to the SIP 500 (Server Internal Error) response as specified in IETF RFC 3261 [4] and skip the rest of the steps;

2)	if FD requested and if the received SIP INVITE request has been queued for later transmission, shall include warning text set to "215 request to transmit is queued by the server" in a Warning header field as specified in clause 4.9, in the SIP 100 (Trying) response, and shall send the SIP 100 (TRYING) response towards the originating participating MCData function according to 3GPP TS 24.229 [5] and not continue with the remaining steps in this clause. Otherwise, continue with the rest of the steps;

3)	shall determine if the media parameters are acceptable and the MSRP URI is offered in the SDP offer and if not reject the request with a SIP 488 (Not Acceptable Here) response and skip the rest of the steps;

4)	shall reject the SIP request with a SIP 403 (Forbidden) response and not process the remaining steps if:

a)	an Accept-Contact header field does not include the g.3gpp.mcdata.sds media feature tag for standalone SDS or SDS session;

b)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.sds" for standalone SDS or SDS session;

c)	an Accept-Contact header field does not include the g.3gpp.mcdata.fd media feature tag for FD; or

d)	an Accept-Contact header field does not include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcdata.fd" for FD;

5)	if FD requested and the incoming SIP INVITE request does not contain an application/vnd.3gpp.mcdata-signalling MIME body with the FD SIGNALLING PAYLOAD as described in clause 6.2.2.3, shall reject the SIP INVITE request with appropriate reject code;

6)	if the originating user identified by the MCData ID is not authorised to initiate the adhoc group session if the <allow-adhoc-group-data-comn> element of the <actions> element of a <rule> element of the <ruleset> element of the MCData user profile document identified by the MCData ID of the originating MCData user (see the MCData user profile document in 3GPP TS 24.484 [12]) is set to a value of "false", shall send a SIP 403 (Forbidden) response with the warning text set to: "238 user not authorised to initiate the adhoc group data communication" in a Warning header field as specified in clause 4.9 and skip the rest of the steps below;

7)	if the MCData service does not support the adhoc group data communication, which is indicated by the <allow-adhoc-group-data-comn-support> element of the <adhoc-group-data-comn> element of the <anyExt> element contained in the <OnNetwork> element of the MCData service configuration document (see the service configuration document in 3GPP TS 24.484 [12]) is set to a value of "false" or if it does not exists, shall send a SIP 403 (Forbidden) response with the warning text set to: "239 the MCData system do not support adhoc group data communication" in a Warning header field as specified in clause 4.9 and skip the rest of the steps below;

8)	if the application/resource-lists+xml MIME body with the MCData ID of the invited MCData users to be called exists in the incoming SIP INVITE request, shall check if the number of invited participants is within the configured limit as specified in the <max-no-participants> element of the <adhoc-group-data-comn> element of the <anyExt> element contained in the <OnNetwork> element of the MCData service configuration document (see the service configuration document in 3GPP TS 24.484 [12]). If not within the configured limit, shall send a SIP 403 (Forbidden) response including warning text set to "242 maximum number of allowed adhoc group participants exceeded" in a Warning header field as specified in clause 4.9 and shall skip the rest of the steps;

9)	if the application/resource-lists+xml MIME body with the MCData ID of the invited MCData users to be called exists in the incoming SIP INVITE request and the <comn-participants-criteria> element with one or more criteria as a comma separated into <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body in the SIP INVITE request exists, shall send a SIP 403 (Forbidden) response including warning text set to "240 can’t determine the adhoc group participants" in a Warning header field as specified in clause 4.9 and shall skip the rest of the steps;

9A)	if the <adhoc-grp-emg-alert-grp-ind> element of the <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body received in the SIP INVITE request exists with the value set to "true" and adhoc group identified using adhoc group identity does not exist, shall send a SIP 403 (Forbidden) response including warning text set to "240 can’t determine the adhoc group participants" in a Warning header field as specified in clause 4.9 and shall skip the rest of the steps;

9B)	if an ad hoc group session is ongoing using the same value of criteria, may, based on local policy continue with step 18);

10)	shall cache SIP feature tags, if received in the Contact header field and if the specific feature tags are supported;

11)	shall start the SIP Session timer according to rules and procedures of IETF RFC 4028 [38];

12)	shall maintain a local counter of the number of SIP 200 (OK) responses received from invited members and shall initialise this local counter to zero;

13)	if the <adhoc-grp-emg-alert-grp-ind> element set to a value of "false" or does not exists, shall create the adhoc group and generate the group identity to be associated with the adhoc group if the identity of adhoc group included in the <mcdata-request-uri> element of the <mcdata-Params> element of the <mcdatainfo> element containing in an application/vnd.3gpp.mcdata-info+xml MIME body received in the SIP INVITE request is not acceptable or not included;

14)	if originating MCData user has requested for end-to-end security by including the <end-to-end-security> element in the <anyExt> element of the <mcdata-Params> element of the <mcdatainfo> element containing in an application/vnd.3gpp.mcdata-info+xml MIME body received in the SIP INVITE request with the value set to "true", shall determine the preconfigured group from which security related materials can be used by the adhoc group data communication participants to communicate securely in the adhoc group session;

15)	shall determine the members to invite to the adhoc group session:

a)	if the application/resource-lists+xml MIME body with the MCData ID of the invited MCData users to be called exists in the incoming SIP INVITE request, shall consider each entry of the MCData users to be invited to the adhoc group session;

b)	if the application/resource-lists+xml MIME body includes entries that are determined to be functional aliases:

NOTE 1:	How the controlling function determines that the MCData ID is a functional alias is out of scope of this specification.

i)	shall identify the MCData ID(s) of the MCData user(s) that have activated the invited functional aliases by performing the actions specified in clause 22.2.2.2.8;

ii)	if unable to determine any MCData ID that has activated any received functional alias of the MCData user to be invited to the call, shall reject the "SIP INVITE request for controlling MCData function of a adhoc group call" with a SIP 403 (Forbidden) response including a warning text set to "145 unable to determine called party" in a Warning header field as specified in clause 4.9, and shall not continue with the rest of the steps;

iii)	if the number of identified MCData IDs is not within the configured limit as specified in the <max-no-participants> element of the <adhoc-group-call> element of the <anyExt> element contained in the <OnNetwork> element of the MCData service configuration document (see the service configuration document in 3GPP TS 24.484 [12]), shall send a SIP 403 (Forbidden) response including warning text set to "189 maximum number of allowed adhoc group participants exceeded" in a Warning header field as specified in clause 4.9 and shall skip the rest of the steps;

iv)	shall include a <called-functional-alias-URI> element into the <anyExt> element of the <mcdata-Params> element of the <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body with the functional alias used to identify the MCData ID; and

v)	shall consider all identified MCData IDs to be invited to the adhoc group session, if the same MCData IDs appear multiple times, only one occurrence shall be considered;

NOTE 2:	How the controlling MCData function selects the appropriate MCData ID and associated functional alias is outside the scope of 3GPP;

c)	if the application/vnd.3gpp.mcdata-info+xml MIME body with the criteria for determining the list of MCData users to be called exists in the incoming SIP INVITE request included in the <comn-participants-criteria> element of the <anyExt> element of <mcdata-Params> element of <mcdatainfo> element of the application/vnd.3gpp.mcdata-info+xml MIME body, shall determine the users as specified in the clause 24.4.5 and consider each of the determined MCData users meeting the specified criteria and also may be based on the local policy to be invited to the adhoc group session. If unable to determine any users meeting the specified criteria within the data communication setup duration, shall send a SIP 403 (Forbidden) response including warning text set to "240 can't determine the adhoc group participants" in a Warning header field as specified in clause 4.9 and shall skip the rest of the steps; or

d)	if the application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdata-request-uri> element set to an identity of the adhoc group and <adhoc-grp-emg-alert-grp-ind> element with the value set to "true" exists in the incoming SIP INVITE request, shall consider each member of the adhoc group to be invited to the adhoc group session;

16) shall invite each adhoc group member determined in step 15) above, to the adhoc group session, as specified in clause 24.4.2.1.1;

17) shall consider all the invited members as implicitly affiliated to the adhoc group; and

18) shall interact with the media plane as specified in 3GPP TS 24.582 [15] clause 6.3.1 for standalone SDS or clause 6.3.2 for SDS session or clause 7.3 for FD.

Upon receiving a SIP 200 (OK) response for a SIP INVITE request as specified in clause 24.4.2.2, and if the MCData ID in the SIP 200 (OK) response matches to the MCData ID in the corresponding SIP INVITE request, and the SIP final response is not yet sent to the inviting MCData client, the controlling MCData function:

1)	shall invoke the procedure in clause 6.3.7.1.23 with an indication that the applicable MCData subservice is Short Data Service using media (standalone SDS) or Short Data Service using session (SDS session) or File Distribution, in order to generate a SIP 200 (OK) response to the received SIP INVITE request according to 3GPP TS 24.229 [5] with clarification below:

a)	shall include in the application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <mcdata-calling-group-id> element set to the adhoc group identity as determined in this clause; and

ii)	if end-to-end security requested for the data communication, the <anyExt> element with the <preconfigured-group-id> element set to the preconfigured group identity as determined in this clause;

2)	shall send the generated SIP 200 (OK) response to the inviting MCData client according to 3GPP TS 24.229 [5]; and

3)	shall generate a notification to the MCData clients, which have subscribed to the conference state event package that the inviting MCData User has joined in the MCData group session, as specified in clause 25.4; and

4)	shall send a SIP NOTIFY request to each MCData client according to 3GPP TS 24.229 [5].

Upon receiving a SIP 200 (OK) response for a SIP INVITE request as specified in clause 24.4.2.2, and if the warning text set to "232 communication is stored for later delivery" is received in a Warning header field as specified in clause 4.9, and the SIP final response is not yet sent to the inviting MCData client, the controlling MCData function:

1)	shall invoke the procedure in clause 6.3.7.1.23 with an indication that the applicable MCData subservice is Short Data Service using media (standalone SDS) or Short Data Service using session (SDS session) or File Distribution, in order to generate a SIP 200 (OK) response to the received SIP INVITE request according to 3GPP TS 24.229 [5] with clarification below:

a)	shall include in the application/vnd.3gpp.mcdata-info+xml MIME body with the <mcdatainfo> element containing the <mcdata-Params> element with:

i)	the <mcdata-calling-group-id> element set to the adhoc group identity as determined in this clause; and

ii)	if end-to-end security requested for the data communication, the <anyExt> element with the <preconfigured-group-id> element set to the preconfigured group identity as determined in this clause; and

2)	shall send the generated SIP 200 (OK) response to the inviting MCData client according to 3GPP TS 24.229 [5].

NOTE 3:	When requested to release the associated media plane resources and to tear down the MCData session, the controlling MCData function stores the INVITE session information that is established between the participating function and the controlling function for later delivery.

==============End of change==============