3GPP TSG-CT WG1 Meeting #159	C1-260364

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.5.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.301 | CR | 4670 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MT terminated signalling while S&F wait timer is running |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | C1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Network can process MT data or SMS when feeder link is available and can send TRANSPORT message to the UE when UE and network are connected. Further, if the UE does not stop S&F wait timer when it receives MT terminated signalling like TRANSPORT message, the S&F wait timer is blocking the UE’s messages to network, although the UE is connected and feeder link is available between satellite and ground station. |  |  |  |  |  |  |  |  |
| Summary of change: |  | When S&F wait timer is running and the UE receives MT terminated signalling, then the UE can stop S&F wait timer. Backward compatibility analysis: The CR is backward compatible since no impact on protocol messages or procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE is unnecessarily blocking NAS signalling messages even if feeder link is available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.11.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 4.11.5.2	Handling of timers

The UE maintains timer T3451 and timer T3452, which are started with the value of the S&F wait time duration received from the network during an attach procedure, a tracking area updating procedure or a service request procedure. If timer T3451 is started, the UE shall stop timer T3452 and if timer T3452 is started, the UE shall stop timer T3451.

NOTE 1:	The S&F wait time duration value takes into account the time taken to synchronize the UE's context between MME(s) as specified in clause O.2 of 3GPP TS 23.401 [10]).

NOTE 2:	Although the theoretical max possible orbital period of a MEO satellite exceeds the maximum possible value for the two octet S&F wait time duration IE, two octets are sufficient for practical deployments.

If the UE attempting a NAS procedure receives an ATTACH REJECT, TRACKING AREA UPDATE REJECT or SERVICE REJECT message without integrity protection with EMM cause value #83 and with the S&F wait time duration value before the network has established secure exchange of NAS messages for the NAS signalling connection:

a)	the UE shall start the timer T3451:

1)	at first attempt with the S&F wait time duration value received from the network, if the value does not exceed UE implementation specific maximum value (e.g., 90 minutes); and

2)	at following attempts, or if the S&F wait time duration value received from the network at the first attempt exceeds the UE implementation specific maximum value:

i)	with the S&F wait time duration value received from the network; or

ii)	with a random value uniformly drawn from the range between 30 minutes and 60 minutes; and

b)	after an implementation-specific number of attempts greater than or equal to 1, the UE may search for a suitable cell from another tracking area according to 3GPP TS 36.304 [21] or perform PLMN selection.

While T3451 is running, the UE shall not initiate NAS procedures over S&F satellite E-UTRAN cells in tracking area(s) which are part of the TAI list, unless the S&F satellite E-UTRAN cell broadcasts the same satellite ID broadcasted by the S&F satellite E-UTRAN cell from which the S&F satellite operation parameters IE was received.

NOTE 3:	As an implementation option for the UE, if NAS detects that NAS procedures cannot be initiated in the current cell because T3451 is running, NAS can indicate this to AS.

While T3452 is running,

i)	if the tracking area(s) broadcast by the S&F satellite E-UTRAN cell when the S&F satellite operation parameters IE was received was not part of the TAI list, the UE shall not initiate NAS procedures over S&F satellite E-UTRAN cells in tracking area (s) which are part of the tracking area(s) broadcast by the S&F satellite E-UTRAN cell from which the S&F satellite operation parameters IE was received; or

ii)	if the tracking area(s) broadcast by the S&F satellite E-UTRAN cell when the S&F satellite operation parameters IE was received was part of the TAI list, the UE shall not initiate NAS procedures over S&F satellite E-UTRAN cells in tracking area (s) which are part of the TAI list.

The UE shall stop the timer T3451 and T3452, if running, when the UE:

1)	successfully performs an attach procedure or a tracking area updating procedure:

a)	via a cell which is not a S&F satellite E-UTRAN cell;

b) 	via a cell being in a TA which is not in the list of TAs; or

c)	in a new PLMN;

2)	receives a paging;

3)	successfully performs intersystem change from S1 mode to N1 mode or A/Gb mode or Iu mode;

4)	receives DETACH REQUEST message from network; or

5)	receives mobile terminated signalling or downlink data from the network.

The UE stores the S&F wait time duration in the non-volatile memory of the ME as described in annex C.

==============End of change==============