3GPP TSG-CT WG1 Meeting #159	C1-260379

# Goa, India, 9th – 13th February 2026

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 24.560 | CR | 0011 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the uri of the HFL training subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | CT1 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_App |  |  |  |  |  | Date: |  |  | 2026-02-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The uri of the newly created resource for the Individual HFL training subscription is incorrect and misalignment with the OpenAPI file. For backward compatibility considerations, it is proposed to correct the resource uri in the main body. Backward compatibility analysis: This proposal is backwards compatible since it only corrects the resource uri in the main body. In addition, the OpenAPI definition of the enumerations defined for the Aimlec_AIMLEClientParticipation API currently does not follow the guidelines of clause 5.3.12 of TS 29.501 with regards to description fields. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the attribute name in the main body. - Apply additional corrections and updates to the description text in the OpenAPI file. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between the main body and OpenAPI file. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2.3.1, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR introduces a backward compatible correction to the following API: - TS24560_Aimlec_AIMLEClientParticipation.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

# *** 1st Change ***

6.2.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.2.3.2.3.1-1.

Table 6.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 6.2.3.2.3.1-2 and the response data structures and response codes specified in table 6.2.3.2.3.1-3.

Table 6.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| HflTrngSub | M | 1 | Represents the " HFL training subscription" resource. |

Table 6.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| HflTrngSub | M | 1 | 201 Created | Successful case. The creation of an Individual HFL training subscription resource is confirmed, and a representation of that resource is returned. An HTTP "Location" header that contains the URI of the created resource shall also be included. |
| NOTE: The mandatory HTTP error status codes for the HTTP POST method listed in table 5.2.6-1 of 3GPP TS 29.122 [5] also apply. |  |  |  |  |

Table 6.2.3.2.3.1-4: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/aimlec-hfl-trng/<apiVersion>/subscriptions/{subscriptionId} |

# *** Next Change ***

A.2	Aimlec_AIMLEClientParticipation API

openapi: 3.0.0

info:

title: Aimlec_AIMLEClientParticipation

version: 1.0.1

description: |

API for Client Participation Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 24.560 V19.0.0; Artificial Intelligence Machine Learning (AIML) Services – Service

Enabler Architecture Layer for Verticals (SEAL); Protocol Specification; Stage 3.

url: 'https://www.3gpp.org/ftp/Specs/archive/24_series/24.560/'

servers:

- url: '{apiRoot}/aimlec-cp/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 5.2.4 of 3GPP TS 29.122.

security:

- {}

- oAuth2ClientCredentials: []

paths:

/participation:

post:

summary: >

Used by AIMLE server to manage the participation of AIMLE clients in AIML operations.

operationId: ClientParticipation

tags:

- AIMLE client for participation of the AIML operations request(Collection)

requestBody:

description: Contains information of the AIMLE client participates the AIML operations.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/AimlecParticipationReq'

responses:

'200':

description: Contains the outcome of the successful AIMLE client participation.

content:

application/json:

schema:

$ref: '#/components/schemas/AimlecParticipationResp'

'307':

$ref: 'TS29122_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29122_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29122_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29122_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29122_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29122_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29122_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29122_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29122_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29122_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29122_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29122_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29122_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{tokenUrl}'

scopes: {}

schemas:

# Structured data types

AimlecParticipationReq:

description: Represents the participation request for the AIML operations.

type: object

required:

- requesterId

- clientSetId

- clientSetPart

- mlModelId

- schedAimlOperations

- dataSetReq

- serviceReq

properties:

requesterId:

description: Represents the requester identity.

type: string

clientSetId:

description: Represents the AIMLE client set identity.

type: string

clientSetPart:

$ref: '#/components/schemas/ClientSetPart'

mlModelId:

description: Identifies the identity of the ML model for AIML operation.

type: string

schedAimlOperations:

type: array

items:

$ref: '#/components/schemas/SchedAimlOperation'

minItems: 1

dataSetReq:

$ref: 'TS29482_AIMLES_AIMLEClientDiscovery.yaml#/components/schemas/DatasetRequirement'

serviceReq:

$ref: 'TS29482_AIMLES_AIMLEClientDiscovery.yaml#/components/schemas/ServiceRequirement'

AimlecParticipationResp:

description: Represents the participation response for the AIML operations.

type: object

required:

- clientStatus

properties:

clientStatus:

description: true indicates the willingness and false indicates the denial.

type: boolean

SchedAimlOperation:

description: Represents the scheduled AIML participation type.

type: object

required:

- aimlOperation

properties:

aimlOperation:

$ref: 'TS24560_Aimles_AIMLEClientRegistration.yaml#/components/schemas/AimlOperation'

aimlOperSched:

$ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime'

# Enumerations

ClientSetPart:

anyOf:

- type: string

enum:

- JOIN

- DEPART

- type: string

description: >

This string provides the participation request for the AIMLE client set.

description: |

Identifies the type of request for participating the AIMLE client set.

Possible values are:

- JOIN: Request to join the AIMLE client set.

- DEPART: Request to depart from the AIMLE client set.

# *** End of Changes ***